3GPP TSG RAN WG1 #106-e			R1- 2106698

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.3.1.2

Source: 	Spreadtrum Communications

Title: 	Discussion on CSI feedback enhancements

Document for:	Discussion and decision

Introduction

During RAN #92e meeting, a recommendation 1 for CSI was endorsed [1], which recommend RAN1 to further investigate subband CQI and delta-MCS [2] for Rel-17 IIoT & eURLLC.

- Revised Recommendation1: Provide the following RAN guidance on CSI feedback enhancement [RAN1]

  - Focus subsequent working group discussions on the schemes proposed in RP-211297.

    - Details (e.g. how to calculate delta-MCS) are up to further working group discussions.

Proposal: RAN confirms the following as a guidance to RAN1 for CSI enhancement in Enhanced URLLC/IIoT WI:

RAN1 to further investigate the following for CSI enhancements for IIoT/URLLC:

- Increasing the number of bits used for the reported subband CQI (3-bits differential subband CQI or 4-bits CQI)

- Reporting of delta-MCS:

  - Report consists of delta-MCS for a TB received with MCS index IMCS:

    - delta-MCS is calculated from the difference between IMCS_tgt and IMCS, where IMCS_tgt is the largest MCS index such that the estimated BLER for a TB received with this MCS index would be smaller than or equal to a BLER target, and IMCS is the MCS index of the received TB.

This contribution provides considerations on subband CQI and delta-MCS for CSI feedback enhancements.

Discussion

Case 1: Increasing granularity of subband CQI

Considering increasing granularity of subband CQI, differential subband CQI with 3 bits or full 4-bit CQI can be reported instead of 2 bits. Basically speaking, there is little specification effort, and little UE complexity. We are fine to support any one of them.

- Increasing granularity of subband CQI, e.g. 3-bits differential subband CQI or 4-bit CQI, can be supported.

Case 2: Delta-MCS

Delta-MCS indicates the difference between the actual MCS for the PDSCH and the required MCS to achieve a specific BLER target. It is a good method to report the margin between MCS of scheduling PDSCH and UE expects. From the maturity point of view, delta-MCS is the most direct way to feedback the decoding margin and easy to define the new type of report. Thus, we support it for Rel-17 URLLC.

- Delta-MCS can be supported.

In addition, some details according to Delta-MCS are still open. We would like to provide our considerations for it.

Measure and report resources

The measurement resource of Delta-MCS is based on PDSCH. The radio between PDSCH and Delta-MCS can be further studied.

From our understanding, one to one mapping of PDSCH and its Delta-MCS can provide the most helpful and finest margin. However, this kind of reports need large payload compared to HARQ-ACK feedback only in Rel-15/16. So multiple PDSCHs to one Delta-MCS can be further studied. For example,

Another dynamic indication that whether or not a PDSCH need the Delta-MCS can also in the scope. Because not all PDSCHs need the Delta-MCS information, gNB can decide the subset to collect this feedbacks.

- Further study multiple PDSCHs to one Delta-MCS feedback.

- Further study dynamic enable or disable PDSCH resources for Delta-MCS

How to trigger Delta-MCS

DL-related DCI is supported considering PDSCH is used for generating Delta-MCS. It can reuse similar CSI request field in UL DCI, e.g. a new CSI request field in DL DCI can be introduced. The association of CSI report and measurement resource can be further study.

- DL DCI is used to trigger the A-CSI report of Delta-MCS.

Feedback resource of Delta-MCS

The feedback resource of Delta-MCS can be PUCCH. This PUCCH can be same or different with PUCCH with PDSCH HARQ-ACK. But from the delay requirement of HARQ-ACK and CSI report, there is little demand for CSI feedback earlier than HARQ-ACK. Therefore, the simplest way is to multiplex HARQ-ACK and Delta-MCS in one PUCCH resource and send to gNB.

According to Delta-MCS is within HARQ-ACK codebook or outside HARQ-ACK codebook. We prefer Delta-MCSs are feedback independently from HARQ-ACK codebook. One reason is Delta-MCS is a new report from HARQ-ACK, there is no necessity to combine HARQ-ACK and Delta-MCS together or do joint coding. Next, it has not decided yet whether every PDSCH need Delta-MCS report. If not, it is not reasonable to report Delta-MCS within HARQ-ACK codebook. Because HARQ-ACK is required for every unicast PDSCH. But Delta-MCS does not required for every unicast PDSCH, additionally, multicast PDSCHs need or not is still open. Last, Delta-MCS belongs to CSI report. Its mapping and multiplexing would be reuse the CSI report mechanism.

- Multiplexing HARQ-ACK and Delta-MCS in one PUCCH resource.

- Delta-MCS is outside HARQ-ACK codebook, and can be part of CSI report.

Conclusion

In this contribution, we made the following proposals.

- Increasing granularity of subband CQI, e.g. 3-bits differential subband CQI or 4-bit full band CQI, can be supported.

- Delta-MCS should be supported.

- Further study multiple PDSCH to one Delta-MCS feedback.

- Further study dynamic enable or disable PDSCH resources for Delta-MCS

- DL DCI is used to trigger the A-CSI report of Delta-MCS.

- Multiplexing HARQ-ACK and Delta-MCS in one PUCCH resource.

- Delta-MCS is outside HARQ-ACK codebook, and can be part of CSI report.

References

RP-211546	Resulting recommendations from RP-21530	RAN1 chair (Samsung)

RP-211297	Way forward on CSI feedback enhancements for enhanced URLLC/IIoT	InterDigital, Inc., Ericsson, Motorola Mobility, OPPO, Qualcomm, Samsung, SONY, Spreadtrum

R1-2106177, Feature lead summary#4 on CSI feedback enhancements for enhanced URLLC/IIoT, Moderator (InterDigital).