3GPP TSG RAN WG1 #106-e	R1-2106729

e-Meeting, August 16th – 27th, 2021

Source: 	ZTE

Title: 	Discussion on Rel-17 UL Tx switching

Agenda item:	5.1

Document for:	Discussion/Decision

Introduction

During RAN#90-e meeting, one objective was approved in NR_RF_FR1_enh WI [1] to support UL Tx switching for SUL and CA for both 2-carrier case and 3-carrier case in Rel-17. During RAN1#105-e meeting, some agreements were reached for this topic [2]. In this contribution, we present our further analysis for the remaining open issues for Rel-17 UL Tx switching.

2-carrier scenario

During RAN1#104b-e meeting, the following table was agreed for CA Option1 and SUL for 2-carrier scenario.

Agreements:

For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |
| Case 3 | 2T+0T | 2P+0P, 1P+0P |

Meanwhile, the following table was agreed for CA Option2.

Agreements:

For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |
| Case 3 | 2T+0T | 2P+0P, 1P+0P |

During RAN1#105-e meeting, the following agreements were achieved.

Agreement: - For a UE configured with higher layer parameter supplementaryUplink and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.3 of TS 38.214 is reused. Agreement: - For a UE configured with UL CA Option 1 and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.2 of TS 38.214 is reused with the following add-on. - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. Agreement: - For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured: - For option 2 of mapping between UL transmission ports and Tx chain - The switching period is only applicable in the following cases: - If the current state of Tx chains is 1Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on either carrier 1 or carrier 2. - If the current state of Tx chains is 0Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 1. - If the current state of Tx chains is 2Tx on carrier 1 and 0Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 2. - For other cases, the state of Tx chains of last UL transmission is assumed. - Note: For SUL, UL CA option 1 and UL CA option 2, in RAN1 understanding, no spec change to power configuration and power control. Agreement: - For a UE configured with 2Tx-2Tx UL Tx switching between two uplink carriers and configured with UL CA Option 2, if the state of Tx chains after UL Tx switching is not unique, a rule to determine the state of Tx chains after Tx switching is to be specified. - FFS: The state of Tx chains with the most of Tx chains on the most important uplink carrier is assumed, e.g. the carrier with uplinkTxSwitchingPeriodLocation configured as false.

For UL CA Option2, there is one remaining issue, i.e., how to address the ambiguity issue for 1P+0P/0P+1P since they are put in two cases. From our perspective, to simplify the implementation, one option is to predefine the state of Tx chains after Tx switching in the specifications. Since UE supports both 1-port and 2-port transmission on carrier2 and carrier1 under Case2 and Case3, respectively, to avoid unnecessary switching between 1-port and 2-port transmission, it is more appropriate to predefine the target case as Case2 and Case3. More specifically, if UE is under Case3 now and the next transmission is 0P+1P, UE switches to Case2; if UE is under Case2 now and the next transmission is 1P+0P, UE switches to Case3. Thus, we have the following proposal.

Proposal 1: For Rel-17 2Tx-2Tx switching between two uplink carriers for UL CA Option 2,

- if the UE is under case2 and it is to transmit a 1-port transmission on carrier1, the UE switches to case3

- if the UE is under case3 and it is to transmit a 1-port transmission on carrier2, the UE switches to case2.

The following TP is proposed to capture the above agreements and proposal 1 into specification.

Proposal 2: Introduce TP-1 (in section 6.1) for Rel-17 2Tx-2Tx switching between two uplink carriers for UL CA in S6.1.6.2 of TS38.214.

3-carrier scenario

Case1 & Case2 for CA and SUL

During RAN1#104b-e meeting, the following table was agreed for CA Option1 and SUL for 3-carrier scenario for case1/case2.

Agreement:

For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows.

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+(0P+0P) |
| Case 2 | 0T+2T | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |

Meanwhile, the following table was also agreed for CA Option2.

Agreement:

For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows.

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+(0P+0P), 1P+(1P+0P), 1P+(0P+1P), 1P+(1P+1P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P) |
| Case 2 | 0T+2T | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |

The applicability of switching gap can be summarized as the following.

Proposal 3: For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, for SUL and UL CA Option 1, the switching gap is needed for the following cases:

1. The next transmission is a 1-port or 2-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port transmission on carrier1;

2. The next transmission is a 1-port transmission on carrier1 and the preceding transmission was a 1-port or 2-port transmission on carrier2 or carrier3.

Proposal 4: For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, for UL CA Option 2, the switching gap is needed for the following cases:

1. The next transmission is a 1-port or 2-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port transmission on carrier1, carrier2 or carrier3 and UE is under Case1

2. The next transmission is a 1-port transmission on carrier1 and the preceding transmission was a 1-port or 2-port transmission on carrier2 or carrier3 and UE is under Case2 or Case3.

Similarly, to avoid the unnecessary debate on whether to reuse the previous “triggering mechanism” and the definition/scope of “triggering mechanism”, we propose to discuss the TP directly.

Proposal 5: Introduce TP-2 (in Section 6.2) for Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B for UL CA in S6.1.6.2 of TS38.214.

Case2 & Case3 for CA and SUL

During RAN1#104b-e meeting, the following table was agreed for CA Option1 and SUL for 3-carrier scenario for case2/case3.

Agreement:

For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows.

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- |
| Case 2 | 0T+2T | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |
| Case 3 | 2T+0T | 2P+(0P+0P), 1P+(0P+0P) |

The applicability of switching gap can be summarized as the following.

Proposal 6: For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, for SUL and UL CA Option 1, the switching gap is needed for the following cases:

1. The next transmission is a 1-port or 2-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port or 2-port transmission on carrier1;

2. The next transmission is a 1-port or 2-port transmission on carrier1 and the preceding transmission was a 1-port or 2-port transmission on carrier2 or carrier3.

Similarly, to avoid the unnecessary debate on whether to reuse the previous “triggering mechanism” and the definition/scope of “triggering mechanism”, we propose to discuss the TP directly.

Proposal 7: Introduce TP-3 (in Section 6.3) for Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B for UL CA in S6.1.6.2 of TS38.214.

Case1 & Case2 & Case3 for CA

During RAN1#104b-e meeting, the following table was agreed for CA Option1 and SUL for 3-carrier scenario for case1/case2/case3.

Agreement:

For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows.

|  | Number of Tx chains in WID (band A + band B) | Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+(0P+0P), 1P+(1P+0P), 1P+(0P+1P), 1P+(1P+1P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P) |
| Case 2 | 0T+2T | 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P) |
| Case 3 | 2T+0T | 2P+(0P+0P), 1P+(0P+0P) |

For the above UL CA Option2, there is one remaining issue, i.e., how to address the ambiguity issue for 1P+(0P+0P) since it is put in two cases. 0P+(1P+0P), 0P+(0P+1P) and 0P+(1P+1P) have similar ambiguity issue. From our perspective, to simplify the implementation, one option is to predefine the switching direction for them in the specification. And more specifically, if UE is under Case3 now and the next transmission is 0P+(1P+0P), 0P+(0P+1P) or 0P+(1P+1P), UE switches to Case2; if UE is under Case2 now and the next transmission is 1P+(0P+0P), UE switches to Case3.

Proposal 8: For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B,

- if the UE is under case2 and it is to transmit a 1-port transmission on carrier1, the UE switches to case3

- if the UE is under case3 and it is to transmit a 1-port transmission on carrier2 and/or carrier3, the UE switches to case2.

The applicability of switching gap can be summarized as the following.

Proposal 9: For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, for UL CA Option 2, the switching gap is needed for the following cases:

1. The next transmission is a 1-port carrier1 and the preceding transmission was a 1-port or 2-port transmission on carrier2 or carrier3 and UE is under case2;

2. The next transmission is a 1-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port or 2-port transmission on carrier1 and the UE is under case3;

3. The next transmission is a 2-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port or 2-port transmission on carrier1;

4. The next transmission is a 2-port transmission on carrier1 and the preceding transmission was a 1-port or 2-port transmission on carrier2 or carrier3;

5. The next transmission is 2-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port transmission on carrier1, carrier2 or carrier3 and UE is case1;

6. The next transmission is 2-port transmission on carrier1 and the preceding transmission was a 1-port transmission on carrier1, carrier2 or carrier3 and UE is case1.

Similarly, to avoid the unnecessary debate on whether to reuse the previous “triggering mechanism” and the definition/scope of “triggering mechanism”, we propose to discuss the TP directly.

Proposal 10: Introduce TP-4 (in Section 6.4) for Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B for UL CA Option2 in S6.1.6.2 of TS38.214.

Conclusion

In this contribution, we discuss and analyse Rel-17 UL Tx switching with the following proposals.

2-carrier scenario

Proposal 1: For Rel-17 2Tx-2Tx switching between two uplink carriers for UL CA Option 2,

- if the UE is under case2 and it is to transmit a 1-port transmission on carrier1, the UE switches to case3

- if the UE is under case3 and it is to transmit a 1-port transmission on carrier2, the UE switches to case2.

Proposal 2: Introduce TP-1 (in section 6.1) for Rel-17 2Tx-2Tx switching between two uplink carriers for UL CA in S6.1.6.2 of TS38.214.

3-carrier scenario: Case1 & Case2 for CA and SUL

Proposal 3: For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, for SUL and UL CA Option 1, the switching gap is needed for the following cases:

1. The next transmission is a 1-port or 2-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port transmission on carrier1;

2. The next transmission is a 1-port transmission on carrier1 and the preceding transmission was a 1-port or 2-port transmission on carrier2 or carrier3.

Proposal 4: For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, for UL CA Option 2, the switching gap is needed for the following cases:

1. The next transmission is a 1-port or 2-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port transmission on carrier1, carrier2 or carrier3 and UE is under Case1

2. The next transmission is a 1-port transmission on carrier1 and the preceding transmission was a 1-port or 2-port transmission on carrier2 or carrier3 and UE is under Case2 or Case3.

Proposal 5: Introduce TP-2 (in Section 6.2) for Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B for UL CA in S6.1.6.2 of TS38.214.

3-carrier scenario: Case2 & Case3 for CA and SUL

Proposal 6: For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, for SUL and UL CA Option 1, the switching gap is needed for the following cases:

1. The next transmission is a 1-port or 2-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port or 2-port transmission on carrier1;

2. The next transmission is a 1-port or 2-port transmission on carrier1 and the preceding transmission was a 1-port or 2-port transmission on carrier2 or carrier3.

Proposal 7: Introduce TP-3 (in Section 6.3) for Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B for UL CA in S6.1.6.2 of TS38.214.

3-carrier scenario: Case1 & Case2 & Case3 for CA

Proposal 8: For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B,

- if the UE is under case2 and it is to transmit a 1-port transmission on carrier1, the UE switches to case3

- if the UE is under case3 and it is to transmit a 1-port transmission on carrier2 and/or carrier3, the UE switches to case2.

Proposal 9: For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, for UL CA Option 2, the switching gap is needed for the following cases:

1. The next transmission is a 1-port carrier1 and the preceding transmission was a 1-port or 2-port transmission on carrier2 or carrier3 and UE is under case2;

2. The next transmission is a 1-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port or 2-port transmission on carrier1 and the UE is under case3;

3. The next transmission is a 2-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port or 2-port transmission on carrier1;

4. The next transmission is a 2-port transmission on carrier1 and the preceding transmission was a 1-port or 2-port transmission on carrier2 or carrier3;

5. The next transmission is 2-port transmission on carrier2 or carrier3 and the preceding transmission was a 1-port transmission on carrier1, carrier2 or carrier3 and UE is case1;

6. The next transmission is 2-port transmission on carrier1 and the preceding transmission was a 1-port transmission on carrier1, carrier2 or carrier3 and UE is case1.

Proposal 10: Introduce TP-4 (in Section 6.4) for Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B for UL CA Option2 in S6.1.6.2 of TS38.214.

Reference

- RP‑202799, New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1), RAN#90e.

- Chairman note of RAN1#105-e meeting.

Proposed TP

TP-1

- 6.1.6.2 Uplink switching for carrier aggregation For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-T0ffset or based on a higher layer configuration(s): - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - When the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL' or configured with [RRC_R17_CA Option1_2carrier], when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - For the UE configured with [RRC_R17_CA Option1_2carrier] or [RRC_R17_CA Option2_2carrier], when the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - For the UE configured with [RRC_R17_CA Option2_2carrier], when the UE is to transmit a 1-port on one uplink carrier and if the preceding uplink transmission was a 1-port or 2-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the another uplink carrier, then the UE switches to the operation state in which 2-port transmission can be supported on the uplink carrier and the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - For the UE configured with [RRC_R17_CA Option2_2carrier], when the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same or another uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in either carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the two carriers. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier, and any transmission on another uplink carrier. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

TP-2

- 6.1.6.2 Uplink switching for carrier aggregation For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with two uplink carriers for uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - When the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier, and any transmission on another uplink carrier. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with three uplink carriers for uplink carrier aggregation, where carrier1 is in one band and carrier2 and carrier3 are in another band: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-T0ffset or based on a higher layer configuration(s): - For the UE configured with [RRC_R17_CA Option1_3carrier-case1-case2], when the UE is to transmit a 1-port or 2-port transmission on carrier2 or carrier3 and if the preceding uplink transmission was a 1-port transmission on carrier1, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the three carriers. - For the UE configured with [RRC_R17_CA Option1_3carrier-case1-case2], when the UE is to transmit a 1-port on carrier1 and if the preceding uplink transmission was a 1-port or 2-port transmission on carrier2 or carrier3, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the three carriers. - For the UE configured with [RRC_R17_CA Option2_3carrier-case1-case2], when the UE is to transmit a 2-port transmission on carrier2 or carrier3 and if the preceding uplink transmission was a 1-port transmission on carrier1, carrier2 or carrier3 and the UE is under the operation state in which 2-port transmission can’t be supported on any uplink carrier, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the three carriers. - For the UE configured with [RRC_R17_CA Option2_3carrier-case1-case2], when the UE is to transmit a 1-port on carrier1 and if the preceding uplink transmission was a 1-port or 2-port transmission on carrier2 or carrier3, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the three carriers. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on carrier1, and any transmission on carrier2 or carrier3. The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on carrier2 or carrier3, and any transmission on carrier1. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

TP-3

For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with three uplink carriers for uplink carrier aggregation, where carrier1 is in one band and carrier2 and carrier3 are in another band: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-T0ffset or based on a higher layer configuration(s): - For the UE configured with [RRC_R17_CA Option1_3carrier-case2-case3], when the UE is to transmit a 1-port or 2-port transmission on carrier2 or carrier3 and if the preceding uplink transmission was a 1-port or 2-port transmission on carrier1, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the three carriers. - For the UE configured with [RRC_R17_CA Option1_3carrier-case2-case3], when the UE is to transmit a 1-port or 2-port transmission on carrier1 and if the preceding uplink transmission was a 1-port or 2-port transmission on carrier2 or carrier3, then the UE is not expected to transmit for the duration of NTX1-TX2 on any of the three carriers. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on carrier1, and any transmission on carrier2 or carrier3. The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on carrier2 or carrier3, and any transmission on carrier1. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

TP-4

For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with three uplink carriers for uplink carrier aggregation, where carrier1 is in one band and carrier2 and carrier3 are in another band, for the UE configured with [RRC_R17_CA Option1_3carrier-case1-case2-case3]: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-T0ffset or based on a higher layer configuration(s): - When the UE is to transmit a 1-port transmission on uplink carrier1 and if the preceding uplink transmission was a 1-port or 2-port transmission on uplink carrier2 or carrier3 and the UE is under the operation state in which 2-port transmission can be supported in carrier2 and carrier3, the UE switches to the operation state in which 2-port transmission can be supported on carrier1 and the UE is not expected to transmit for the duration of on any of the three carriers. - When the UE is to transmit a 1-port transmission on uplink carrier2 or carrier3 and if the preceding uplink transmission was a 1-port or 2-port transmission on uplink carrier1 and the UE is under the operation state in which 2-port transmission can be supported in carrier1, the UE switches to the operation state in which 2-port transmission can be supported on carrier2 and carrier3 and the UE is not expected to transmit for the duration of on any of the three carriers. - When the UE is to transmit a 2-port transmission on uplink carrier2 or carrier3 and if the preceding uplink transmission was a 1-port or 2-port transmission on uplink carrier1, then the UE is not expected to transmit for the duration of on any of the three carriers. - When the UE is to transmit a 2-port transmission on uplink carrier1 and if the preceding uplink transmission was a 1-port or 2-port transmission on uplink carrier2 or carrier3, then the UE is not expected to transmit for the duration of on any of the three carriers. - When the UE is to transmit a 2-port transmission on uplink carrier2 or carrier3 and if the preceding uplink transmission was a 1-port transmission on uplink carrier1, carrier2 or carrier3 and the UE is under the operation state in which 2-port transmission cannot be supported on any of carrier, then the UE is not expected to transmit for the duration of on any of the three carriers. - When the UE is to transmit a 2-port transmission on uplink carrier1 and if the preceding uplink transmission was a 1-port transmission on uplink carrier1, carrier2 or carrier3 and the UE is under the operation state in which 2-port transmission cannot be supported in any of carrier, then the UE is not expected to transmit for the duration of on any of the three carriers. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on carrier1, and any transmission on carrier2 or carrier3. The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on carrier2 or carrier3, and any transmission on carrier1. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.