# 3GPP TSG RAN WG1 #106-e							R1-2106761

# e-Meeting, August 16th – August 27th, 2021

Agenda item:	8.15.2

Source: 	Qualcomm Incorporated

Title: 	Timing relationship enhancements

Document for:	Discussion and Decision

# Background

In RAN#92-e, the following RAN1 objectives were approved for eMTC/NB-IoT over NTN in the WID [1]:

Specify the following time and frequency synchronization enhancements, using NR_NTN_solutions WI agreements as baseline, according to Section 8 in TR 36.763:

-	UE pre-compensation including ephemeris format (orbital / Position -Velocity)

-	UE pre-compensation for UL synchronization in RRC_IDLE and RRC_CONNECTED states based at least on its GNSS-acquired position and the serving satellite ephemeris

-	Timing advance formula (granularity of the timing advance may be different)

-	Combination of Open (i.e. UE autonomous TA estimation, and common TA estimation) and Closed TA (i.e., received TA commands) control loops in RRC_CONNECTED state

Agreements on the above are up to the decision in NR_NTN_Solutions WI and will be used for IoT NTN with minimum changes, if any.

Specify the following time and frequency synchronization enhancements that are not covered by NR_NTN_Solutions WI agreements, according to Section 8 in TR 36.763:

-	Long PUSCH and PRACH Transmission enhancements: segmented UE pre-compensations, new UL gaps and/or implementation solutions, time units and duration of segments.

-	Validity timer for UL synchronization: satellite ephemeris, and potentially other aspects

-	DL synchronization enhancements: A single solution will be selected between: new channel raster, (part of) ARFCN-indication-in-MIB.

-	GNSS Measurements: Validity of a GNSS position fix and details of acquiring a GNSS position fix, duration of validity, in RRC CONNECTED mode for sporadic short transmission

Specify the following IoT NTN specific timing relationships enhancements according to Section 8 in TR 36.763:

-	Timing relationships for NB-IoT / eMTC: as listed in Section 6.6.3 in TR 36.763

-	UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD

-	Signalling aspects in UE-specific TA maintenance and reporting, techniques to reduce the signalling load and determination of the UE-specific TA.

In this contribution, we provide our views on timing relationship enhancements.

# Reporting of UE-specific TA to the network

To handle UL/DL collisions for half-duplex UEs, it is essential that the UE-specific TA (which the UE determines autonomously) be made known to the network and updated subsequently if the value changes significantly. This is important since any UE-specific K_offset that the network may configure a UE with, depends on accurate knowledge of the UE-specific TA.

Observation 1: UE-specific K_offset is derived based on the UE-specific TA. As a result, the UE-specific TA is the fundamental quantity that needs to be reported (and updated) by the UE to the network, for the network to ensure causal and collision-free scheduling of UL/DL communications for half-duplex UEs.

Having established the need for UE-specific TA reporting by the UE, we must specify the mechanism by which such reporting shall occur. These comprise aspects such as what the triggering conditions for the reporting are, and how (e.g., in which messages) such a report is sent.

In terms of triggering conditions, a natural option is for the UE to report a new value of UE-specific TA when it determines that the last reported value differs from the current value by a certain threshold. This mechanism ensures that the TA is tracked accurately by the base-station, in an on-demand basis. The network can use this value to update the UE-specific K_offset to maintain causality and avoid collisions in half-duplex scheduling.

Proposal 1: A UE reports the UE-specific TA to the network when the previously reported value differs from a current value by a pre-determined threshold.

In terms of reporting the TA, a solution that is attractive for NTN (since it avoids round-trip delays in case an uplink resource is to be requested) is to use an uplink semi-persistent scheduling (SPS) message (such as the one specified for NB-IoT to transmit a buffer status report (BSR)) on which the UE can transmit this TA report when required.

Proposal 2: The UE-specific TA is reported in an uplink semi-persistent scheduling (UL-SPS) message, such as the one used for reporting buffer status reports (BSRs) in NB-IoT.

# Definition of interrupted downlink subframes

Due to large, UE-specific TAs, DL subframes interrupted due to an UL transmission are UE-specific. Current specs assume a small TA (e.g., up to 1 ms), and do not incorporate a “TA term” in defining these subframes. However, as illustrated in Fig. 1, for NTN, the interrupted subframes need to be defined using a term that captures the impact of this very large TA. In the figure, we want X_DL to Y_DL to be the interrupted DL SFs—i.e., the subframes where the UE is not required to monitor another PDCCH, since it must get ready to transmit the PUSCH starting at Y_UL. Without the “-TA” term for Y_DL, the interrupted SFs would be from X_DL +TA to Y_DL +TA, while it would (problematically) be expected to monitor PDCCH from X_DL to Y_DL!

Figure 1: Definition of DL interrupted subframes in the presence of large NTN-specific TAs.

Examples (for NB-IoT) where this needs to be reflected include DL subframes before and after a NPUSCH or NPRACH transmission, DL subframes corresponding to half-duplex guard periods, etc.

Proposal 3: The definition of downlink interrupted subframes (e.g., those before and after a PUSCH, PRACH, PUCCH, and half-duplex guard periods), where a half-duplex UE is not expected to monitor PDCCH, is modified, in accordance with the large UE-specific TAs in NTN.

# Need for UE-specific K_offset

There has been some discussion as to whether a UE-specific K_offset is essential for IoT-NTN. In our view, it is indeed essential, since without a UE-specific K_offset, the throughput will suffer significantly when the differential delay between the UEs in an NTN cell is high—such as in GEO-based deployments—since the cell-specific K_offset has to cater to the worst-case, i.e., ensure that the UE with the maximum round-trip delay can be scheduled causally.

Proposal 4: Specifiy UE-specific K_offset(s) for IoT-NTN.

# Conclusion

In this contribution we presented our views on enhancements for NB-IoT/eMTC over NTN as it relates to timing relationship enhancements. We summarize our proposals below.

Observation 1: UE-specific K_offset is derived based on the UE-specific TA. As a result, the UE-specific TA is the fundamental quantity that needs to be reported (and updated) by the UE to the network, for the network to ensure causal and collision-free scheduling of UL/DL communications for half-duplex UEs.

Proposal 1: A UE reports the UE-specific TA to the network when the previously reported value differs from a current value by a pre-determined threshold.

Proposal 2: The UE-specific TA is reported in an uplink semi-persistent scheduling (UL-SPS) message, such as the one used for reporting buffer status reports (BSRs) in NB-IoT.

Proposal 3: The definition of downlink interrupted subframes (e.g., those before and after a PUSCH, PRACH, PUCCH, and half-duplex guard periods), where a half-duplex UE is not expected to monitor PDCCH, is modified, in accordance with the large UE-specific TAs in NTN.

Proposal 4: Specifiy UE-specific K_offset(s) for IoT-NTN.

# References

[1] RP-211601, WID on NB-IoT/eMTC support for NTN