3GPP TSG RAN WG1 #106-e		  R1-2106796

e-Meeting, August 16th – 27th, 2021

Agenda Item:					    8.2.2

Source:	Sony

Title:	PDCCH enhancement for NR from 52.6GHz to 71GHz

Document for:	Discussion

# Introduction

In RANP#90e, the WID of NR operating in 52.6GHz-71GHz frequency has been established in [1], and the PDCCH enhancement was captured as below under RAN1’s working scope

According to the outcome of the study item on Supporting NR above 52.6GHz and leveraging FR2 design to the extent possible, this WI extends NR operation up to 71GHz considering, both, licensed and unlicensed operation, with the following objectives: - Physical layer aspects including [RAN1]: - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

During the RAN1#104bis-e meeting [2], the following agreement has been made for 120 kHz SCS PDCCH monitoring capability:

Agreement: For 120 kHz SCS in 52.6-71 GHz, the BD/CCE budget is the same as that for 120 kHz in FR2.

Moreover, PDCCH monitoring capability definition for SCS of 480 kHz and 960 kHz was further discussed based on three candidate solutions, but without concluding:

Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability - Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability. - Each slot group consists of X slots - Slot groups are consecutive and non-overlapping - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g. the Y [symbols or slots] always start at the first slot within a slot group - FFS: Further definition of capabilities - Alt 2: Use an (X, Y) span as the baseline to define the new capability - X is the minimum time separation between the start of two consecutive spans - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots] - Y <= X - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary. - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots. - FFS: Further definition of capabilities - Alt 3: Use a sliding window of X slots as the baseline to define the new capability. - The capability indicates the BD/CCE budget within the sliding window - The sliding unit of the sliding window is [1] slot. - FFS: Further definition of capabilities - Specific numbers for X, Y may depend on UE capability and gNB configuration - Examples: - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS

In this contribution, we would like to present our further view on PDCCH enhancements for above 52.6GHz.

# PDCCH monitoring enhancement

From RANP#90 meeting, for systems operating at 52.6GHz to 71GHz carrier frequency, larger subcarrier spacing greater than 120kHz with 480 kHz and 960 kHz were introduced, then the time duration for one slot is reduced further. Take 960 kHz for illustration; an increase in SCS to 960 kHz may require processing eight times as many slots as 120 kHz systems for the same duration. As such, if the PDCCH processing procedure and the capability limits remain unchanged, the UE monitoring capability and power consumption will grow much complicated. Considering the limitations of UE processing capability, PDCCH enhancement was agreed upon for supporting larger SCS.

## Multi-slot PDCCH monitoring capability

With a comprehensive consideration of the UE monitoring complexity reduction, the UE power-saving, and the DL performance, defining a new time unit like multi-slot could be a proper solution for larger SCS (480 kHz and 960 kHz) in the 52.6GHz-71GHz frequency range. More specifically, three elements may need to be considered to specify the multi-slot PDCCH monitoring scheme:  1. A minimum gap X slots between the first symbol of two multi-slot monitoring. 2. A Y consecutive units for each multi-slot monitoring, and 3. If pattern (X, Y) or even starting position of the Y consecutive units of this multi-slot monitoring could be flexible.

A minimum gap of X units between the first symbol of two multi-slot monitoring is to avoid increasing the UE monitoring complexity with larger SCSs. For example, to maintain the same complexity with 120 kHz, X=8 slots for 960 kHz and X = 4 slots for 480 kHz would be needed.

Similarly, there is also a need to define the number of consecutive Y units that UE needs to be monitored with multi-slot monitoring. Different values of Y can be selected for 480 kHz and 960 kHz, respectively, e.g., 2 or 3 slots (28-42 symbols) slots for 960 kHz and 1or2 slots (14-28 symbols) for 480 kHz. Y consecutive units for each multi-slot monitoring can centralize monitoring to reduce the latency in PDCCH blind detection, thus reducing the delay of data interaction.

Pattern (X, Y) defines the combination of X and Y, which can be fixed in slot group patterns or supports some flexibility in span patterns.

- : Illustrations of the slot group structure of a fixed slot group pattern and a flexible (X, Y) span

In the last meeting, three possible multi-slot PDCCH monitoring schemes have been proposed [3]:

- Alt 1: Use a fixed pattern in a slot group as the baseline to define the new capability.

- Alt 2: Use an (X, Y) span as the baseline to define the new capability

- Alt 3: Use a sliding window of X slots as the baseline to define the new capability.

To our understanding, the motivation of Alt 3 (sliding window) is to avoid the back-to-back monitoring occasion while ensuring enough flexibility for network scheduling. However, those concerns can be well addressed by Alt 1 or Alt 2 by carefully selecting the value of multiple slots monitoring span according to the email discussion in RAN1#104bis-e meeting [3]. Therefore, we focus on Alt 1 and Alt 2 in the discussion below:

Both Alt. 1 and Alt. 2 cover the aspect of defining X, Y, and they are feasible solutions. Illustrations of the slot group structure of a fixed slot group pattern and a flexible (X, Y) span are shown in Fig. 2, where it is assumed that X = 8 slots and Y = 3 slots for the fixed pattern.

Considering the simplicity of the fixed slot group pattern in Alt. 1, it can be taken as a baseline for Rel-17 multi-slot PDCCH monitoring. Different combinations of (X, Y) may still be specified, which can be up to UE capabilities, e.g. (8,3), (4,3), (4,2), so that the network can benefit from advanced UE designs. Moreover, the back-to-back monitoring occasion can be simply avoided by setting Y to be smaller than X, and consecutive Y units can always start from the beginning of each slot group X to further reduce the complexity of the system. Though the proposed method may lack flexibility compared to other alternatives, it may predict that the limited number of unexpected MOs can eventually contribute to a better power efficiency on UE operations.

On the other hand, for the span-based monitoring, after UE reporting span capability, the Y consecutive units' start position can be flexibly configured by the network as long as the gap between the first symbol of two continuous spans is no less than X, which improves the scheduling flexibility. However, as stated above, the lack of a fixed pattern could result in a large increase in UE processing complexity as the UE may have to plan for its processing highly dynamic, and the micro-sleep opportunity decreased for devices.

- : Using a fixed pattern (Alt.1) in a slot group provides the simplest scheme, while Use an (X, Y) span (Alt.2) can provide higher flexibility of scheduling.

Overall, considering the short symbol duration of 480kHz and 960 kHz SCS, a fixed pattern according to Alt 1 may provide a better trade-off between flexibility and system complexity than the flexible multiple slot span pattern described in Alt. 2.

Proposal 1: Adopting Alt. 1 as a baseline for PDCCH multi-slot span for 480 kHz and 960 kHz with the additional constraints below:

- X=4 slots for 480 kHz, and X=8 slots for 960 kHz

- 1<=Y< X, while Y always starts from the first slot of each X slot group.

## PDCCH monitoring location and duration

For multi-slot monitoring, there also had been some discussions from RAN1#104 on monitoring location and duration of OFDM symbols:

Further discussion on multi-slot span capabilities, monitoring periodicities, corresponding number and location of OFDM symbols for Cases 1-1 and 1-2. - Case 1: PDCCH monitoring of all SS sets monitored in a slot occurs within 3 consecutive OFDM symbols that have fixed positions in each slot - Case 1-1: PDCCH monitoring limited to within first three OFDM symbols of a slot - Case 1-2: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot - For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot - Case 2: PDCCH monitoring cases other than Case 1

With a limited location of PDCCH monitoring, Case 1-1 is simple for realization, while Case 1-2 is more flexible for gNB scheduling. Thus, we suggest Case 1-1 can be the baseline, and Case 1-2 can be discussed with further benefits evaluation of flexible scheduling.

- : PDCCH monitoring limited to within the first several OFDM symbols of a slot can be supported as the baseline.

As aforementioned, large SCSs with 480kHz and 960kHz cause a relatively short time duration of a symbol. Therefore,  if CORESET duration remains up to 3 symbols as in R16, the real-time duration for PDCCH monitoring is quite small, which also puts extra time limitation of UE blind decoding. Therefore, we suggest a large CORESET duration with more than 3 symbols for SCS 480kHz and 960kHz alleviate UE processing capability for PDCCH decoding. Thus, we suggest PDCCH monitoring with a maximum duration of more than 3 OFDM symbols per PDCCH monitoring occasion.

- : If CORESET duration remains up to 3 symbols as in R16, the real-time duration for PDCCH monitoring is quite small due to the short symbol duration with large SCS.

Proposal 2: PDCCH monitoring with a maximum duration of more than 3 OFDM symbols per PDCCH monitoring occasion is more suitable.

# Multi Beam aspects

As haven been mentioned by multiple companies in the last meeting [5]-[7]: In Rel-16 NR-U, several fields such as CO duration, SS-group switching trigger, RB-sets, etc., were introduced to DCI format 2_0. In the frequency range above 52.6 GHz, that information can be beam-dependent due to the utilization of beamforming. Therefore, there is a need to consider per beam indication of DCI format 2_0.

Proposal 3: Support per beam indication of DCI format 2_0 for above 52 GHz unlicensed operation.

# Conclusions

Finally, allow us to repeat our observations and proposals to draw attention.

- : Using a fixed pattern (Alt.1) in a slot group provides the simplest scheme, while Use an (X, Y) span (Alt.2) can provide higher flexibility of scheduling.

- : PDCCH monitoring limited to within the first several OFDM symbols of a slot can be supported as the baseline.

- : If CORESET duration remains up to 3 symbols as in R16, the real-time duration for PDCCH monitoring is quite small due to the short symbol duration with large SCS.

Proposal 1: Adopting Alt. 1 as a baseline for PDCCH multi-slot span for 480 kHz and 960 kHz with the additional constraints below:

- X=4 slots for 480 kHz, and X=8 slots for 960 kHz

- 1<=Y< X, while Y always starts from the first slot of each X slot group.

Proposal 2: PDCCH monitoring with a maximum duration of more than 3 OFDM symbols per PDCCH monitoring occasion is more suitable.

Proposal 3: Support per beam indication of DCI format 2_0 for above 52 GHz unlicensed operation.

# Reference

- RP-202925: Revised WID: Extending current NR operation to 71 GHz.

- R1-2103980 Session notes for 8.2 (Study on supporting NR from 52.6 GHz to 71 GHz)”, Ad-Hoc Chair (Ericsson)

- R1-2104057	 Feature lead summary #2 for [104b-e-NR-52-71GHz-02] Email discussion/approval on PDCCH Monitoring Enhancements”, 	Moderator (Lenovo)

- R1-2102559 	PDCCH monitoring enhancements	  Nokia, Nokia Shanghai Bell

- R1-2102997 	PDCCH monitoring enhancements for NR from 52.6 GHz to 71GHz	  Lenovo, Motorola Mobility

- R1-2103097	 Discussion on PDCCH Enhancements for above 52.6 GHz  	Apple

- R1-2103230	 PDCCH monitoring enhancements for NR from 52.6 GHz to 71 GHz 	 Samsung

- R1-2103340 	PDCCH monitoring enhancements to support NR above 52.6 GHz 	 LG Electronics