3GPP TSG RAN WG1 #106-e		                           		    R1-2106811

e-Meeting, August 16th –27th, 2021

Agenda Item: 	8.5.3

Source:		Sony

Title:			Considerations on DL-AoD enhancements

Document for:	Discussion & Decision

# Introduction

The work item on NR positioning enhancements was updated in RANP#91e [1]. The objective of this work item is to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning enhancements for improving positioning accuracy, latency, network and/or device efficiency. One of the objectives of this work item is:

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy [1].

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

Note: RAN1 will discuss the candidate solutions and provide updates for this objective, with status to be reviewed in RAN#91e.

In RAN1#105-e. The following agreements were made [2]:

Agreement: For both UE-based and UE-assisted DL-AOD, the UE can be requested subject to UE capability to measure and report (for UE-assisted) the PRS RSRP of the first path - FFS: Details of measurement and reporting of PRS RSRP of the first path For UE-assisted DL-AOD positioning method, select one or more of the following to enhance the signalling to the UE for the purpose of PRS resource(s) measurement and reporting: - Option 1: the LMF explicitly identify adjacent beams in the assistance data (AD) - Option 2: the LMF send the beam information in the AD with an order of priority of PRS resources. - Option 3: the LMF includes boresight direction information for each PRS resource in the assistance data. - Option 4: the LMF send the beam information in the AD with indicated subset of PRS resources. - FFS: Detailed signalling and procedure - FFS: How to define adjacent beams For the beam/antenna information to be optionally provided to the LMF by the gnodeB, select one or more of the following: - Option 1: the gNB reports the antenna configuration including at least the following parameter: - the number of antenna elements (vertical and horizontal) - antenna spacing dh and dv - FFS: For DFT-based beams, precoder information for each PRS resource - Check whether the already reported boresight directions are sufficient, or whether more information is needed - FFS: Antenna Element pattern Information - FFS: Details - FFS: If additional information about panel/orientation is needed - Option 2: the gNB reports a mapping of angle and beam gains for each of the PRS resources. - FFS: representation of the mapping (e.g. parametric function approximating the beam response, or gain/angle table, beamwidth, intersection point of multiple beams (angle, RSRP)intersection point) - Other options are not precluded - In either option, the gNB beam/antenna information can optionally be provided to the UE by the LMF for UE-based DL-AoD - For both UE-based and UE-assisted DL methods, at least for two-stage PRS beam sweeping, study further at least the following: - Enhancements in the association between resources belonging to two DL PRS resource sets of the same TRP - Companies are encouraged to evaluate whether other potential enhancements in this subagenda or other subagendas (e.g. additional beam information, on-demand PRS framework) could be used to enable this feature (potentially by implementation). - Note: Two-stage PRS beam sweeping corresponds to different DL PRS resource sets

This contribution firstly analyzes the remaining issues left from the #105e meeting and then discusses some potential DL-AoD enhancements to improve UE positioning accuracy in relation the progress made in the previous meeting.

# Discussions

## Discussion on assistance data (AD)

The discussion on assistance data (AD) sent from LMF to UE during RAN1 #105e is still around enabling LMF to indicate which PRS resources to be measured. In our view, the intention of this method is to assist the UE to select the adjacent beams that could be based on the LMF’s historical knowledge. LMF may have the knowledge of the UE location or the channel quality from the historical measurement. Based on this knowledge, LMF can estimate the adjacent beam (e.g., Tx beams) of the associated PRS resources. In another example, the adjacent beams can be the beams adjacent to the LOS beam. If the UE reports the adjacent beams, the LMF can obtain an angular domain with a denser granularity. Hence, a better positioning accuracy can be achieved. However, we also consider some potential issues of indicating specific beams:

- Require associated gNB ID: The indicated PRS resource ID are gNB specific. The LMF need to inform the UE of the association information of the indicated PRS resources with the gNB ID.

- Low resource efficiency: Normally, the indicated beams are only small part of the transmitted beams. The beams that are not being indicated by LMF cause a waste of the PRS resources.

- Mismatch between UE’s and LMF’s decision: UE could also have its own estimation on the reported beams based on the spatial channel measurement. It generates mismatch between UE’s estimation and LMF’s indication if they are not the same.

- Limited number of indicated PRS resources: As defined by LPP [3], the maximum number of reported beams is 8 which means LMF can’t indicate more than 8 beams.

We consider using on-demand PRS or two-stage beam sweeping as a better method to measure specific beams. In case the LMF knows the expected AoD (e.g., the range within which the boresight direction may occur), the LMF can inform this information to the gNB. The gNB would allocate the PRS resources and perform beam sweeping in the range accordingly. As an alternative, LMF can also directly indicate the PRS resources. This method has some advantages over the aforementioned method. First of all, LMF can indicate more than 8 PRS resources. The maximum number of indicated beams can be up to 64 which is the limitation of the beams used in one measurement occasion. And secondly, LMF’s indication is transparent to the UE. Therefore, there will be no mismatch issue. This method would also be more resource efficient, since all the transmitted beams are indicated by LMF.

Observation 1: Enabling LMF to indicate adjacent beams have some potential issues, such as low resource efficiency and mismatch between UE’s and LMF’s decision.

Proposal 1: Down-prioritize the usage of assistance data (AD) to indicate adjacent beam. The similar enhancements can be obtained by on-demand PRS operation and/or two-stage beam sweeping to enable LMF to indicate specific PRS resources.

## Two-stage beam sweeping

Two-stage beam sweeping were also briefly discussed in RAN1#105e. We consider this approach is beneficial to improve positioning accuracy. In our view, beam sweeping can be divided into two stages. In the first stage, gNB preforms normal beam sweeping covering the full range (0~ in FR2 and 0~180º in FR1).  This is performed in the first resource set. In the second stage, gNB performs beam sweeping in a smaller range in the second resource set. Due to the greater beam density than the first stage, UE can obtain an estimated angular domain with a higher granularity in the second stage. Hence, the accuracy can be improved.

To illustrate the enhancement by increasing beam density, we provide following simulation results. It should be noted that the beam density here is defined as the number of beams swept in a unit of degree. The simulation is performed in INF-SH scenario and FR2 frequency range. It is assumed that the beam sweeping range is 0~120º. In the first case, we used only 4 Tx sweeping beams. In the following four cases, we increased the number of beams in each case by 4 gradually.

Figure 1: DL-AoD positioning performance evaluated by using different Tx beam density (number of beam / sweeping range).

From the figure, we can observe that the horizontal error becomes smaller as the beam density increases. Besides that, one can also find that the performance gain is less significant as the beam density increases. At 16beams/120º, the gain by further increasing the beam density is negligible. This limitation could be due to the beam width.

This observation also proves that if the two-stage beam sweeping can increase the beam density in the second stage, it will improve the positioning accuracy. Furthermore, if the gNB can configure a smaller sweeping range (e.g. smaller than 120º), less beams would be needed to achieve the optimal density. From the resource usage perspective, it is more efficient.

Observation 2: DL-AoD positioning accuracy can be improved by increasing beam density (number of beam / sweeping range). Simulation also shows that the improvement becomes insignificant when the beam density reaches a certain value (e.g., 16beams/120º in InF-SH, FR2).

In order to facilitate two-stage beam sweeping operation, we consider LMF to assists the gNB in operating the beam sweeping for PRS transmission. After the first stage of beam sweeping, UE reports the beams that covers the whole range to LMF. LMF estimates a UE location and the corresponding uncertainty. These can also be transferred to an expected AoD range. Based on the expected AoD range, LMF can configure certain PRS resources by on-demand PRS. As an alternative, LMF can also choose to inform the gNB of the expected AoD range via assistance information. gNB can configure the sweeping beam by itself.

Proposal 2: Support LMF to assist gNBs  to facilitate the two-stage beam sweeping operation. It can be performed such as LMF configures sweeping beam directly by on-demand PRS, or LMF sent assistance information to gNB (e.g., the expected AoD range, beam width).

As two-stage beam sweeping is basically based on PRS transmission. We consider two-stage beam sweeping can be applied to DL-based positioning method, such as DL-AOD and DL-TDOA.

Proposal 3: Support two-stage beam sweeping for DL-AOD and DL-TDOA positioning

## First Path RSRP

In RAN1 #105e, the usage of first path RSRP in DL-AoD has been agreed upon. The remaining issue is how to define the first path RSRP. In our view, the first path RSRP is referring to the power/amplitude of the first arrival path in the unnormalized estimated power delay domain. The unnormalized estimated power delay domain is purely the cross-correlation between the reference signal and the received signal. Those two signals are captured in time windows with the same length. In practice, UE measures the first path delay from the power delay profile. Furthermore, it selects the first arrival path in the unnormalized power delay profile based on the delay and also capture the power of this path as the first path RSRP.

Proposal 4: First path RSRP is the power/amplitude of the first arrival path in the estimated power delay domain. The estimated power delay domain is given by the cross-correlation between reference PRS signal and received PRS signal.

A time window is used for measuring first path RSRP. We consider the intention is to define a specific region in the estimated power delay domain in which UE selects the first arrival path. In our view, this time window can be defined by the UE itself and not necessary to be configured by other sources (e.g., gNB or LMF). In the TDOA positioning, some algorithms have been applied in the UE to identify the signal arrival region, such as moving sum-based methods and some relative variants. We consider the DL-AoD positioning shall keep using the legacy method and leave the time window as a UE implementation aspect. Depending on UE implementation, the UE can select the first path from the measurement result. However, LMF can provide assistance information to assist the UE to select the first path. For example, LMF can provide the criteria of the first path,

Proposal 5: Time window for PRS-RSRP and selection of the first path are UE implementation aspect.

Proposal 6: Support assistance information from LMF to UE to assist UE in selecting the first path.

## Tx Beam information reporting

In RAN1#105, it was agreed that gNB reports the beam/antenna information to the LMF. We consider it is needed as the legacy AoD estimation in LMF is based on PRS-RSRP reported by the UE. The beam information is the beam when gNB transmits PRS. The legacy Rel-16 only provide the spatial direction when TRP transmit PRS resources. In case the LMF also obtain additional beam configuration, such as beamwidth information (e.g. HPBW) and the gain of the associated beam, this can improve AoD estimation in LMF. LMF can also improve AoD estimation by utilizing antenna information from gNB. The antenna information can be the codebook information associated with the PRS resources, gNB Tx antenna configuration (e.g. number of elements, antenna pattern).

Proposal 7: For UE-A DL-AoD positioning: support gNB to report the TX antenna configuration (e.g., antenna codebook configuration, number of elements, and antenna pattern) and TX beam configuration (e.g. beamwidth and gain). For UE-B DL-AoD positioning: gNB sends this information to the UE.

# Conclusion

In this contribution, we discuss some aspects to improve the accuracy of DL-AoD positioning method.  We have made the following observations and proposals:

Observation 1: Enabling LMF to indicate adjacent beams have some potential issues, such as low resource efficiency and mismatch between UE’s and LMF’s decision.

Observation 2: DL-AoD positioning accuracy can be improved by increasing beam density (number of beam / sweeping range). Simulation also shows that the improvement becomes insignificant when the beam density reaches a certain value (e.g., 16beams/120º in InF-SH, FR2).

Proposal 1: Down-prioritize the usage of assistance data (AD) to indicate adjacent beam. The similar enhancements can be obtained by on-demand PRS operation and/or two-stage beam sweeping to enable LMF to indicate specific PRS resources.

Proposal 2: Support LMF to assist gNBs  to facilitate the two-stage beam sweeping operation. It can be performed such as LMF configures sweeping beam directly by on-demand PRS, or LMF sent assistance information to gNB (e.g., the expected AoD range, beam width).

Proposal 3: Support two-stage beam sweeping for DL-AOD and DL-TDOA positioning

Proposal 4: First path RSRP is the power/amplitude of the first arrival path in the estimated power delay domain. The estimated power delay domain is given by the cross-correlation between reference PRS signal and received PRS signal.

Proposal 5: Time window for PRS-RSRP and selection of the first path are UE implementation aspect.

Proposal 6: Support assistance information from LMF to UE in order to assist UE in selecting the first path.

Proposal 7: For UE-A DL-AoD positioning: support gNB to report the TX antenna configuration (e.g., antenna codebook configuration, number of elements, and antenna pattern)and TX beam configuration (e.g. beamwidth and gain). For UE-B DL-AoD positioning: gNB sends this information to the UE.

# References

RP-210903, Revised WID on NR Positioning Enhancements

RAN1 Chairman’s Notes, RAN1#105, e-Meeting, 10th - 27th May 2021.

3GPP TS 37.355 V16.5.0 (2021-06) 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP) (Release 16)