3GPP TSG RAN WG1 #106-e		R1-2106834

e-meeting, August 16th – 27th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	Beam-management enhancements for NR from 52.6 GHz to 71GHz

Agenda Item:   8.2.4

Document for:	Discussion

# 1	Introduction

In RAN#92-e [1], a revised WI has been approved with the following objectives related to beam-management and multi-scheduling:

- Physical layer aspects including [RAN1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

  - Time line related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timeing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively.

  - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range.

  - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP.

    - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access.

    - Note: coverage enhancement for SSB is not pursued.

  - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

    - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed

    - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided

Furthermore, following agreements related to beam-management were made in RAN1#104bis-e [2]:

Agreement:

Introduce new parameter values for additional beam switching time delay d, when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS or PDSCH

Agreement:

For timeDurationForQCL, beamSwitchTiming and beamReportTiming,

- Following candidate values of FR2 are reused for 120 kHz:

  - timeDurationForQCL: 14 and 28 symbols

  - beamSwitchTiming: 14, 28, 48, 224 and 336 symbols

  - beamReportTiming: 14, 28 and 56 symbols

- For 480 kHz

  - Support at least the candidate values for 120 kHz scaled by 4x

  - FFS: Support for additional candidate value(s)

- For 960 kHz

  - Support at least the candidate values for 120 kHz scaled by 8x

  - FFS: Support for additional candidate values(s)

- FFS: UE capability signaling details

- Note: The scaled values 224 and 336 symbols for beamSwitchTiming are used as in Rel-16 (defined in Rel-15 with updates in Rel-16).

Agreement:

For multiple PDSCHs/PUSCHs scheduled by a single DCI, at least for single TRP, support indication of only a single TCI state/SRI in DCI

- FFS: number of TCI states/SRIs in a single DCI scheduling multiple PDSCHs/PUSCHs for multi-TRP

In this contribution, we provide our views on enhancements needed for beam management for NR operation between 52.6 GHz to 71 GHz with newly agreed SCS values including 480kHz and 960kHz.

# 2	Discussion

## Beam-management enhancements for initial access

Currently supported SCS values for FR2 are limited up to 240kHz for SSB and up to 120kHz for data channels. For SSB beam switching/sweeping procedure with SCS up to 240kHz, no time gaps between the contiguous SSB candidates are required and the beam switching occurs during the CP of the next OFDM symbol. RAN4 (TS38.817-2) specified the required beam switching time considering the analogue phase shifters delay of about 100ns. For SCS of 240kHz, the CP is long enough to handle the beam switching delay. However, for beyond 52.6GHz bands, new higher SCS values are being considered to mainly compensate against phase noise at high frequencies.  If these high SCS values are applied, a beam switching issue would appear between the contiguous transmissions since the CP length would not be enough for beam switching, and an extra gap needs to be added to prevent performance degradation. Furthermore, similar issue can arise when multiple contiguous transmission occasions (including repetitions/retransmissions) are to be transmitted on different beams and high SCS.

Observation 1: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, for the agreed higher subcarrier spacings (numerologies) such as 960kHz, beam switching issue would appear between the contiguous transmissions (such as SSB beams) since the CP length would not be enough for beam switching, and an extra gap might be needed to prevent performance degradation

One possibility could be introducing a symbol gap to accommodate the beam switching delay. However, it might not be a spectral efficient solution as using the entire symbol gap might be unnecessary, depending upon the SCS value. Other possibility that can be considered is to adapt the length of a gap between two contiguous SSBs that are on two different beams. Another possibility is to add a post-fix at the end of the last OFDM symbol of the SSB and the last PDSCH-SIB symbol in case of multiplexing patterns 2/3.

Proposal 1: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, if higher subcarrier spacings (numerologies) are adopted for SSB, then to allow the beam switching between contiguous SSBs, a gap (for example a symbol gap or post prefix) should be supported between contiguous SSB at least for 960kHz SCS

## Beam indication with m-TRP for multi-PDSCH

In RAN1#104bis-e, it has been agreed that a single TCI state (SRI) is indicated for multiple scheduled PDSCH/PUSCH via single DCI in case of single TRP scenario. However, for multiple TRP scenario, it should be further discussed how the TCI indication is done and how it is applied to multiple scheduled PDSCH. If a UE is scheduled to receive multiple PDSCHs, where some PDSCHs are received from one TRP, while some PDSCHs are received from second TRP, then the corresponding TCI states need to be indicated. Based on Rel-16 TCI state indication for multiple TRP, one TCI codepoint can indicate two TCI states, where one TCI state corresponds to one TRP, while other TCI state corresponds to other TRP. Similar indication can be followed for multiple PDSCH scheduling via single DCI, where it is allowed to have one TCI index indicating two TCI states.

Proposal 2: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, when multiple PDSCHs are scheduled via single DCI to be received from two TRPs (not all PDSCHs received from one TRP), then indication of at least two TCI states should be supported

Furthermore, it should be discussed when the TCI state (corresponding Rx beam at UE) needs to be updated i.e., which PDSCHs are received from first TRP and which are received from second TRP. One method could be to extend the signaling to also configure the duration for which first TCI state is valid and the remaining duration for which second TCI state is valid. An example is shown in Figure 1, where the TCI state 1 is applied to PDSCH1 and PDSCH2, and then the TCI state is switched for PDSCH 3 to TCI state 2. Such switching point needs to be indicated to the UE.

Figure 1: Example of multiple TCI indication by DCI for multiple scheduled PDSCHs

Proposal 3: For NR operation between 52.6 GHz and 71 GHz when multiple PDSCHs are scheduled via single DCI to be received from two TRPs (not all PDSCHs received from one TRP), then the duration for which each TCI state is valid should also be indicated

## Multiple default beams association for multi-PDSCH/PUSCH over multiple slots

Another issue that has been discussed during RAN1#104-e [3] related to PDSCH (PUSCH) beam is related to determination of default beam(s).

Agreement:

Further study the following:

- For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL.

- For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL

- Note: If the current Rel-16 behavior would be extended to multiple-PDSCH scheduling, it could result in a different QCL assumption for each PDSCH due to the fact the that the CORESET with the lowest ID can be different for different slots, resulting in a potentially different TCI state for each slot

- Note: Applicability to multi-TRP can be discussed further

In our view, if the UE is expected to monitor different CORESETs with the lowest IDs during the PDSCH transmission across multiple slots, then UE should not be expected to change the default beam based on the lowest CORESET ID monitored after the first PDSCH transmission has started. The default QCL assumption should be based only on the QCL assumption associated with lowest CORESET ID that is monitored before the start of PDSCH transmission. However, this default QCL assumption might not be suitable for the entire duration of multiple PDSCH transmissions across multiple slots. Therefore, one straightforward solution could be to associate (lowest) CORESET ID with multiple QCL assumptions (multiple beams). Currently, in Rel-17 MIMO WI, it is already agreed to support association of CORESET with two QCL assumptions. For the purpose of multiple PDSCH scheduling by single DCI across multiple slots, it should be considered to support the association of more than two QCL assumptions with CORESET ID and additionally associate the duration for which each of the associated QCL assumption (beam) can be applied as default beam for multiple PDSCH across multiple slots. An example is illustrated in Figure 2, where two QCL assumptions are associated with the lowest CORESET ID and the first QCL1 is applied as default QCL/beam for PDSCH1 and PDSCH2 (until the applicable duration expires). Following QCL1, QCL2 is applied as default QCL for PDSCH3. The timeDurationForQCL is greater than the scheduling offset for all three PDSCHs, therefore, default QCL is applied to all three of the PDSCHs.

Figure 2: Example of multiple default beams switching

Proposal 4: For NR operation between 52.6 GHz and 71 GHz with high subcarrier spacing values such as 480kHz and 960kHz, specify enhancements to support multiple default beams association for multiple PDSCHs scheduled by single DCI:

- PDCCH CORESET can be associated with multiple QCL assumptions (beams) that can be used to determine multiple default beams based on lowest CORESET ID

- Duration/applicability for each of the default beams can also be associated to allow UE to determine when to switch from one default beam to another during the duration of multiple PDSCH transmission

## Beam update for consecutive CG and DG PUSCH transmissions

One more issue that can happen is when a UE may need to transmit a set of consecutive PUSCH transmissions including both dynamically scheduled PUSCH transmissions and CG-PUSCH transmissions, and the CG-PUSCH transmissions and dynamically scheduled PUSCH transmissions may be indicated to use different UL Tx beams. For example, as shown in Figure 3, a UE is scheduled to transmit a PUSCH transmission by an UL grant following a CG-PUSCH transmission. In this case, beam 0 is indicated to the UE by higher layer signaling for CG-PUSCH transmission and then, beam 1 is indicated to the UE by UL grant for dynamically scheduled PUSCH transmission. There may not be sufficient gap for switching UL Tx beam from beam 0 to beam 1, especially when higher SCS values such as 480kHz and 960kHz are considered. Therefore, before transmitting the set of consecutive PUSCH transmissions, it’s better for the UE to determine one common UL Tx beam to transmit all the consecutive UL CG and DG transmissions. In this example, the UE can use the beam 1 to transmit the two PUSCH transmissions because the beam 1 is indicated later and may be more suitable for the current channel condition. Above all, we propose that the UE can use the latest indicated UL Tx beam to transmit a set of consecutive UL transmissions including dynamically scheduled PUSCH transmissions and CG-PUSCH transmissions.

Proposal 5: For NR operation between 52.6 GHz and 71 GHz with high subcarrier spacing values such as 480kHz and 960kHz, if a UE is going to transmit a set of consecutive PUSCH transmissions including both dynamically scheduled PUSCH transmissions and CG-PUSCH transmissions, the UE can select the latest indicated UL Tx beam to transmit the consecutive UL CG and DG transmissions

Figure 3: Example of beam update for continuous CG and DG PUSCH transmissions

## Beam-management enhancements for periodic RS with LBT

Another aspect that has been discussed is how to handle periodic transmissions of beamformed RS such as P-TRS when there is LBT failure. In RAN1#104-e, following options have been discussed to handle this issue, but no agreement has been made:

- Termination of periodic RS transmission

- Aperiodic RS transmission to patch a non-transmitted periodic RS (e.g., TRS, CSI-RS, BFD-RS q0, and NBI-RS q1)

- Dynamic switching of QCL assumption of periodic RS

- Multiple RS transmission opportunities

- Multi-slot or multi-resource set RS transmission by a single DCI

- Note: Other enhancements are not precluded.

In our view, it makes sense to either terminate the periodic RS transmission on resources where continuous LBT failures are encountered. This will not require any additional effort in terms of additional dynamic signaling. A step further would be to allow for dynamic switching of the QCL assumption (beam) for periodic RS transmission (using same resources, but different beam) where continuous LBT failures are encountered. In order to support dynamic switching of QCL assumption of periodic RS without any additional dynamical solution, one solution could be to associate the periodic RS resource with multiple QCL assumptions (multiple beams) such that the beam switch can be triggered once the continuous number of LBT failures reach a certain threshold value.

Proposal 6: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, the following potential enhancements related to periodic transmissions of RS such as P-TRS should be specified to deal with LBT failure:

- Termination of periodic RS transmission on beams where consecutive LBT failures are encountered

- Dynamic switching of the QCL assumption (beams) for periodic RS transmission where consecutive LBT failures are encountered, where:

  - Multiple QCL assumptions (multiple beams) can be configured to the RS resource and beam switch can be triggered once the continuous number of LBT failures reach a certain threshold value

## Rel-17 TCI framework considerations

DCI based unified TCI framework has been agreed to be supported in Rel-17 MIMO, where a common TCI state shall be indicated by DCI format 1_1/1_2 for all PDCCH and PDSCH reception in the active BWP of the serving cell. It means that the TCI state indicated by one DCI format 1_1/1_2 shall not be directly applied to the scheduled PDSCH. The following agreement regarding to how to determine the application time for the indicated common TCI state was achieved in RAN1#105-e [2]:

Agreement

On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the first slot that is at least X ms or Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication.

Note: The gap between the last symbol of the beam indication DCI and that first slot shall satisfy the UE capability

Figure 4 illustrates an example for the common TCI state change. The UE receives DCI#0 using the current TCI state, while the DCI indicates a new TCI state different from the current TCI state and the ACK for DCI#0 is reported by a PUCCH transmitted in slot n+2. According to the agreement, the new indicated TCI state shall be applied from the first slot that is at least Y symbols after the last symbol of the PUCCH carrying the ACK for the DCI indicating the new TCI state. In the given example, the new indicated TCI state shall be applied for all PDCCH and PDSCH from slot n+6.

Figure 4: Example of common TCI state application (Rel-17 TCI framework)

For multi-PDSCH scheduling by single DCI, it has been agreed in RAN#104bis to support the indication of only single TCI state via DCI for single TRP scenario. It means that single TCI state is indicated for all the scheduled PDSCH for single TRP scenario. Based on this agreement and the support of common TCI indication according to Rel-17 TCI framework, it should be straightforward to support Rel-17 common TCI state indication with multi-PDSCH scheduling.

Proposal 7: For NR operation between 52.6 GHz and 71 GHz, Rel-17 common TCI state indication should be supported for multi-PDSCH scheduling

In unified TCI state framework, the indicated common TCI state shall be applied from the first slot that is at least Y symbols after the last symbol of the PUCCH carrying the ACK for the DCI indicating the new TCI state. An illustration with this constraint of beam application timing and multi-PDSCH scheduling is illustrated in Figure 5.

Figure 5: Example of common TCI state application for multi-PDSCH scheduling

Different from the scheduling case provided in Figure 4, PDSCH#1, PDSCH#2, PDSCH#3 and PDSCH#4 are scheduled by a same DCI in 4 continuous slots. The new common TCI state indicated by DCI#0 shall be applied to PDSCH#3 and PDSCH#4 if we go with the first agreement. It requires the UE has the capability to switch it RX beam within the CP of the 1st symbol of slot n+6, it may be challengeable for the UE with 960kHz SCS with normal CP, which corresponds to an 81.25ns CP length. It is noted that the worst-case beam switching time is estimated as < 100ns as described in TR38.817. If the required RX beam switching time is larger than the CP length, the new indicated common TCI state cannot be applied to PDSCH#3 and PDSCH#4 although they are received after the application time of the new indicated TCI state.

Observation 2: For NR operation between 52.6 GHz and 71 GHz, the new indicated common TCI state may not be applicable for the scheduled PDSCHs even the PDSCHs are received after the application time when the UE cannot switch it RX beams to the new indicated common TCI state between two continuous PDSCH transmissions

Based on this observation, we have the following proposal:

Proposal 8: For NR operation between 52.6 GHz and 71 GHz, how to determine the applied TCI state for the multiple PDSCH in continuous slots when the indicated common TCI state is changed but the UE cannot switch it RX beam within the CP should be further discussed

Another issue is how to determine the acknowledgment for the DCI indicating the common TCI state indication with multi-PDSCH scheduling. When the DCI indicating a new common TCI state and also scheduling a PDSCH, it has been agreed that the ACK/NACK of the scheduled PDSCH is used as the ACK for the DCI as the following agreement:

Agreement

On beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework:

- Support L1-based beam indication using at least UE-specific (unicast) DCI to indicate joint or separate DL/UL beam indication from the active TCI states

  - The existing DCI formats 1_1 and 1_2 are reused for beam indication

  - Support a mechanism for UE to acknowledge successful decoding of beam indication

    - The ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI

FFS: Whether any additional specification support is needed

If multiple PDSCHs are scheduled by the DCI indicating a new TCI state, and different TB(s) are carried by different PDSCHs, the ACK/NACK of any one scheduled PDSCH can be used as the ACK for the DCI.

Proposal 9: For NR operation between 52.6 GHz and 71 GHz, when multiple PDSCHs with different TBs are scheduled by the DCI indicating a common TCI state, the ACK/NACK of any one scheduled PDSCH can be used as the ACK for the DCI

# 3	Conclusion

Here we summarize the observations and proposals from the above sections:

Observation 1: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, for the agreed higher subcarrier spacings (numerologies) such as 960kHz, beam switching issue would appear between the contiguous transmissions (such as SSB beams) since the CP length would not be enough for beam switching, and an extra gap might be needed to prevent performance degradation

Observation 2: For NR operation between 52.6 GHz and 71 GHz, the new indicated common TCI state may not be applicable for the scheduled PDSCHs even the PDSCHs are received after the application time when the UE cannot switch it RX beams to the new indicated common TCI state between two continuous PDSCH transmissions

Proposal 1: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, if higher subcarrier spacings (numerologies) are adopted for SSB, then to allow the beam switching between contiguous SSBs, a gap (for example a symbol gap or post prefix) should be supported between contiguous SSB at least for 960kHz SCS

Proposal 2: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, when multiple PDSCHs are scheduled via single DCI to be received from two TRPs (not all PDSCHs received from one TRP), then indication of at least two TCI states should be supported

Proposal 3: For NR operation between 52.6 GHz and 71 GHz when multiple PDSCHs are scheduled via single DCI to be received from two TRPs (not all PDSCHs received from one TRP), then the duration for which each TCI state is valid should also be indicated

Proposal 4: For NR operation between 52.6 GHz and 71 GHz with high subcarrier spacing values such as 480kHz and 960kHz, specify enhancements to support multiple default beams association for multiple PDSCHs scheduled by single DCI:

- PDCCH CORESET can be associated with multiple QCL assumptions (beams) that can be used to determine multiple default beams based on lowest CORESET ID

- Duration/applicability for each of the default beams can also be associated to allow UE to determine when to switch from one default beam to another during the duration of multiple PDSCH transmission

Proposal 5: For NR operation between 52.6 GHz and 71 GHz with high subcarrier spacing values such as 480kHz and 960kHz, if a UE is going to transmit a set of consecutive PUSCH transmissions including both dynamically scheduled PUSCH transmissions and CG-PUSCH transmissions, the UE can select the latest indicated UL Tx beam to transmit the consecutive UL CG and DG transmissions

Proposal 6: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, the following potential enhancements related to periodic transmissions of RS such as P-TRS should be specified to deal with LBT failure:

- Termination of periodic RS transmission on beams where consecutive LBT failures are encountered

- Dynamic switching of the QCL assumption (beams) for periodic RS transmission where consecutive LBT failures are encountered, where:

  - Multiple QCL assumptions (multiple beams) can be configured to the RS resource and beam switch can be triggered once the continuous number of LBT failures reach a certain threshold value

Proposal 7: For NR operation between 52.6 GHz and 71 GHz, Rel-17 common TCI state indication should be supported for multi-PDSCH scheduling

Proposal 8: For NR operation between 52.6 GHz and 71 GHz, how to determine the applied TCI state for the multiple PDSCH in continuous slots when the indicated common TCI state is changed but the UE cannot switch it RX beam within the CP should be further discussed

Proposal 9: For NR operation between 52.6 GHz and 71 GHz, when multiple PDSCHs with different TBs are scheduled by the DCI indicating a common TCI state, the ACK/NACK of any one scheduled PDSCH can be used as the ACK for the DCI

# 4	References

[1]	3GPP RP-211584, “Revised WID:  Extending current NR operation to 71 GHz”, Qualcomm, Intel

[2] 	3GPP RAN1#105-e, “Chairman Notes”

[3] 	3GPP RAN1#104bis-e, “Chairman Notes”

[4] 	3GPP RAN1#104-e, “Chairman Notes”