3GPP TSG RAN WG1 #106-e	R1- 2106837

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.3.1.2

Source: 	Quectel, Langbo

Title: 	Discussion on CSI Feedback Enhancements

Document for:	Discussion and Decision

- Introduction

At RAN#92-e, the following guidance to RAN1 was endorsed:

- Provide the following RAN guidance on CSI feedback enhancement [RAN1]

  - Focus subsequent working group discussions on the schemes proposed in RP-211297.

    - Details are up to further working group discussions.

The schemes proposed in RP-211297 [1] are recapped below for convenience:

RAN1 to further investigate the following for CSI enhancements for IIoT/URLLC:

- Increasing the number of bits used for the reported subband CQI (3-bits differential subband CQI or 4-bits CQI)

- Reporting of delta-MCS:

  - Report consists of delta-MCS for a TB received with MCS index IMCS:

    - delta-MCS is calculated from the difference between IMCS_tgt and IMCS, where IMCS_tgt is the largest MCS index such that the estimated BLER for a TB received with this MCS index would be smaller than or equal to a BLER target, and IMCS is the MCS index of the received TB.

In this document, design details for these schemes are provided.

- Sub-band CQI

In the existing system, a sub-band CQI is derived differentially from the wide-band CQI so that overhead for CSI report can be saved. According to evaluations and analysis in previous meetings [2], finer granularity for sub-band CQI can improve the accuracy of sub-band CQI reports with minimal specification efforts. As per the guidance, 3-bit sub-band differential CQI or 4-bit sub-band full CQI can be considered to increase the sub-band CQI report accuracy. 4-bit sub-band full CQI can provide better accuracy than 3-bit sub-band differential CQI at the expense of larger overhead. Although the performance difference between 3-bit sub-band differential CQI and 4-bit sub-band full CQI would be marginal, such trade-offs can be left for gNB implementation. Moreover, the impact of interference could be hardly predicted, the configurability brings larger flexibility for gNB to resist the interference.

Proposal 1:The gNB configures a UE to report 2-bit sub-band differential CQI, 3-bit sub-band differential CQI or 4-bits sub-band full CQI.

For existing 2-bit sub-band differential CQI, when sub-band CQI reporting is configured, one CQI for each codeword is reported for each sub-band in a configured CSI reporting band. When more than one sub-band CQI reporting schemes (e.g., 2-bit sub-band differential CQI, 3-bit sub-band differential CQI or 4-bit sub-band full CQI) are supported, a scheme can be configured per sub-band or per subset of sub-bands. This could save CQI reporting overhead when interference could be somehow manageable in frequency domain, e.g., interference (or strong interference) could be constrained to certain sub-bands by inter-gNB coordination.

Proposal 2: A sub-band CQI reporting scheme (e.g., 2-bit sub-band differential CQI or 3-bit sub-band differential CQI) is configured per sub-band or per subset of sub-bands.

An existing 2-bit sub-band differential CQI is mapped to an offset level which is the offset (or offset range) between the sub-band CQI index and the wideband CQI index according to the following table.

Table 1: Mapping 2-bits sub-band differential CQI value to offset level

| Sub-band differential CQI value | Offset level |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | ≥ 2 |
| 3 | ≤-1 |

The mapping table could be simply expanded to 3-bit sub-band differential CQI case. The offset levels mapped to respective differential CQI value could be determined based on a certain distribution of CQI absolute values. However, as interference strength and interference appearance (in both time and frequency domain) may not follow any deterministic distribution and hence not predictable, the impact of interference to CQI distribution should be taken into account for the offset level setting.

Proposal 3: The impact of interference is considered in offset level setting for 3-bit sub-band differential CQI.

- Delta-MCS

The delta-MCS is mainly targeted to assist the gNB to acquire more information for a PDSCH transmission (or retransmission) in addition to the ACK/NACK feedbacks, which may facilitate the link adaption to converge to the optimal operating point faster than conventional ACK/NACK based OLLA. This kind of delta-MCS information can be considered as some kind of soft information besides HARQ-ACK feedback. In theory, this kind of soft information could be helpful for gNB scheduler. However, the successful decoding or failed decoding may be resulted by different reasons (e.g. beam blockage, frequency selection fading, interference spike, channel fading hole, etc), so in practical deployments it may not be efficient to always report the delta-MCS especially considering the extra overhead.

Proposal 4: Triggering or enabling/disabling mechanisms for reporting of delta-MCS needs be defined.

Furthermore, it also needs to be discussed whether the delta-MCS is reported for each PDSCH transmission when delta-MCS report is enabled/triggered. In general, delta-MCS enabling/disabling per HARQ process or even per PDSCH transmission could provide larger flexibility, which would be beneficial in case performance of a PDSCH transmission is dominated by frequency selection fading and interference spike. However, this will quite complicate the UCI multiplexing design or HARQ-ACK codebook design. And also, interference could be hardly predicted, which will largely offset the potential gain expected by a more flexible configuration.

Proposal 5: Delta-MCS is enabled/disabled in a UE dedicated manner.

One other open issue for delta-MCS is whether the delta-MCS is regarded as a part of CSI reporting or a part of HARQ-ACK feedback. If delta-MCS is transmitted as a new type of CSI, current channel coding designs, rate matching designs and multiplexing/resource mapping designs may need to be revisited. In addition, aperiodic CSI on PUCCH may have to be introduced to timely report the delta-MCS. The aperiodic CSI on PUCCH is likely to further complicate the whole system, for example, priority assigned to the aperiodic CSI needs to be considered for intra-UE dropping and/or multiplexing designs. Considering the limited time left for this WI and the complications for new CSI type, it would be a more sensible choice to transmit the delta-MCS as a part of HARQ-ACK feedback.

Proposal 6: Delta-MCS reports are integrated into HARQ-ACK codebook.

- A-CSI triggering on PUCCH

The support of aperiodic CSI triggering on PUCCH have been mentioned in a number of contributions and no progress or conclusion was made due to divergent views. This is not a new topic and the pros and cons are quite clear. As delta-MCS is expected to be transmitted as a part of HARQ-ACK feedback and due to lack of time, the support of A-CSI reporting on PUCCH may not be strongly motivated at least in Rel-17.

Proposal 7: Aperiodic CSI reporting on PUCCH is not supported in Rel-17.

- Conclusion

In this contribution, we discuss design aspects for CSI feedback enhancements based on the guidance from RAN and a number of proposals are made in the following:

Proposal 1:The gNB configures a UE to report 2-bit sub-band differential CQI, 3-bit sub-band differential CQI or 4-bits sub-band full CQI.

Proposal 2: A sub-band CQI reporting scheme (e.g., 2-bit sub-band differential CQI or 3-bit sub-band differential CQI) is configured per sub-band or per subset of sub-bands.

Proposal 3: The impact of interference is considered in offset level setting for 3-bit sub-band differential CQI.

Proposal 4: Triggering or enabling/disabling mechanisms for reporting of delta-MCS needs be defined.

Proposal 5: Delta-MCS is enabled/disabled in a UE dedicated manner.

Proposal 6: Delta-MCS reports are integrated into HARQ-ACK codebook.

Proposal 7: Aperiodic CSI reporting on PUCCH is not supported in Rel-17.

- References

- RP-211297, Way forward on CSI feedback enhancements for enhanced URLLC/IIoT, InterDigital, Inc., Ericsson, Motorola Mobility, OPPO, Qualcomm, Samsung, SONY, Spreadtrum.

- R1-2106177, Feature lead summary #4 on CSI feedback enhancements for enhanced URLLC/IIoT, Moderator (InterDigital, Inc.).