3GPP TSG RAN WG1 #106-e	R1-2106858

e-Meeting, August 16th –27th, 2021

Agenda item:	7.2.4

Source: 	Samsung

Title: 	Discussion on interpretation of frequency resource indication value for NR sidelink

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss on the calculation of FRIV.

Discussion

How to determine TRIV and FRIV is specified in TS38.214 as below.

- 8.1.5 UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields 'Frequency resource assignment', 'Time resource assignment' of the associated SCI format 1-A as described below. 'Time resource assignment' carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl_MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows: if N=1 TRIV=0 elseif N=2 TRIV=t1 else if t2-t1-1≤15 TRIV=30t2-t1-1+t1+31 else TRIV=3031-t2+t1+62-t1 end if end if where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31. The starting sub-channel nsubCH,0start of the first resource is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels for each of the N resources LsubCH≥1 and the starting sub-channel indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where. If sl-MaxNumPerReserve is 2 then FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i If sl-MaxNumPerReserve is 3 then FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2 where - nsubCH,1start denotes the starting sub-channel index for the second resource - nsubCH,2start denotes the starting sub-channel index for the third resource - N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter sl-NumSubchannel If TRIV indicates N < sl-MaxNumPerReserve, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used. ** omitted **

As highlighted above as yellow, FRIV is calculated by using the length of the allocated sub-channels and the starting indexes of the sub-channel.

If sl-MaxNumPerReserve is 2 and N is 1, i.e., the actual number of allocated resource is 1, nsubCH,1start is not used. If sl-MaxNumPerReserve is 3 and N is 1, i.e., the actual number of allocated resource is 1, nsubCH,1start and nsubCH,2start are not used. If sl-MaxNumPerReserve is 3 and N is 2, i.e., the actual number of allocated resource is 2, nsubCH,1start is not used.

But, it is not clear how to calculate FRIV when sl-MaxNumPerReserve is 2 and N is 1, or when sl-MaxNumPerReserve is 3 and N is less than 3. There are two approached for those cases.

- Approach 1. nsubCH,1start  and/or nsubCH,2start are set to zero, if N < sl-MaxNumPerReserve

- Approach 2. A transmitter can choose any possible value for nsubCH,1start  and/or nsubCH,2start, if N < sl-MaxNumPerReserve

It can be considered that the intention was Approach 1 to set nsubCH,1start  and/or nsubCH,2start to zero, if N < sl-MaxNumPerReserve.

Proposal 1. The starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are assumed zero if TRIV indicates N < sl-MaxNumPerReserve.

Proposal 2. The text proposal in [1] is adopted.

Conclusions

In this contribution, we discuss how to determine FRIV with starting indexes.

Proposal 1. The starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are assumed zero if TRIV indicates N < sl-MaxNumPerReserve.

Proposal 2. The text proposal in [1] is adopted.

- References

[1] R1-2106859, “Draft CR on interpretation of frequency resource indication value for NR sidelink,” RAN1#106-e, Aug. 2021