3GPP TSG RAN WG1 #106-e	R1-2106920

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.15.1

Source: 	Samsung

Title: 	On enhancements to time and frequency synchronization

Document for:	Discussion and Decision

# Introduction

In RAN1 #105-e meeting, many agreements were achieved on time and frequency synchronization for IOT over NTN. But, there are still some items for further study or discussion, e.g., whether to support common TA indication, whether to support TA reporting, details of segmented UE pre-compensation, and etc. This paper will discuss related issues and provide our proposals based on the discussion.

# Discussion

In the WID description [1], transparent payload is identified as an applicable scenario as showed as in Figure 1. In that case, the full TA includes common TA (2*Delay_0, i.e., roundtrip delay of feeder link) and UE specific TA (e.g., 2*Dealy_1 for UE-A, 2*Delay_2 for UE-B, i.e., roundtrip delay of service link). If TA estimation is based on timestamp indicated by network and UE’s local time derived via GNSS implementation, the estimated roundtrip delay between UE and network can include UE specific roundtrip delay of service link and common roundtrip delay of feeder link. If TA estimation is based on Satellite’s position derived by Satellite ephemeris indication and UE’s position derived via GNSS implementation, only UE specific roundtrip delay of service link and UE can be estimated, and the common roundtrip delay of feeder link cannot be estimated. If full TA needs to be compensated by UE to align UL and DL frame timing at eNB side, common TA should be indicated by eNB to cover the roundtrip delay of feeder link. In addition, the common TA drift, which is determined by relative velocity between Satellite and Gateway, can also be indicated to UE to update the common TA.

Proposal 1: Common TA should be indicated to cover the roundtrip delay of feeder link at least for position based TA estimation.

Figure 1    A scenario with transparent payload

In the RAN1#104-e meeting, one agreement on Koffset was achieved as “Update of K_offset after initial access is supported” for NR NTN. This means UE-specific Koffset are supported for NR NTN. Thus, for IOT over NTN, UE-specific Koffset should be further studied.

If UE-specific Koffset is supported to reduce scheduling delay, TA reporting should be supported to assist gNB to configure UE-specific Koffset. At gNB side, UE-specific Koffset should be derived based on UE’s full TA, i.e., combination of UE specific TA and cell common TA. So, UE’s estimated TA, i.e., UE specific TA should be reported to gNB. UE power consumption caused by TA reporting is an issue especially for IOT device. Actually, TA estimation and reporting may not be frequently needed during RRC connected mode, wherein closed loop TA command signaling can maintain TA and make the same understanding of TA value between UE and gNB, except that UL synchronization is lost.  Therefore, TA estimation and reporting can be only needed during initial access, RRC connection setup and UL synchronization loss case. Thus, the overall UE power consumption on TA reporting can be acceptable.

Proposal 2: Reporting of UE’s estimated TA should be supported at least during initial access.

In the last RAN1 meeting, segmented UE pre-compensation per N time units for long PUSCH and long PRACH was agreed, and one FFS is “Value of N for the number of time units and what is the time unit for the segmented UE pre-compensation”. The segmented UE pre-compensation includes UL timing pre-compensation and UL frequency timing pre-compensation. For PUSCH, the time unit should be one slot to avoid transmission broken during one slot. For PRACH, the time unit can be symbol group. Regarding the value of N, i.e., the number of time units, it can be configured by network, e.g., configured in system information or via UE specific RRC signaling. In theory, the value of N should be dependent on the maximum tolerated transmission timing/frequency error and timing/frequency drift rate. If timing/frequency drift rate and maximum tolerated transmission timing/frequency error are configured by network, UE and eNB can have the same understanding on these parameters. Thus, the value of N can be derived based on these parameters.

Proposal 3: For segmented UE pre-compensation per N time units, the value of N is configured by network or determined according to timing/frequency drift.

In addition, considering maximum tolerated transmission timing error and maximum tolerated transmission frequency error are different, and timing drift rate and frequency drift rate are also different, the value of N should be different for segmented UL timing pre-compensation and segmented UL frequency pre-compensation. For example, the value of N can be separately configured by network for segmented UL timing pre-compensation and segmented UL frequency pre-compensation.

Proposal 4: For segmented UE pre-compensation per N time units, the value of N can be different for UL timing pre-compensation and UL frequency pre-compensation, e.g., separately configured by network.

For segmented UE timing pre-compensation, signal may be overlapped between different TA segments. To handle this issue, one solution is to introduce a UL gap to avoid signal dropping. However, this will cause resource waste since at least one symbol is needed for gap reservation. Considering the overlapped range of signal is not large, signal dropping is acceptable, e.g., the last signal is dropped.

Proposal 5: For segmented UE timing pre-compensation, if signal is overlapped between different TA segments, the last one is dropped.

# Conclusions

In this contribution, we discussed the enhancements on UL time and frequency synchronization. Our proposals are summarized below:

Proposal 1: Common TA should be indicated to cover the roundtrip delay between Satellite and Gateway at least for position based TA estimation.

Proposal 2: Reporting of UE’s estimated TA should be supported at least during initial access.

Proposal 3: For segmented UE pre-compensation per N time units, the value of N is configured by network or determined according to delay/frequency drift.

Proposal 4: For segmented UE pre-compensation per N time units, the value of N can be different for UL timing pre-compensation and UL frequency pre-compensation, e.g., separately configured by network.

Proposal 5: For segmented UE timing pre-compensation, if signal is overlapped between different TA segments, the last one is dropped.

# References

[1] RP-193235, New study WID on NB-IoT/eMTC support for NTN, MediaTek Inc.