3GPP TSG RAN WG1 #106-e                                            R1-2106978

e-Meeting, August 16th – 27th, 2021

Source:	CATT

Title:	Discussion on reduced number of Rx branches

Agenda Item:	8.6.1.2

Document for:	Discussion and Decision

Introduction

In RAN#90-e, a new Rel-17 WI on support of reduced capability NR devices, i.e. RedCap, was approved [1]. The latest WID is revised in RAN#92-e [2]. In RAN1#105-e, the following agreements and conclusion were achieved on the reduced number of Rx branches [3]:

Agreement: - Redcap UE is mandated to support at least DCI format 0_0/1_0. Agreement: Regarding DCI format 0_1/1_1 and DCI format 0_2 and 1_2, - DCI format 0_1/1_1 are mandatory as in legacy. DCI 0_2/1_2 are optionally supported. Agreement: For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework. - Detailed signalling is up to RAN2 Conclusion - No consensus to support early identification of the number of Rx branches in Msg1/Msg3/MsgA for Redcap UE in Rel-17

In this contribution, we provide our views on the remaining aspects related to the reduced number of Rx branches, based on the agreements so far.

Discussion

On modification of DCI fields

It was agreed that DCI format 0_0/1_0/0_1/1_1 are mandatory for RedCap UE. The RedCap UE can optionally support DCI format 0_2/1_2 as well. Due to the reduced number of Rx branches, the number of the DL MIMO layers is also reduced to 2 or 1. Several companies proposed to modify some DCI fields to reflect this change. Our views on the modification of DCI fields are listed as follows:

- DCI format 0_0/1_0

DCI format 0_0 and 1_0 are fallback DCIs which serve as basic scheduling tools, and have already been designed with relative small payload size. In principle, the fields of existing DCI formats 0_0/1_0 should not be modified. Specifically, there is no Antenna Port(s) field in DCI format 1_0.

- DCI format 0_1/1_1/0_2/1_2

For DCI formats 0_1/1_1 and DCI formats 0_2/1_2, additional fields are included based on the RRC configuration. However, these DCI formats are also flexible enough to accommodate the need for RedCap UE. For example, if carrier aggregation is not configured, the number of bits of Carrier indicator field is 0 automatically. For another example, the length of frequency domain resource allocation (FDRA) field will be reduced if a smaller bandwidth is configured for a BWP. Especially for DCI formats 0_2/1_2, most of the bit widths of the fields are variable according to the detailed configuration.

Some companies may propose to reduce the bitwidth of Antenna Port(s) field. Currently, for DCI format 1_1, the bitwidth of Antenna Port(s) field can be 4/5/6 bits. Similarly, for DCI format 1_2, the bitwidth of Antenna Ports can be 0 bit or 4/5/6 bits. Ideally, for 1 Rx UE, the bitwidth to indicate DMRS may be reduced compared to the case of 4/5/6 bits. However, the bit reduction may not be huge. For example:

  - For a RedCap UE with 2 Rx branches, 4 bits are required to indicate 10 different states of the ports, i.e. {0, 1, 01} for 1 CDM group without data and port {0, 1, 2, 3, 01, 23, 02} for 2 CDM groups without data. Hence there is no bit reduction compared to the current Antenna Port(s) field.

  - For a RedCap UE with 1 Rx branch, it may still need 3 bits to indicate 6 different states of the ports, i.e. port {0, 1} for 1 CDM group without data and port {0, 1, 2, 3} for 2 CDM groups without data. There is only 1 bit reduction compared to the current Antenna Port(s) field (assuming 4 bits).

So, for the case of 4/5/6 bits Antenna Port(s) field in DCI format 1_1/1_2, the coverage improvement is marginal with potential bit reduction for RedCap UEs. It is not worthy to bring additional complexity to the standard, at least for Rel-17. We have the following proposal:

Proposal 1: Modification on fields of existing DCI formats for reduced number of Rx branches is not considered in Rel-17.

On PDCCH blocking issue

Several solutions were proposed to tackle the potential PDCCH blocking issue in RAN1#104bis-e. However, there was no consensus whether there will be serious PDCCH blocking issue based on the previous study during the SI phase [4]. According to the current WID, there is no requirement to introduce solution for PDCCH blocking issue [2].

Note that PDCCH blocking rate is highly related to simulation assumptions and scheduling scheme. As a result, companies agreed to utilize current DCI formats (specifically compact DCI format 0_2/1_2) to alleviate the potential PDCCH blocking issue in RAN1#104bis-e [5].

Agreements: - Reuse at least the existing DCI formats 0_x/1_x (including Rel-16 DCI format 0_2/1_2) applicable to Redcap devices as a starting point. - FFS Whether and how potential modification on fields of existing DCI formats is considered to reduce PDCCH block issue, if any. - FFS: Which DCI formats are mandatory for the RedCap UEs to support.

Other than compact DCI, many existing methods can be reused to reduce the blocking rate. For example, the gNB can configure non-overlapped CORESET/USS to normal UE and RedCap UE within the same BWP. For another example, different BWPs can be configured to normal UE and RedCap UE in RRC_CONNECTED state. We do not see strong need to pursue standardized solutions to reduced PDCCH blocking issues in Rel-17.

Proposal 2: No new standardized method is pursued to reduce PDCCH blocking issues in Rel-17.

Conclusion

In this contribution, we provide our view on the remaining aspects related to the reduced number of Rx branches. The proposals are summarized as follows:

Proposal 1: Modification on fields of existing DCI formats for reduced number of Rx branches is not considered in Rel-17.

Proposal 2: No new standardized method is pursued to reduce PDCCH blocking issues in Rel-17.

Reference

- RP-202933, New WID on support of reduced capability NR devices, Ericsson, RAN#90-e, e-Meeting, December 7th – 11th, 2020.

- RP-211574, Revised WID on support of reduced capability NR devices, Ericsson, RAN#92-e, e-Meeting, June 14th – 18th, 2021.

- Chairman's Notes RAN1#105-e, 3GPP TSG RAN WG1 Meeting #105-e, e-Meeting, May 10th – 27th, 2021.

- 3GPP TR 38.875, Study on support of reduced capability NR devices, v2.0.0, March, 2021.

- Chairman's Notes RAN1#104bis-e, 3GPP TSG RAN WG1 Meeting #104bis-e, e-Meeting, April 12th – 20th, 2021.