3GPP TSG RAN WG1 #106-e                                            R1-2106988

e-Meeting, August 16th – 27th, 2021

Source:	CATT

Title:	Discussion on enhancements on PUSCH repetition type A

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

Introduction

Rel-17 NR aims to improve the coverage of UL channels, where the latest WID was agreed in RAN#92-e [1]. For PUSCH, it was agreed that both directions for repetition type A (i.e., increasing the maximum repetition number, and counting the number of repetitions on the basis of available UL slots) are to be specified.

- Specification of PUSCH enhancements [RAN1, RAN4] - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1] - Increasing the maximum number of repetitions up to a number to be determined during the course of the work. - The number of repetitions counted on the basis of available UL slots.

In this contribution, we provide our views on PUSCH repetition type A enhancements for both directions.

Discussion

Increasing the maximum number of repetitions

In RAN1#105-e meeting, the following agreement on the maximum number of repetitions and the configurable number set were reached [2]:

Agreement: In addition to {1, 2, 3, 4, 7, 8, 12, 16} and {32}, the following additional value set for repetition factor is supported in Rel-17. - {20, 24, 28}

From the above agreement, the maximum number of repetition and the configurable value set are both clear. However, which RRC parameters are to be extended for supporting the increased maximum number of repetitions remains unclear. In RAN1#104-e meeting, it was agreed to support the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index, indicated either by the configured grant configuration or by TDRA field in a DCI. To be more specific, the IE numberOfRepetitions in the TDRA entry supports the increased numbers of repetitions [3]. But whether to increase the number of repetitions with repetition factor configured in PUSCH-Config or ConfiguredGrantConfig is still FFS.

In current NR, if the UE is indicated/configured with TDRA entry with numberOfRepetitions, the repetition number K follows numberOfRepetitions, while the repetition factor configured in PUSCH-Config (i.e. pusch-AggregationFactor) or ConfiguredGrantConfig (i.e. repK) will be ignored:

Table 1 Repetition of DG-PUSCH and CG-PUSCH in Rel-16

| Case | Description in TS 38.214 [3] |
| --- | --- |
| DG-PUSCH | For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberOfRepetitions-r16 is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions-r16; - elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor; - otherwise K=1. |
| CG-PUSCH | For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberOfRepetitions-r16 is present in the table; otherwise K is provided by the higher layer configured parameters repK. |

Considering that numberOfRepetitions has higher priority in repetition number determination, extending the configurable number of numberOfRepetitions seems sufficient for coverage enhancement. The gNB can always configure and indicate a large value in numberOfRepetitions in TDRA entry to compensate the deep coverage loss, regardless of the pusch-AggregationFactor and repK. Therefore, good coverage compensation and flexibility can be achieved.

Observation 1: Increasing the number of repetitions in numberOfRepetitions already achieves good coverage compensation and flexibility.

Note that, pusch-AggregationFactor and repK can be regarded as the ‘default’ repetition number, if configured. In case a large value is configured in pusch-AggregationFactor or repK, if numberOfRepetitions is not presented in the TDRA entry, the UE will have to transmit PUSCH with large number of repetitions. This may cause waste of UL resource, since the CE-capable UE may not be a static UE. In addition, even if the maximum repetition number of pusch-AggregationFactor and repK are extended, it does not provide further coverage enhancement on the basis of the already agreed numberOfRepetitions with extended values of repetition factor. This is due to the fact that the maximum repetition number is always 32, regardless of indication by numberOfRepetitions, pusch-AggregationFactor or repK.

Hence, we suggest not increasing the repetition number in pusch-AggregationFactor or repK. It brings no further benefit for coverage.

Proposal 1: Do not increase the number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Repetition number counted on the basis of available UL slots

In RAN1#105-e meeting, the following agreements on the repetition number counted on the basis of available UL slots were reached [2]:

Agreement: - RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17 Agreement: - Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. - RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. Agreement: - If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot. Agreement: Select one from the following (further refinement of the alternatives can be further discussed), for the procedure of Rel-17 PUSCH repetition Type A (other alternatives are not precluded) - Alt 1-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - Alt 1-B’ consisting of two steps - Step 1: Determine K repetitions based on available slots, where the available slot is the UL slot and flexible slot indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated. - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability - Alt 2-A consisting of a single step - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic signaling (e.g. SFI, UL CI, DCI for high priority channel) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Alt 2-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic SFI in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - FFS timeline for the dynamic signalling - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Procedure of available slot determination

For the procedure of available slot determination, a total number of four alternatives were agreed for further down-selection, as listed above. Our views on these alternatives are summarized as follows:

- Alt 1-B: For Alt 1-B, the determination of available UL slots is based on semi-static RRC parameters only, which guarantees the alignment of available slot determination between the gNB and UE. Even if dynamic signaling (e.g. SFI/CI) is supported, the total candidate slots for PUSCH repetitions still depends on reliable RRC parameters and will not be changed due to successfully reception of SFI/CI. When misdetection of SFI/CI happens, the misalignment of transmission/omission of PUSCH repetitions between gNB and UE is limited within the slots/symbols indicated by SFI/CI only.

- Alt 1-B’: Alt 1-B’ is similar to Alt 1-B. The main difference is that Alt 1-B’ only considers tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated as the RRC parameters to determine the available slot. Other RRC parameters or dynamic indication only take part in the omitting/dropping procedure. However, as agreed in the last meeting, at least SSB should also be considered during the available slot determination procedure.

- Alt 2-A & Alt 2-B: For both Alt 2-A and Alt 2-B, the determination of available UL slots will be based on not only semi-static RRC parameters but also dynamic signaling. The determination based on dynamic indication, however, is equivalent to enabling dynamic postponing. Hence, if misdetection happens, the misalignment of PUSCH repetitions will spread to the subsequent slot(s), leading to serious waste of resource and error propagation. Figure 1 illustrates this case when a SFI/CI is taking part in the determination of available slot. It can be seen that error propagation occurs since the happening of misdetection of SFI/CI. Note that the PUSCH repetitions have different RV versions and cannot replace each other.

Figure 1 Available UL slots determined by RRC parameters and dynamic signals.

Based on the above analysis, we propose to take Alt 1-B for the procedure of Rel-17 PUSCH repetition Type A.

Proposal 2: Support Alt 1-B for the procedure of Rel-17 PUSCH repetition Type A, which consists of the following two steps:

- Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI.

- Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

RRC parameters for available slot determination

For determination of available slot, it was agreed that at least SSB should be considered in addition to tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. But it is FFS whether other RRC parameters can also be considered.

We should pay more attention on the RRC parameter candidates. Generally, the more RRC parameters in consideration, the more slots become unavailable. In principle, only cell-specific channels/signals which will be certainly transmitted can be considered in the determination of available slots. Therefore, SSB, tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated should be enough.

It may be arguable whether Type0-PDCCH CSS should also be considered. In our view, there is no need to consider any CSS, since it is up to the gNB to transmit PDCCH in the search space, i.e. not an always-on signal/channel. This is well reflected in TS 38.213 [5], where we can see that Type0-PDCCH CSS does not directly determine whether the PUSCH can be transmitted or not, but just not expect to be configured as uplink.

For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

Current PUCCH repetition mechanism also provides a good example, as copied below from TS 38.213[5]. We can find that when determining the available slots for PUCCH repetition, only SSB is taken into consideration to determine whether flexible symbol(s) can be considered as available or not. It does not consider Type0-PDCCH CSS set, either.

For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in Clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in Clause 9.2.4 for SR reporting or in Clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having - an UL symbol, as described in Clause 11.1, or flexible symbol that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and - consecutive UL symbols, as described in Clause 11.1, or flexible symbols that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4

We think it is beneficial to have a unified determination of available slots for PUSCH and PUCCH, which helps to simplify the specification and implementation. Hence, we propose:

Proposal 3: The determination of available slots for PUSCH repetition only depends on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and SSB symbols.

Combination of the two features

In this section, we discuss the combination of increased maximum repetition number and counting repetition number based on available slots. The following agreement was reached in RAN1#105-e meeting [2]:

Agreement: - Down-selection in RAN1#106-e: - Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method, - Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

In Rel-15 and/or Rel-16, slot-level repetition is supported for both PUSCH and PUCCH. For PUSCH, slot-level repetition (a.k.a. repetition type A) is counted based on continuous physical UL slots. For PUCCH, slot-level repetition is counted based on the available UL slots. Anyway, for each channel, the same RRC parameters are applied to both TDD and FDD systems. For instance, the maximum number of repetitions and the configurable set of repetition numbers of PUSCH are the same for both TDD and FDD systems. Thus it can be seen that for PUSCH repetition type A, a unified design for TDD and FDD is applied throughout Rel-15 and Rel-16, which can lighten the normative work and simplify the device/chip development.

It may be argued that, a maximum number of 32 will not be needed for TDD with counting based on available slots. However, there is no harm to the network, either. Note that the actual repetition number is still under the control of gNB. Even if the maximum number of 32 is supported, the gNB can configure an actual repetition number less than 32 to a UE, if desired. For comparison, we would like to remind that low complexity UEs such as LTE-MTC or NB-IoT UEs support a repetition number far larger than 32, e.g. 1024. There should not be serious complexity issue for a UE to support a repetition number of 32.

It is suggested to achieve unified design on the maximum number of Rel-17 PUSCH repetition type A, regardless of available slot counting method or duplex mode. The related aspects include: increased maximum repetition number, configurable set of repetition number, etc.

Proposal 4: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method.

Conclusion

In this contribution, we provide our view on mechanisms for enhancements on PUSCH repetition type A. The observation and proposals are summarized as follows:

Observation 1: Increasing the number of repetitions in numberOfRepetitions already achieves good coverage compensation and flexibility.

Proposal 1: Do not increase the number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Proposal 2: Support Alt 1-B for the procedure of Rel-17 PUSCH repetition Type A, which consists of the following two steps:

- Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI.

- Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Proposal 3: The determination of available slots for PUSCH repetition only depends on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and SSB symbols.

Proposal 4: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method.

Reference

- RP-211566, Revised WID on NR coverage enhancements, China Telecom, RAN#92-e, e-Meeting, June 14th – 18th, 2021.

- Chairman's Notes RAN1#105-e, 3GPP TSG RAN WG1 Meeting #105-e, e-Meeting, May 10th – 27th, 2021.

- Chairman's Notes RAN1#104-e, 3GPP TSG RAN WG1 Meeting #104-e, e-Meeting, January 25th – February 5th, 2021.

- 3GPP TS 38.214, Physical layer procedures for data, v16.6.0, June, 2021.

- 3GPP TS 38.213, Physical layer procedures for control, v16.6.0, June, 2021.