3GPP TSG RAN WG1 #106-e	R1-2107001

e-Meeting, 16th - 27th August, 2021

Title:	Discussion on the PDCCH monitoring enhancements for 52.6 to 71 GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.2

Document for:	Discussion and Decision

Introduction

According to the outcome of the study item on supporting NR above 52.6 GHz [1], in 3GPP RAN #91 plenary e-meeting in March 2021, an update WID RP-210862 [2] was approved to extend NR operation up to 71 GHz considering both licensed and unlicensed operation. In the WID, some objectives related to PDCCH monitoring enhancements were given and are excerpted as follows.

- Physical layer aspects including [RAN1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

  - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

In the RAN1 #104bis e-meeting in April 2021, the following agreements related to PDCCH monitoring were reached.

Agreement:

Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability

- Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability.

  - Each slot group consists of X slots

  - Slot groups are consecutive and non-overlapping

  - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately

  - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X

  - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g. the Y [symbols or slots] always start at the first slot within a slot group

  - FFS: Further definition of capabilities

- Alt 2: Use an (X, Y) span as the baseline to define the new capability

  - X is the minimum time separation between the start of two consecutive spans

  - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots]

  - Y <= X

  - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary.

  - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots.

  - FFS: Further definition of capabilities

- Alt 3: Use a sliding window of X slots as the baseline to define the new capability.

  - The capability indicates the BD/CCE budget within the sliding window

  - The sliding unit of the sliding window is [1] slot.

  - FFS: Further definition of capabilities

- Specific numbers for X, Y may depend on UE capability and gNB configuration

  - Examples:

    - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS

Conclusion:

For 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed.

Agreement:

For 120 kHz SCS in 52.6-71 GHz, the BD/CCE budget is the same as that for 120 kHz in FR2.

In this contribution, we give our views on PDCCH monitoring enhancements for NR between 52.6 GHz ~ 71 GHz.

Discussion on PDCCH monitoring enhancements

  - 2.1 	PDCCH Blind detection

In Rel-15/16 NR, the maximum number of non-overlapped CCEs per slot for a DL BWP with SCS 15~120 kHz is given by Table 2.1-1. The maximum number of monitored PDCCH candidates per slot for a UE in a DL BWP with SCS 15~120 kHz is given by Table 2.1-2. As shown in two Tables, the maximum number of non-overlapped CCEs and monitored PDCCH candidates per slot decreases with increasing SCS, respectively.

Table 2.1-1: Maximum number  of non-overlapped CCEs per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of non-overlapped CCEs per slot and per serving cell |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

Table 2.1-2: Maximum number MPDCCHmax,slot,μ of monitored PDCCH candidates per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of monitored PDCCH candidates per slot and per serving cell MPDCCHmax,slot,μ |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

As given in Table 2.1-1, for SCS 120 kHz (i.e. μ = 3), there are only 32 non-overlapped CCEs per slot. For NR operation above 52.6 GHz, this number can be further expected to decrease with increasing value of SCSs. According to this decreasing trend,  for SCS 960 kHz is likely to be smaller than 16, which is even less than the number of CCEs needed for a PDCCH with CCE aggregation level of 16. In this case, the highest level i.e. CCE aggregation level 16 in NR becomes impossible. It will obviously affect the detection performance of PDCCH. Even for PDCCH SCS = 960 kHz, assuming that  is greater than or equal to 16, it can be expected that the PDCCH monitoring capability will be further reduced and the number of PDCCH candidates per slot could be quite small, which will impact the scheduling for multiple UEs.

Observation 1: For supporting NR operation in 52.6-71 GHz with 480/960 kHz SCS, the PDCCH monitoring capability will be further relaxed, the maximum number of non-overlapped CCEs and PDCCH candidates per slot could be further limited, and the use of the highest CCE aggregation level may be affected.

For the newly introduced SCS i.e. 480/960 kHz in Rel-17 NR above 52.6 GHz, the length of the slot becomes shorter. If a UE detects PDCCH in each slot, PDCCH monitoring will be too frequent, which will not only increase the power consumption of the UE, but also increase the UE detection complexity and impact UE capability. Moreover, if we continue to reduce the maximum number of non-overlapped CCEs and PDCCH candidates per slot for above higher SCSs along the downward trend shown in the Table 2.1-1 and Table 2.1-2, the flexibility of scheduling will be affected and the PDCCH load will be too heavy resulting in blocking. There are several ways discussed and summarized in RAN1#104bis e-meeting to solve the above problems:

- Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability

In Alt 1, each slot group with a fixed pattern consists of X slots and slot groups are consecutive and non-overlapping. The capability indicates the BD/CCE budget within Y consecutive symbols or slots in each slot group separately. If parameter X and Y are properly restricted, we can not only avoid back-to-back issue, but also enable gNB flexibility to configure CSS/USS.

The values of X should be configurable for more flexible operation in above 52.6 GHz band, that depend on UE capability. Firstly, X= 4 slots for 480 kHz and X= 8 slots for 960 kHz to align the absolute time of a slot with 120 kHz SCS with the existing PDCCH monitoring capability should be supported. In addition, smaller numbers of slots of X can be investigated. For example, X=2 slots for 480 kHz and X=2 or 4 slots for 960 kHz can also be considered.

For whether to restrict that PDCCH monitoring occasion (MO) can only be configured in a subset of slots/symbols (i.e. Y consecutive slots/symbols) with fixed positions in a slot group with X slots, in principle, we think a gNB should be allowed to configure PDCCH MO in any slots/symbols in the slot groups, that is, Y can be equal to X slots. The location and number of PDCCH MO slots/symbols can be decided by the gNB configuration. But considering the potential back-to-back monitoring issues, it may be best to set Y≦X/2 and the location of PDCCH monitoring can be restricted to always be on the first Y slot(s) within the slot group, i.e. Y=1 slot, as shown in Figure 1. The capability indicates the BD/CCE budget within the first Y slot(s) in each slot group. Considering larger Y value results in better gNB flexibility but higher UE complexity, we propose that Y should be a configurable value.

Although the number of BD/CCE shall be strictly limited for per slot PDCCH monitoring, it still has many usage scenarios, such as in low latency traffic and flexible scheduling case. Per-slot PDCCH monitoring can be considered as a special case of multi-slot PDCCH monitoring, thus UE capability for per slot PDCCH monitoring (X = 1) should not be precluded.

As Alt 1 is the simplest and has the lowest standardization complexity, so it could be the basis for defining multi-slot PDCCH monitoring capability.

Figure 1: A fixed pattern of slot groups

- Alt 2：Use an (X, Y) span as the baseline to define the new capability

Alt 2 uses the Rel-16 capability (pdcch-Monitoring-r16, (X, Y) span) as the baseline to define the new capability for multi-slot PDCCH monitoring, wherein X is the minimum time separation between the start of two consecutive spans.

The main difference between Alt 2 and Alt 1 is that: Alt 1 uses a fixed pattern of slot groups each with X slots. If the X slots consisting a slot group in Alt 1 are regarded as a window, then the length of different windows is constant, and the different windows are consecutive and non-overlapping. However, for Alt 2, the different windows are still non-overlapping, but may not be consecutive if the X slots in Alt 2 are regarded as a window too. The reason is that X is the minimum span separation and the actual time span separation is not constant, as shown in Figure 2.

As the same as Alt 1, X=4 slots separation for 480kHz and X=8 slots separation for 960kHz should be supported to align the absolute time with per slot UE capability for PDCCH monitoring with 120 kHz. In order to acquire lower latency performance and sufficient scheduling flexibility, the values of X equal to {1, 2} for 480kHz SCS and {1, 2, 4} for 960kHz SCS can also be considered. Y can also be equal to X but it may result in back to back slot configuration, thus the values of Y slots can be restricted. The capability indicates the BD/CCE budget within a span of at most Y consecutive slots. Y should be equal to or less than X/2 for multi-slot PDCCH monitoring condition.

Compared with Alt 1, Alt 2 is more complex. It is not easy to determine the actual span separations, the length of spans, repeated span pattern, as well as crossing slot boundaries. Therefore, we think that Alt 2 could be de-prioritized.

Figure 2: Multi-slot span

- Alt 3：Use a sliding window of X slots as the baseline to define the new capability

Alt 3 with a sliding window mainly solves the problem that the number of BD/CCE in back-to-back slot configuration may exceed UE capability of PDCCH monitoring. It requires that the maximum number of BD/CCE in each sliding window should not exceed UE capability. In Alt 3, the different windows are sliding and can overlap. The basic unit of a sliding window can be one slot. Alt 3 ensures that the BD/CCE budget will not be exceeded over any X slots duration on the UE side.

Alt 3 makes PDCCH monitoring more complex, especially in PDCCH monitoring dropping and power efficiency. Furthermore, back-to-back issue can be avoided by proper configuration in Alt 1 and Alt 2. Thus we don’t see there is a need to support a sliding window in Alt 3.

Proposal 1: Alt 1 using a fixed pattern of slot groups is preferred as the baseline to define the new capability for PDCCH monitoring with the new SCSs 480/960 kHz:

- Each slot group consists of X slots

- X={1, 2, 4} slots for 480 kHz SCS

- X= {1, 2, 4, 8} slots for 960 kHz SCS

- The capability indicates the BD/CCE budget within Y consecutive slots in each slot group

- For multi-slot PDCCH monitoring, Y ≦ X/2 and is always the first Y slot(s) within each slot group

- The locations of the PDCCH monitoring symbols should not be restricted

  - 2.2 	Search space set configuration

If a fixed pattern of slot groups to define the new capability for PDCCH monitoring is adopted, when configuring the search space set by higher layer parameter monitoringSlotPeriodicityAndOffset, the gNB needs to ensure that PDCCH monitoring periodicity ks, the duration TS is an integral multiple of X slots (X slots consists a slot group), or is an integral multiple of slot groups, i.e. ks and TS are in the units of slot group. For example, if a slot group includes four slots (X=4), the duration TS can be configured as 4, 8, 12, 16, ... of slots. Alternatively, the duration TS can be configured as 1, 2, 3, 4, ... of slot groups, i.e. the basic granularity of the duration TS should be defined as a slot group. Figure 3 gives one configuration type in a slot group, e.g., PDCCH MO is configured in the first slot within the slot group.

Figure 3: Configurations if a fixed pattern of slot groups is supported

Multiple PDSCH/PUSCH scheduling with a single DCI being discussed in agenda item 8.2.5 can not only save DCI overhead, but also reduce PDCCH monitoring frequency without sacrificing scheduling flexibility. Therefore, the design of the new UE capability for PDCCH monitoring, search space set configuration can be considered in combination with multiple PDSCH/PUSCH scheduling by a single DCI.

Proposal 2: PDCCH monitoring periodicity ks and the duration TS of the search space sets should be configured as an integral multiple of a slot group, if a fixed pattern of slot groups to define the new capability for PDCCH monitoring is supported.

  - 2.3  Search space set group switching

Search Space Set Group (SSSG) Switching was introduced in Rel-16 NR-U. In order to reduce access delay and save power, the SSSG monitoring periodicity/granularity is increased or decreased according to the channel access procedure. UE has to support frequent PDCCH monitoring outside gNB-COT and infrequent PDCCH monitoring inside COT. The existing SSSG switching is applied for SCS 15/30/60 kHz For SSSG switching on large SCS values like 120, 480 and 960 kHz, UE needs to dynamically change between two PDCCH monitoring periodicity/granularity. SSS switching operation is also related to whether per slot PDCCH monitoring is supported. Besides, the switching time Pswitch  with all SCSs supported in above 52.6 GHz needs to be defined if SSSG switching is agreed to be supported.

Proposal 3: Support SSSG switching for SCS 120/480/960 kHz, and the following points can be further studied:

- SSSG switching between multi-slot and per-slot monitoring if per-slot monitoring is supported for 480/960 kHz

- The switching time Pswitch  with all SCSs supported in above 52.6 GHz

  - 2.4 	Cross-carrier scheduling

The cross-carrier scheduling here mainly refers to cross-carrier scheduling of a cell within 52.6-71GHz band from/to a cell within FR1/FR2. Although a cell within 52.6-71GHz band cross-carrier schedules other cells within FR1/FR2 is less likely, it should not be ruled out unless we find enough proof. Similarly, we do not think that we can rule out the case of a cell with low SCS (e.g. 15kHz) cross-carrier schedules other cells with high SCS (e.g. 480kHz) in this phase before sufficient research.

Another problems related to cross-carrier scheduling are minimum PDSCH scheduling delay and minimum A-CSI- RS trigger delay. In Rel-15/16 NR, cross-carrier scheduling only supports four cases of PDCCH with u = 0, 1, 2 and 3, as given in Table 5.5-1 and Table 5.2.1.5.1a in TS 38.214. The 120kHz SCS in above 52.6GHz band can reuse the value of u = 3. But the values of µPDCCH with 480/960kHz SCS needs to be determined. The same values of µPDCCH for minimum PDSCH scheduling delay and minimum A-CSI- RS trigger delay can be used for 480/960kHz SCS.

TS 38.214 Table 5.5-1: Npdsch as a function of the subcarrier spacing of the scheduling PDCCH

| µPDCCH | Npdsch [symbols] |
| --- | --- |
| 0 | 4 |
| 1 | 5 |
| 2 | 10 |
| 3 | 14 |

TS 38.214 Table 5.2.1.5.1a: Ncsirs as a function of the subcarrier spacing of the triggering PDCCH

| µPDCCH | Ncsirs [symbols] |
| --- | --- |
| 0 | 4 |
| 1 | 5 |
| 2 | 10 |
| 3 | [14] |

Proposal 4: The values of µPDCCH with 480/960kHz SCS for minimum PDSCH scheduling delay and minimum A-CSI- RS trigger delay should be determined.

Conclusion

In this contribution, we discuss PDCCH monitoring enhancements for NR above 52.6 GHz band operation and have the following observations and proposals.

Observation 1: For supporting NR operation in 52.6-71 GHz with 480/960 kHz SCS, the PDCCH monitoring capability will be further relaxed, the maximum number of non-overlapped CCEs and PDCCH candidates per slot could be further limited, and the use of the highest CCE aggregation level may be affected.

Proposal 1: Alt 1 using a fixed pattern of slot groups is preferred as the baseline to define the new capability for PDCCH monitoring with the new SCSs 480/960 kHz:

- Each slot group consists of X slots

- X={1, 2, 4} slots for 480 kHz SCS

- X= {1, 2, 4, 8} slots for 960 kHz SCS

- The capability indicates the BD/CCE budget within Y consecutive slots in each slot group

- For multi-slot PDCCH monitoring, Y ≦ X/2 and is always the first Y slot(s) within each slot group

- The locations of the PDCCH monitoring symbols should not be restricted

Proposal 2: PDCCH monitoring periodicity ks and the duration TS of the search space sets should be configured as an integral multiple of a slot group, if a fixed pattern of slot groups to define the new capability for PDCCH monitoring is supported.

Proposal 3: Support SSSG switching for SCS 120/480/960 kHz, and the following points can be further studied:

- SSSG switching between multi-slot and per-slot monitoring if per-slot monitoring is supported for 480/960 kHz

- The switching time Pswitch  with all SCSs supported in above 52.6 GHz

Proposal 4: The values of µPDCCH with 480/960kHz SCS for minimum PDSCH scheduling delay and minimum A-CSI- RS trigger delay should be determined.

Reference

- RP-193259, New SID: Study on supporting NR from 52.6GHz to 71GHz. Intel, Sitges, Spain, RAN#86, December 9 – 12, 2019

- RP-210862, Revised WID: Extending current NR operation to 71 GHz, CMCC, 3GPP TSG RAN Meeting #91-e, 16 - 26 March 2021

- RAN1 #104-bis-e chairman’s notes

- R1-2104057, Feature lead summary #2 for [104b-e-NR-52-71GHz-02] Email discussion/approval on PDCCH Monitoring Enhancements