3GPP TSG RAN WG1 #106-e	R1-2107028

e-Meeting, August 16th – 27th, 2021

Agenda Item:	7.2.4

Source: 	Fujitsu

Title:	Maintenance for physical layer procedures for NR sidelink

Document for:	Discussion/Decision

Introduction

In this contribution, we share some views on the remaining issue on the prioritization between PUSCH carrying SL HARQ-ACK and the SL transmission.

Prioritization between PUSCH carrying SL HARQ-ACK and SL transmission

The prioritization rule for SL and UL has been captured in TS 38.213 Clause 16.2.4.3 “simultaneous SL and UL transmissions ”. However, one missing part is the prioritization related to PUSCH carrying SL HARQ-ACK. In the following, we share some discussions on that. As for PUSCH carrying SL HARQ-ACK, it actually carries both SL HARQ-ACK and UL-SCH since SL HARQ-ACK is piggybacked on PUSCH.

Figure 1. An example showing simultaneous PUSCH carrying SL HARQ-ACK and SL transmission

An example is shown in Figure 1 where PUSCH carrying SL HARQ-ACK and a SL transmission overlap in the time domain. The target is to determine which one is prioritized. In this case, it can still be treated as if the prioritization is done between SL and UL. In this case, SL includes both SL HARQ-ACK and the SL transmission, and the priority of SL is the highest priority between SL HARQ-ACK and the SL transmission. If SL is prioritized over UL, SL with the highest priority (PUSCH carrying SL HARQ-ACK or SL transmission) is prioritized. Otherwise, UL-SCH (PUSCH carrying SL HARQ-ACK) is prioritized. The aforementioned prioritization rule is also equivalent to the following expression.

- If the SL transmission is prioritized over both SL HARQ-ACK and UL-SCH, the SL transmission is prioritized.

- Otherwise, PUSCH carrying SL HARQ-ACK is prioritized.

Proposal 1: The prioritization rule between PUSCH carrying SL HARQ-ACK and a SL transmission is defined as follows.

- If the SL transmission is prioritized over both SL HARQ-ACK and UL-SCH, the SL transmission is prioritized.

- Otherwise, PUSCH carrying SL HARQ-ACK is prioritized.

--------------------------------- Text Proposal for Clause 16.2.4.3 of TS 38.213------------------------------

16.2.4.3	Simultaneous SL and UL transmissions

*** Unchanged text omitted ***

If a UE

-	is capable of simultaneous transmissions on the UL and on the SL in two respective carriers,

-	would transmit on the UL and on the SL in the two respective carriers,

-	the transmission on the UL would overlap with the transmission on the SL over a time period, and

-	the total UE transmission power over the time period would exceed PCMAX

the UE

-	reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the UL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX

-	reduces the power for the SL transmission prior to the start of the SL transmission, if the UL transmission has higher priority than the SL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX

If a UE

-	is capable of simultaneous transmissions on the UL and on the SL of two respective carriers of a serving cell, or of two respective serving cells,

-	would transmit PUSCH with the SL HARQ-ACK and the UL-SCH on the UL and transmit on the SL of the two respective carriers of the serving cell, or of the two respective serving cells,

-	the transmission on the UL would overlap with the transmission on the SL over a time period, and

-	the total UE transmission power over the time period would exceed PCMAX

the UE

-	reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the SL HARQ-ACK and the UL-SCH as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX

-	reduces the power for the SL transmission prior to the start of the SL transmission otherwise, so that the total UE transmission power would not exceed PCMAX

16.2.4.3.1	Prioritizations for sidelink and uplink transmissions/receptions

*** Unchanged text omitted ***

------------------------------------------------ End Text Proposal --------------------------------------------------

Conclusions

In this contribution, we share some views on the remaining issue on the prioritization rule related to PUSCH carrying SL HARQ-ACK. The proposal is highlighted as follows. The text proposals are given in Section 4.

Proposal 1: The prioritization rule between PUSCH carrying SL HARQ-ACK and a SL transmission is defined as follows.

- If the SL transmission is prioritized over both SL HARQ-ACK and UL-SCH, the SL transmission is prioritized.

- Otherwise, PUSCH carrying SL HARQ-ACK is prioritized.

Text proposals

The text proposals are provided below where the new paragraphs marked with the red color are added to Clause 16.2.4.3 of TS 38.213.

--------------------------------- Text Proposal for Clause 16.2.4.3 of TS 38.213------------------------------

16.2.4.3	Simultaneous SL and UL transmissions

*** Unchanged text omitted ***

If a UE

-	is capable of simultaneous transmissions on the UL and on the SL in two respective carriers,

-	would transmit on the UL and on the SL in the two respective carriers,

-	the transmission on the UL would overlap with the transmission on the SL over a time period, and

-	the total UE transmission power over the time period would exceed PCMAX

the UE

-	reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the UL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX

-	reduces the power for the SL transmission prior to the start of the SL transmission, if the UL transmission has higher priority than the SL transmission as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX

If a UE

-	is capable of simultaneous transmissions on the UL and on the SL of two respective carriers of a serving cell, or of two respective serving cells,

-	would transmit PUSCH with the SL HARQ-ACK and the UL-SCH on the UL and transmit on the SL of the two respective carriers of the serving cell, or of the two respective serving cells,

-	the transmission on the UL would overlap with the transmission on the SL over a time period, and

-	the total UE transmission power over the time period would exceed PCMAX

the UE

-	reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the SL HARQ-ACK and the UL-SCH as determined in Clause 16.2.4.3.1, so that the total UE transmission power would not exceed PCMAX

-	reduces the power for the SL transmission prior to the start of the SL transmission otherwise, so that the total UE transmission power would not exceed PCMAX

16.2.4.3.1	Prioritizations for sidelink and uplink transmissions/receptions

*** Unchanged text omitted ***

------------------------------------------------ End Text Proposal --------------------------------------------------