3GPP TSG-RAN WG1 Meeting #106-e	Tdoc R1-2107049

e-Meeting, 16th – 27th August, 2021

Agenda Item:	7.2.2

Source:	Ericsson

Title:	Correction related to wideband operation

Document for:	Discussion, Decision

# 1	Introduction

This contribution contains a correction related to wideband operation under the Rel-16 NR-U maintenance agenda item. Two alternatives are discussed, and RAN1 should discuss and decide which one to adopt.

# 2	Correction Related to Wideband Operation

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap

-	if a CORESET is not associated with any search space set configured with freqMonitorLocations, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-Offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-Offset.

-	if a CORESET is associated with at least one search space set configured with freqMonitorLocations, the first NRBG,set0size  bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in each RB set k in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position  RBs0+k,DLstart,μ  [6, TS 38.214], where the first common RB of the first group of 6 PRBs has common RB index RBs0+k,DLstart,μ+NRBoffset and k is indicated by freqMonitorLocations if provided for a search space set; otherwise, k=0. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-Offset or NRBoffset=0 if rb-Offset is not provided. If a UE is provided RB sets in the DL BWP, the UE expects

In 38.213 Section 10.1, there is a procedure for the UE to determine the frequency domain resources of a CORESET, and the procedure depends on whether or not the Rel-16 RRC parameter rb-Offset is provided to the UE. The relevant text is as follows:

The intention of the yellow highlighted text is that for either a Rel-15 UE or for a Rel-16 UE operating in licensed spectrum, the frequency domain resources of the CORESET should be configured in the legacy (Rel-15) manner, i.e., always aligned with the boundaries of a 6-CRB grid. Note that for such a UE operating in licensed spectrum, no search space set is configured with freqMonitorLocations.

The yellow text is supposed to be executed for the case when rb-Offset is not provided. However, according to 38.331, this condition never happens for a Rel-16 UE since the physical layer is always provided with a value for rb-Offset due to the following field description in 38.331:

rb-Offset Indicates the RB level offset in units of RB from the first RB of the first 6RB group to the first RB of BWP (see 38.213 [13], clause 10.1). When the field is absent, the UE applies the value 0.

The field description in 38.331 states that when the field is absent – meaning when the UE is not signaled rb-Offset from higher layers – the UE should apply the value of 0 for rb-Offset. In other words, for a Rel-16 UE, the physical layer is always provided a value for rb-Offset. According to the text from 38.213 above, this means that the yellow text is never executed, and instead, the cyan text is executed. The consequence is that the frequency domain resources of the CORESET are aligned with the CRB corresponding to the start of the BWP rather than the boundaries of the 6-CRB grid as in Rel-15.  This is counter to previous RAN1 agreements (see 1st paragraph of the relevant agreement in Appendix A), and is not backwards compatible, thus unnecessarily complicating gNB and UE implementations.

A simple RAN1-based fix to this problem is given by the draft CR shown in Appendix B. The fix ensures that the yellow text is executed in case rb-Offset is NOT signaled from higher layers. With this fix, the UE will ignore the default value of 0 for rb-Offset since the yellow text does not depend on any value of rb-Offset.

## 2.1	Alternative Correction

An alternative correction is to make no change to RAN1 specifications, and instead request that RAN2 makes the following change to the field description of the rb-Offset parameter within the ControlResourceSet IE.

rb-Offset Indicates the RB level offset in units of RB from the first RB of the first 6RB group to the first RB of BWP (see 38.213 [13], clause 10.1). When the field is absent, the UE applies the value 0.

In this way, if the parameter rb-Offset is absent (not configured), the yellow text in 38.213 is still executed as desired, since from a physical layer perspective, the UE is not provided with rb-Offset. Furthermore, in the 2nd paragraph of 38.213 text shown above, the default value is assigned to rb-Offset according to the green text. This solution has the benefit that duplication of the default value assignment is avoided between 38.331 and 38.213. We point out that this fix is also consistent with the RAN1 agreement in Appendix A (2nd paragraph).

RAN1 to down-select to one of the following alternatives:

- Alt-A (RAN1 Fix): Agree on draft CR to 38.213 Section 10.1 shown in Appendix B

- Alt-B (RAN2 Fix): No change to 38.13. Request RAN2 to make the above change to the field description of the parameter rb-Offset in the ControlResourceSet IE.

# Appendix A – Relevant RAN1 Agreement

RAN1 agreed on the following in RAN1#100-e:

Agreement:

- If CORESET p is not configured with rb-offset, and is not associated with any search space set configured withfreqMonitorLocation-r16,

  - The bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart, where the first common RB of the first group of 6 consecutive RBs has common RB index 6∙NBWPstart6, i.e., same as in Rel-15.

- If CORESET p is not configured with rb-offset, and is associated with at least one search space set configured with freqMonitorLocation-r16,

  - The bits of the first A bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset, where the UE assumes the default value rb-Offset = 0.

- If CORESET p is configured with rb-offset, and is not associated with any search space set configured withfreqMonitorLocation-r16,

  - The bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset.

- If CORESET p is configured with rb-offset, and is associated with at least one search space set configured with freqMonitorLocation-r16,

  - The bits of the first A bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset.

- Note: A bits in above bullets is defined as floor({the number of available PRBs in the first RB set (accounting for rb-Offset) for the BWP}/6), as per previous agreement.

- TS 38.213 editor to implement this agreement

# Appendix B – Draft CR for Alt-A (RAN1 Fix)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CORESET configuration when UE is not signaled rb-Offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | TSG RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If a Rel-16 UE operates in licensed spectrum, no search space set is configured with freqMonitorLocations. Additionally, if the parameter rb-Offset is not signaled by higher layers, the UE assumes a default value of 0 according to 38.331; in other words, a value of rb-Offset is always provided to the physical layer. In this case, the frequency domain resources of a CORESET associated with the search space set(s) cannot be aligned with the boundaries of the 6-CRB grid as in Rel-15 unless the BWP also starts on the 6-CRB grid. This is counter to previous RAN1 agreements, incurs significant reduction in gNB flexibility, and is not backwards compatible to Rel-15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the wording "if rb-Offset is not provided" to "if rb-Offset is not signaled by higher layers" to ensure that legacy (Rel-15) CORESET configuration is achievable for a Rel-16 UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Frequency domain resources of a CORESET for a Rel-16 UE cannot be aligned to the 6-CRB grid as in Rel-15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text omitted ***

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap

-	if a CORESET is not associated with any search space set configured with freqMonitorLocations, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-Offset is not signaled by higher layers; otherwise, the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-Offset.

-	if a CORESET is associated with at least one search space set configured with freqMonitorLocations, the first NRBG,set0size  bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in each RB set k in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position  RBs0+k,DLstart,μ  [6, TS 38.214], where the first common RB of the first group of 6 PRBs has common RB index RBs0+k,DLstart,μ+NRBoffset and k is indicated by freqMonitorLocations if provided for a search space set; otherwise, k=0. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-Offset or NRBoffset=0 if rb-Offset is not provided. If a UE is provided RB sets in the DL BWP, the UE expects that the RBs of the CORESET are within the union of the PRBs in the RB sets of the DL BWP.

*** Unchanged text omitted ***