3GPP TSG RAN WG1 Meeting #106-e	R1-2107086

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.13.2

Source:	Futurewei

Title:	Support efficient activation/de-activation mechanism for SCells

Document for:	Discussion/Decision

# Introduction

In the past few 3GPP RAN1 meetings, agreements and working assumptions on efficient activation/de-activation mechanism for SCells were achieved, a few of which are listed below:

Agreements:

TRS is selected as temporary RS for Scell activation

·         If more functionalities are confirmed to be supported by temporary RS, other RS candidates, e.g. aperiodic CSI-RS, P/SP-CSI RS, SRS and RS based on SSS/PSS, are not precluded.

·         The TRS should be triggered by DCI or MAC-CE. FFS which exact triggering command.

Agreements:

As working assumption, with respect to efficient SCell activation, reuse existing Rel-15/16 TRS structure for temporary RS

- FFS: how many burst/symbols are required for both AGC settling and Time/Frequency tracking for different cases, e.g. FR1 and FR2, known and unknown SCell

  - A burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214

    - “2-slot with four CSI-RSs resources (4 samples)” for FR1

    - either “1-slot with two CSI-RSs resources (2 samples)” or “2-slot with four CSI-RSs resources (4 samples)” for FR2

- The working assumption can be confirmed after RAN4 check. (A LS for such request is planned).

Working Assumption

For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell

- FFS: QCL type

- FFS: the case of unknown SCell

- FFS: other QCL source, e.g. the SSB/P-TRS of another active cell

Agreement

For efficient activation of Scells, the triggered temporary RS is aperiodic.

Agreement

For efficient activation of a Scell (in known Scell case), at least the number of temporary RS bursts is indicated by a field in new MAC-CE

- The number of temporary RS bursts is RRC configurable.

- FFS: which field in MAC-CE is used and how this field is associated with the number of bursts

- For the purpose of designing temporary RS Scell activation, there is no RAN1 specification impact for the case where the number of indicated temporary RS bursts is smaller than what is expected by the UE

Agreement

To trigger temporary RS for efficient activation of SCells, the contents of the triggering MAC-CE(s) in a single PDSCH provide at least the following information (explicitly or implicitly):

- Whether or not temporary RS is triggered

- FFS detailed Information of temporary RS, e.g.:

  - Resources used for triggered Temporary RS

  - Triggering time offset of triggered Temporary RS

  - QCL source for triggered Temporary RS

- FFS: Detailed signaling structure of the triggering MAC-CE(s) including the down-selection between the following example options and whether the decision should be made in RAN1 or RAN2

  - Opt. 1.1: One new MAC CE for both SCell activation triggering and corresponding temporary RS triggering

  - Opt. 1.2: One R15/16 SCell activation MAC CE for SCell activation triggering and one new MAC CE (in the same PDSCH) for corresponding temporary RS triggering

Agreement

For efficient activation of a Scell (in known Scell case), the triggering offset of temporary RS is indicated by a field in new MAC-CE

- The candidate value(s) of triggering offset(s) is RRC configurable

- FFS: which field in MAC-CE is used and how this field is associated with the value of triggering offset

Agreement

For the reference slot for triggering offset of temporary RS

- Option 2: the last DL slot of the to-be-activated Scell overlapping with slot n+k as defined in 38.213 sub-clause 4.3

- FFS: the earliest slot no earlier than the reference slot for a UE to receive a triggered temporary RS

Agreement

If a UE measures a temporary RS triggered by a MAC-CE during SCell activation procedure, the measurement is performed within the BWP bandwidth of BWP indicated by firstActiveDownlinkBWP-Id

In this contribution, further discussions on these enhancements are provided.

# Discussion

## Scenario clarification

Based on previous discussions, there has been confusion on the applicable scenarios for SCell fast activation, such as known cell / unknown cell scenarios, FR1 / FR2 scenarios, etc. It would be useful to clarify the understanding of the scenarios and decide which scenarios are to be supported in Rel-17 and which are not. Otherwise, it may be difficult for companies having different scenarios in mind to agree on how to proceed.

### Known/unknown SCell

Whether a cell is known or unknown is specified by RAN4 in TS 38.133. For example, some related excerpts are provided below:

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5 for intra-frequency handover and Clause 9.3.4 for inter-frequency handover. … In FR2, the target cell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the handover command: - the UE has sent a valid measurement report for the target cell and - One of the SSBs measured from the NR target cell being configured remains detectable according to the cell identification conditions specified in clause 9.3, - One of the SSBs measured from the target cell also remains detectable during the handover delay according to the cell identification conditions specified in clause 9.3. otherwise it is unknown. …

There seems to be many different cases in which known/unknown cells are defined. Some companies wish to only cover known SCell cases, but some other companies do not want to be limited by that. Moreover, one major confusion in RAN1 is whether the gNB and UE have the same understanding of a to-be-activated SCell being known or not. Some companies believe the gNB and UE can have different understandings and hence it may not be meaningful to provide a design assuming a SCell is known or unknown.

In our view, regardless of whether there are different understandings or not, at least the following are certain: 1) If the UE has communicated with the SCell within the last x ms (e.g., x = 400), the SCell is definitively known, and the UE/gNB must have the same understanding; 2) If the UE has not communicated with the SCell within the last y ms (e.g., y = 2000), the SCell is definitively unknown, and the UE/gNB must have the same understanding. For these certain cases, RAN1 design can be provided accordingly. For the cases in between, i.e., the UE has communicated with the SCell between x and y ms and no longer since then, RAN1 can always provide a bit conservative design to ensure fast activation always works, e.g., by transmitting one AP SSB as part of a temporary RS. If this can be agreeable, then the known/unknown cell issue can be fully resolved in RAN1, but if not, RAN4 input seems necessary.

Proposal 1: Clarify RAN1 understanding on the definition / support of known / unknown SCell cases:

- Alt 1: Support all cases, with conservative design for cases in which the SCell has not been used for more than x ms, and FFS x; OR

- Alt 2: Send an LS to RAN4 to inquire whether the gNB and UE have the same understanding of a to-be-activated SCell being known or not.

### Scenarios related to RAN4 LS replies

The following table provides a summary of RAN4 LS replies on the needed number of TRS bursts for several cases. All other cases are still TBD in RAN4.

Table 1 Summary of RAN4 requirements on the number of 2-slot TRS bursts

There are still a few entries in the table that are unclear from RAN4 inputs. We suggest to prioritize the cases with complete answers from RAN4 for now, and work on other cases when RAN4 decisions are available.

Proposal 2: Prioritize the following cases before further decisions from RAN4 are made:

- FR1 known SCell

- FR1 unknown SCell with an activated intra-band contiguous cell

- FR2 known SCell with an activated intra-band cell

## Candidate temporary RS other than TRS

### Candidate temporary RS considerations for an unknown SCell

In addition to TRS which has been agreed as a temporary RS, the following candidate temporary RS may be considered for an unknown SCell.

- SSB

A to-be-activated SCell may be known to the UE or unknown to the UE. If it is known, there is no need for cell search / cell detection. If the SCell is unknown, however, then the SSB may be triggered and transmitted as a temporary RS for cell search / cell detection. According to RAN4 reply, in some cases, the cell detection is not needed, but for other cases when the cell detection is needed, SSB can be used as in legacy procedures, and aperiodic SSB which is transmitted on demand can reduce the latency compared with periodic SSB. The potential latency reduction becomes more significant if the periodic SSB’s periodicity is longer. Thus, aperiodic SSB should be considered as temporary RS for unknown cells.

For the aperiodic triggering of SSB, due to the potential timing uncertainty, the precise timing of the RS based SSS/PSS may not be known to the UE in some cases. Hence, a search time window for the RS based on SSS/PSS may be configured or signalled to the UE for the case of activation of an unknown SCell.

Aperiodic SSB may also be used for AGC and/or time-frequency tracking, in addition to cell detection, though the aperiodic SSB for AGC/time-frequency tracking may be substituted by AP TRS burst(s).

- Cross-carrier RS

A temporary RS may rely on a cross-carrier signal, e.g., if the to-be-activated SCell is unknown, the AP TRS on the SCell may be seen as QCLed with a cross-carrier SSB or a cross-carrier P/SP TRS. The cross-carrier SSB or P/SP TRS should be on an activated carrier which is generally an intra-band carrier (or even stronger, intra-band contiguous/adjacent carrier), and may be configured/received for the UE before the activation process starts. Based on network configuration and standard specification, the UE can assume some common properties across multiple serving cells, and thus a good amount of information about a to-be-activated SCell can be derived by the UE.

However, the UE cannot assume any common properties across serving cells without network assistance or standardized UE behavior (including UE capability reporting). Then Rel-17 can consider to specify network signaling to the UE about the side information and the UE assumptions with the side information. There are some approaches (not mutually exclusive) to support this, which are potential ways to specify cases/conditions utilizing intra-band (contiguous or non-contiguous) activated serving cell to assist SCell activation according to RAN4 LS.

- Configure/assume cross-carrier QCL properties

A RS sent by a cell may be configured/assumed to QCL with another RS sent by another cell. For example, the TRS on a to-be-activated SCell may be QCLed to the SSB on an activated serving cell, where the activated serving cell may be on a carrier that is intra-band and contiguous (adjacent) with the to-be-activated SCell for FR1, or intra-band with the to-be-activated SCell for FR2. Note that cross-carrier QCL was discussed in RAN1; see [1] for a summary.

To configure/assume cross-carrier QCL, association between the SSBs for the carriers may be considered. That is, SSB 1 in cell 1 is QCLed with SSB 2 in cell 2, or vice versa. The UE does not really need to utilize one SSB to receive the other SSB, but the QCL relation is to inform the UE that cross-carrier QCL can be assumed. With the SSBs associated with each other, the other signals QCLed to SSB can be assumed to have implied QCL relationships with the cross-carrier SSB.

Figure 1 Example of bi-directional cross-carrier QCL (relationships shown by dashed lines may be assumed and not need to be configured)

Figure 1 shows an example. In the example, on each carrier, legacy QCL relationships are configures as usual, e.g., TRS 1 is QCLed with Type A to SSB 1, and TRS 2 is QCLed with Type A to SSB 2. When cell 1 and cell 2 are in a band with intra-band CA, or are on carriers with intra-band contiguous CA, or have shared PA, etc., SSB 1 and SSB 2 may be configured / assumed with cross-carrier QCL. Then during the SCell activation, TRS 1 of the to-be-activated cell 1 may be assumed to QCL with Type C/D to SSB 2 of the activated cell 2, and vice versa.

The QCL between SSB 1 and SSB 2 should be interpreted as bi-directional, i.e., the source and target in the RS pair may be flipped. This may be defined as a new QCL type. For example if cell 1’s SSB is configured as QCL with cell 2’s SSB, since such a relationship is reciprocal between cell 1 and cell 2, it is not necessary to configure the reverse direction and it can be assumed by the UE.

- Introduce new QCL types for pathloss, RSRP, and frequency/time synchronization across carriers

The existing QCL types can be generalized to define that new properties as listed above may be assumed by the UE if signalled by the network. The QCL relationship may be signalled based on reference to cell index, SSB, or RS. If no additional signaling is to be adopted, then the relationship may be specified in standards for intra-band and contiguous carriers for FR1 and intra-band carriers for FR2.

- Introduce cell sets sharing common properties

More generally, the above described common properties are shared among multiple cells, such as intra-band adjacent cells (for FR1, for example), intra-band cells (for FR2), or cells in adjacent bands if they share the same PA/RF. Therefore, it may be useful to introduce cell sets with common properties. This may be done resembling TAGs, that is, the cells are configured into multiple TAGs, and the cells within the same TAG share the same TA.

- Introduce offset values

Even if some properties are not the same between two cells, there may be a fixed offset (or an offset bounded by an upper bound) between them and known to the network or can be derived by the UE. The network can signal the offset value to the UE. For example, if the symbol boundary of cell 1 is x ms ahead of that of cell 2, the x value can be signalled to the UE and the UE can apply it for acquiring rough timing. For another example, if the pathloss for cell 1 is y dB higher than that for cell 2, the y value can be signalled to the UE for estimating its initial AGC setting. Alternatively, the UE may derive the offset value, e.g., based on the separation of the cells.

Proposal 3: Support aperiodic triggering of SSB search time window for the case of unknown SCell.

Proposal 4: Support cross-carrier RS and cross-carrier properties (e.g., bi-directional cross-carrier QCL between SSBs) for efficient activation according to RAN4 LS inputs.

### Aperiodic CSI-RS, CSI-IM, and SRS as temporary RS

In addition to AP TRS which has been agreed as a temporary RS, the following candidate aperiodic RS can also be considered:

- Aperiodic CSI-RS for a to-be-activated SCell

CSI acquisition is a necessary component of SCell activation, and hence in LTE and NR it is always required that a valid CSI report is transmitted as part of the RAN4 SCell activation delay requirements. Reducing the CSI acquisition time can reduce the SCell activation delay. CSI acquisition may be done by DL CSI-RS (and CSI-IM resource; to be further discussed) and UE CSI reporting, or alternatively by UL SRS (to be further discussed). For DL based CSI acquisition, aperiodic CSI-RS and periodic/semi-persistent CSI-RS may be considered. However, periodic/semi-persistent CSI-RS can only be transmitted with predetermined periodicity and slot offset, which may not lead to activation delay reduction in general. On the other hand, aperiodic CSI-RS are more flexible and can be triggered right when the activation procedure starts. Therefore, aperiodic CSI-RS should be included as an additionally configurable temporary RS.

Aperiodic CSI-RS as temporary RS may not have to be triggered every time during a SCell activation. If a quick report of DL CSI is desirable by the network, then the network should be able to trigger an additionally configurable aperiodic CSI-RS, but otherwise the network can decide not to trigger it.

If one transmission of aperiodic CSI-RS is not enough, multiple aperiodic CSI-RS can be triggered (via one trigger if overhead reduction is considered). The total energy consumption associated with receiving the aperiodic CSI-RS is lower than periodic CSI-RS, and especially so if the deactivation duration is long and the periodicity is short. With a sufficient triggering offset guaranteed by MAC CE based CSI-RS triggering (with at least 3 ms timing offset and additional K0 slot offset), the UE does not have to be ready for receiving an aperiodic CSI-RS and generating an aperiodic CSI-RS report all the time. The triggering offset can allow enough time for the UE to respond to the aperiodic trigger.

- Aperiodic CSI-IM and aperiodic CSI

In order for the UE to generate a valid CSI report, CSI-IM resource is needed. Therefore, each AP CSI-RS may also be configured with an AP CSI-IM resource(s), and the AP CSI-IM resource(s) is also transmitted when the AP CSI-RS is transmitted. Therefore, the AP CSI-IM resource(s) is also viewed as a part of a temporary RS or temporary RS resource. In this sense, the L1 AP CSI-RS trigger discussed in RAN1 so far may be better interpreted as a L1 AP CSI trigger. An AP CSI trigger triggers a combination of an AP CSI-RS(s) and an AP CSI-IM resource(s), and a CQI and/or L1 SINR is to be reported according to the CSI-RS/CSI-IM. The timing relation and configurations for the AP CSI trigger, the AP CSI-RS, the AP CSI-IM, and the AP CSI report are generally the same as the conventional ones.

- Aperiodic SRS for a to-be-activated SCell

It is known that SRS can be useful for providing DL full MIMO CSI in TDD systems, UL CSI in TDD/FDD, maintaining UL TA, UL power control, and UL/DL beam. Therefore, SRS can be very important for a to-be-activated SCell to re-acquire CSI, TA, power control, and beam. In the case that the pathloss RS for the SRS is on an activated cell, the UE may be able to transmit the SRS on the to-be-activated SCell without waiting for the DL AGC to settle. In fact, a SCell may not be fully usable as a “fully activated” SCell until its UL is also ready, which can be achieved by SRS transmissions. (Note that, however, if the SCell activation is considered as completed only after a valid CQI report is sent, an early CSI acquisition via AP SRS still cannot shorten the activation delay.) In addition, utilizing SRS in activation can shift some of the processing from the UE to the network, thus reducing UE processing complexity. Regarding aperiodic SRS versus periodic SRS, similar to the CSI-RS analysis, aperiodic SRS is more suitable for SCell activation delay reduction. Hence, AP SRS should also be considered as an additionally configurable temporary RS.

Aperiodic SRS as temporary RS may not have to be configured / triggered for every to-be-activated SCell. If UL CSI for FDD / TDD, or DL CSI for TDD, or UL TA, or UL power control, or UL beam management is required by the network, then the network should be able to configure / trigger an additionally configurable aperiodic SRS, but otherwise the network can decide not to configure / trigger it.

Therefore, we suggest to add AP CSI-RS, AP CSI-IM, AP SRS, and AP SSB (discussed before) as optional temporary RS / RS resources. Note that the temporary RS may not be always needed for activation, and whether a specific temporary RS is supported/used or not depends on network configuration / UE capability. The only temporary RS that shall be always triggered during SCell activation is TRS.

Proposal 5: Support aperiodic CSI-RS as additionally configurable temporary RS, to be triggered if the SCell activation requires a valid DL CSI with reduced delay.

Proposal 6: Support AP CSI-IM as additionally configurable temporary RS resource if AP CSI-RS is supported as temporary RS.

Proposal 7: Support aperiodic SRS as additionally configurable temporary RS, to be triggered if the SCell activation requires a valid UL CSI or DL CSI for TDD or UL TA / PC / beam with reduced delay.

### On long-periodicity RS

Periodic RS may help maintain the connection between the UE and the deactivated SCell, which will in turn facilitate SCell activation. Whether periodic RS may be included as “temporary RS” can be further discussed, but (only) long-periodicity RS may be considered for a deactivated SCell to avoid frequent monitoring of the RS while avoiding the SCell to become unknown.

- Periodic CSI-RS / TRS for a deactivated SCell

In essence, periodic CSI-RS and TRS (or likewise, SP CSI-RS and TRS) act like LTE CRS and thus can shorten the activation latency. The pros include that their occurrences are fully predictable, which can help reduce PDCCH monitoring and PDCCH overhead, and simplify UE design. The cons include that it is difficult to set the periodicity: if it is too long then the latency reduction is not significant, but if it is too short then the overhead and energy consumption are high. To overcome the periodicity issue, long-periodicity CSI-RS / TRS may be used jointly with aperiodic CSI-RS / TRS for the best tradeoff between short latency and reduced UE power consumption and overhead.

Therefore, optional long-periodicity CSI-RS (also including long-periodicity TRS) of periodicity at least 100 TTIs may be configured for a deactivated SCell to reduce energy consumption while keeping the link alive. The P/SP TRS configured for the deactivated SCell can also serve as the source RS for the AP TRS during the activation process.

Note that when periodic CSI-RS is configured for CSI acquisition, periodic CSI-IM resources should also be configured.

Finally, when periodic CSI-RS / CSI-IM are configured for a deactivated SCell, the UE is required to monitor the CSI-RS / CSI-IM as configured.

- A combination of P/SP/AP CSI-RS for a to-be-activated SCell

This provides the network with most flexibility/capability.

Comparing CSI-RS with TRS, we note that CSI-RS is needed for CSI measurement, while TRS is for tracking. Since in most SCell activation, CSI measurement and reporting are needed, CSI-RS is still needed even when TRS is used as a temporary RS for activation. Thus, at least one of AP CSI-RS and P/SP CSI-RS should be supported for efficient SCell activation, and it seems a combination of long-periodicity P/SP CSI-RS and AP CSI-RS may be able to achieve the best tradeoff among fast activation and reduced UE power consumption.

- Periodic SRS for a deactivated SCell

Similar to the periodic / aperiodic CSI-RS / TRS described above, for the SRS as temporary RS, it is also preferred to combine long-periodicity SRS with aperiodic SRS.

Periodic SRS may not have to be configured for every deactivated SCell. If UL CSI for FDD / TDD, or DL CSI for TDD, or UL TA, or UL power control, or UL beam management is required by the network, then the network should be able to configure an optional periodic SRS, but otherwise the network can decide not to configure it.

- Periodic SSB for a deactivated SCell

Similar to the periodic / aperiodic CSI-RS / TRS / SRS described above, for SSB, it is also preferred to combine long-periodicity SSB with aperiodic SSB.

Proposal 8: Support long-periodicity RS (e.g., TRS / CSI-RS / CSI-IM / SRS / SSB) as additional RS / RS resources to facilitate efficient SCell activation.

## Triggering signaling

As seen above, different scenarios exist, and different functionalities are required. The scenarios include at least known SCell and unknown SCell, which implies that the activation procedure may or may not need SSB. In addition, for AGC settling, usually it needs TRS and/or CSI-RS. For time/frequency tracking, usually it needs TRS. And for CSI measurement/acquisition, usually it needs CSI-RS and CSI reporting, and/or SRS. However, to mandate all these RSs be transmitted and processed during the activation may not be realistic due to the high RS overhead and operation/processing complexity. This means that under difference circumstances, different RSs and different procedures should be adopted. Therefore, it is needed to provide a flexible framework for configuration and triggering temporary RS with reasonable overhead and complexity. A general L2 triggering signaling design and its associated triggering mechanism can provide such flexibility with low triggering overhead and complexity, which is described below.

SCell activation triggering (i.e., activation command) configuration and temporary RS configuration are supported. For a configured SCell, a TRS as the default temporary RS is always configured, and optionally an AP CSI-RS (and CSI-IM resource) is configured, and optionally an AP SRS is configured. These RSs will be useful during the SCell activation procedure. At least the TRS will be always transmitted during the SCell activation, and other RS may be optionally transmitted. Therefore, there are the following components and possible ways:

- Minimum standardized components for SCell activation:

  - Legacy MAC CE activation command to activate the SCell is supported.

  - One AP TRS is configured with the SCell, and it will be transmitted during the SCell activation, i.e., triggered by the same legacy MAC CE for activation.

- Additional components that can be additionally configurable and used for SCell activation:

  - One or more additional AP TRS may also be configured for the SCell. A new MAC CE may select an AP TRS to be triggered during the SCell activation (depending on the design, this MAC CE or another MAC CE initiates SCell activation).

  - One or more AP CSI-RS may be configured for the SCell. MAC CE may select an AP CSI-RS to be triggered during the SCell activation. One CSI-RS may be the default, so if the SCell activation starts without additional indication, the default AP CSI-RS is also sent.

  - One or more AP SRS may be configured for the SCell. MAC CE may select an AP SRS to be triggered during the SCell activation. One SRS may be the default, so if the SCell activation starts without additional indication, the default AP SRS is also sent.

Figure 2 illustrates the framework for SCell activation.

- The SCell supports the minimum standardized components and mechanism for SCell activation, including that legacy MAC activation command to activate the SCell is always supported, and a default AP TRS is always configured with the SCell, so it is always transmitted during the SCell activation, regardless of how the activation starts.

- Additional components that can be optionally configured and used for SCell activation include AP TRS L2 trigger, AP CSI-RS, AP CSI-RS L2 trigger, AP SRS, AP SRS L2 trigger.

  - If an AP TRS L2 trigger is received, in which the TRS selected by the TRS trigger may be different from the default TRS configured for the SCell, then the TRS selected by the TRS trigger is sent and the default TRS is not sent.

  - If an AP CSI-RS L2 trigger is received, then the selected AP CSI-RS will be sent during the SCell activation, and the default AP TRS will also be sent.

  - If an AP SRS L2 trigger is received, then the selected AP SRS will be sent during the SCell activation, and the default AP TRS will also be sent.

  - Similarly, if AP RS for SSS / PSS is supported, the above framework can also incorporate the AP RS for SSS / PSS and its associated trigger.

Figure 2 Illustration of SCell activation triggering commands and temporary RS. Green parts are the minimum components for the enhanced SCell activation, and red parts are additional components that can be optionally configured and used for the enhanced SCell activation

Regarding the details of L2 activation and triggering design, several possibilities may be considered:

- Alt 1: one legacy MAC CE for activation, which also triggers one pre-configured temporary RS

- Alt 2: one legacy MAC CE for activation + one new MAC CE for temporary RS triggering. The new MAC CE for temporary RS triggering may not be useful for scenarios other than SCell activation, so it may not be used without accompanying the SCell activation MAC CE. That is, the new MAC CE for temporary RS triggering should only be expected in a PDSCH also carrying the SCell activation MAC CE.

- Alt 3: one new joint MAC CE for activation and temporary RS triggering

- Alt 4: left to RAN2

We remark that MAC CE for temporary RS triggering is not the same as activation of some RS; triggering is a one-shot transmission of the RS but activation activates periodic / semi-persistent transmissions of the RS with pre-configured periodicity and slot offset.

We think at least Alt 1 should be supported, and further detail of new MAC CE design (including Alt 2 or Alt 3 details) may be left for RAN2 based on RAN1 input of functionalities. RAN1 may make recommendation of which parameters are mandatory/optional in a MAC CE. Below we analyze the parameters:

- Whether or not temporary RS is to be triggered, or which resources are to be used for triggered temporary RS

These two questions are related. If n different temporary RS are to be configured for a SCell, then log(n+1) bits are needed in the MAC CE to indicate which or none is triggered. However, if 0 temporary RS is configured, then temporary RS is not triggered (i.e., legacy activation procedure). Furthermore, if only 1 temporary RS is configured, or a default temporary RS is to be configured, then this field is optional in MAC CE if the new Rel-17 temporary RS based activation procedure is to be used, or one bit can be used to indicate whether the one temporary RS is triggered or not if both legacy and new Rel-17 activation procedures are to be used. The default temporary RS may be the first temporary RS in the configured list.

- Triggering time offset

As agreed, the candidate value(s) of triggering offset(s) is RRC configurable. If c value(s) is configured, then log(c) bit is needed in the MAC CE. If c=1 or a default can be configured, then 0 bit is needed, i.e., the field may be optional. The default value may be the first value in the configured list. An upper bound for c should be decided by 3GPP, such as at most 4 or 8 offset values.

- QCL source

QCL source may be configured by RRC for each temporary RS and may not need to be present in the MAC CE. However, if needed, more than one TCI state may be configured and one of them is selected by the MAC CE.

Below we analyze the MAC CE designing options/alternatives:

- It seems that although in general, some parameters may be included in the MAC CE for temporary RS triggering, none of the parameters needs to be always presented in the MAC CE. At least for some cases, all the parameters may be RRC configured and the default may be triggered by the legacy MAC CE without any new design of MAC CE. So in this case, when only the legacy MAC CE is sent, it implies that the default temporary RS with its default parameters is triggered, provided that the default temporary RS and parameters are configured; if no default temporary RS is configured, then no temporary RS is triggered.

- A new MAC CE to only trigger a temporary RS, if supported, should not be sent if it is not within a PDSCH carrying the legacy SCell activation MAC CE. As agreed before, the temporary RS triggering should only be for SCell activation, so a temporary RS triggering MAC CE shall not be expected by the UE without accompanying the SCell activation MAC CE.

- A new MAC CE design should balance between high flexibility (too many parameters) and high overhead. To this aim, the RRC configuration may limit the temporary RS parameters (resources, time offset, etc.) in MAC to a few combinations, and the MAC CE selects one combination. See later section for more details.

In general, we think the MAC CE design can be up to RAN2, but if RAN1 will provide more discussions, we suggest to discuss the parameters one by one first to determine if a parameter is necessary to present in MAC CE.

We have the following proposal:

Proposal 9: Support the legacy MAC CE activation command to jointly trigger SCell activation and one default AP TRS configured by RRC.

Proposal 10: Additionally, support MAC CE to select/trigger one of the additional AP TRSs configured by RRC, select/trigger one of AP CSI-RSs additionally configured by RRC, and select/trigger one of AP SRSs additionally configured by RRC.

- Send an LS to RAN2 on the requirements for MAC CE / RRC design to support the activation and temporary RS selection/triggering.

## Triggering procedure and timeline enhancements

The activation and triggering procedures, timeline, and UE behavior with a L2 signaling can be enhanced. There can be many cases and not all of them have to be explicitly handled in RAN1 standards (they are to be explicitly handled in RAN4 TS 38.133). RAN1 provides an overall timeline for RAN2 TS 38.321 activation timing and refers to RAN4 TS 38.133 for activation delay requirement. An example is provided in Figure 3, assuming FR1 known cell with 15 kHz SCS.

Figure 3 Example triggering procedure and timeline (for FR1 known cell with 15 kHz SCS)

Details of some components are described below.

- MAC CE triggering time: ending in slot n

This is the slot when the PDSCH carrying MAC CE(s) for activation and temporary RS selection/triggering ends. Note that before the PDSCH reception, there is DCI reception.

- HARQ, initial MAC processing, and RF delays: ending in slot n + k

The slot n + k is still defined as one slot after the required MAC decoding and processing time, which is the same as the current TS 38.213 of slot n + k, i.e., slot n + k1 + 3 Nslot,subframe, µ + 1. The initial MAC processing refers to MAC CE decoding and other MAC related operations but not those specified in clause 5.9 of TS 38.321 for CSI/PDCCH/SRS/PUCCH/etc. operations.

By this time, temporary RS is not transmitted.

- AGC, gap, time/frequency tracking, fine timing, cell search, etc., after slot n + k

Temporary RS needs to be transmitted after slot n+k. RAN1 had an earlier agreement “UEs measure the triggered temporary RS during Scell activation procedure no earlier than a slot m: FFS timeline values m which may need coordination with RAN4”. Thus, m should be equal to n + k, i.e., m = n + k.

For FR1 known cell, if the measurement cycle is no more than 160 ms, then AGC is not needed, but otherwise 1 burst of AP TRS is needed. After the AGC is set, a minimum gap of 2 slots is also needed.

Then time/frequency tracking based on another burst of AP TRS is needed.

In some cases, fine timing may be needed, cell search for an unknown cell is needed, some uncertainties (for MAC, RRC, etc.) are also needed as defined in TS 38.133.

Some details of these operations are not to be specified in RAN1. The role of RAN1 here may be to ensure 1 or more AP TRS bursts with desired design are provided. RAN1 may inform RAN2/RAN4 about X AP TRS bursts with some gaps in between, and the X may be 1, 2, or more, and the TRS bursts should be for the same BWP, not for different BWPs. The MAC signaling details and timing details can be left to RAN2/RAN4.

- Time offset for the first AP TRS

In order for the UE to receive an AP TRS, the TRS should not be sent before the initial MAC processing and RF delays are done. So the earliest possible timing may be immediately after slot n + k, e.g., slot n + k + 1. However, it may be useful to allow some flexibility here, such as the UE may take some time to get ready for the AP TRS. This is a bit similar to L1 AP TRS triggering offset; a larger offset can allow the UE to have more time to respond. Therefore, we suggest to transmit the AP TRS in slot n + k + kTRS, where kTRS is an additional slot offset for AP TRS (also referred to as K0 or aperiodicTriggeringOffset for CSI-RS/TRS) and may be configured by RRC signaling or specified by MAC signaling. The kTRS may also be a fixed value specified by the standards, or may take one value out of several specified by the standards, and the selection is via RRC or MAC signaling. The kTRS slot offset may also be modified to consider only the “available slot” or “admissible slot”, such as the slots configured as downlink by MIB/SIB/RRC by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated but not those flexible slots.

In more details, the current mechanism for TRS triggering offset may be largely reused. In the current mechanism, the maximum possible triggering slot offset for a CSI-RS is 24 or 31, and MinSchedulingOffsetK0-Values can take one or two values up to 16, where each minimum K0 parameter denotes minimum applicable value(s) for the TDRA table for PDSCH and for A-CSI RS triggering Offset(s). When multiple minimum K0 parameters are configured, one of them is indicated in the triggering DCI. When no minimum K0 parameter is configured, the default offset specified by the standard specification is to be used. Then for AP TRP triggering via MAC CE, 0, 1, or more minimum K0 parameters may be configured for the configured TRS, and the MAC CE can select one which will be the kTRS described in the above.

- SRS, CSI, PDCCH, PUCCH, etc.

At the final stage of activation after slot n + k, according to TS 38.133, operations related to SRS, CSI, PDCCH, PUCCH, etc., can start. Note that CSI reporting related operations can start as early as slot n + k.

Other temporary RS, such as AP CSI-RS/SRS and even more AP TRS may be transmitted, and AP CSI reporting with a valid CQI may be performed.

- Time offset for AP CSI-RS

Similar to the timing offset design for AP TRS above, the AP CSI-RS may be transmitted in slot n + k + kCSI-RS, where kCSI-RS is an additional slot offset for AP TRS, and the AP SRS in slot n + k + kSRS, where kSRS is an additional slot offset for AP SRS. These offsets may be configured by RRC signaling or specified by MAC signaling. These offsets may also be a fixed value specified by the standards, or may take one value out of several specified by the standards, and the selection is via RRC or MAC signaling.

In general, the offset for CSI-RS should be larger than that for TRS, and ensure that the CSI-RS is sent only after any prerequisite TRS burst(s) is completed. For example, if 2-slot AGC with TRS is needed, followed by 2-slot gap and another TRS burst of 2 slots for tracking, the CSI-RS offset should be after the TRS burst for tracking, e.g., kCSI-RS ≥ kTRS + 6 slots. Generally, as the network controls the activation process and temporary RS transmissions under various conditions (e.g., those specified in TS 38.133), the network should guarantee that the offsets for the signals can meet appropriate signal transmission/reception requirements.

The CSI-RS triggering offset mechanism may be similar to the above TRS triggering offset mechanism. To have more flexibility, the configured AP CSI-RS may be allowed to have a different offset than the AP TRS triggering offset. Generally, the offsets may be configured differently for different TRS/CSI-RS resources in RRC, and may be indicated differently/separately for different TRS/CSI-RS resources in the MAC CE.

- Time offset for AP SRS

The offset for SRS generally may be larger than that for TRS, but if the prerequisites are already available/valid for the SRS, the SRS may be sent as early as the TRS. For example, for SRS with usage “antennaSwitching” or “codebook” in FR1, if the pathloss RS for the SRS and other prerequisites (e.g., timing advance) are available/valid, the SRS may be sent without waiting for AGC or tracking to be completed; note that the RSRP/pathloss estimate may still be available/valid for the SRS especially if long-periodicity SSB/CSI-RS are still transmitted during deactivation, or if the pathloss RS is allowed to be configured on a different serving cell which is activated, and the TA may be valid if the SCell is in a TAG with an activated serving cell. However, for “nonCodebook”, a prerequisite is the associated CSI-RS (on the same carrier as the SRS) to be transmitted, so the SRS with “nonCodebook” can only be after the associated CSI-RS. Again, the network can guarantee the proper time order with sufficient flexibility in selecting the offsets.

The AP SRS triggering offset may be based on one or more configured slotoffset values, and one value is to be selected by the MAC CE. However, as the SRS can only be transmitted in UL slot or on UL symbols in a flexible slot in TDD, the triggering slot offset may also be only accounting for UL slots and flexible slots with sufficient OFDM symbols to fit the AP SRS transmission.

- CSI reporting

CSI reporting, based on AP/P/SP CSI-RS and the associated CSI-IM, may be done after the CSI-RS transmission(s) and an additional time delay for UE processing. The time relation between the CSI-RS transmission and the CSI reporting as defined in current standards can be reused.

The CSI reporting may be on a different carrier or on the carrier being activated. For P/SP CSI reporting, current standardized procedures can be followed. For AP CSI reporting triggered by MAC CE, the reporting parameters should be mainly configured by RRC and the rest few (if any) by the MAC CE. A straightforward design is to add a CSI request field in the MAC CE and the design of the field is similar to that in a DCI.

We have the following proposal.

Proposal 11: For SCell activation, support

- The earliest slot for a UE to receive a triggered temporary RS is slot n + k

- On and after slot n + k (according to current TS 38.213) + kTRS, transmission of X AP TRS bursts with a gap between the bursts, the value of X includes at least 1 and 2; additional values may be from RAN4

- On slot n + k + kSRS, transmission of AP SRS if configured or triggered by MAC CE

- On slot n + k + kCSI-RS which is also after a number of AP TRS bursts, transmission of AP CSI-RS/CSI-IM if configured or triggered by MAC CE, and reporting a valid CQI based on existing reporting mechanism

- The values of kTRS, kSRS, and kCSI-RS are determined by the network and provided to the UE by RRC or MAC signaling

- Inform RAN2/RAN4 on RAN1 decisions in an LS.

## Aperiodic TRS and QCL for aperiodic TRS

An AP TRS is generally not a standalone RS, i.e., it relies on a P/SP TRS. In particular, the AP TRS is QCLed to the P/SP TRS. We now examine the reliance for a to-be-activated SCell when the AP TRS is a temporary RS used to facilitate the activation.

First, for a known SCell, the time/frequency synchronization is still maintained, and the channel properties derived from the SSB and/or associated P/SP TRS are still valid for the AP TRS to use. Then there is no issue with the QCL for AP TRS. This has been confirmed by RAN4 for FR1/FR2. For FR1, if the known SCell measurement cycle is no larger than 160 ms, 1 burst of 2 slots with 4 AP CSI-RS resources as TRS is sufficient for time/frequency tracking, and if the known SCell measurement cycle is larger than 160 ms, 2 bursts of AP CSI-RS resources as TRS are sufficient for AGC and time/frequency tracking. For FR2, 1 burst of 2-slot AP CSI-RS resources as TRS is sufficient for fine timing tracking. Therefore, the SSB and/or associated P/SP TRS of the to-be-activated SCell is the QCL source for the temporary AP TRS in case of known SCell.

If the above assumption does not hold for some cases, then other solutions need to be provided for the QCL of the AP TRS. One is that the AP TRS can rely on a cross-carrier signal as described before, such as the AP TRS is QCLed with a cross-carrier SSB or a cross-carrier P/SP TRS, and the QCL type may be Type C (and also D for FR2) or further discussed. The cross-carrier SSB or P/SP TRS should be on an activated carrier which is generally an intra-band carrier and should be configured/received for the UE before the activation process starts. In another solution, in case of the cross-carrier SSB or P/SP TRS is not available, the AP TRS may be transmitted in consecutive slots according to the design of the associated P/SP TRS so that the UE can derive sufficient tracking information from the TRS.

After the transmission of AP TRS during activation, it should serve as the QCL source for other RS(s) after it, including P/SP TRS, P/SP/AP CSI-RS, P/SP/AP SRS. However, after a transmission of P/SP TRS, the P/SP TRS, or the P/SP and AP TRS jointly, can be used as the QCL source for RS(s) after it.

For an unknown SCell, most likely the SSB (or RS based on SSS/PSS) needs to be transmitted, preferably based on a trigger over periodic SSB, and then the P/SP/AP TRS may be transmitted. The activation latency will be longer, so it may be desirable to avoid the case of a SCell becomes unknown via a long-periodicity RS.

Proposal 12: The associated P/SP TRS and SSB of the to-be-activated SCell is the QCL source for the temporary AP TRS in case of known SCell.

Proposal 13: The AP TRS serves as the QCL source for other RS following it, including P/SP TRS if sent after the AP TRS, and the AP/P/SP TRS serves as the QCL source for other RS after the P/SP TRS.

## TRS structure

RAN1 had a working assumption that Rel-15/16 TRS structure will be reused for temporary RS, with each TRS burst being 2-slot with 4 CSI-RS resources or 1-slot with 2 CSI-RS resources. Based on RAN4 LS replies, only 2-slot with 4 CSI-RS resources is needed for TRS. Thus, we propose to confirm this part of the working assumption.

Proposal 14: Confirm the working assumption as: with respect to efficient SCell activation, reuse existing Rel-15/16 TRS structure for temporary RS, and a burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214, with “2-slot with four CSI-RSs resources (4 samples)” for FR1/2.

## The BWP clarification

By default, in legacy design, when the SCell is activated, the UL BWP and DL BWP that are being activated are based on firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id. For this reason, RAN1 made the following agreement:

Agreement

If a UE measures a temporary RS triggered by a MAC-CE during SCell activation procedure, the measurement is performed within the BWP bandwidth of BWP indicated by firstActiveDownlinkBWP-Id

However, the agreement does not say that a temporary RS has to be on the BWP with firstActiveDownlinkBWP-Id; it only says that the measurement of the temporary RS, e.g., a TRS, is within the bandwidth of the BWP with firstActiveDownlinkBWP-Id. Thus, it seems possible that, say, the BWP with firstActiveDownlinkBWP-Id is BWP 1 but the TRS is configured on BWP 2, and the UE just performs measurement of the TRS on the overlapped bandwidth of BWP 1 and BWP 2. This can create some issues:

- If BWP 1 and BWP 2 have no overlap, then the TRS is not a useful TRS.

- If BWP 1 and BWP 2 have overlap but the overlapped bandwidth is small, the measurement results are not reliable.

- If BWP 1 and BWP 2 have overlap but have different numerologies, it is unclear how the UE can perform measurement on the TRS or need a BWP switching.

Whether the standards or implementation should deal with this case needs discussion, which could be unnecessarily complicated. Alternative, restrictions may be added to the agreement. It seems some companies intended to use only TRS configured for the BWP with firstActiveDownlinkBWP-Id as temporary RS, but this was never clarified. We suggest to clarify / confirm this. However, if this cannot be confirmed, for example, some companies may think this is too restrictive, then we suggest to relax the agreement to allow activation into other BWPs. We also point out that there can be benefit of having the flexibility to activate into other BWPs, for example, different BWPs may be preferred by the network for different traffic demands or spectrum utilization considerations. Then activation into only BWP 1 and measuring/reporting CSI for BWP 1 may not help much if the gNB wishes to use BWP 2 but BWP 1 and BWP 2 may have different bandwidths and numerologies; even worse, the BWPs may be on orthogonal frequencies in this carrier. Further discussion is needed.

Proposal 15: Further clarify / strengthen the previous agreement on BWP to include:

- All TRS(s) as temporary RS(s) can only be configured on the BWP with firstActiveDownlinkBWP-Id;

- The SCell always activates into the BWP with firstActiveDownlinkBWP-Id.

## Summary of RS transmissions during activation for different cases

As described above, for different cases with different requirements on AGC/cell detection/tracking, RS transmissions may be different. A summary is provided in below table.

Table 2 Summary of the number of 2-slot TRS bursts and number of SSB transmissions (numbers in the table are for 2-slot TRS bursts unless otherwise specified)

From the table, we can see at least 2-slot TRS burst(s) should be supported, and m1 SSB bursts and/or m2 SSB bursts should be supported, where m1 and m2 may be 1 or more. The cases (i.e., rows) in the table may be assigned with IDs and the IDs may be selected by the MAC CE to reduce MAC signaling overhead.

Proposal 16: Support at least 1 burst and 2 bursts of 2-slot aperiodic TRS, and m SSB bursts as temporary RS, and FFS m.

# Conclusion

In this contribution, we discussed Rel-17 efficient SCell activation mechanism. The following are proposed:

Proposal 1: Clarify RAN1 understanding on the definition / support of known / unknown SCell cases:

- Alt 1: Support all cases, with conservative design for cases in which the SCell has not been used for more than x ms, and FFS x; OR

- Alt 2: Send an LS to RAN4 to inquire whether the gNB and UE have the same understanding of a to-be-activated SCell being known or not.

Proposal 2: Prioritize the following cases before further decisions from RAN4 are made:

- FR1 known SCell

- FR1 unknown SCell with an activated intra-band contiguous cell

- FR2 known SCell with an activated intra-band cell

Proposal 3: Support aperiodic triggering of SSB search time window for the case of unknown SCell.

Proposal 4: Support cross-carrier RS and cross-carrier properties (e.g., bi-directional cross-carrier QCL between SSBs) for efficient activation according to RAN4 LS inputs.

Proposal 5: Support aperiodic CSI-RS as additionally configurable temporary RS, to be triggered if the SCell activation requires a valid DL CSI with reduced delay.

Proposal 6: Support AP CSI-IM as additionally configurable temporary RS resource if AP CSI-RS is supported as temporary RS.

Proposal 7: Support aperiodic SRS as additionally configurable temporary RS, to be triggered if the SCell activation requires a valid UL CSI or DL CSI for TDD or UL TA / PC / beam with reduced delay.

Proposal 8: Support long-periodicity RS (e.g., TRS / CSI-RS / CSI-IM / SRS / SSB) as additional RS / RS resources to facilitate efficient SCell activation.

Proposal 9: Support the legacy MAC CE activation command to jointly trigger SCell activation and one default AP TRS configured by RRC.

Proposal 10: Additionally, support MAC CE to select/trigger one of the additional AP TRSs configured by RRC, select/trigger one of AP CSI-RSs additionally configured by RRC, and select/trigger one of AP SRSs additionally configured by RRC.

- Send an LS to RAN2 on the requirements for MAC CE / RRC design to support the activation and temporary RS selection/triggering.

Proposal 11: For SCell activation, support

- The earliest slot for a UE to receive a triggered temporary RS is slot n + k

- On and after slot n + k (according to current TS 38.213) + kTRS, transmission of X AP TRS bursts with a gap between the bursts, the value of X includes at least 1 and 2; additional values may be from RAN4

- On slot n + k + kSRS, transmission of AP SRS if configured or triggered by MAC CE

- On slot n + k + kCSI-RS which is also after a number of AP TRS bursts, transmission of AP CSI-RS/CSI-IM if configured or triggered by MAC CE, and reporting a valid CQI based on existing reporting mechanism

- The values of kTRS, kSRS, and kCSI-RS are determined by the network and provided to the UE by RRC or MAC signaling

- Inform RAN2/RAN4 on RAN1 decisions in an LS.

Proposal 12: The associated P/SP TRS and SSB of the to-be-activated SCell is the QCL source for the temporary AP TRS in case of known SCell.

Proposal 13: The AP TRS serves as the QCL source for other RS following it, including P/SP TRS if sent after the AP TRS, and the AP/P/SP TRS serves as the QCL source for other RS after the P/SP TRS.

Proposal 14: Confirm the working assumption as: with respect to efficient SCell activation, reuse existing Rel-15/16 TRS structure for temporary RS, and a burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214, with “2-slot with four CSI-RSs resources (4 samples)” for FR1/2.

Proposal 15: Further clarify / strengthen the previous agreement on BWP to include:

- All TRS(s) as temporary RS(s) can only be configured on the BWP with firstActiveDownlinkBWP-Id;

- The SCell always activates into the BWP with firstActiveDownlinkBWP-Id.

Proposal 16: Support at least 1 burst and 2 bursts of 2-slot aperiodic TRS, and m SSB bursts as temporary RS, and FFS m.

# References

R1-1718863, Summary of QCL, Nokia, Nokia Shanghai Bell, RAN1#90bis, Prague, Czech Republic, Oct. 2017.