3GPP TSG RAN WG1 Meeting #106-e	R1-2107091

eMeeting, Aug. 16 –Aug 27, 2021

Agenda Item:	8.11.1.1

Source:	FUTUREWEI

Title:	Power consumption reduction for sidelink resource allocation

Document for:	Discussion and decision

# Introduction

At RAN#86, a WI on sidelink enhancements was agreed for Rel-17 [1] with further updates in [4]. In this WI, there is an objective on resource allocation enhancements to reduce power consumption:

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

At RAN1#105-e, a set of agreements was reached and is listed in the Appendix.

In this contribution, we first discuss the conditions for random resource selection for different types of UE. We then discuss periodic-based partial sensing including the determination of candidate slots for partial sensing and periodic sensing occasions. Following that, we discuss contiguous based partial sensing for both periodic traffic and aperiodic traffic, as well as the impact of SL-DRX on UE sensing.

# Discussions

## Random resource selection

### UE types and switch between different resource selection

At RAN1#104-e, it was agreed that random resource selection is applicable to both periodic and aperiodic transmissions. Obviously, performance wise, random resource selection is inferior to partial sensing or full sensing based resource selection in terms of PRR. However, as a simple resource selection scheme, random resource selection reduces transmission delay as well as power consumption.

Also, at RAN1#104-e, it was agreed that three types of UEs are supported for power savings evaluation in Rel-17, i.e., Type A, Type B, and Type D UEs. Since a type A UE is not capable of receiving any SL signals, random resource selection is adopted. Therefore, no other configuration is needed for a type A UE to enable/disable random resource selection.

For a Type B UE, since it is capable of receiving PSFCH, if PSFCH is available, e.g., for unicast and groupcast transmissions, it can then reselect resources when consecutive NACK feedback is received, which indicates a high probability of resource conflicts. At the #104-e meeting, several companies proposed to extend the PSFCH to indicate the conflict and trigger resource reselection. If this is adopted, a Type B UE can perform random resource reselection after receiving the extended PSFCH with conflict indication. However, for broadcast transmissions, there is no feedback. Then the Type B UE performs random resource selection as a Type A UE does. For groupcast with HARQ option 2 or unicast transmissions, if the PSCCH is not successfully received at receiving UE, there is no ACK/NACK. Then Type B UE also performs random resource reselection.

For a type D UE, partial sensing or full sensing is supported. In general, random resource selection is not preferred. However, it is better to keep it as an option for additional power savings. Therefore, random resource selection can be either configured by upper layers or is used under certain conditions.

First, conditions to switch between partial/full sensing and random resource selection can be specified. The resource selection procedure attempts to select a certain number of sub-channels based on sensing evaluations. When the system load is high, a UE can switch to random resource selection instead of relying on the results of sensing because the sensing RSRP thresholds were increased a lot in order for a UE to find available resources. Based on this observation, one approach is to perform initial sensing to determine the system load from the sensing results. If the results indicate a high load, the benefits of sensing over random resource selection are limited. When the load is low, there is no or a small number of resources occupied by other nearby UEs. Under this condition, the UE can switch to random resource selection to reduce power consumption. Note that UE should be able to monitor the PSCCH in order to make this determination. The conditions can be set by a threshold on CR or available resource ratio. To switch back to partial/full sensing, a predefined timer can be activated once the UE switches to random resource selection. For better power savings, we propose to switch back to partial sensing with a minimum number of slots configured. Then gradually increase the timer for switching back to partial/full sensing from the random resource selection  based on sensing results.

Secondly, for a UE, if the remaining PDB is not sufficient for constructing a configured partial sensing window, the UE then performs random resource selection.

Another condition is for low power levels (e.g., battery levels). In such a case, when the battery is about to drain out or is below a certain level, to reduce power consumption, it is benefit that this UE perform random resource selection instead of partial sensing/full sensing. For low power level UEs, they may also be configured to perform random resource selection all the time and not to switch between partial/full sensing and random resource selection.

When a UE performs random resource selection, it may mitigate conflicts with re-evaluation and pre-emption process. However, we do not support re-evaluation and pre-emption during random resource selection because of the following reasons: 1) re-evaluation and pre-emption require PSCCH monitoring which requires more power. 2) Monitoring PSCCH is essentially a sensing function. 3) In the scenarios described above, e.g., extreme high or low load, insufficient time for sensing due to PDB, re-evaluation and pre-emption do not bring any benefit.

Proposal 1: UE can switch between partial/full sensing and random resource selection when certain conditions are satisfied, e.g., a threshold on the available resource ratio or CR. A timer or counter can be preconfigured to switch back to partial/full sensing.

Proposal 2: When UE performs random resource selection, re-evaluation and pre-emption are disabled.

### Consecutive packet loss

UEs performing random resource selection do not monitor the PSCCH to detect possible collisions. A problem arises when two UEs with the same traffic periodicity or when a periodicity is a multiple of the other periodicity happen to select the same resources. The initial collision is then repeated until resource reselection, resulting in a consecutive packet loss.

Although with HARQ, UE can perform retransmission to alleviate the performance degradation, it is still not very efficient use of the resources to continue retransmissions until resource reselection. On the other hand, for Type A UE and broadcast transmissions, a UE is not capable of receiving the PSFCH or there is not PSFCH configured.

To improve the performance of random resource selection, instead of always using the same resources, the UE alternates between two resources. Two consecutive transmissions use different resources, as shown in Figure 1, the red resource for odd-numbered transmissions, the blue resource for even-numbered transmissions.  This way, the probability of having at least half of the packets correctly received is greatly improved.

Proposal 3:  For random resource selection with periodic traffic, the transmitting UE uses two different resources for two consecutive transmissions.

Figure 1. Periodic transmission with alternating resources

### Low priority transmissions in mixed RA

As summarized in [5], one issue for random resource selection is mixing different resource allocation schemes in a same resource pool where the power saving UEs with the random resource selection and UEs with full or partial sensing select resources for their transmissions.  One problem arises when the low priority transmission from a power saving UE performing random resource selection or with re-evaluation or pre-emption disabled collides with a high priority transmission from a sensing UE. With re-evaluation and pre-emption, a sensing UE can perform resource reselection to avoid some collisions with other UEs including the UE with random resource selection. However, since power saving UEs with random resource selection either do not have reception/PSCCH monitoring capability (e.g., Type A, B UEs) to perform re-evaluation and pre-emption or on the power saving mode without enabling re-evaluation and pre-emption, they cannot detect the resource reserved for a higher priority transmission to perform resource reselection. The sensing UE with a higher priority transmission will not free the resource either when detecting a collision with a lower priority transmission.  The two transmissions will collide at the intended recipients and cause a performance loss.

We propose to introduce one-bit indicator in SCI to combat the collisions between random selection UEs and sensing UEs. We can add new field in SCI for the indicator or use one of the reserved bits in SCI. The indicator can be used to indicate that the transmission is from a power saving UE with random resource selection or with the re-evaluation/pre-emption disabled. Then a sensing UE detects the indicator, and based on the value of indicator, increase the priority of packet via an offset or a priority mapping rule from the original priority so that the sensing UE performs the pre-emption check based on the updated priority. Since the new indicator cannot be detected by Rel-16 sensing UEs, it will not alleviate the collision problem for Rel-16 UEs. Therefore, alternatively, the power saving UE can increase the priority and send a higher priority in the priority field in SCI so that the legacy sensing UE will avoid the collision with pre-emption check if its priority is lower than the updated priority from the power saving UE. However, the receiving UE may need the original data priority. With the new indicator and a priority mapping, the receiving UE can retrieve the original priority without ambiguity.

Proposal 4:

- For SL transmissions, increase the priority of UEs performing random resource selection

  - A 1-bit field in the SCI indicates that the UE is performing random resource selection.

  - The offset of priority increase is applied either

    - Option 1: by the detecting UE

    - Option 2: in the SCI priority field directly

  - Details FFS

Another way to solve the problem is to partition the resource pool into two sub-pools or resource zones, sub-pool A and sub-pool B as shown in Figure 2. Then a UE with random resource selection performs resource selection in one sub-pool, e.g., sub-pool A. The sensing UE performs resource selection in the other sub-pool, e.g., sub-pool B. Since Rel-16 sensing UE with high priority data does not have the knowledge of such resource pool partitioning, its transmissions will still collide with the UE with random selection. However, since Rel-16 UE selects a resource from the entire resource pool, the collision probability between Rel-16 sensing UE with high priority and low priority UE with random resource selection is lower than that without resource partition. Moreover, we can assign a priority threshold on the sub-pool for random resource selection. A UE with random resource selection selects the resource in a sub-pool, i.e., sub-pool A when the data priority is lower than the priority threshold of that sub-pool. A random selection UE with a priority higher than the threshold can select the resource in the other sub-pool, i.e., sub-pool B, or select a resource from the entire resource pool.

Proposal 5: Partition the resource pool into several resource subsets, i.e., sub-pools, UE with random resource selection selects the resource from a sub-pool.

Proposal 6: Partition the resource pool into several resource subsets, i.e., sub-pools, and assign a priority threshold on the sub-pool for random resource selection

Figure 2. Resource pool partition for mixed resource allocations for power saving UEs and sensing UEs

## Periodic-based partial sensing

### Determination of Y candidate slots for periodic transmission

As shown in Figure 3, with periodic based partial sensing, a UE monitors one or more periodic occasions with Y candidate slots on each occasion. Based on sensing results, UE forms a set of candidate resources on a set of Y slots in the resource selection window and selects one for sidelink transmission.

Also as shown in Figure 3, in a resource pool (pre-)configured with at least partial sensing, if a UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, the UE monitors slots of at least one periodic sensing occasion, where a periodic sensing occasion is a set of slots according to

ty-k×PreserveSL,

where the set of k values indicate the sensing occasions. For example, when k{1, 2}, the UE performs partial sensing on ty-1×PreserveSL and ty-2×PreserveSL. If k is specified as a bitmap as in LTE V2X, i.e., {i1,i2, …, ik, …,}, ik{0,1}, the UE performs partial sensing on the occasion of ty-k×PreserveSL, where ik=1.

Figure 3. Periodic based partial sensing

At RAN1#104-e, it was agreed that when (pre)-configured with partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window.

UE may choose Y in a range with a minimum value for Y is (pre-)configured from a range of values. To have reliable sensing results, a different minimum value for Y can be configured for each priority. Due to discrete values of priorities, we then select a set of minimum values of Y corresponding to different priorities in the list. For high priority, setting one minimum value of Y to accommodate all possible traffic loads, heavy or low, may result in a large value on the minimum of Y, which may be inefficient for power saving. It is then better to configure a small value of minimum Y, but UE needs to adapt Y according to the sensing results, e.g., the available resource ratio threshold for partial sensing as described below.

Due to a shorter sensing time in partial sensing, the sensing results may be unreliable, particularly for the available resource ratios. Also due to supporting aperiodic transmissions, with a small sensing window, it is highly possible there will be a resource collision particularly when the available resource ratio from partial sensing is small. For example, with 20% available resources from partial sensing, 80% are occupied, indicating a high system load. With a shorter sensing time, the variance of the actual available resource ratio for 20% is much larger than for full sensing. Hence, the collision probability based on this sensing result could be much higher compared with full sensing. Another issue is due to the smaller candidate pool for a small value Y. For the same available resource ratio, e.g., 20, the available candidate resources for partial sensing are much less than that for full sensing. The collision probability can also be higher if another UE with new data reserves the resource in the same region. Then for partial sensing with a small Y, a larger threshold than 20% on the available resource ratio should be used. Therefore, it is beneficial to specify a new table for the available resource ratio list of available resource ratio X% and a relationship between configured Y (or Ymin in the specification) and minimum value of X, i.e., Xth(Y), Or at least to restrict the lowest value of X from the existing table sl-TxPercentageList specified in R16 when Y<Yth(Xmin).

Therefore, for better power consumption, UE may start with Y from a configured minimum value. If the available resource ratio X from the sensing results is smaller than configured X (prioTX, Y) for the smallest Y, then UE increases Y to the next value in the list or determine next Y1, based on sensing results and perform partial sensing in the next round of sensing.

Instead of specifying the list of X for partial sensing based on Y and number of partial sensing window, we could alternatively specify the list based on the ratio of the number of partial sensing slots to the number of full sensing slots.

Proposal 7: Specify a new list of X for partial sensing or set new rules for partial sensing on X with the existing list sl-TxPercentageList.

### Periodic sensing occasions

In R16, NR sidelink supports periodic traffic and the reservation period is configured from the list sl-ResourceReservePeriodList, where the list includes the values 0, 100, 200 … 1000ms, and integer values in [1, …, 99].  Due to shorter reservation periods available in NR, periodic-based partial sensing should be able to detect such traffic. A natural way is then to let Preserve include all values from the configured set sl-ResourceReservePeriodList. If the value is transformed to the slot, the list of Preserve needs to support different SCS to accommodate the RRI in the ResourceReservePeriodList.

At RAN1#105-e, it was agreed that if no (pre-)configuration or by default, Preserve corresponds to all values from the (pre-) configured set sl-ResourceReservePeriodList. Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList.

Regarding the sensing occasions, it was also agreed at RAN1#105-e that by default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction. Also, multiple sensing occasions can be configured, i.e., if (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value. Then, a working assumption is agreed at RAN1#105-e that the possible values for (pre-)configured sensing occasions include last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included.

From the agreement in RAN1#105-e, one sensing occasion, i.e., the most recent sensing occasion, is determined for periodic partial sensing for each period in the sl-ResourceReservePeriodList or its subset if (pre-)configured. Based on the definition of the sensing occasion, as shown in Figure 3, for different periodicities, the k value representing the most recent sensing occasion is different. Also as shown in Figure 3, one sensing occasion is a set of slots corresponding to the Y slots for resource selection. Since the periodic partial sensing is restricted at least before the first slot of Y slots, for the slot within the Y slots, the corresponding most recent sensing occasion may also have different k values. Therefore, k value for most recent sensing occasion is dependent on the periodicity Preserve and the slot location in the Y slots. From Figure 3, we can see that for some small Preserve and some slot closer to the end of Y slots, the corresponding k value for the most sensing occasion can be very large, indicating that the most sensing occasion for that particular periodicity and particular slot is the many periods ahead of the target reservation slot in the Y slots. The agreement on using the most recent occasion instead of the first periodic set is to include at least one sensing slot for a periodicity and each slot in the Y slots and also because that the sensing on a slot that is multiple-periods (> 2 periods) ahead is still effective for detecting a potential resource collision. The principle shall be applied for all periodicity values and for all slots in the Y slots. On the other hand, for different scenarios, e.g., the system with a high traffic load or system with a low traffic load, the number of sensing occasions required to achieve the same PRR can be different. For a high load system, it is possible that the UE may miss the detection of the SCI, resulting in a performance loss on PRR with only one or two sensing occasions. Therefore, maximally more than 2 sensing occasions shall be allowed. To adapt to different scenarios, a flexibility on configurations of the number of sensing occasions and values of the sensing occasions is desired. One additional advantage for flexible configurations is to align with DRX active period as much as possible. Therefore, the working assumption should not be confirmed as is. We propose to have the number of and value of the sensing occasions that can be (pre-)configured In additional to the most recent sensing occasions, we propose that the additional sensing occasions are not limited to the last periodic sensing occasion prior to the most recent and can be configured other than the last periodic sensing occasions prior to the most recent.

For the set values of sensing occasions, we propose to represent the values with a bitmap. Denote k’ as the index of sensing occasion and k’=1 represents the most recent sensing occasion. Then the set of sensing occasions can be represented as the bitmap {i1, i2,…, ik’,…,iK’}. If ik’ = 1, the k’th recent sensing occasion prior to ty0 is determined as a sensing occasion for a given periodicity and for a slot within the Y slots. Since including the most recent sensing occasion by default is agreed, we have i1 =1.   If the partial sensing configuration needs to be signaled to other UE via a PHY signal for some use cases, e.g., inter-UE coordination, a bitmap with a fixed bit size is desirable. Compared with a numeric list of k values, the bitmap representation could also reduce the number of bits when multiple k values are configured. To achieve better power saving performance, a maximum number of sensing occasions for periodic partial sensing can be specified, i.e., specifying maximum number of k’ values or the maximum number of 1’s in the bitmap,  i.e., at most kmax 1’s in the bitmap can be configured. For example the bitmap of k’ is denoted as {i1, i2,…, ik’,…,i10}, with kmax =3, we have k=110ik≤kmax=3. With maximum number of sensing occasions and flexible configurations of sensing occasions, the periodic based partial sensing can be adaptive to different scenarios with better balancing/tradeoff between power consumption and throughput/delay performance.

Proposal 8: For sensing occasions in periodic partial sensing, maximally more than two sensing occasions can be configured. Besides the most recent sensing occasions, the additional sensing occasions are not limited to the last periodic sensing occasion prior to the most recent and can be configured on the one other than the last periodic sensing occasions prior to the most recent.

Proposal 9: The set of values for sensing occasions can be represented with a bitmap.  For better power saving, maximum number of sensing occasions should be specified.

## Contiguous based partial sensing

Aperiodic/dynamic transmission is supported in sidelink communications. For sidelink resource allocation, a UE should detect possible aperiodic traffic from other UEs to avoid resource conflicts. For this purpose, a UE performs contiguous based partial sensing for resource (re)selection. It was noted in RAN1#104-e that contiguous based partial sensing can be specified for resource allocation for both periodic traffic and aperiodic traffic for the sensing UE.

### Contiguous based partial sensing for periodic traffic

For the sensing UE with periodic traffic, as shown in Figure 4 , when the resource (re)selection is triggered at slot n, the UE will select the resource from a set of Y candidate slots within a resource selection window [n+T1, n+T2]. The starting slot of the Y candidate slots is at slot ty. If a UE performs contiguous based partial sensing, it monitors the slots between [TCPS,st, TCPS,end].

Figure 4. Illustration of timing for contiguous based partial sensing for SL transmissions with periodic traffic

For aperiodic traffic, the UE selects multiple candidate resources, but with the restriction that the gap between two consecutive candidate resources must be smaller than 32 slots. For example, when a resource is selected on slot m1, meaning that the other candidate resource is located in the range of slots [m1-31, m1+31]. Therefore, to select resources in the set of Y candidate slots, it is meaningless to monitor the slot ty0-32 or before.  Thus, the starting point of contiguous partial sensing can be TCPS,st= ty0-31. Since for periodic traffic, n is known in advance, it is fine that ty0-31<n. Given the time to complete the sensing process and resource selection processing, the ending slot for contiguous partial sensing TCPS,end= ty0-Tproc,0 - Tproc,1. Contiguous partial sensing is similar to the re-evaluation process, to provide the better resource selection, we can limit the processing time to Tproc,1.  We then have TCPS,end= ty0- Tproc,1. The sensing window for the contiguous partial sensing is [ty0-31, ty0- Tproc,1]. From the agreement in RAN1#104-e for contiguous partial sensing, as in the option 1, a UE monitors the slots between [n+TA, n+TB]. With these notations, we then have TA= -n+ ty0-31, TB=-n+ ty0- Tproc,1.

As aforementioned, monitoring a slot can only detect resource occupancy or reservation from aperiodic traffic within 32 slots. Then monitoring slot ty0-31 is only useful for resource selection on ty0, and monitoring slot ty0-30 is only useful for slots ty0, ty0+1, and so on. The slot in the sensing window closest to ty0 has largest coverage in the resource selection region. For this reason, to achieve better power saving, we may reduce the window size for contiguous partial sensing. For contiguous partial sensing for periodic traffic, sensing may start later than ty0-31. For flexibility, the sensing starting point can be configured from a predefined range or a predefined list, with the earliest point being on slot ty0-31. For example a list of TCPS,st or TA can then be specified, e.g. TCPS,st = ty0-a×32+1, or TA = -n+ty0-a×32+1, where a =1, ½, 1/4,….

It is possible that there is an overlap between the slots for contiguous partial sensing and periodic partial sensing occasions. Based on the sensing results from contiguous partial sensing and periodic partial sensing if available, the UE selects the resource from the set of Y candidate slots within the resource selection window. After selecting the resource, the UE then performs re-evaluation and pre-emption if configured.

Again, the sensing results may be unreliable due to the short sensing window size. On the other hand, the value of Y can be larger than 32. Contiguous partial sensing does not provide any benefit for resource selection on slots [ty0 -Tproc,1+32, ty0+Y-1]. Also, if the sensing UE detects many aperiodic traffic within the sensing window [ty0-31, ty0- Tproc,1], resulting in many resources being occupied on [ty0, ty0-Tproc,1+31], relying on periodic sensing results for resource allocation on [ty0 -Tproc,1+32, ty0+Y-1] will cause many resource conflicts. Therefore, a larger threshold on the available resource ratio X% is desirable than that in the full sensing for the data with the same priority. Continuing partial sensing will be beneficial if the number of available resources on [ty0 , ty0 -Tproc,1+31] is small. However, different from re-evaluation process where the transmission resource is allocated, e.g., m, here we do not have that information in advance. Setting the sensing slot based on m is not appropriate. Although it is possible to reporting available candidate resource set SA to the MAC layer on some slot and getting a grant on slot m, this two-stage process may functionally overlap with the re-evaluation process. Since the purposes of initial sensing and re-evaluation are different, with one for resource allocation and the other for checking resource conflicts, it is better to separate them in the specification. Therefore, a UE is allowed to continue sensing till TCPS,end. Although the UE can continue sensing until the slot TCPS,end= ty0+Y-1- Tproc,1, which may only leave a maximum of 1 slot for resource selection, it is better to set an offset, i.e., TCPS,end= max(ty0- Tproc,1 ,ty0+Y-1- Tproc,1 – TCPS,offset ). TCPS, offset, can be set or fixed to 31. A UE can report the available resource any time after slot ty0- Tproc,1 to the MAC layer. Then based on TCPS,end and the notation in option 1 from RAN1#104e, we then have TB=-n+ max(ty0- Tproc,1 , ty0+Y-1- Tproc,1 – TCPS,offset).

Based on above descriptions, we propose to set minimum and maximum values related to the sensing boundary, i.e., TB,min and TB,max for contiguous partial sensing for the transmission with periodic traffic, where TB,min  =ty0- Tproc,1 , TB,max =-n+ max(ty0- Tproc,1 , ty0+Y-1- Tproc,1 – TCPS,offset). A UE reports available resource set any time on [TB,min, TB,max ] . After a resource is selected, it is up to UE to perform re-evaluation or pre-emption.

Proposal 10: For contiguous partial sensing to support resource selection for periodic traffic, to achieve better power saving, the sensing start point is specified from a slot later than ty0-31 can be configured from a predefined range or a predefined list.

Proposal 11: For contiguous partial sensing to support resource selection for periodic traffic, a minimum and a maximum on TB, TB,min  and TB,max, UE is allowed to continue sensing after n+ TB,min  , and performs resource selection on any slot in n+ [TB,min  TB,max ].

As agreed in RAN1#105-e, for contiguous partial sensing, TA and TB values can be zero, positive or negative. The resource selection is performed based on the sensing results after contiguous partial sensing. For periodic transmissions, as we discussed, it is beneficial that UE performs contiguous partial sensing within Y slots, the resource reservation can be performed after contiguous partial sensing. If the initial configured TB is positive and within the Y candidate slots, the candidate resource set SA should be initialized with the slots after TB. Therefore, the candidate resource set SA can be initialized based on the subset of Y candidate slots.

Proposal 12: The candidate resource set (SA) can be initialized based on the Y or a subset of Y candidate slots.

### Contiguous based partial sensing for aperiodic traffic

For UE with aperiodic traffic, a data packet could arrive at any time without any prior knowledge. Therefore, it is impossible for a UE to know in advance when resource selection is triggered at slot n. Thus, contiguous based partial sensing for aperiodic traffic can only start after n, i.e, TCPS,st >n, as shown in Figure 5. The earliest possible starting point is TCPS,st  = n+1, i.e., TA=1.

Figure 5. Illustration of timing for contiguous based partial sensing for SL transmissions with aperiodic traffic

When UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, to achieve maximum power saving, the UE may perform the partial sensing with a minimum window size to have a reliable sensing results for resource selection. The sensing results for aperiodic traffic from other UEs are only beneficial for the resource selection on the slots [TCPS,end  + Tproc,1 , TCPS,end  +31]. For the resources on slots [TCPS,end +32, n+T2], it is equivalent to random resource selection. On the other hand, the sensing window size TCPS,end  -TCPS,st  +1 may also impact the reliability of reported candidate resources on [TCPS,end  + Tproc,1 , TCPS,end  +31].

The available resource ratio on [TCPS,end  + Tproc,1 , TCPS,end  +31] derived from contiguous partial sensing is an important factor for resource selection. If the ratio is small, the available resources ratio on slots [TCPS,end  +32, n+T2 ] may also be small. Assuming that the resources on slots [ TCPS,end  +32, n+T2 ] are all available, reporting them in Sa will lead to a high conflict rate. To solve this issue, first it is beneficial to specify a larger threshold on the available resource ratio X%.  Second, if the available resource ratio is not large enough, the UE shall continue sensing instead of reducing the RSRP threshold. The UE stops sensing when the available resources is enough for the resource selection. The sensing window can be increased in a predefined value. Another solution is to restrict the resource selection window in the effective range of the contiguous partial sensing, i.e., [TCPS,end  + Tproc,1 , TCPS,end  +31].

Also, since the value of T2 is left to UE implementation, it is difficult to specify a maximum slot for the UE performing contiguous partial sensing. In this case, we can specify the minimum sensing window to achieve better power saving. For efficient sensing and power consumption, it is better to reduce the sensing windows size and, in this case, start the resource selection window earlier. Another advantage for having a smaller sensing window for aperiodic traffic is to have low latency, which is one of key features for dynamic transmission in sidelink. Therefore, the minimum sensing window size can be specified with a fixed value smaller than 31- Tproc,1 or a set of predefined values with the largest one being 31- Tproc,1 . If sensing is specified with [n+TA, n+TB], and TA =1, TB can be a fixed value smaller than 31- Tproc,1 being or a configured from a list with the largest one being 31- Tproc,1.

Proposal 13: For contiguous partial sensing to support resource selection for aperiodic traffic, the sensing window size can be fixed with a value smaller 31- Tproc,1  or configured from a set of predefined values with the largest one being 31- Tproc,1

## Impact of SL-DRX on UE sensing

SL DRX is also a mechanism for SL power saving.  A UE receives SL signals only when DRX is on. One question is whether UE performs partial sensing during the DRX off period. If the DRX on/off setting and partial sensing are independent from each other, not performing partial sensing during the DRX off period will impact the resource selection performance significantly. However, performing partial sensing during the DRX off will affect the power saving efficiency. Therefore, it would be better to align partial sensing with DRX on period as much as possible assuming that partial sensing is also performed during the DRX off period.

Proposal 14: Allow UE to perform sensing during the DRX inactive period.

Since DRX is configured by upper layer, it would be easier if sensing occasions are configured by upper layer with a bitmap of k. On the other hand, it is currently not clear about the DRX specification in RAN2. If alignment is difficult, then sensing during the DRX period off may reduce the power saving efficiency. To mitigate this issue, two minimum values on Y can be configured, one for DRX on period and one for DRX off period, so that the UE can adjust the sensing window on the occasions on either DRX on or DRX off period. In addition, aforementioned criterion on the sensing results can be adopted for the UE to change the size of sensing slots to balance the sensing reliability and power saving gain.

On the other hand, to reduce the sensing in DRX inactive period, the number of sensing occasions in DRX inactive periodic can be limited with another configuration of multiple number of sensing occasions kmax,DRX. For example, if the kmax,DRX = 1, the sensing occasion for each periodicity is limited to one, as long as one sensing occasion corresponding to ik’, for a periodicity for a slot within Y slots can be performed at DRX active period, the sensing is disabled in the DRX inactive period for that periodicity for that slot. If no sensing occasion can be found in DRX active period for a periodicity for a slot within Y slots, sensing can be done at the sensing in DRX inactive period on the sensing occasion corresponding to the smallest k’ with ik’=1 in the configured sensing occasions for periodic partial sensing

Proposal 15: Different settings can be configured for periodic partial sensing in DRX active and inactive periods, e.g., maximum number of sensing occasions.

It would also be beneficial if there is a certain alignment on the DRX between the transmitting UE and receiving UE so that the receiving UE will achieve better power saving with long DRX inactive time without have much packet loss.  Such alignment can be achieved for the periodic traffic for which the transmitting UE knows traffic arrival time for a certain duration. The transmitting UE can signal the data periodicity to the receiving UE, e.g., in the second stage SCI or some MAC CE, or the approximate time of the next transmission from the slot where such information is delivered. The receiving UE can align the DRX configurations with the data transmission periodicity for the data reception. Note that the periodicity is not the exact transmission slot, rather approximation transmission time. For example, it can be resource selection triggering time n, while the exact transmission is on a slot within the resource selection window [n+T1, n+T2], or [ty0, ty0+Y-1] for periodic partial sensing, which is unknown in advance. However, the approximate next transmission time, e.g., the time difference between the next data triggering time and the timing of the slot this time difference is delivered is good enough to have a certain DRX alignment. The timing difference can also be quantized to a list values so that it can be signaled with few bits.

Proposal 16: For periodic traffic, the transmitting UE can signal the time when the receiving UE expects the next transmission so that the receiving UE can align the DRX with the data reception for better power saving.

# Conclusion

Random resource allocation and partial sensing for power saving were discussed. We propose the following:

Proposal 1: UE can switch between partial/full sensing and random resource selection when certain conditions are satisfied, e.g., a threshold on the available resource ratio or CR. A timer or counter can be preconfigured to switch back to partial/full sensing.

Proposal 2: When UE performs random resource selection, re-evaluation and pre-emption are disabled.

Proposal 3:  For random resource selection with periodic traffic, the transmitting UE uses two different resources for two consecutive transmissions.

Proposal 4:

- For SL transmissions, increase the priority of UEs performing random resource selection

  - A 1-bit field in the SCI indicates that the UE is performing random resource selection.

  - The offset of priority increase is applied either

    - Option 1: by the detecting UE

    - Option 2: in the SCI priority field directly

  - Details FFS

Proposal 5: Partition the resource pool into several resource subsets, i.e., sub-pools, UE with random resource selection selects the resource from a sub-pool.

Proposal 6: Partition the resource pool into several resource subsets, i.e., sub-pools, and assign a priority threshold on the sub-pool for random resource selection

Proposal 7: Specify a new list of X for partial sensing or set new rules for partial sensing on X with the existing list sl-TxPercentageList.

Proposal 8: For sensing occasions in periodic partial sensing, maximally more than two sensing occasions can be configured. Besides the most recent sensing occasions, the additional sensing occasions are not limited to the last periodic sensing occasion prior to the most recent and can be configured on the one other than the last periodic sensing occasions prior to the most recent.

Proposal 9: The set of values for sensing occasions can be represented with a bitmap.  For better power saving, maximum number of sensing occasions should be specified.

Proposal 10: For contiguous partial sensing to support resource selection for periodic traffic, to achieve better power saving, the sensing start point is specified from a slot later than ty0-31 can be configured from a predefined range or a predefined list.

Proposal 11: For contiguous partial sensing to support resource selection for periodic traffic, a minimum and a maximum on TB, TB,min  and TB,max, UE is allowed to continue sensing after n+ TB,min  , and performs resource selection on any slot in n+ [TB,min  TB,max ].

Proposal 12: The candidate resource set (SA) can be initialized based on the Y or a subset of Y candidate slots.

Proposal 13: For contiguous partial sensing to support resource selection for aperiodic traffic, the sensing window size can be fixed with a value smaller 31- Tproc,1  or configured from a set of predefined values with the largest one being 31- Tproc,1

Proposal 14: Allow UE to perform sensing during the DRX inactive period.

Proposal 15: Different settings can be configured for periodic partial sensing in DRX active and inactive periods, e.g., maximum number of sensing occasions.

Proposal 16: For periodic traffic, the transmitting UE can signal the time when the receiving UE expects the next transmission so that the receiving UE can align the DRX with the data reception for better power saving.

# References

RP-201385, “WID revision: NR sidelink enhancement”, RAN#88-e, June 29-July 3, 2020.

R1-2005295, “Power consumption reduction for sidelink resource allocation”, FUTUREWEI, RAN1#102e, Aug. 17 - 28, 2020.

R1-2100047, “Views on resource allocation enhancements for sidelink communication,” FUTUREWEI, RAN1#104-e, Jan. 25-Feb. 5, 2021.

RP-202846, “WID revision: NR sidelink enhancement”, RAN#90-e, Dec. 7-11, 2020.

R1-2104093, “FL summary for AI 8.11.1.1 - resource allocation for power saving (final)”, OPPO, RAN1#104b-e, Apr. 12-20, 2021.

# Appendix

The agreements from RAN1#105-e are listed below.

Agreement:

- For the set of Preserve values in periodic-based partial sensing,

  - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-) configured set sl-ResourceReservePeriodList.

  - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList

    - This is per mode 2 Tx resource pool (pre-)configuration

Agreement:

- In periodic-based partial sensing for resource (re)selection, the UE at least monitors in periodic sensing occasion(s) for a given reservation periodicity before the first slot of the selected Y candidate slots subject to processing time restriction for the identification of candidate resources.

  - The processing time restriction includes Tproc,0SL  and Tproc,1SL.

  - Aspects relating to sensing during SL DRX are to be discussed separately

- Relationship to re-evaluation and pre-emption operation for periodic-based partial sensing to be discussed separately

  - FFS details including whether monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection or re-evaluation and pre-emption checking

Agreement:

- For the k value in periodic-based partial sensing for resource (re)selection,

  - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction.

  - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value

    - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included.

    - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions)

    - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring.

  - FFS relationship between periodic-based partial sensing occasions and SL-DRX

  - Note:

    - This is for the case when the resource (re)selection triggering slot n is expected by UE

Agreement:

- For random resource selection,

  - Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation.

  - SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any)

    - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected.

- FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions).

  - Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.).

Agreement:

In contiguous partial sensing for resource (re)selection, TA and TB values can be zero, positive or negative

- TA and TB values or range depend on different operating scenarios or conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc)

  - FFS details

- FFS: details of how periodic-based partial sensing and contiguous partial sensing are used for re-evaluation and pre-emption checking. Including how to reduce UE’s power consumption (caused by additional sensing operation of re-evaluation/pre-emption) after its resource selection, with the considerations of different operating scenarios or conditions (e.g., pre-emption enabled/disabled, HARQ-ACK enabled/disabled, etc).