3GPP TSG RAN WG1 Meeting #106-e	R1-2107099

e-Meeting, Aug 16th – Aug 27th, 2021

Agenda Item:	8.2.3

Source:	FUTUREWEI

Title:	Resource mapping and sequences for PUCCH formats 0/1/4 for 52.6GHz to 71GHz

Document for:	Discussion and decision

# Introduction

A working item (WI) has been approved with the aim of extending NR up to 71 GHz by 3GPP TSG RAN Meeting #90 [1]. As a part of the objectives of the WI, one sub-agenda is to support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation.

The RAN1#105-e meeting has mainly covered the issues involving the maximum number of RBs for enhanced PF0/1/4, configuration granularity on number of RBs, RE mapping for enhanced PF0/1/4 for 120 kHz SCS, sequence construction for enhanced PF0/1, rate matching for enhanced PF4, and PUCCH resource sets prior to RRC configuration [2], [3]. The online GTW time was mainly allocated on the discussion of the issue of RE mapping for enhanced PF0/1/4 for 120 kHz SCS, during which limited input for sub-PRB resource mapping (Alt-2) was provided by proponents of sub-PRB resource mapping for comparison over the full-RE resource mapping (Alt-1) in terms of the MIL and UE multiplexing performance.

Issues related to  the maximum number of RBs such as the selection between a single long sequence (Alt-1) and repeated sequence with the cycling of cyclic shifts (Alt-2) and rate matching [4], [5], were deferred for lack of RAN4 feedback on recommended power levels by RAN1#105-e, and the feedback is now available. In this document, we continue the discussion on the recommendation for the maximum number of RBs and provide our views on the sequence selection and alternatives for rate matching. We also provide further discussion on the PUCCH resource set configuration prior to RRC configuration.

# Discussion

## Maximum number of RBs

From RAN1#104b-e meeting Agreement: - The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least: - 12 RBs for 120 kHz SCS - 3 RBs for 480 kHz SCS - 2 RBs for 960 kHz SCS - FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

Extending the number of RBs for PF0/1/4 to allow multi-RB for each format was among the initial goal of this WI and had been the key issue of discussion till the RAN1#104b-e meeting, during which an LS was sent to RAN4 on the inquiry of whether RAN4 will specify UE power class(es) with larger values for maximum UE conducted power and maximum UE EIRP [6]. In the reply RAN4 sent back after RAN1-#105-e meeting, reference values of FR2-1 the bands on maximum EIRP, maximum TRP, among answers to RAN1’s inquiry are provided in [7], although for band beyond 52.6GHz there are no agreed values at RAN4 since it is still under discussion. Thus, RAN4 asks RAN1 to take the information on FR2-1 bands into account. For power classes including handheld UE and vehicular UE, the reference values for maximum EIRP and maximum TRP are both 43dBm and 23dBm, respectively. With the large value of 43dBm, it is unlikely that the calculations for maximum allowed bandwidth is limited by maximum EIRP (but instead UEP) based on the equation provided by RAN1 FL for this agenda item as listed in [5]

Maximum PUCCH BW MHz=min⁡{10UEEIRP-TxBF-710, 10UEP-CM-710}

- Based on the maximum EIRP = 43dBm in the feedback (which can be used in US but not necessarily ETSI), if RAN1 continues to use the conducted power (25dBm) as recommended by FL in email discussions, and CM takes value from the set {1, 2, 3} (dB), the set of maximum numbers for 120kHz/480kHz/960kHz correspondingly are {35/9/5, 28/7/4, 22/6/3} (RBs); If RAN1 modifies the conducted power with TRP (23dBm), and CM takes value from {1, 2, 3} (dB), the set of maximum numbers for 120kHz/480kHz/960kHz are {22/6/3, 18/5/3, 14/4/2} (RBs). Note that there is no consensus on which of those CM values should be used for the calculations, although a CM value larger than 3dB is unlikely based on simulations.

- The values provided in the above sets can both serve as candidate sets for down-selection in this meeting. It is seen that the recommended RB values, i.e., 22/6/3 in our previous contribution document [6] are within the set no matter if conducted power or maximum TRP is considered by RAN1. Thus, we keep the original recommendation of our previous proposal as the maximum numbers of RBs foreach SCS .

Proposal 1. Support 22RBs/6RBs/3RBs as the maximally allowed numbers for 120kHz/480kHz/960kHz for enhanced PF0/1/4 of B52/FR2-2, taken the RAN4 feedback further into account.

## Resource mapping for Enhanced PF0/1/4 for 120kHz SCS

From RAN1#105-e meeting Agreement: - For 120 kHz SCS: - Support at least Alt-1 for enhanced PF0/1 for both PUCCH resources before and after dedicated PUCCH resource configuration - FFS: Whether or not Alt-2 is additionally supported for PF0/1 for either or both of the following: - PUCCH resources before dedicated PUCCH resource configuration - PUCCH resources after dedicated PUCCH resource configuration - FFS: Supported RE mapping scheme(s) amongst {Alt-1, Alt-2} for enhanced PF4 including design details - Notes: - Alt-1 = all REs within each RB are mapped - Alt-2 = a subset of REs within each RB are mapped (sub-PRB interlaced mapping) - Which RE mapping scheme(s) to support for PF0/1/4 to be concluded in RAN1#106 - Note: No further enhancements on RB shortage issue and frequency hopping distance issue should be considered for PUCCH resource sets prior to RRC configuration. Agreement: For DMRS of enhanced PF4, a Type-1 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4 (Alt-1 in agreement from RAN1#104-e). Agreement: User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion.

On the issue of resource mapping for enhanced (multi-RB) PF0/1/4, it was agreed during RAN1#104-e meeting that for 480kHz/960kHz SCS, all REs within each RB are mapped and sub-PRB interlaced mapping is not considered further. The remaining issue of resource mapping for 120kHz SCS was discussed during RAN1#104b-e and RAN1#105-e meetings and became the issue that consumed most of the online and email discussion time during the latter meeting.

For PF0/1, the latest agreement on the issue decides to support at least Alt-1 (all REs within each RB are mapped) for both PUCCH resources before and after dedicated PUCCH resource configuration.  Whether to support Alt-2 (sub-PRB interlaced mapping) additionally is the current study point.

For PF4, it was agreed that  DMRS Type-1 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is used, but it has not been agreed whether all REs within each RB are mapped or to adopt sub-PRB resource mapping, thus Alt-1 and Alt-2 remain viable candidates.

Three aspects have been the main discussion points for comparison between the two alternatives: coverage, user multiplexing, and implementation complexity. One of the main benefits that was claimed to associate with Alt-2 is that it could offer better user multiplexing capability with FDM than Alt-1 with CDM. However, given that it was agreed user-multiplexing can be considered but lower priority compared to the maximum isotropic loss for PUCCH as a design criterion, the focus of comparison for now is coverage instead of user multiplexing, as presented in the next subsections.

Regarding implementation complexity, it was identified that additional support of Alt-2 for 120kHz SCS will lead to gNB and UE implementation complexity (Alt-1 is now supported). Therefore, our view is that for PF0/1, only if Alt-2 provides notable MIL gain over Alt-1 should it be worthwhile to consider support two schemes for 120kHz SCS. While for PF4, since there is neither of the two alternatives was agreed and it is favorable to not support two schemes at the same time, we recommend supporting one alternative that has better MIL for more cases.

Proposal 2. For PF0/1, consider support Alt-2 additionally for 120kHz SCS only if it provides notable MIL gain over Alt-1; for PF4, support one alternative that has better MIL for more cases, and we are inclined to Alt-1 based on the simulation results.

Another issue is that for PF0/1 if Alt-2 is additionally supported, whether to support it for either or both the PUCCH resources before dedicated PUCCH resource configuration and PUCCH resource after dedicated PUCCH resource configuration. Note that if Alt-2 is supported for initial access, it would be mandatory for the UE to support. Our view is that Alt-2 can be considered for before and after dedicated PUCCH resource configurations only if notable MIL gain is observed. If there is only marginal MIL gain or no MIL gain over Alt-1, Alt-2 should not be supported for either before or after PUCCH resource allocation.

Proposal 3. Alt-2 can be considered for before and after dedicated PUCCH resource configurations only if notable MIL gain is observed. If there is only marginal MIL gain or no MIL gain over Alt-1, Alt-2 should not be supported for either before or after PUCCH resource allocation.

### PUCCH Format 0

The link-level simulations are configured based on the parameters specified in Section 2.3 of R1-2102127. Under 120kHz SCS, 256 RBs (400MHz), one OS, one-bit ACK, the corresponding SNRs at 1% miss detection and cubic metrics are summarized in Table 1 and MIL performance results are plotted in Figures 1 and 2, under the coarser granularity with number of RBs in set {1, 2, 4, 8, 16, 22}, as we proposed in the last contribution document [5]. Note that the trends in general regarding comparisons between full-RE and sub-PRB that are revealed in this section do not alter if continuous RB is eventually agreed instead of coarser RB granularity. It is seen from Table 1 that for PF0, the sub-PRB can offer better detection performance, and the gain grows if sparser interlaces are utilized and the comb-6 attains the best performance for all tested RBs. Also, the full-RE and sub-PRB resource mappings lead to very close CM such that it is not the major factor to consider for the comparison. The maximum gain that sub-PRB can attain is around 2dB for the 4 RBs case, but the sub-PRB is not guaranteed to have MIL gain for all cases under PF0.

Observation 1. For PF0, sub-PRB resource mapping can attain MIL gains with comb-2, comb-4, and comb-6 interlaces, and the gain grows if sparse interlaces are utilized.

Proposal 4. For PF0, sub-PRB resource mapping can provide marginal MIL gains for 120kHz SCS, thus can be considered for both before and after dedicated PUCCH resource configurations.

Table 1. SNR@ 1% ACK miss rate and CM for SCS 120kHz, 256RBs, 1 OS, PF0, coarse RB granularity

|  | RB = 1 | RB = 2 | RB = 4 | RB = 8 | RB = 16 | RB = 22 |
| --- | --- | --- | --- | --- | --- | --- |
| SNR@1% ACK miss (Full-RE) | 6.8 | 4.2 | 4.3 | 1.4 | -0.6 | -2.6 |
| SNR@1% ACK miss (Comb-2) | 6.4 | 5.5 | 2.9 | 0.2 | -1.8 | -2.4 |
| SNR@1% ACK miss (Comb-4) | 6.4 | 4.2 | 2.1 | -0.5 | -0.4 | -3.4 |
| SNR@1% ACK miss (Comb-6) | 5.4 | 3.7 | 2.0 | -0.6 | -2.1 | -3.5 |
| CM (Full-RE) | 1.15 | 0.91 | 1.06 | 1.14 | 1.24 | 2.19 |
| CM (Comb-2) | 1.10 | 0.92 | 1.08 | 1.14 | 1.23 | 2.19 |
| CM (Comb-4) | 1.12 | 0.94 | 1.08 | 1.14 | 1.23 | 2.18 |
| CM (Comb-6) | 1.10 | 0.94 | 1.07 | 1.14 | 1.23 | 2.19 |

Figure 1. MIL performance for PF0 with coarser RB granularity under US regional regulations

Figure 2. MIL performance for PF0 with coarser RB granularity under South Korea regional regulations

### PUCCH Format 1

Under 120kHz SCS, 256 RBs (400MHz), 14 OS, two-bit ACK, the corresponding SNRs at 1% miss detection and cubic metrics are summarized in Table 2 and MIL performance results are plotted in Figures 3 and 4, under the coarser granularity. It is seen from Table 2 that for PF1, the full-RE offers better detection performance for almost all the tested cases. The full-RE and sub-PRB resource mappings lead also to very close CM. It is shown by the figures that the maximum gain that full-RE can attain is above 4dB for the one-RB case, and increasing the sparsity of the sub-PRB interlace mostly does not lead to MIL increment as with the PF0 cases. Thus, it does not seem reasonable to consider sub-PRB for PF1.

Observation 2. For PF1, full-RE attains MIL gains over the sub-PRB resource mapping for almost all tested number of RBs.

Proposal 5. Support only the full-RE resource mapping for PF1. Sub-PRB resource mapping for PF1 is not considered due to inferior MIL performance.

Table 2. SNR@ 1% ACK miss rate and CM for SCS 120kHz, 256RBs, 14 OS, PF1, coarse RB granularity

|  | RB = 1 | RB = 2 | RB = 4 | RB = 8 | RB = 16 | RB = 22 |
| --- | --- | --- | --- | --- | --- | --- |
| SNR@1% ACK miss (Full-RE) | -5.8 | -7.0 | -8.0 | -8.7 | -10.3 | -10.6 |
| SNR@1% ACK miss (Comb-2) | -5.0 | -6.1 | -6.9 | -8.5 | -10.1 | -10.7 |
| SNR@1% ACK miss (Comb-4) | -1.0 | -3.7 | -6.0 | -8.3 | -9.5 | -10.6 |
| SNR@1% ACK miss (Comb-6) | -0.9 | -4.1 | -6.1 | -8.0 | -9.3 | -10.6 |
| CM (Full-RE) | 1.01 | 1.04 | 1.61 | 1.18 | 1.07 | 1.69 |
| CM (Comb-2) | 1.03 | 1.04 | 1.62 | 1.18 | 1.07 | 1.68 |
| CM (Comb-4) | 1.00 | 1.03 | 1.63 | 1.18 | 1.08 | 1.69 |
| CM (Comb-6) | 1.01 | 1.02 | 1.63 | 1.18 | 1.08 | 1.69 |

Figure 3. MIL performance for PF1 with coarser RB granularity under US regional regulations

Figure 4. MIL performance for PF1 with coarser RB granularity under South Korea regional regulations

### PUCCH Format 4

Under 120kHz SCS, 256 RBs (400MHz), 14 OS, 22 CSI bits, the corresponding SNRs at 1% BLER and cubic metrics are summarized in Table 2 and MIL performance results are plotted in Figures 3 and 4, under the coarser granularity. Unlike the non-coherent PF0, the PF1/PF4 use DMRS for channel estimation and both the UCI and the associated DMRS resources/tones adopt the sub-PRB mapping.

It is seen from Table 2 that for PF4, the full-RE offers better detection performance for all the tested cases. The full-RE and sub-PRB resource mappings, still, lead to very close CM. It is shown by the figures that for PF4 the MIL gains that full-RE attains over the sub-PRB counterparts are even larger than the PF1, and changing the sparsity of the sub-PRB interlace does not help MIL performance. Thus, it does not recommend only support full-RE mapping for PF4.

Table 3. BLER 1% and CM for SCS 120kHz, 256RBs, 14 OS, PF4, coarse RB granularity

|  | RB = 1 | RB = 2 | RB = 4 | RB = 8 | RB = 16 | RB = 22 |
| --- | --- | --- | --- | --- | --- | --- |
| SNR@BLER 1% (Full-RE) | 3.9 | -5.0 | -9.2 | -13.2 | -15.4 | -16.2 |
| SNR@BLER 1% (Comb-2) | 5.8 | -1.1 | -7.8 | -13.1 | -14.5 | -14.7 |
| SNR@BLER 1% (Comb-4) | 9.5 | 1.2 | -5.1 | -11.4 | -12.4 | -11.4 |
| SNR@BLER 1% (Comb-6) | 11.7 | 2.0 | -3.1 | -8.9 | -9.6 | -9.8 |
| CM (Full-RE) | 1.73 | 1.38 | 1.88 | 1.73 | 1.73 | 1.64 |
| CM (Comb-2) | 1.74 | 1.36 | 1.90 | 1.72 | 1.74 | 1.64 |
| CM (Comb-4) | 1.71 | 1.36 | 1.88 | 1.72 | 1.74 | 1.64 |
| CM (Comb-6) | 1.72 | 1.35 | 1.88 | 1.72 | 1.74 | 1.64 |

Observation 3. For PF4, full-RE attains MIL gains over the sub-PRB resource mapping for the tested number of RBs.

Proposal 6. Support only the full-RE resource mapping for PF4. Sub-PRB resource mapping for PF4 should not be supported  for due to its inferior MIL performance.

Figure 5. MIL performance for PF1 with coarser RB granularity under US regional regulations

Figure 6. MIL performance for PF1 with coarser RB granularity under South Korea regional regulations

## Rate matching for enhanced PF4

From RAN1#105-e meeting Agreement: - The configured number of RBs for enhanced PF 0/1/4 is denoted NRB - The minimum value of NRB is 1 for PF 0/1/4 for all subcarrier spacings - The maximum value of NRB depends on subcarrier spacing - FFS: maximum value for each SCS and each of PF0/1/4 - FFS: Allowed values of NRB within the [min/max] range - FFS: Details of indication of NRB by cell-specific (for PF0/1) and dedicated signaling (PF0/1/4) - FFS: Whether or not multiplexing of users with misaligned RB allocations is supported, where “misaligned” also includes users with different # of RBs. - For PF4: - The actual number of RBs used for a PUCCH transmission is equal to NRB, i.e., the actual number of RBs does not vary dynamically based on PUCCH payload - NRB fulfils the following: N=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers - Note: if frequency hopping is enabled, NRB is the number of RBs per hop Note: decisions on the maximum value of NRB for each SCS and PUCCH format shall take into account link budgets Companies were invited to provide their views on the issue of rate matching for PF4 based on the table below during the email discussion. FL Recommendation Further discuss rate matching for enhanced (multi-RB) PF4 under the constraint that the actual number of RBs does not vary dynamically based on PUCCH payload (as agreed in RAN1#104-e).

During RAN1#105-e, the moderator draws companies’ attention to and encouraged further discussion on the issue of rate matching for the PF4, based on four alternatives proposed by the company. The rate matching for PF4 issue has been decomposed into two questions:

1. Whether to support the same restriction (upper limit) on the UCI payload as in Rel-15/16 PF4 or support different restriction (upper limit);

2. Whether to use the existing rate matching to the configured number of RBs N_RB (similar to Rel-16 rate matching to the fixed number of RBs N = 10/11 of an interlace for PF3) or to allow a new rate matching mechanism, e.g., some form of repetition.

Despite that the two questions and the down-selections are supposed to be addressed for enhanced PF4 for B52, it does not seem to be straightforward to solve them based on the current progress of the agenda item. The original intention of this AI is to enhance PF0, PF1, and PF4 for better coverage, because the prior releases only allow for 1 RB for these three formats. PF2 and PF3 are left unenhanced because they allow up to 16 RBs. However, this may lead to, or is already leading to a larger scale issue that whether PF2 or PF3 are still needed to perform the same role as they were performing for prior releases. For example, if PF3 is still expected to accommodate a large UCI payload, we find it less motivated to remove the restriction (upper limit) on the UCI payload for PF4. Otherwise, if PF4 is enhanced further to address both moderate and large payload, there is less need for PF3. While in order not to waste more standard effort on the enhancement of PF3 for B52, it is much more convenient to keep the same restriction (upper limit) on the UCI payload for PF4.

Observation 4: If PF3 is still expected to accommodate large UCI payload, we find it less motivated to remove the restriction (upper limit) on the UCI payload for PF4.

Proposal 7: From the standard effort perspective, it is recommended to keep the same restriction (upper limit) on the UCI payload for PF4, and use PF3 for a larger UCI payload, similar to Rel-16.

Regarding whether to allow repetition as a new rate matching mechanism, our understanding is that the concern comes from the highlighted text in the above agreement, i.e.,

“The actual number of RBs used for a PUCCH transmission is equal to NRB, i.e., the actual number of RBs does not vary dynamically based on PUCCH payload”,

because in case that the configured value NRB has to be large for higher payload for coverage reason, and the small payload is agreed to use the same large NRB configured, a waste of resources is incurred since the code-rate becomes too low, and therefore, repetition seems helpful to improve performance for the small payload transmissions. Considerations of whether to explore such potential gains with repetition include, on the one hand, performing rate matching over all PRBs rather than repetition is adopted for NR-U enhanced PF3 based on interlace, despite that code-rate there can also be low. On the other hand, the potential gain of using repetition is expected to be larger if the final agree value of maximum NRB is larger than 16 RBs, which is now the upper limit for Rel-16 PF1/PF3, and this situation is likely given the RAN4 feedback for RAN1 to take into account. Therefore, we recommend allowing repetition as a new rate matching mechanism only if the agreed value of maximum NRB is much larger than 16 for the smaller payload case. Also, considering the standard effort and potentially larger-scale issues to be resolved, there seems no benefit of further investigate other rate matching mechanisms beyond repetition despite the potentially agreed value of maximum NRB.

Proposal 8: Consider repetition as a new rate matching mechanism for the small payload case only if the final agreed value of maximum NRB is larger than 16.

Proposal 9: For standard effort consideration, not to further investigate other rate matching mechanisms except for repetition despite the potentially agreed value of maximum NRB .

## PUCCH Resource Sets Prior to RRC Configuration

From RAN1#105-e meeting Proposal Agree to the following - For a PUCCH resource set prior to RRC configuration, down-select to one of the following alternatives on the number of RBs, N_RB: - Alt-1: N_RB is configured via SIB - Alt-2: N_RB is hardwired by specification, and is potentially different for different rows of the PUCCH configuration table - FFS: Supported values of N_RB Agreement: - For 120 kHz SCS: - Support at least Alt-1 for enhanced PF0/1 for both PUCCH resources before and after dedicated PUCCH resource configuration - FFS: Whether or not Alt-2 is additionally supported for PF0/1 for either or both of the following: - PUCCH resources before dedicated PUCCH resource configuration - PUCCH resources after dedicated PUCCH resource configuration - FFS: Supported RE mapping scheme(s) amongst {Alt-1, Alt-2} for enhanced PF4 including design details - Notes: - Alt-1 = all REs within each RB are mapped - Alt-2 = a subset of REs within each RB are mapped (sub-PRB interlaced mapping) - Which RE mapping scheme(s) to support for PF0/1/4 to be concluded in RAN1#106 - Note: No further enhancements on RB shortage issue and frequency hopping distance issue should be considered for PUCCH resource sets prior to RRC configuration.

A few issues relating to PUCCH resource sets prior to RRC configuration were discussed during RAN1#105-e, among which the issues of further enhancements on RB shortage and frequency hopping distance were agreed not to be pursued by the highlighted Note in the agreement above. The main remaining issue is the indication of the number of RBs, i.e., whether N_RB is configured via SIB or is hardwired by specification and is potentially different for different rows of the PUCCH configuration table. Another issue raised by a company is whether a UE-specific design for PRB number determination prior to RRC configuration should be considered to allow, for example, different numbers of PRBs for different UE types with different maximum transmission power.

Configuring N_RB is a flexible option taken into consideration different regulatory regions and use scenarios comparing with hardwired by specification by allowing different values for different rows of the PUCCH configuration table. Besides, with the highlighted Note which disallows further enhancement to address the RB shortage issue that correlates with enhanced multi-RB PUCCH formats, it is unsure at this stage whether the original table can be reused for different N_RB values being hardwired into without causing an issue. Therefore, we support the Alt-1 to configure N_RB through SIB1 prior to RRC configuration.

Observation 5: Given that further enhancement to address the RB shortage issue is not allowed, it may not be suitable to further consider using the PUCCH configuration table to hardwire the N_RB into different rows.

Proposal 10: Support the Alt-1 to configure N_RB through SIB1 prior to RRC configuration.

Regarding whether a UE-specific signaling is needed to allow UE-specific number of RBs, we view it beneficial since such design caters to various UE types and geometry that UEs operate with, i.e., UEs far away from the gNB probably needs more PUCCH RBs for coverage reason. Otherwise, if UE-specific signaling is not introduced, all UEs may have to allocate an unnecessarily large number of RBs to satisfy the worst-case scenario, which is clearly a waste of resources. The downside of allowing UE-specific signaling, on the other hand, is the requirement of capability signaling from UE, such that gNB can indicate to the UEs with different numbers of RBs by the PUCCH resource indicator field of a scheduling DCI.

## Sequence Construction for Enhanced PUCCH formats 0/1

From RAN1#104-e meeting Agreement: - For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives: - Alt-1: A single sequence of length equal to the total number of mapped REs of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured. - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction: - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact Proposal 3 For enhanced PF 0/1 for 120 kHz, down-select to one of Alt-1 and Alt-2. The decision should be based on consideration of coverage vs. user multiplexing. Agreement: User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion.

The discussion on the issue of sequence construction for enhanced PF0/1, similar to the issue of resource mapping, has started at RAN1#104-e, but due to splits of company views down-selection still has not been made to one of the two alternatives, i.e., Alt-1 using a single sequence of length equal to the total number of mapped REs of the PUCCH resource; and Alt-2 using a single sequence of length equal to the number of mapped REs per RB of the PUCCH resource and the sequence is repeated in each RB through the cycling of cyclic shifts across RBs in a similar way as for Rel-16.

An intermediate Proposal 3 was made during RAN1#105-e and an FL recommendation was followed up on waiting for RAN4 feedback (which is now available) to decide between Alt-1 and Alt-2, therefore, there is now little reason to let this issue be consuming more standard effort, provided with sufficient observations and conclusions from the simulation results gathers during multiple meetings.

In our view, the issue can at least be simplified with better focus

- UE multiplexing capability comparison between the two alternatives are still alive and given weight to during the RAN1#105-e email discussions, even if it was already agreed in the previous meeting that user-multiplexing can be considered but as a lower priority compared to the maximum isotropic loss for PUCCH as a design criterion

- CM comparison between the two alternatives was not clearly identified as not being an independent factor from the MIL (but only a part of MIL) during the email discussion, which unnecessarily complicates the comparison

- Given the above two points are just minor (which should be removed soon to focus on MIL), the existing summarized simulation results show that both Alt-1 has clear merits in terms of MIL, i.e., Alt-1 has better MIL than Alt-2 for 480/960kHz SCS in all regions and better MIL than Alt-2 for 120kHz SCS in regions except for cases when N_RB ≥ 12

Therefore, only in the case that the number of RBs is no less than 12 will Alt-2 have merit in terms of MIL. In this case N_RB ≥ 12, given that both alternatives are seen as extensions of Rel-15 or Rel-16 and not different in spec complexity, there seems no reason not to not consider Alt-2 for cases it assoicates with better MILs, even if the original intention was to down-select from the two (for all scenarios). Therefore, we recommend that for cases with N_RB < 12, only Alt-1 one is supported, and consider Alt-2 as the only supported alternative for cases with N_RB ≥ 12, unless it is strongly favored by the majority that only one alternative should be supported for all N_RB values, in which case we slightly lean towards supporting Alt-1 only. Meanwhile, it can be beneficial for the meeting to identify a bar value of MIL gain that can be viewed as large enough such that if the Alt-2 attains the bar for cases N_RB ≥ 12, it becomes the straight-forward option despite if any potential downside of supporting different alternative for different cases is associated.

Observation 6: This issue of sequence construction for PF0/1 can be simplified by not to further discuss UE-multiplexing comparison between Alt-1 and Alt-2 and not treating CM as an independent factor from MIL, but instead focus solely on MIL.

Proposal 11: For cases with N_RB < 12, only Alt-1 one is supported, and consider Alt-2 as the only supported alternative for cases with N_RB ≥ 12, unless it is strongly favored by the majority that only one alternative should be supported for all N_RB values, in which case we slightly lean towards supporting Alt-1 only.

# Conclusion

This document continues discussion for enhanced PUCCH formats on the maximum allowable number of RBs, down-selection for resource mapping between full-RE and sub-PRB, rate matching, PUCCH resource sets prior to RRC configuration, and sequence construction.

Observation 1. For PF0, sub-PRB resource mapping can attain MIL gains with comb-2, comb-4, and comb-6 interlaces, and the gain grows if sparse interlaces are utilized.

Observation 2. For PF1, full-RE attains MIL gains over the sub-PRB resource mapping for almost all tested number of RBs.

Observation 3. For PF4, full-RE attains MIL gains over the sub-PRB resource mapping for the tested number of RBs.

Observation 4: If PF3 is still expected to accommodate large UCI payload, we find it less motivated to remove the restriction (upper limit) on the UCI payload for PF4.

Observation 5: Given that further enhancement to address the RB shortage issue is not allowed, it may not be suitable to further consider using the PUCCH configuration table to hardwire the N_RB into different rows.

Observation 6: This issue of sequence construction for PF0/1 can be simplified by not to further discuss UE-multiplexing comparison between Alt-1 and Alt-2 and not treating CM as an independent factor from MIL, but instead focus solely on MIL.

Proposal 1. Support 22RBs/6RBs/3RBs as the maximally allowed numbers for 120kHz/480kHz/960kHz for enhanced PF0/1/4 of B52/FR2-2, taken the RAN4 feedback further into account.

Proposal 2. For PF0/1, consider support Alt-2 additionally for 120kHz SCS only if it provides notable MIL gain over Alt-1; for PF4, support one alternative that has better MIL for more cases, and we are inclined to Alt-1 based on the simulation results.

Proposal 3. Alt-2 can be considered for before and after dedicated PUCCH resource configurations only if notable MIL gain is observed. If there is only marginal MIL gain or no MIL gain over Alt-1, Alt-2 should not be supported for either before or after PUCCH resource allocation.

Proposal 4. For PF0, sub-PRB resource mapping can provide marginal MIL gains for 120kHz SCS, thus can be considered for both before and after dedicated PUCCH resource configurations.

Proposal 5. Support only the full-RE resource mapping for PF1. Sub-PRB resource mapping for PF1 is not considered due to inferior MIL performance.

Proposal 6. Support only the full-RE resource mapping for PF4. Sub-PRB resource mapping for PF4 should not be supported due to its inferior MIL performance.

Proposal 7: From the standard effort perspective, it is recommended to keep the same restriction (upper limit) on the UCI payload for PF4, and use PF3 for a larger UCI payload, similar to Rel-16.

Proposal 8: Consider repetition as a new rate matching mechanism for the small payload case only if the final agreed value of maximum NRB is larger than 16.

Proposal 9: For standard effort consideration, not to further investigate other rate matching mechanisms except for repetition despite the potentially agreed value of maximum NRB .

Proposal 10: Support the Alt-1 to configure N_RB through SIB1 prior to RRC configuration.

Proposal 11: For cases with N_RB < 12, only Alt-1 one is supported, and consider Alt-2 as the only supported alternative for cases with N_RB ≥ 12, unless it is strongly favored by the majority that only one alternative should be supported for all N_RB values, in which case we slightly lean towards supporting Alt-1 only.

# References

- RP-202925, “Revised WID: Extending current NR operation to 71 GHz”, CMCC, RAN#90-e.

- “RAN1 Chairman’s Notes”, 3GPP RAN1#105-e.

- R1-2106288, “FL Summary 2 for [105-e-NR-52-71GHz-02 Email discussion/approval]”, 3GPP RAN1#105-e.

- “RAN1 Chairman’s Notes”, 3GPP RAN1#104-e.

- R1-2103784, “FL Summary for Enhancements for PUCCH formats 0/1/4”, 3GPP RAN1#104b-e.

[6]   R1-2104061, “LS to RAN4 on maximum UE conducted power and maximum UE EIRP for operation in the 52.6 – 71 GHz band”, 3GPP RAN1#104b-e.

[7] R4-2107950, “LS reply on maximum UE EIRP and conducted power,” 3GPP TGS-RAN WG4 Meeting #99-e, May 2021.

[8] R1-2104211, “Enhancements of PUCCH formats for beyond 52.6GHz”, FutureWei, 3GPP RAN1#105-e.