3GPP TSG RAN WG1 Meeting #106-e	R1-2107102

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.2.6

Source:	Futurewei

Title:	Channel access for shared spectrum Beyond 52.6 GHz

Document for:	Discussion/Decision

# Introduction

In RAN # 90 the extensions to WI [1] for NR operation up to 71GHz were approved. According to [1] RAN1 should define:

“Physical layer procedure(s) including [RAN1]:

Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access.

Study, and if needed specify, energy detection threshold enhancement”

Multiple Agreements we reached in RAN1#104-e, RAN1#104b-e and RAN1#105-e based on the above principles. In this document we provide our views on the open issues after recalling the relevant agreements.

# EDT Computation

In RAN1 #104-e the following agreements on energy detection threshold were made:

Agreement:

The baseline ED threshold can be computed as

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS definition of Operating Channel BW

- FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi.

- FFS: EDT when the COT has time varying transmission beams and varying EIRP

Subsequently in RAN1 #104b-e the following working assumption was reached.

Working assumption:

- For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

In RAN1#105-e the working assumption was sought to be modified to the following:

- For Pout in EDT determination, define Pout to be at least the maximum of mean EIRP of each transmission burst during the COT at the node initiating the COT.

There was no agreement on this proposed modification. Our concern on this proposed modification is that there is no formal definition of a “burst” in B52. In absence of any definition, a COT initiator will essentially reduce the max_over_(mean burst-EIRP) operation to mean-EIRP-over-COT.  This is because when using the max_over_(mean burst-EIRP) operation over any given set of transmissions in a COT, partitioning them into as few bursts as possible will reduce Pout, and therefore increase EDT and improve access likelihood for that initiating node.  However, as illustrated in Fig. 1, we can have skewed combinations of  beam EIRPs and their durations. Using the mean over all per-beam EIRPs for such a combination can violate the principle behind the ETSI BRAN specification [3]. The latter statement is in the sense that for a substantial duration of the acquired COT the EIRP can be above Pout, contrary to ETSI BRAN, which we note considers a single transmission burst during the COT and mandates that its EIRP must not exceed Pout.

Therefore, there is a need for a common understanding of  a “burst”. In our opinion a burst can be defined as “ a contiguous sequence of transmissions along any transmit beam” where the contiguity can be relaxed to allow for small gaps that are below some threshold. Then,  the mean used in the proposed modification would be over instantaneous powers that would depend on transmit data symbol realizations. With this burst definition the proposed modification is acceptable.

Proposal 1: Define a transmission burst to be a contiguous sequence of transmissions along any transmit beam.

Fig 1. Skewed bursts within COT

On the other hand, neither the working assumption nor its proposed modification seem to be well matched towards a scenario in which multiple LBT sensing beams (considering different intended transmit beams) are implemented by an initiator node. The latter scenarios have not been precluded and are still being considered for further study. Their main intention is to utilize spatial reuse opportunities, i.e., sensing along one or more beams may be successful (in finding the channel idle) while others are not. For such a scenario it is evident that using a separate EDT for each sensing beam is beneficial. In particular, the Pout definition that is used in the EDT computed for each sensing beam must consider the transmit power and beamforming gain of the intended transmit beam (via a specific EIRP). Using a common maximal EIRP across all sensing beams may degrade spatial reuse possibilities thereby negating the main advantage of multiple sensing stages. This is illustrated by an example depicted in Fig.2 where the Tx upon acquiring the COT intends to transmit in a TDM fashion to Rx-A (burst-1) and Rx-B (burst-2). Rx-A is relatively closer and has LoS to the Tx but Rx-B can only be reached via a reflected path and hence requires higher effective transmit power to overcome the larger propagation loss and additional loss due to reflection.

This is depicted in Fig.3 where the burst-2 associated with transmission to Rx-B has significantly higher average EIRP than the burst-1 associated with transmission intended for Rx-A. Then, using a common EDT based on Pout computed using the higher or maximal EIRP (EIRP-2) can degrade acquiring channel along Tx beam-1 by being overly conservative. We capture this in our following observation and then present our proposal.

Observation 1. Using common Pout (common EDT) for multiple sensing beams can limit spatial reuse.

Proposal 2: Utilize a separate EDT for each sensing beam.

Fig 2. Multiple beam transmissions within COT

Fig 3.  Disparate EIRPs within COT

# Relation between Sensing and Transmit Beams

In RAN1#105-e discussions on defining the relation between sensing and transmit beams were held. The following reflects the final status from the FL summary (where we have labeled the FFS options in red for clarity):

3GPP specification defines at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define sensing beam for LBT, where at least sensing beam(s) “covers” the transmission beam(s), considering following alternatives

- Alt 1: Specify necessary requirement/test procedure to guarantee sensing beam “covers” the transmission beam

  - FFS: this is handled in RAN1 and/or RAN4

  - Opt-1 FFS: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam.

  - Opt-2 FFS: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain

  - Opt-3 FFS: the sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A  [FFS] dB of the peak transmission beam gain. the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.

  - Opt-4 FFS: sensing beam has the minimum [3]dB beamwidth which at least contains all beam peak directions of transmission beams

  - Other mechanisms not precluded

- Alt 2. Extending the beam correspondence framework and/or QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s)

  - FFS: details on how to extend the beam correspondence framework and/or QCL/TCI/ SpatialRelationInfo framework

We support both Alt-1 and Alt-2. However, agreeing on some modification of Alt-1 seems to be more   within reach. This is because Alt-2  involves defining beam correspondence for gNB which hitherto has not been done, and in addition it also entails extending beam correspondence to correspondence between one receive and one or more transmit beams. Considering the timeline and need for progress our preference is to focus first on Alt-1.

Regarding Alt-1 there is already a baseline of using Omni type wide sensing beam in NRU and 802.11ad and a one-to-many cover relation (between one sensing and multiple intended transmit beams)  is thus already present. However, the challenge now is to allow directional sensing beams while respecting ETSI BRAN principles. Here we note that there are two points: (i) a sensing beam must have enough sensing gain in the directions in which proposed transmissions during the COT will inject interference and (ii) conversely the sensing gain can be smaller in directions that will see no or negligible transmit power. Out of these two requirements we believe the first one is critical since the initiating device will already be biased towards ensuring (ii). With this understanding let us examine the four options under Alt-1.

The first three options (Opt-1 to Opt-3) under Alt-1 specify relations between a sensing and transmit beam pair, so in the case one sensing beam covers a set of multiple transmit beams these relations are defined pair-wise between the (common) sensing beam and each of the transmit beams it seeks to cover. Among these our preference is slightly more for the third option (Opt-3) since we believe it seeks to specifically check a set of prominent directions (i.e., in which directions higher proportion of transmit power would be imposed).

The final fourth option under Alt-1 (Opt-4) attempts to directly specify one-to-many cover requirement but is not fully specified since a formulation for designing the sensing beam has not been provided. We note that an alternate well posed one-to-many cover formulation (cf. [6]) can be  the following:

Sensing beam minimizes white noise gain while satisfying  distortion-less constraints for all beam peak directions of transmission beams.

It can be verified that the constraints in the above formulation (which ensure enough sensing gain in prominent directions) are a special case of the third option (Opt-3) listed under Alt-1.

Proposal 3: Consider cover relation based on the following relation between a sensing beam and each one of its intended transmit beams:

  - The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak transmission beam EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.

Another issue that was discussed in RAN1#105-e is further adjustment of ED threshold based on the sensing beam and the transmission beam with the caveat that further adjustment should not violate EDT requirements as per regulations. Two alternatives were considered:

- Alt A: support additional adjustment to Energy Detection computation/threshold to include transmit beamforming and/or sensing beam

  - FFS how to adjust

- Alt B: no additional adjustment to Energy Detection computation introduced (Energy measurement directly compared with baseline EDT agreed no matter which transmit beamform(s) and sensing beam(s) are used

Our preference is for Alt-A. Gain of the sensing beam (based on its relationship with respect to transmit beams) should be accounted for in the EDT computation. Equivalently, the sensed energy can be adjusted before comparing it to the unmodified EDT. Either choice can be pursued based on implementation, but both should be compatible with a common RAN4 test.

One argument against further adaptation to EDT is that transmit beamforming via EIRP is already accounted for and any more adjustments can only be made to make the EDT lower (i.e., more conservative).This argument in our opinion is misleading. Clearly if the sensing gain is relatively higher along on or more directions then the initiator can adjust its sensed energy accordingly (which is equivalent to adjusting the EDT based on sensing gain and its relation to transmit beamforming gain).  In our previous contribution we highlighted useful analytical relations to formally define  sensing beam cover for one or more transmit beams [5].

RAN1 should strive to capture to capture a cover definition. If no agreement can be made at the very least RAN1 should provide basic guidance to RAN4. For that scenario, we propose the following modified version of a high-level outline proposed in [4] during the last meeting where we have shaded our changes in green. The parts shaded in yellow are changes in [4] made over the ETSI BRAN specification.

A channel access procedure is a procedure based on sensing that evaluates the availability of a channel for performing transmissions. The basic unit for sensing is a sensing slot with a duration Tsl=5us. The sensing slot duration Tsl is considered to be idle if an eNB/gNB or a UE senses the channel during the sensing slot duration with adequate sensing gain in at-least the intended transmit directions, and determines that the detected power at-least in the intended transmission directions for at least X us within the sensing slot duration is less than energy detection threshold XThresh. Otherwise, the sensing slot duration Tsl is considered to be busy.

# Energy Sensing

In RAN1#105-e the following agreement was reached:

For energy measurement in 8us deferral period, continue down-selection between the following alternatives

- Alt 1. Two energy measurements are required, with one measurement in the first 3us and one measurement in the last 5us

- Alt 2. One measurement is required

  - FFS where the measurement is located

Note: By implementation, it is possible to support longer than 8us deferral period (Intend to cover Alt 3 as implementation choice for either Alt 1 or Alt 2)

Our preference is for Alt-2.  This is because only one energy measurement is mandated within the 8us deferral period in 802.11ad specification. There is no clear motivation for adding an additional energy measurement within 8us deferral period for NR operation up to 71GHz. We thus think performing one energy measurement within 8us deferral period is enough. Further details on the location and minimal duration must be discussed further.  As baseline, the same location and duration as in the 5us observation slot can be considered.

Proposal 4: For energy measurement in 8us deferral period one measurement is required

  - FFS where the measurement is located and its minimal duration.

# LBT Bandwidth

With regards to the operating bandwidth to be used in LBT, the following alternatives were discussed for the single carrier case:

For LBT for single carrier transmission:

- Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth)

- Alt SC.2. gNB/UE performs LBT over the transmission bandwidth (from the lowest RB to the highest RB used for the transmission)

- Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth.

Our preference is for alternative 1. We realize that alternative 3 allows partial usage in the case that a subset of the checked LBT units is found to be free. However, as has been pointed out before, idle but dis-contiguous  chunks cannot be used in the uplink and there is more specification effort involved in terms of deciding appropriate LBT bandwidth unit values (one fixed value was argued to be infeasible for covering the vast range of carrier bandwidths while respecting UE hardware limitations). We note here that LBT bandwidth unit choice cannot be left to gNB and/or UE implementation as that can create co-existence issues arising from different bandwidth unit assumptions among devices. In addition, further effort is required to refine Coreset structures for chosen LBT bandwidth unit values and to define  related UE capability signaling. Moreover, implementing multiple LBT processes requires new procedures for handling unequal completion times and partial CCA check failures across LBT processes.

Proposal 5: In LBT for single carrier transmission gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth).

Regarding LBT for multi-carrier transmission in intra-band CA, the following alternatives were discussed in RAN1 #104b-e.

- Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately

- Alt CA.2. gNB/UE performs single LBT over all CCs

- Alt CA.3. gNB/UE performs multiple LBT, one for each CC over the transmission bandwidth (from the lowest RB in to the highest RB used for the transmission in the CC)

- Alt CA.4. gNB/UE performs LBT over the transmission bandwidth over all CCs (from the lowest RB in the lowest CC to the highest RB in the highest CC used for the transmission)

- Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

The down selection among the alternatives for multi-carrier LBT should happen once the same for single-carrier LBT has been accomplished. Our preference is for Alt CA.1 which we view as an intuitive extension of Alt SC.1. We also recognize the benefit of Alt CA.2 which can reduce overall LBT overhead at-least in the low load case wherein the probability of finding all channel bandwidths to be idle is relatively higher. The other alternatives can result in increased complexity and signaling at-least in terms of exploiting subsets of bandwidth units found to be idle across carriers as well as in handling different completion times.

Proposal 6: In LBT for multi carrier transmission gNB/UE support:

- gNB/UE performs multiple LBT, one for each channel bandwidth separately,

- gNB/UE performs single LBT over all CCs.

# Cat 2 LBT and Maximum gap

In our opinion Cat 2 LBT, i.e., a short deterministic LBT is useful for a quick evaluation of the channel availability. Such short LBT may be as short as a symbol duration, that is one-shot LBT can be a Cat 2 LBT.

Proposal 7: Introduce Cat 2 LBT for 60GHz unlicensed band operation.

With a Cat 2 LBT supported one can define the device behavior during the COT that facilitates fair coexistence in unlicensed band.

In ETSI EN 302 567 [3] the total time that the equipment initiating transmission makes use of an operating channel is defined as the Channel Occupancy Time. This Channel Occupancy Time (COT) shall be less than 5 ms, after which the initiating device shall perform a new CCA Check.  The document does not specify if the device can have gaps of the channel usage (transmissions) during COT and how long such gaps may be.

For the CCA check procedure a device observes the channel for at least 8us and if the channel is found not occupied defers for a random number (0 to Max number) of empty (not occupied) slots (5us each), where the Max number is 3 or larger.  Thus, the CCA check operation can take up to 23us for the Max number equal to 3. Consequently, if there is a significant gap (i.e., no transmissions from the original node that acquired the channel) during the COT, it might result in another LBT initiator to detect the channel as idle and begin its transmissions. The transmissions from the second initiator node can interfere with and get interfered by the transmissions from the original one when the latter resumes transmitting. A one-shot LBT by the first node will protect the transmissions of the second node. This arrangement also dis-incentivizes adoption of a proactive excessive channel acquisition policy by a node, i.e., a policy under which a node (gNB) acquires a COT (with a relatively long duration of 5ms) even though it anticipates sparse usage. Without a Cat 2 LBT such a node may resume transmissions after a long gap and disrupt transmissions of other co-channel nodes that have initiated transmissions in the interim. However, if a Cat 2 LBT is required before resuming transmissions after a long gap, then that node may be forced to refrain from transmitting once it detects the channel as busy.

Proposal 8: Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, a one-shot LBT is needed to share the COT:

- FFS: Specific value of Y.

Another use case when CAT 2 LBT can be supported is when per-beam LBT before acquiring a channel is allowed. The justification here is similar to that given for the previous proposal. In particular, a significant gap between the time LBT is carried out for a beam and the time that beam is eventually applied can result in other nodes in the interference footprint of that beam initiating transmission.

Proposal 9:   When independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, an additional requirement on Cat 2 LBT before switching to new beam  during the COT should be specified if the time duration from that beam’s LBT sensing exceeds a threshold.

# Contention Exempt Short Control Signaling

Agreement:

- Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS.

  - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals)

  - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell

  - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective

- FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

Our preference is for Alt-1 since otherwise there would not be any bound on the contention exempt signaling at a cell level. Enforcement of the limit (including signaling requirements) needs to be further studied.

Proposal 10: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell.

# RX Assistance

It was agreed in the previous RAN1 meetings [2] that the receiver assisted LBT may be beneficial. Further, for the receiver to help, channel sensing and reporting need to be performed.

The goal of the receiver (a.k.a. responding device) assistance is to assess the local resource availability prior to the transmission and inform the transmitter if that resource is available.  A key advantage of receiver assistance is that it allows for mitigation of the hidden node problem. It is recognized that while directional sensing based LBT at the transmitter side reduces number of exposed nodes, it can still be sensitive to the hidden node problem.  Indeed, given the highly directional nature of transmission and reception, interference reciprocity does not hold in many instances, i.e., the transmitter via sensing may be unable to sense a source of interference to the receiver.  In such instances assistance from the receiver is beneficial.   NR already offer multiple tools for interference assessment such as CSI-IM, and NZP-CSI-RS with flexible resource and time configurations (aperiodic, periodic, or semi-persistent). Therefore, a straightforward way to implement a receiver assisted LBT is by extending the existing mechanisms (such as CSI-IM) to the specifics of shared spectrum operation.

Proposal 11: For receiver assisted LBT, support NR CSI-IM based reporting for the clear channel assessment at the receiver.

To provide a report on the resource availability, the receiver needs to measure the received RF power (for instance RSSI) over the provisioned resources (frequency, spatial filter) for a specific duration based on the LBT type and to compare the received energy with the energy detection threshold, and to decide if the resource is clear (available) or not. Then the receiver must report to the transmitter the resource availability for a transmission. The report from the receiver should be timely and as short as possible (to avoid errors and overhead), therefore a simple map of the resource availability is a better tradeoff. Notice here that the transmitter side LBT when done on a per-LBT bandwidth basis can also yield availability information on a per-LBT bandwidth basis, but it incurs the penalty of multiple LBT processes with unequal completion times and furthermore does not fully mitigate the problem of hidden nodes.  Lastly, receiver-side only LBT can address the hidden node problem (by accurately sensing the interference level at that receiver) but it need not be sufficient to protect ongoing transmissions.

Proposal 12: For receiver assisted LBT, the receiver shall report the resource availability prior to the transmission. The RSSI measurement definition may be extended to assess the resource availability, where the resources, type of measurement (for instance Cat2 LBT) shall be provided by the transmitter.

# LBT and No-LBT Signaling and Related Procedures

In RAN1#105-e the following agreement was reached.

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

We do not see enough justification for further allowing a per-beam LBT enable/disable signaling. This provides additional option (compared to per-UE LBT enable/disable) mainly in the scenario when a single user communicates using more than one beams, but then it opens up multiple issues in terms different completion times for multiple LBT processes at the same user.

In our opinion operating without LBT and without any additional policy to address coexistence (or managing interference) can result in degraded performance in several cases. In this context, we note that while it is tempting to assume that there is an absence of significant interference in the beyond 52.6 GHz (NR_ext_to_71GHz) frequency range due to narrow beams and high propagation loss, it so happens that such optimistic assumptions need not always hold. Indeed, as demonstrated in [6] for legacy (802.16ad) WiFi devices operating in 60GHz, beams realized via practical devices have considerable sidelobes which, along with out-of-band emissions, can be a source of significant interference. Such interference is particularly detrimental since it is a spurious transmission that does not benefit any receiver.  Moreover, the commonly used default mode in several legacy Wifi devices, at-least in the indoor settings, is to use omni RX beams. This is because while it is recognized that finer RX beamforming can alleviate the interference problem, the caveat is that there can be increased training overhead and higher susceptibility to blocking upon using such beamforming. Thus, there can be vast disparities in the capabilities (in terms of out-of-band emission suppression, TX and RX beamforming capability etc.) and operational settings among co-existing devices. In addition, there is an asymmetry inherent in interference among co-existing links, i.e., one TX-RX link can suffer much more interference from a coexisting active link while imposing substantially less interference on that link. Together, these factors imply that operating without LBT and without any form of interference mitigation policies can lead to reduced system throughput and/or degraded fairness. One approach that seems feasible is for each device to adopt a policy of a certain channel vacation after each acquired CoT, and such a policy will be more effective if it accounts for the disparity prevalent among co-existing devices.

Proposal 13: For regions where LBT is not mandated, indication of UE specific per-beam LBT/no-LBT indication from the gNB is not supported.

Proposal 14:  In deployments without LBT consider specification of channel vacation policies accounting for disparity among co-existing devices.

- Conclusions

Proposal 1: Define a transmission burst to be a contiguous sequence of transmissions along any transmit beam.

Observation 1. Using common Pout (common EDT) for multiple sensing beams can limit spatial reuse.

Proposal 2: Utilize a separate EDT for each sensing beam.

Proposal 3: Consider cover relation based on the following relation between a sensing beam and each one of its intended transmit beams:

- The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak transmission beam EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.

Proposal 4: For energy measurement in 8us deferral period one measurement is required

FFS where the measurement is located and its minimal duration.

Proposal 5: In LBT for single carrier transmission gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth).

Proposal 6: In LBT for multi carrier transmission gNB/UE support:

- gNB/UE performs multiple LBT, one for each channel bandwidth separately,

- gNB/UE performs single LBT over all CCs.

Proposal 7: Introduce Cat 2 LBT for 60GHz unlicensed band operation.

Proposal 8: Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, a one-shot LBT is needed to share the COT:

- FFS: Specific value of Y.

Proposal 9:   When independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, an additional requirement on Cat 2 LBT before switching to new beam  during the COT should be specified if the time duration from that beam’s LBT sensing exceeds a threshold.

Proposal 10: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell.

Proposal 11: For receiver assisted LBT, support NR CSI-IM based reporting for the clear channel assessment at the receiver.

Proposal 12: For receiver assisted LBT, the receiver shall report the resource availability prior to the transmission. The RSSI measurement definition may be extended to assess the resource availability, where the resources, type of measurement (for instance Cat2 LBT) shall be provided by the transmitter.

Proposal 13: For regions where LBT is not mandated, indication of UE specific per-beam LBT/no-LBT indication from the gNB is not supported.

Proposal 14:  In deployments without LBT consider specification of channel vacation policies accounting for disparity among co-existing devices.

- References

- RP-202925, Revised WID: Extending current NR operation to 71 GHz, CMCC, RAN#90e, 2020.

- Chairman's Notes RAN1#105-e

- ETSI EN 302 567 Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum.

- R1-2104463,	Channel Access Mechanisms, Ericsson.

- R1-2104213,	Channel access for shared spectrum Beyond 52.6 GHz, Futurewei

- P. Le Son, On the Design of Sparse Arrays with Frequency-Invariant Beam Pattern, IEEE Trans. 2021