3GPP TSG RAN WG1 #106-e	R1-2107109

e-Meeting, August 16th – 27th, 2021

Agenda item:		8.2.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Channel access mechanism

Document for:		Discussion and Decision

# Introduction

The revised work item on supporting NR from 52.6 GHz to 71 GHz [1] was approved at RAN#92-e. Before that, 3GPP  carried out a study on required changes to NR using existing DL/UL waveform to support operation between 52.6 GHz and 71GHz, reported in [2]. This contribution deals with the following objective of the WID:

- Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

  - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

  - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access

  - Study, and if needed specify, energy detection threshold enhancement

We consider first the LBT channel access mechanism and related procedures, focusing on the transmitter LBT procedure in Section 2, LBT beamforming in Section 3, channel access within COT in Section 4, receiver assistance in Section 5. Then we discuss the no-LBT channel access in Section 6.

It should be noted that there are only three RAN1 meetings left before the targeted RAN1 completation date for the work item, while a considerable number of channel access features remain open. Hence, we see that the focus should be on those features that are essential for baseline operation of channel access.

Proposal 1: Completing the design for features essential for baseline channel access operation should be prioritized.

# Transmitter LBT procedure design

In this section, we consider LBT channel access mode and address basic design aspects for LBT procedure at the transmitter. We consider first in Section 2.1 the LBT procedure used to initiate the COT, that is, Cat-3 or Cat-4 LBT procedure. We discuss Cat-2 LBT in Section 2.2, energy detection threshold in Section 2.3, LBT bandwidth in Section 2.3, and multi-channel LBT procedure in Section 2.4. We also consider short control signalling and SSB related channel access in Sections 2.5 and 2.6, respectively.

## LBT procedure initiating COT

In TR38.808 [2], it is stated on the LBT procedure:

Use the CCA check procedure in EN 302 567 as the baseline for channel access for 60GHz band when LBT is applied. The following can be discussed further during normative work:

-	whether CAPC and contention window adjustment mechanisms are introduced,

-	whether contention window range needs to be adjusted.

EN 302 567 [3] presents only a minor restriction to the LBT contention window size: the contention window for random back-off must be at least 3. In TR36.889 [4], channel access schemes were categorized into 4 categories. From those categories, both Cat-3 LBT: LBT with random back-off with a contention window of fixed size and Cat-4 LBT: LBT with random back-off with a contention window of variable size can meet the EN 302 567 LBT requirements. Cat-4 LBT adjusts the contention window size to trade-off between LBT overhead and collision probability: if a collision on the channel access is detected, contention window size is increased, increasing LBT overhead but decreasing the probability of a further collision. Obviously this requires contention window (CW) adjustment procedures including determination of a collision. As the possible collisions would relate to a certain beam or beams only, there would be a need to revise the CW adjustment procedures for the beam based operation. These procedures are not needed with Cat-3 LBT using a fixed contention window.

Interference and channel access contention are expected to be less severe on 60 GHz than on 5 GHz unlicensed band due to more directive transmissions and stronger signal attenuation. Hence, channel access collisions can be expected to be infrequent even when the random back-off is drawn from a fixed-size contention window. The further benefit from variable contention window size would be marginal, but would nevertheless require specification and implementation efforts for the revising the CW adjustment procedures. Hence, we prefer Cat-3 LBT allowing for a simple LBT design.

Observation 1: We do not see a need for contention window adjustment mechanism for mitigating channel access collisions.

Proposal 2: LBT procedure uses fixed contention window size for random back-off.

The actual size of the random back-off contention window is one of the open items that is essential to agree. EN 302 567 [3] sets a limit that the contention window for random back-off must be at least 3. A longer contention window is needed when there is a considerable number of nodes contending for channel access, and there is a risk that multiple nodes would occupy the channel simultaneously and interfere each others transmissions. However, presented coexistence results do not indicate that high contention would be faced frequently. On other hand, relatively short contention window limits the LBT overhead. Further, a relatively short contention window reduces need to introduce any CAPCs.

A contention window size of 4 would allow to perform LBT on a vacant channel in 28 us or less, meaning that LBT could be performed in 1 slot with 480 kHz SCS, in 2 slots in case of 960 kHz SCS, or in the 4 symbol gap between consecutive SSB pairs in case of 120 kHz SCS.

Observation 2: With sufficiently short contention window size, there is no need for CAPCs

Proposal 3: Contention window size of [4] is used in the LBT procedure

There remain some issues open on the channel sensing structures. In RAN1#105e, it was agreed that for energy measurement in 8us deferral period, one of the following alternatives is selected:

- Alt 1. Two energy measurements are required, with one measurement in the first 3us and one measurement in the last 5us

- Alt 2. One measurement is required (FFS where the measurement is located)

The purpose of deferral period measurement is to determine whether the channel is vacant for the node to proceed to the channel contention within the contention window. In a portion of cases, the node determines the random number of 5 us observation slots to be 0, in which case the deferral period measurement is the only channel sensing performed during LBT. With Alt. 2, it is possible that the measurement occurs during a gap (up to 3 us) of WiFi transmission. However, this is rather unlikely case that will not occur persistently in consecutive LBTs while Alt 2 is simpler solution with smaller implementation impact. Hence, we prefer Alt. 2.

Two energy measurements within 8 us deferral period aims to ensure that a short transmission as 802.11ad SIFS is not missed. We see that the channel occupancy determination should be rather robust on the deferral period and hence prefer two energy measurements.

Considering the measurement duration and location within the 5 us observation slot, we see that there is no need to specify the location of the measurement. Instead that can be left for the implementation.

Proposal 4: For energy measurement in 8us deferral period, one measurement according to 5 us observation slot is required.

## On Cat-2 LBT procedure

During RAN1#104e [13], the following Cat-2 LBT use cases were identified for further study:

- Initiating device resuming transmission after a gap

- Responding device channel access when sharing a COT

- Prior to switching to a new transmission beam or resuming transmission on a previously used beam.

- Channel sensing at a responding device for providing Rx-assistance

We discuss these use cases in more detail in Sections 3, 4, and 5. It can be noted that each use case relates to either UE or gNB but not both: UE is not expected to resume transmission after a gap as an initiating device but instead it is acting as responding device sharing a COT or providing Rx-assistance for gNB. On the other hand, gNB may be switching it’s transmission beams several times within a COT.

As gNB and UE have separate use cases for Cat-2 LBT, we see that also Cat-2 LBT support should be decided separately for gNB and UE. When discussing on the need of Cat-2 LBT, also the specific use case for Cat-2 LBT should be identified and agreed upon. We do not see that support for Cat-2 LBT should be agreed without agreeing also on the use case(s) for Cat-2 LBT.

Therefore, we propose:

Proposal 5: Decide on Cat-2 LBT support separately for gNB and UE.

Proposal 6: Decide on Cat-2 LBT support together with the specific Cat-2 LBT use case(s).

The reported simulations show a considerably lower impact of interference than there was in 5GHz NR-U and no significant and consistent benefit is seen from LBT at the transmitter. Still, the introduction of the LBT procedure leads to considerable complexity, as it challenges the scheduling-oriented network operation by enabling the last moment interruption of the scheduled actions, if the LBT procedure reports the channels as “busy”.

With this in mind, there is no clear motivation to introduce Cat-2 LBT procedure to the UE side, as there have not been yet identified a use case, where a related procedure cannot be efficienty handled without a Cat-2 LBT. Hence, the potential minor gains introduced in a limited number of scenarios do not pay off the rapidly increased levels of complexity and uncertainty if the gNBs and, especially, UEs start performing Cat-2 LBT. For example, if UE starts performing Cat-2 LBT before every transmission within the COT then during this transmission the gNB is not aware if the UE transmission has started or was dropped due to “busy” indication from the UE-side Cat-2 LBT. This creates additional uncertainty to the gNB operation.

It is also important to note that introduction of Cat-2 LBT to the UEs has also a considerable implementation impact. Although Rel-16 NR-U supports UE-side LBT procedure, it is not certain that UE supporting 60 GHz bands supports also Rel-16 NR-U or benefits from the Rel-16 NR-U implementation. Without a major gain, such additions to the UE functionality are not preferred. Finally, if the Cat-2 LBT operations are supported, then there is also a need to support the UEs without the LBT capabilities, hence the signalling for UEs with and without LBT capabilities has to be developed. The simpler solution with this regard is not to support LBT at the UE side at all.

Therefore, we propose:

Proposal 7: Do not support Cat-2 LBT at the UE side.

Regarding the gNB side, the results in Figures 4 and 5 do not show any tangible performance improvement from gNB performing Cat-2 LBT when switching the transmission beam as discussed in Section 3.3. With the lack of clear performance improvement, we are not convinced that Cat-2 LBT procedure should be introduced for beam switching or resuming transmission.

Further, the difference between Cat-2 LBT and Cat-3 LBT is reduced when short contention window size is defined for Cat-3 LBT. Additionally, CAT-2 LBT based channel access does not fulfil the requirements of EN 302 567, limiting its possible use to outside Europe or to cases where further channel access restrictions beyond the regulatory requirements are imposed on NR. We do not see any need for such further restrictions, especially as no tangible performance improvements from them are shown.

Therefore, we propose:

Proposal 8: Do not support Cat-2 LBT at the gNB side.

## Energy Detection Threshold

In RAN1 #104-e meeting [13], energy detection threshold was discussed with the following agreement.

Agreement: The baseline ED threshold can be computed as EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz) Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax. - FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations) - FFS: If Pout is max output EIRP of the device or instantaneous output EIRP - FFS definition of Operating Channel BW - FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi. - FFS: EDT when the COT has time varying transmission beams and varying EIRP

In RAN1#104bis-e [14], a related working assumption was made:

Working assumption: For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

It can be noted that the working assumption leads to unnecessarily large EDT and potentially conservative channel access, as on EN 302 567 Pout is defined as RF output power, which is further defined as “The RF output power is the mean equivalent isotropically radiated power (EIRP) for the equipment during a transmission burst.”

At same time, it can be noted that the use of mean EIRP has also some problems:

- EIRP may vary between different beams used during the COT. Using just mean EIRP over all beams may result in too low Pout and too high EDT when considering the channel access from the viewpoint of the beam with the highest EIRP. This can be solved by defining Pout as maximum of beam specific mean EIRPs

- Another issue to consider is that the estimated mean EIPR may differ from the actual mean EIRP of COT, e.g. if Cat-2 LBT is required during the COT and blocks some of the intended transmissions. When Pout is defined as the maximum of beam specific mean EIRPs, we do not see this to likely result in cases where the Pout is determined to be significantly smaller than actually realized.

Following the RAN1#105e discussion, we propose to update the Pout to be at least the maximum of mean EIRP of each COT initiating node transmission burst during the COT as discussed in RAN1#105e FL summary [12]. The proposed Pout definition would allow also the use of maximum EIRP in the Pout definition, if that is seen to yield simpler implementation at price of more conservative EDT.

Proposal 9: For Pout in EDT determination, define Pout as at least the maximum of mean EIRPs of the transmission bursts of the node initiating the COT during the COT.

Observation 3: Proposal 8 allows also for implementation according to RAN1#104bis working assumption.

Considering the other remaining open items on the EDT determination:

- One of the controversial open items on EDT is the potential EDT adjustment based on the sensing beam and transmission beam. The EDT is a function of transmitted EIRP. However, the increase in EIRP does not differentiate between increase in Tx power or in beamforming gain, although EIRP increase by beamforming can be expected to cause less interference to other systems than an increase by Tx power. Further, any difference on the sensing beam and transmission beam is not reflected on the EDT determination.

On other hand, the potential EDT adjustment shall not result in EDT that would violate EN 302 567 requirement. That means that any modification could only reduce the current EDT and penalize the affected equipment in the channel access. In other words, we would further restrict channel access for some of NR equipment while the system improvement from such penalization remains unclear. Further, specifying EDT dependence on beamforming is far from a trivial task and would mean a large standardization effort. We do not see any reason to take such large effort for the sake of unclear benefits.

Proposal 10: Further adjustment of EDT based on the sensing and transmission beams is not specified.

The determination of Pout based on the EIRP of the following transmission means that the EDT and channel access probability can depend considerably on the characteristics of the transmission. For example, CG PUSCH configurations of different number of RBs can have a considerable difference in EDT. If UE supports LBT and is configured with CG PUSCH, it would be attractive to use a configuration with high EDT outside of gNB initiated COT and other one with higher Tx power within gNB initiated COT, allowing for higher MCS and larger TBS. Hence, the configuration and operation of CG PUSCH should take the advantage of EDT dependency on Pout but without imposing considerable impacts on UE or gNB processing. This can be simply achieved by adding to the CG PUSCH configuration an indication whether the configuration is to be used inside or outside of a gNB initiated COT.

Proposal 11: CG PUSCH configuration shall include indication of whether the CG PUSCH configuration is used inside or outside of a gNB initiated COT, or both.

## LBT bandwidth

In RAN1 #104bis-e meeting [14],  the following options were captured for LBT bandwidth.

For LBT for single carrier transmission, continue down selection between - Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) - Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth For LBT for multi-carrier transmission in intra-band CA, continue down selection between - Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately - Alt CA.2. gNB/UE performs single LBT over all CCs - Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

However, no consensus was achieved on LBT bandwidth in RAN1#105-e meeting. The key concern focuses on whether it’s necessary to define a unit of LBT bandwidth for Alt SC.3 and Alt CA.5.

In our view of point, on one hand the introduction of LBT BW unit will cause additional implementation complexity and energy consumption for LBT operation since gNB/UE needs to perform multiple parallel LBTs over the channel bandwidth (or BWP bandwidth); on the other hand, it will also complicate the discussion and design. For example, it needs to be further discussed whether a guard band shall be further introduced between LBT bandwidth units within a carrier and how to deal with LBT failure on a some of the LBT bandwidth units. Although flexible channelization may be beneficial from channel access perspective, the benefits will be marginal especially in the case of low traffic load or/and UL. However, definition of LBT subbands involves a lot of specification efforts.

In contrast, one LBT over channel bandwidth would simplify the LBT procedure substantially. As the channel bandwidth is defined flexibly enough to accommodate any BWP bandwidth by EN 302 567, it’s not necessary to introduce to a LBT bandwidth unit in terms of flexibility, complexity and specification efforts of LBT operation in 60GHz band operation. Hence, we prefer to support LBT bandwidth equal to the channel bandwidth.

Therefore, only Alt SC.1 shall be supported for single carrier transmission. As gNB/UE shall perform one LBT over the channel bandwidth or BWP for single carrier transmission, there are no LBT bandwidth seelection and indication issues which makes the LBT procedure easier.

As for multiple carrier transmission, the LBT bandwidth should follow the same design as that of single carrier transmission. Therefore, it is reasonable to support at Alt CA.1 for multiple carrier LBT operation. Moreover, considering that a single LBT over all the carriers is required instead of multiple parallel LBTs per CC, Alt CA.2 can be considered to further simply the LBT procedure in comparison with Alt CA.1 specifically in low traffic load cases where the chance of LBT failure is low. Furthermore, the LBT bandwidth selection could be left to gNB/UE’s implementation based on e.g., interference environment and other considerations such as UE power status or/and HARQ ID allocation of multiple carriers. Apart from COT sharing, there is no strong motivation to indicate the LBT bandwidth adopted by gNB/UE. For gNB-initiated COT sharing, the LBT bandwidth adopted by the gNB can be implicitly indicated by the COT information via DCI Format 2_0. As for UE-initiated COT sharing, UE will not always occupy the whole LBT bandwidth for 60GHz band operation. If gNB shares the LBT bandwidth, collision may happen with other devices on the bandwidth that UE doesn’t actually occupy. For this, it makes sense for gNB to share the channel bandwidth where UE actually transmit without indication of LBT bandwidth for simplification.

Proposal 12: Support Alt SC.1 for single carrier transmission and both Alt CA.1 and Alt CA.2 for multiple carrier transmission.

Proposal 13: For multiple carrier transmission, how to perform LBT is left to implementation and there is no motivation to indicate the LBT bandwidth adopted by gNB/UE.

## Multi-channel LBT

In RAN1 #104-e meeting [13], the following agreement has been reached about multi-channel LBT:

Agreement: Define Type A and Type B multi-channel channel access as: - Type A: Perform independent eCCA for each channel - Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot Down-selection between - Alt1: Support Type A multi-channel channel access only - Alt2: Support both Type A and Type B multi-channel channel access. Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

For operation in the 52.6-71 GHz band, it is beneficial to support multi-channel LBT to facilitate wideband operation as already supported in Rel-15/16. Therefore, NR-U at 60 GHz should support multiple-channel access operation at least for the case of wideband carrier, e.g., in the range of 400 MHz to 2.16 GHz.

ETSI EN 302 567 specifies support for the Type A channel access operation on 60GHz unlicensed band. This kind of operation was also used for Rel-16 NR-U. Hence, the Type A channel access operation should definitely be supported for multi-channel operation.

Type B operation corresponds to the channel bonding case for multiple-channel access as defined in 5 GHz specifications. However, ETSI regulation in EN 302 567 doesn’t specify the channel bonding-based multiple-channel access on 60GHz unlicensed band. Furthermore, Type B channel access further relies on an assumption on pre-determined or fixed channelization. For example, in NR-U on 5GHz spectrum, a fixed channelization and LBT BW of 20 MHz are defined for channel bonding based multi-channel access. However, on 60GHz, there is no fixed channelization or nominal channel BW defined in ETSI BRAN regulation where different channel bandwidths are supported. Support of Type B channel access, would impose unnecessary restrictions on the design of supported channel bandwidths for NR-U on 60GHz unlicensed band and cause extra specification efforts in 3GPP RAN1 as well as RAN4.

At the current state, the benefits from Type B channel access are also unclear. As only one eCCA is required on the primary channel, Type B channel access might in principle speed up the multi-channel LBT procedure in certain cases (e.g. in low traffic environments). However, if fixed and relatively small CWSs are assumed, the advantages of Type B channel access will become insignificant in comparison with Type A channel access. Furthermore, Type B channel access decisions mainly rely on the outcome of eCCA on the primary channel. Once the eCCA check is blocked on the primary channel, the transmitter can not access all the channels for transmission. In contrast, Type A channel access performs independent eCCA on each channel, which enables more flexible channel usage.

Based on the considerations above, we have the following proposal:

Proposal 14: Only Type A multi-channel access procedure (i.e. Alt.1 defined in RAN1#104-e meeting) shall be supported in NR-U on 60GHz band.

## Short Control Signalling

Short Control Signals (SCS) were discussed during the Study Item on supporting NR from 52.6 GHz to 71 GHz, as captured into the TR 38.808:

Support contention-exempt short control signalling transmission in 60GHz band for regions where LBT is required 	and short control signaling without LBT is allowed. It should be noted that if regulations do not allow short control 	signaling exemption in a region when operating with LBT, operation with LBT for these short control signals should 	be supported. Restrictions to the transmission, such as, on duty cycle (airtime measured over a relatively long period 	of time), content, TX power, etc. can be discussed when specifications are developed.

As for the definition of Short Control Signals, there has been a recent effort to introduce their support into EN 302 567. In the ETSI BRAN 108 meeting, text as copied below was introduced to the latest draft EN 302 567 v2.2.0 [3]. Short control signalling transmission, as defined by ETSI, are control and management transmission, that are not required to undergo LBT procedure, but can instead be transmitted without channel sensing, as long as the total duration of SCS transmissions over a 100-ms observation interval does not exceed 10%, as the excerpt from EN 302 567 v2.2.0 below shows.

### 4.2.6 Short Control Signalling Transmissions #### 4.2.6.1 Applicability The present requirement applies to all equipment within the scope of the present document. #### 4.2.6.2 Definition Short Control Signalling Transmissions are transmissions used by the equipment to send management and control frames without sensing the channel for the presence of other signals. #### 4.2.6.3 Limits The use of Short Control Signalling Transmissions is constrained as follows: - within an observation period of 100 ms, - the total duration of the equipment's Short Control Signalling Transmissions shall be less than 10 ms within said observation period. #### 4.2.6.4 Conformance The conformance tests as defined in clause 5.3.8 shall be carried out.

Following observation can be made based on the EN 302 567 :

- SCS allowance of 10% within a 100 ms observation period can be used for various (unspecified) types of control and management transmissions

- SCS transmissions do not need to be periodic, as long as the 10 % allowance within 100 ms period is not exceeded

- multiple SCS transmissions are allowed within the 100 ms observation interval, as long as the 10% limit is not exceeded

- SCS can be transmitted by both gNB, as well as UE(s)

The SCS definition in EN 302 567 is written from a single device (UE or gNB) point of view, i.e. in principle there could be multiple devices operating in a cell, each transmitting control transmissions without LBT for up to 10 % of the time.

Observation 4: EN 302 567, v2.2.0 allows for Short Control Signalling transmissions for up to 10% of time within an observation period of 100 ms.

In the context of NR operation on 60 GHz unlicensed spectrum, there are a few clear candidates for signals and channels that can be considered as short control signalling. At RAN1#104bis-e and RAN1#105-e, it was agreed to support short control signalling for transmission of SS/PBCH and RACH Msg1 and MsgA, while applicability of on other signals and channels was left open. Candidate signals include at least:

Downlink:

- SS/PBCH blocks (already agreed)

- PDCCH

- CSI-RS and other reference signals, e.g., for beam management

- SIBs

- Paging

Uplink:

- HARQ-ACK feedback on either PUCCH or PUSCH

- Scheduling Request

- CSI feedback

- Sounding RS, e.g., for beam management

- RACH related transmissions (Msg1 and MsgA already agreed)

Transmitting selected signals/channels as SCS transmissions is highly attractive as deterministic transmission time of the signals critical control and managements transmissions can be maintained, when the channel access uncertainty is removed. This can simplify the design a lot, as the procedures mitigating the impact of blocked channel access for control signals are not needed. Additionally, even for PUSCH data, short control signaling framework could facilitate transmission of SR or a similar indication, when actual PUSCH transmission is not possible due to LBT failure. Furthermore, there is no need to modify e.g. signal design to facilitate the time gap for LBT measurements, making TDM easier. Therefore, we see that NR design for 60 GHz bands should make most of the short control signaling allowance, such that unnecessary modifications to L1 design can be minimized.

Proposal 15: NR-U design for 60 GHz bands supports transmission of the following DL and UL control and management signals as short control signalling without LBT:

- Downlink: SS/PBCH blocks (already agreed), PDCCH, CSI-RS and other reference signals, e.g., for beam management, SIBs, Paging

- Uplink: HARQ-ACK feedback on either PUCCH or PUSCH, Scheduling Request, CSI feedback, Sounding RS, e.g., for beam management, RACH related transmissions

Even if it is not possible to transmit all the signals listed above within the 10 % allowance, it is still beneficial to use short control signaling as often as possible, at least part of transmission instances. Furthermore, especially for the UL transmissions, one needs to consider means for gNB to control that the 10% maximum limit for SCS transmissions of 100 ms observation period is not exceeded. Moreover, it should be discussed if the maximum amount of SCS transmissions in a cell should be limited somehow, in cases where gNB and multiple UEs are all transmitting significant proportion of their transmissions without LBT. Related to this point, at RAN1#105-e, in the context of RACH the following was agreed:

Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

In our view, it is reasonable to apply the 10% limit for short control signaling per link direction in a cell, such that all UEs in a cell shall share the same 10% short control signaling allowance. The network should have sufficient means (i.e. signaling) for controlling that the UEs will not exceed this limit at any point in time. For RACH related transmissions this corresponds to Alt 1 above. The same rule can also be generalized to cover other UL signals and channels.

Proposal 16: For the UL transmissions, the 10% short control signaling allowance is shared by all the UEs in the cell.

## LBT for SSB

In RAN1 #104-e meeting [13], channel access for transmission of SSBs was discussed with the following agreement.

Agreement: - SSB transmission with LBT is supported, at least when the conditions for contention exempt short control signalling based SSB transmission is not met - Note the channel access for SSB with LBT may not be different from a normal COT with multiple beams - FFS: If any difference from a multi-beam COT LBT needs to be introduced

When gNB transmits SSBs as part of normal COT, there is no need for any specific channel access considerations. When gNB transmits SSBs alone, that is, not as part of a normal COT, we can recognize three channel access cases:

- SSBs are transmitted as short control signals without LBT. We cover this case as part of Section 2.6.

- Only part of SSBs can be transmitted as short control signals without LBT while channel access for the remaining part of SSBs is acquired with LBT. This case can be faced in Europe when gNB is compliant with EN 302 567 and overall transmission time for SSBs exceeds 10%.

- Short control signalling contention exemption cannot be used at all and channel access for all SSBs is acquired with LBT.

The SSB time locations for 120 kHz SCS were agreed to follow the baseline of existing FR2 pattern:

Agreement: For SSB with 120kHz SCS for NR 52.6 GHz to 71 GHz, - 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8,16, 20} + 28×n, where index 0 corresponds to the first symbol of the first slot in a half-frame. - For carrier frequencies within 52.6 GHz to 71GHz, support at least 𝑛 = 0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18. - Other values of n (if any) are FFS, and support of additional n values are subject to support of DBTW for 120kHz SSB

The time locations are presented in Figure 1 for 120 kHz SCS, showing that two SSBs are located back-to-back without any gap between them. With the agreed locations, it is not possible to require Cat-2 LBT at the gNB beam switching at the border of these two SSBs. Introduction of LBT gap would require a re-design of SSB time locations, which would mean a considerable additional specification effort and need to revert the earlier agreement.

Proposal 17: One-shot LBT within COT is not required before gNB beam switch between SSBs.

Figure 1. Example of SSB candidate time locations in symbol level for 120 kHz SCS.

When gNB is transmitting SSBs alone, LBT could be performed prior to the half-frame burst of SSBs or “sub-sweep block of SSBs” [17]. The number of slots between these SSB bursts without any SSBs can range from 2 to 4 slots, depending on SCS and 480 kHz / 960 kHz SCS design, which is enough to perform LBT with reasonably sized contention window.

In case of 120 kHz SCS, there is also possibility to perform LBT within the 4 symbols prior the block of two SSBs This would provide additional flexibility in time when LBT can be performed. In case of Cat-3 LBT, rather short contention window of 4 observation slots could be introduced. This means that a LBT gap of 8 us + 4*5us = 28 us is required, while 4 symbols with 120 kHz SCS has duration of 35.7 us. Hence, in the case of 120 kHz SCS, LBT can be performed during the 4 empty symbols preceding the SSBs, but it is clear that there is no space for much longer contention window.

Observation 5: Short contention window of [4] observation slots facilitates flexible LBT timing for SSB transmissions.

When only a part of SSBs are transmitted as short control signals while the transmission of remaining SSBs is conditioned with LBT, it makes sense that the same SSBs are not constantly conditioned with LBT. Instead, use of SCS contention exemption and use of LBT could be cycled over the SSBs to evenly distribute the channel access uncertainty over the SSBs. There are different ways to achieve this; in a fully flexible manner where gNB could choose dynamically in every SSB burst/discovery window that which SSBs are sent based on SCS contention exemption and which are sent (or not) based on LBT, or in a more static or predetermined manner where choise for SSB ‘grouping’ to {SCS or LBT} is done in advance e.g. in a periodic manner. In latter case the expected pattern could be informed to the UE, while in fully flexible case UE would need to assume both options for each SSB. We discuss this further in accompanying contribution [17].

Proposal 18: Use of short control signal contention exemption and use of LBT for SSBs is predetermined or semi-statically determined, distributing the channel access uncertainty over the SSBs.

# Beamforming for LBT

One of the objectives identified for the WI in RP-202925 is to [1]:

Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access

Supporting directional LBT is complicated, the progress of the topic has been very slow, and the companies are still having very differing positions. Directional LBT is not strictly essential for channel access operation but access with wide beam covering all Tx beams can be used instead. Considering the situation, it would be beneficial to deprioritize the directional LBT until baseline channel access methodology is agreed.

Proposal 19: COT initiating LBT with multiple independent per-beam LBT sensing should be deprioritised while completing the design for baseline channel access procedures.

In the following, we present our views on directional LBT in case that directional LBT is discussed in RAN1#105-e.

## Relation between LBT sensing beam and transmission beam

Regarding directional LBT operation in 60 GHz unlicensed band, one of the major issues to clarify is the beamforming for LBT or, particularly, LBT beam relation to Tx beam. Here, setting the LBT beam equal to the gNB transmission beam may appear at first sight as a straightforward solution especially when serving single UE within a COT. However, serving single UE per COT can be inefficient due to UE and gNB processing times and increased LBT overhead and can increase latencies for other UEs. When directional LBT sensing is performed for multiple UEs simultaneously or consecutively at beginning of COT (i.e., using a wider LBT beam or more complex LBT beamforming procedures), transmissions in different beams can be multiplexed in the same COT as agreed in RAN1#103e [11]. This highlights that preferred LBT sensing beam may depend on the multiplexing strategy used by gNB.

Further, the exact beamforming operations are subject to the details of the transceiver and antenna array system utilized at the gNB that may vary a lot between different vendors. The LBT beamforming operations may also vary from a COT to COT depending on the UEs scheduled to be served during the COT. Moreover, the whole NR beam based operation at gNB is defined so that the design of exact beam patterns is left for implementation. It is hard to see that those should be now specified for the sake of LBT.

Beam correspondence has been proposed to be used as means to define dependency between LBT beam and Tx beam. However beam correspondence has been defined only for UEs but not for gNBs. Hence definition and testing of beam correspondence for gNBs would require considerable additional efforts. Also for UEs beam correspondence is not mandatory.

Also using QCL/TCI/SpatialRelationInformation has been proposed. QCL is used to describe dependencies of downlink reference signals. It is not clear how to use it for defining gNB sensing beams. Hence again considerable effort is needed to specify such mechanism.

Therefore, it is proposed to leave the relationship between gNB LBT sensing beam and transmission beam for the directional LBT up to implementation, as long as the relevant region and deployment specific requirements (i.e. ETSI EN 302 567) are fulfilled and tests defined by regulations are passed. Additional, generic requirements can be considered, e.g., that the beam(s) used in the LBT contain the transmission direction(s) intended to be used during the COT. However, it should be left for RAN4 to decide on how to define such beam relationship, the requirement and corresponding test case. We see that RAN1 specifications do not need to define such requirement. However possible L1 signalling support, if necessary, need to be discussed in RAN1.

Proposal 20: Leave the relationship between gNB LBT sensing beam(s) and transmission beam(s) to vendor-specific implementations. Vendors can use different beamforming techniques for their LBT procedures, as long as global or region and deployment specific requirements (i.e., ETSI EN 302 567) are fulfilled.

Observation 6: Generic requirements may be considered, e.g., that the beam(s) used in the LBT contain the transmission direction(s) intended to be used during the COT. However, that should be done in RAN4, not in RAN1.

## LBT beam at the start of the COT

In RAN1#104e [13], it was agreed that:

Agreement: For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both) - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Agreement: Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations - Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - FFS: Details on the definition of "cover" - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

In light of the discussion and Proposal 20 above, we propose:

Proposal 21: For a COT with MU-MIMO (SDM) transmission, support both Alt 1 and Alt 2.

Proposal 22: Within a COT with TDM of beams with beam switching, support both Alt 1 and Alt 2 for LBT operations.

Yet another issue is the requirement of performing Cat 2 LBT before beam switch. Cat 2 LBT before beam switch may in worst case cause considerable overhead. The gNB needs to frequently serve multiple directions in a single COT in a time multiplexed manner due to use of analog or hybrid beamforming. Transmissions on different beams are also heavily interleaved in time. For example, gNB may transmit PDSCH for a first UE, UL grant for a second UE and while the UE is preparing the PUSCH, gNB may transmit a DL assignment and PDSCH for a third UE, receive HARQ-ACK from a the first UE, and then receive PUSCH from the second UE, and receive HARQ-ACK from the third UE, etc, as shown in Figure 2. Cat 2 LBT prior to every change of Tx beam, possibly consuming 1-5 symbol durations depending on subcarrier spacing, would induce considerable overhead. Further, Cat 2 LBT at gNB beam switch cannot be done between the two consecutive SSBs with the SSB time locations agreed for 120 kHz SCS. Requiring Cat-2 LBT at every beam switch would mean that SSB time locations should be re-designed to accomodate for LBT gap in case that short control signalling exemption is not used for SSBs.

Figure 2. gNB transmissions on 3 beams during a shared COT

## Evaluation results for Cat 2 LBT at gNB beam switch

In this section we provide simulation results for use of Cat 2 LBT at gNB beam switch within COT as discussed above. Results are for half size office scenario and 50%/50% DL/UL. We have provided results for 100% DL case in our previous contribution [16]. Simulation parameters are found in Appendix 2. LBT mode alternatives used in simulations are shown in Figure 3. As seen in figure the simulations cover both omni-LBT and directional per-beam LBT with and without additional Cat 2 LBT at gNB beam switch. In simulations, the LBT modes are compared against the case without LBT, i.e. use of beamforming as coexistence method.

Figure 3. Various LBT modes used in simulations.

Median and 5th percentile throughputs are shown in Figure 4 and Figure 5 for DL and UL, respectively. Compared to the results without LBT, the LBT seems to cause a clear loss in most cases. There is gain in DL median throughput due to per-beam LBT in high load, but at price of loss in DL and UL 5 percentile throughput.

Observation 7: Use of LBT provides mostly loss in median throughput compared to no-LBT mode

Observation 8: Use of LBT reduces throughput for cell edge UEs

Results for beam switching within COT both without additional Cat-2 LBT (Alt 1a, Alt2) and with additional Cat-2 LBT (Alt1b, Alt3) are provided for both the omni LBT and per beam LBT. Results show practically no difference in performance with or without Cat-2 LBT and hence no gain from introducing the additional Cat-2 LBT. These results support our conclusion related to introduction of Cat-2 LBT, which is discussed further in Section 2.2.

Observation 9: Simulation results do not show any gain from introduction of additional Cat-2 LBT at gNB beam switch during COT.

Figure 4. FTP model 3 DL throughput median and 5th percentile for low, medium and high load

Figure 5. FTP model 3 UL throughput median and 5th percentile for low, medium and high load

## eCCA with multiple LBT beams

As discussed above, gNB may use Alt. 1 or Alt. 2 when gNB acquires channel access for multiple transmission beams to be served in the same COT. In the case of Alt. 2 and analog beamforming, a question to be addressed is how the CCAs for different beams are carried out in time. Some alternatives illustrated in Figure 6 are:

- gNB carries out separate eCCA check procedure for each beam with independent back-off values. Once the CCA check procedure is completed, gNB enters self-deferral until the CCA check procedure for other beams is completed. The CCA check procedures can be time multiplexed as blocks or interlaced in time, as shown in Figure 6a and 6b, respectively.

- gNB carries out single eCCA check procedure during which each beam is sensed as shown in Figure 6c. It may be determined that each beam is sensed at least N times (gNB may increase drawn back-off value to meet the requirement.).

The point of random back-off is to randomize the channel access times of multiple contending devices after a channel becomes vacant. Multiple beam-specific eCCA procedures would mean in practice that the gNB experiences a longer back-off given by the sum of the individual random back-off values. This would at least increase the LBT overhead and lead to long LBT durations, which may be too long e.g for SSB structure. In the case that gNB would actually experience channel contention, it would also reduce the gNB’s probability to gain channel access on all scheduled beams due to long LBT duration. On other hand, single occupied beam would not prevent gNB to obtain channel access on the other beams. The benefits and drawbacks for a single eCCA procedure over all beams are basically the opposite. The shorter LBT gap is important especially when LBT is performed for block of two SSBs during the gap of 4 symbols. Based on the above, we see that the LBT requirements need to be defined so that different solutions for LBT beamforming and eCCA procedures are equally supported.

Figure 6. CCA check procedures for gNB using 2 LBT beams: separate CCA check procedure for each beam multiplexed in blocks in time (Fig 6a) or interlaced (Fig 6b), single CCA check procedure during which beams are sensed (Fig 6c), and single wide LBT beam covering both transmit beams (Fig 6d).

In RAN1#104bis-e [14], this was considered further and it was agreed for the alternatives in the earlier agreement that:

Agreement: For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed in TDM fashion - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Agreement: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed one after another in time domain - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Related to these agreements what comes to MU-MIMO (SDM) case the gNB should be able to do sensing simultaneously and hence alternative B should be supported.

Proposal 23: For a COT with MU-MIMO (SDM) transmission, support Alt B.

For TDM case the gNB may not have capability for simultaneous processing, meaning that the sensing of beams needs to be arranged in time domain. In general implementation flexibility should be maintained and the options should not be restricted too much. However, some further restrictions and downselections should be done for the alternatives listed above.

For alternative A-1 it is not clear what happens on beam(s) for which eCCA is completed and are in self-deferral while waiting eCCA completion on the last beam. We see that the delay between the last channel sensing on the beam and the actually channel occupation may become too large and, hence, would propose that a further CCA check is performed on all self-deferred beams after completing the last eCCA and before starting the COT as shown in Figure 6a. In our view it would be necessary to ensure the gap between sensing and channel occupancy does not get too high. Hence, we propose following modification to Alt A-1

Proposal 24: Alt A-1 is modified as: The node completes one eCCA on one beam, and directly moves on to the eCCA on the other beam, with no transmission in the middle. After completing eCCA on all beams, a further round robin CCA check is carried out in all beams (except the last beam).

Alternative A-2 does not look like a true multi-beam COT but more like combination of shorter single beam COTs and hence does not need to be supported.

In alternative A-3 it should be clarified if a single contention window is shared by beams or there are separate contention windows for each beam. In first case the CCA check procedure would be like in Figure 6c, whereas in latter case like in Figure 6b. Both options should be supported as discussed above. In the case that single CW is shared by the beams, a further restriction is needed that the last CCA checks on all beams indicate a vacant channel. Otherwise it would be possible to start COT on a beam that is measured to be occupied. Hence, we propose following modification to Alt A-3

Proposal 25: Alt A-3 is modified as: The node performs eCCA of the different beams simultaneous, round robin between different beams.

- single contention window is shared by beams or each beam has a separate contention window.

- the last CCAs shall indicate vacant channel on all beams that are part of the COT

Observation 10: It is important to maintain flexibility of gNB implementation for multi-beam COT

Proposal 26: For a COT with TDM transmission, support the modified Alt A-1 and Alt A-3.

# Channel access within COT

Related to the UE sharing of gNB initiated COT, following agreement was reached in RAN1#105e [15]:

Agreement: On maximum gap within a COT to allow COT sharing without LBT, down-select or support both of the following two alternatives: - Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration - Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT

At 60 GHz band, the benefits from the transmitter LBT are rather unclear. As discussed in previous section, we did not observe tangible benefits from one-shot LBT at the gNB beam switch within COT. Hence, Alt. 1 appears as a reasonable approach.

It is said on COT sharing in EN 302 567 [3]:

- “An equipment (initiating or not initiating transmission), upon correct reception of a packet which was intended for this equipment, can skip the CCA Check, and immediately proceed with the transmission in response to received frames. A consecutive sequence of transmissions by the equipment, without a new CCA Check, shall not exceed the 5 ms Channel Occupancy Time as defined in step 5) above.”

There is no definition for a maximum gap before the UE’s response, or condition on one-shot LBT if a gap is exceeded. In Rel-16 NR-U, Cat-1 channel access is allowed when the time gap between the initiating device and responding device transmissions is at most 16 us. That presents significant restrictions for the use of Cat-1 channel access: Cat-1 channel access cannot be used for PUSCH or for PDSCH HARQ-ACK unless there are some other transmissions between the UL grant or the PDSCH because of the time required for UE processing. We do not see reasonable to impose a maximum time gap for Cat-1 channel access that complicates and restricts the use of it, for coexistence benefits that might not exist. When considering Alt.3, the allowed time gap X should at least be longer that PDSCH processing time and PUSCH preparation time to accommodate for UE that is immediately proceeding with the transmission as said in EN 302 567. In the case of 120 kHz SCS, the PUSCH preparation time is 36 symbols or roughly 320 us.

Proposal 27: On maximum gap within a COT to allow COT sharing without LBT, we support Alt. 1.

Proposal 28: In case of Alt. 3 for COT sharing without LBT, the maximum time gap X is at least longer that PDSCH processing time and PUSCH preparation time.

Implementation of LBT procedure can have considerable impact on UE, especially if Rel-16 NR-U is not supported by the UE. With the use of short control signalling and Cat-1 channel access within COT, UEs without LBT can be supported from the channel access viewpoint. However, gNB should be aware of that so that gNB does not schedule UE to initiate its own COT.

Proposal 29: UEs without LBT functionality are supported.

# Receiver assistance in channel access

Following TR 38.808, in this WI we further discuss Class A receiver assistance channel access scheme – where Rx provides assistance information (signalling) to transmitter only. It was agreed that Class B, B1, B2, and C will not be investigated as a receiver assistance channel access schemes in the scope of Release 17.

Therefore, the Work Item studies only Class A receiver assistance, as defined in TR38.808 [2]:

The following receiver assisted channel access and interference management schemes have been considered and can be further investigated when specifications are developed.

-	Class A) Receiver provides assistance information (signalling) to transmitter only. The following aspects of Class A can be further discussed when specifications are developed.

-	Applicability in the following potential channel access modes:

-	LBT is performed prior to transmission,

-	No LBT is performed prior to transmission.

-	Details of assistance information (e.g., type, timing, content, how the assistance information is obtained etc.).

-	Whether the assistance information can be obtained by LBT performed at the receiver prior to transmission.

-	Whether the assistance information can be obtained by existing layer 1 and layer 3 measurements with enhancements if needed.

-	If any specification changes are needed to support Class A.

Ideal LBT would defer a transmission when the intended receiver is considerably interfered by an on-going transmission, or when the transmission would considerably interfere the reception of already on-going transmission. The basic approximation of transmitter ED-LBT is that channel energy measured on the transmitter is highly correlated with the channel energy at the receiver. While this is a reasonable approximation with omni-directional antennas, the measured channel energy may be very different at transmitter and receiver with beam-based operation.

There might be different types of assistance provided by Rx: (i) RTS-CTS functionality; (ii) additional sensing at Rx side; (iii) existing measurements and signalling. The type (i) of receiver assistance refers to Class C LBT and, according to TR 38.808 [2], will not be further investigated in Rel-17.

Further, from the RAN1 104-e agreements we have the following tools to be considered.

Agreement: For receiver to provide assistance, channel sensing and reporting need to be performed. The following set of tools can be considered for further discussion - Alt 1. Legacy RSSI measurement and reporting with possible enhancements - Alt 2. AP-CSI report with possible enhancements - Alt 3. LBT at receiver - Alt 3.1 eCCA - Alt 3.2 Cat2 LBT

Since the LBT at the receiver introduces additional delay and complexity, it is reasonable to utilize the existing measurements and reports as a part of receiver assistance. Therefore, Alt 1 and Alt 2 from the above agreement can be chosen as options of interest.

Proposal 30: Employ existing RSSI measurements and CSI reporting as the receiver assistance.

Aperiodic CSI on PUCCH is being discussed in URLLC context for Rel-17. Also L1-RSSI measurement and reporting has been proposed to be introduced as part of enhanced AP-CSI report. These, as well as other CSI methods could be utilized as part of receiver assistance, however there is some standardization effort required, even if it is low, to specify such new features.

As explained before there is a need for prioritization in the channel access topic. Since receiver assistance is not mandatory feature for baseline operation we propose to deprioritize introduction of new mechanisms for receiver assistance until more essential parts of the channel access solution have been agreed. After all the necessary channel access framework is in place the work on receiver assistance, e.g., on L1-RSSI, can be continued.

Proposal 31: Deprioritize introduction of new mechanisms for receiver assistance until more essential parts of the channel access solution have been agreed.

In case that design of receiver assistance mechanisms continues, alternatives for a Rx assistance that is tightly incorporated to gNB LBT procedure with tight processing time requirements (for reducing the waste of channel access time due to the handshaking procedure before actual data transmissions) are worth considering that:

- Rx assistance could be configured only to the UEs that are rather frequently detecting high interference. This way the drawbacks from Rx assistance would be limited only to the situations where Rx assistance can provide further information on channel occupancy, hence improving coexistence. Further, gNB could proceed with transmissions towards other UEs (not providing Rx assistance) while waiting for Rx assistance from the UE in challenging location.

- It should be considered that Rx assistance is provided with UE processing times corresponding to PDSCH processing time or CSI computation time. Meanwhile gNB may be allowed to transmit e.g. limited amount of PDSCHs to the UE, while Rx assistance information is pending.

- One significant limitation of Rx assistance being tightly incorporated to LBT procedure is that it fails to detect channel access collisions that start later during the COT. Given the long duration of maximum COT in relation to slot duration, and alternating DL and UL transmissions within COT, it could be beneficial that UE provides Rx assistance on channel conditions periodically throughout the COT, e.g., as part of periodic or aperiodic CSI reporting.

Observation 11: Any Rx assistance scheme should be configurable per UE, so that it could be used only with UEs frequently detecting high interference.

Observation 12: For any new Rx assistance schemes, UE processing time similar to PDSCH processing time (N1) or CSI computation time (N2/Z1Z2) should be considered when providing Rx assistance.

Observation 13: Rx assistance should not be limited to the beginning of COT only.

# Channel access without LBT

60 GHz unlicensed band can be used for a wide variety of NR use cases, some of which, e.g. relaying and IAB backhaul, are relevant for scenarios where transmissions are highly directional. In such scenarios, the need for and benefits from LBT are unclear. However, LBT procedure can have considerable implementation impact for the very high data rate devices to be used on 60 GHz band. Correspondingly, it is stated in WID [1] that channel access mechanism without LBT is supported:

- Physical layer procedure(s) including [RAN1]:

  - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

    - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

As LBT procedure involves considerable implementation impact, the mechanism without LBT should cause only limited additional implementation efforts. Hence it is reasonable that additional sensing mechanisms are not required as also stated by the WID.

The variety of relevant use cases may be seen also in ETSI BRAN work. In addition to the work item on harmonized standard EN 302 567 [3] covering the requirements of at least c1, ETSI BRAN has an ongoing work item developing harmonized standard EN 303 722 for fixed network equipment according to the requirements of at least c3. A new work item for a new harmonized standard (EN 303 753) to cover devices not in scope of the other two standards was adopted by BRAN in July 2020 [5], and the intended harmonized standard is likely suitable for 3GPP NR-U systems. Work on EN 303 753 is expected to continue until late 2021, and the decisions on coexistence mechanisms would take into account latest developments in 3GPP and other relevant SDOs. We see that channel access mechanism without LBT should fulfil the expected requirements of EN 303 722 but also possibly EN 303 753.

Observation 14: Channel access mechanism without LBT should fulfil the expected requirements of EN 303 722 but also possibly EN 303 753.

## Beamforming impact

Beamforming impacts significantly interference as well as co-channel coexistence. Narrow beams or large antenna gain will limit the transmitted signal to a certain direction (from the transmitter). If EIRP is kept constant, the spatial area interfered by a transmitted signal is reduced with more narrow beams. Additionally, narrow Rx beams can suppress interference coming from directions outside the Rx beam.

In [6] we have presented simulation results for single and two operators in the Scenario Indoor-A [7]. Both full load and half-load cases were evaluated. The offered load as well as the number of served UEs was doubled on the indoor office area when the second network was introduced. Nevertheless, drop in mean throughput remained below 30% for DL and below 40% for UL. 30% to 60% throughput losses were observed at 5%ile cell edge. The results show that beamforming alone without LBT, even with a modest number of antenna elements, can provide enough spatial separation for efficient coexistence of two networks. Hence when beamforming is used, simultaneous use of LBT is not necessary.

In [8] we have provided simulation results for single and two operators in both Indoor and Outdoor scenarios. Conclusions of these simulations are very similar to the previous paragraph. BRAN has discussed the coexistence for EN 303 753 based on the beamforming as the primary mechanism and will continue the discussion in next meetings. In [18], we present simulation results in an outdoor scenario with NR-U coexisting with a Fixed Wireless Access (FWA) system. The results show that 13 dBi antenna gain, without additional LBT, results in reasonable coexistence between systems in outdoor deployments.

In Section 3 we have provided simulation results with no-LBT and several LBT modes. Also these results show clearly that use of LBT is not required and beamforming performs similar or even better than LBT as channel access method.

It would be useful to have possibility to implement NR-U UE without mandatory LBT support. In this case the channel access could be based on beamforming. Beamforming gain for a typical UE is lower than for a typical base station and hence UE requirements for channel access could be different. Possibly lower beamforming requirements could be tied e.g. to use of some other limitation e.g. lower EIRP, limited duty cycle or limited response time for base station transmissions.

## Automatic transmit power control and automatic link adaptation

The coexistence mechanisms in EN 303 722 are still under discussion but will include automatic transmit power control and automatic link adaptation.

Automatic transmit power control is considered as effective and cost-efficient coexistence mechanism in [9]. In [9], ATPC is described as a mechanism adjusting the transmit power to the minimum power that is necessary to operate the link with the desired performance. ATPC requires feedback from the receiver but does not involve additional measurements on channel occupancy. The existing NR uplink power control can be seen to provide desired ATPC also from the coexistence viewpoint. However, there are several open issues related to ATPC on downlink, such as, which DL signals and channels the ATPC should be applied to, how dynamic should DL ATPC be and how much changes to NR standards are needed. Power control range in EN 303 722 specification draft approved for European Norm Approval Process (ENAP) seems to be settling to minimum of 3dB [10].

Automatic link adaptation aims to maximize the link throughput by adjusting the modulation and coding scheme according to the channel conditions. This aims for efficient use of spectrum, hence reducing the transmission time for a certain amount of data. Correspondingly, link adaptation facilitates spectrum sharing. Again, there are several open issues related to possible EN 303 722 requirements for link adaptation and, correspondingly, whether any changes are needed to NR standards. Proposed requirements in EN 303 722 specification draft approved for European Norm Approval Process (ENAP) are 10% reduction in duty cycle for a given amount of payload or 3dB reduction in power [10].

Observation 15: NR for 60 GHz band shall be able to fulfil the EN 303 722 requirements for spectrum sharing based on automatic transmit power control and/or automatic link adaptation.

## Conditions for the use of no-LBT channel access mode

It is stated in TR 38.808 [2]:

For operation where LBT is not required, the following can be further discussed when specifications are developed:

-	whether to introduce additional conditions/mechanisms for no-LBT to be used, or whether to leave it for gNB implementation,

-	when no-LBT mode is used, whether to introduce additional restrictions, such as DFS needs to be applied, ATPC needs to be applied, long term sensing needs to be applied, certain duty cycle limitation, certain transmit power limitation, MCOT limits, etc, or leave the restriction for gNB implementation,

-	when no-LBT mode is used, whether to introduce mechanism for the system to fallback to LBT mode, or whether to leave it for gNB implementation.

There have been proposals of restrictions to use of no-LBT mode, or fallback to another mechanism when using no-LBT mode, e.g. DFS, ATPC, automatic link adaptation, duty cycle limitation (MCOT limitation, stay away time) or fallback to LBT e.g. using timer.

It has been shown in the paragraphs above that beamforming works adequately well as channel access method. Also there are regulations and ETSI harmonized standards that do not require additional conditions/mechanisms to be used such as EN 303 753 [5]. Actually for EN 303 753 duty cycle limitation is being discussed for responding devices which do not meet the beamforming gain requirement. Some other like EN 303 722 [10] require methods such as ATPC and automatic link adaptation. Implementation of gNBs are required to follow the regulations applied for the location and the use case. ATPC and automatic link adaptation are such methods that are supported in NR already now, although it is not clear if regulations require some changes to the existing functionality. If 3GPP would also mandate use of additional mechanisms there would be a risk that there are several redundant mechanisms implemented, which would introduce unnecessary complexity. LBT has much higher impact to implementation and hence implementing LBT should not be mandatory for products following regulations that do not require LBT. Hence we propose to leave additional conditions or restrictions on the use of no-LBT mode or additional coexistence mechanisms or fallbacks for no-LBT mode for gNB implementation which anyway has to be compliant with the regulations.

Proposal 32: Leave any additional conditions/mechanisms/restriction/fallback modes on the no-LBT channel access mode for gNB implementation.

# Conclusion

In this contribution, we considered both LBT and no-LBT channel access mechanisms for NR on 60 GHz unlicensed band. We made following observations and proposals:

Proposal 1: Completing the design for features essential for baseline channel access operation should be prioritized.

Observation 1: We do not see a need for contention window adjustment mechanism for mitigating channel access collisions.

Proposal 2: LBT procedure uses fixed contention window size for random back-off.

Observation 2: With sufficiently short contention window size, there is no need for CAPCs

Proposal 3: Contention window size of [4] is used in the LBT procedure

Proposal 4: For energy measurement in 8us deferral period, one measurement according to 5 us observation slot is required.

Proposal 5: Decide on Cat-2 LBT support separately for gNB and UE.

Proposal 6: Decide on Cat-2 LBT support together with the specific Cat-2 LBT use case(s).

Proposal 7: Do not support Cat-2 LBT at the UE side.

Proposal 8: Do not support Cat-2 LBT at the gNB side.

Proposal 9: For Pout in EDT determination, define Pout as at least the maximum of mean EIRPs of the transmission bursts of the node initiating the COT during the COT.

Observation 3: Proposal 8 allows also for implementation according to RAN1#104bis working assumption.

Proposal 10: Further adjustment of EDT based on the sensing and transmission beams is not specified.

Proposal 11: CG PUSCH configuration shall include indication of whether the CG PUSCH configuration is used inside or outside of a gNB initiated COT, or both.

Proposal 12: Support Alt SC.1 for single carrier transmission and both Alt CA.1 and Alt CA.2 for multiple carrier transmission.

Proposal 13: For multiple carrier transmission, how to perform LBT is left to implementation and there is no motivation to indicate the LBT bandwidth adopted by gNB/UE.

Proposal 14: Only Type A multi-channel access procedure (i.e. Alt.1 defined in RAN1#104-e meeting) shall be supported in NR-U on 60GHz band.

Observation 4: EN 302 567, v2.2.0 allows for Short Control Signalling transmissions for up to 10% of time within an observation period of 100 ms.

Proposal 15: NR-U design for 60 GHz bands supports transmission of the following DL and UL control and management signals as short control signalling without LBT:

- Downlink: SS/PBCH blocks (already agreed), PDCCH, CSI-RS and other reference signals, e.g., for beam management, SIBs, Paging

- Uplink: HARQ-ACK feedback on either PUCCH or PUSCH, Scheduling Request, CSI feedback, Sounding RS, e.g., for beam management, RACH related transmissions

Proposal 16: For the UL transmissions, the 10% short control signaling allowance is shared by all the UEs in the cell.

Proposal 17: One-shot LBT within COT is not required before gNB beam switch between SSBs.

Observation 5: Short contention window of [4] observation slots facilitates flexible LBT timing for SSB transmissions.

Proposal 18: Use of short control signal contention exemption and use of LBT for SSBs is predetermined or semi-statically determined, distributing the channel access uncertainty over the SSBs.

Proposal 19: COT initiating LBT with multiple independent per-beam LBT sensing should be deprioritised while completing the design for baseline channel access procedures.

Proposal 20: Leave the relationship between gNB LBT sensing beam(s) and transmission beam(s) to vendor-specific implementations. Vendors can use different beamforming techniques for their LBT procedures, as long as global or region and deployment specific requirements (i.e., ETSI EN 302 567) are fulfilled.

Observation 6: Generic requirements may be considered, e.g., that the beam(s) used in the LBT contain the transmission direction(s) intended to be used during the COT. However, that should be done in RAN4, not in RAN1.

Proposal 21: For a COT with MU-MIMO (SDM) transmission, support both Alt 1 and Alt 2.

Proposal 22: Within a COT with TDM of beams with beam switching, support both Alt 1 and Alt 2 for LBT operations.

Observation 7: Use of LBT provides mostly loss in median throughput compared to no-LBT mode

Observation 8: Use of LBT reduces throughput for cell edge UEs

Observation 9: Simulation results do not show any gain from introduction of additional Cat-2 LBT at gNB beam switch during COT.

Proposal 23: For a COT with MU-MIMO (SDM) transmission, support Alt B.

Proposal 24: Alt A-1 is modified as: The node completes one eCCA on one beam, and directly moves on to the eCCA on the other beam, with no transmission in the middle. After completing eCCA on all beams, a further round robin CCA check is carried out in all beams (except the last beam).

Proposal 25: Alt A-3 is modified as: The node performs eCCA of the different beams simultaneous, round robin between different beams.

- single contention window is shared by beams or each beam has a separate contention window.

- the last CCAs shall indicate vacant channel on all beams that are part of the COT

Observation 10: It is important to maintain flexibility of gNB implementation for multi-beam COT

Proposal 26: For a COT with TDM transmission, support the modified Alt A-1 and Alt A-3.

Proposal 27: On maximum gap within a COT to allow COT sharing without LBT, we support Alt. 1.

Proposal 28: In case of Alt. 3 for COT sharing without LBT, the maximum time gap X is at least longer that PDSCH processing time and PUSCH preparation time.

Proposal 29: UEs without LBT functionality are supported.

Proposal 30: Employ existing RSSI measurements and CSI reporting as the receiver assistance.

Proposal 31: Deprioritize introduction of new mechanisms for receiver assistance until more essential parts of the channel access solution have been agreed.

Observation 11: Any Rx assistance scheme should be configurable per UE, so that it could be used only with UEs frequently detecting high interference.

Observation 12: For any new Rx assistance schemes, UE processing time similar to PDSCH processing time (N1) or CSI computation time (N2/Z1Z2) should be considered when providing Rx assistance.

Observation 13: Rx assistance should not be limited to the beginning of COT only.

Observation 14: Channel access mechanism without LBT should fulfil the expected requirements of EN 303 722 but also possibly EN 303 753.

Observation 15: NR for 60 GHz band shall be able to fulfil the EN 303 722 requirements for spectrum sharing based on automatic transmit power control and/or automatic link adaptation.

Proposal 32: Leave any additional conditions/mechanisms/restriction/fallback modes on the no-LBT channel access mode for gNB implementation.

# References

- RP-211584, “Revised WID: Study on supporting NR from 52.6GHz to 71 GHz”, Qualcomm, Intel

- TR 38.808, “Study on supporting NR from 52.6GHz to 71 GHz", 3GPP

- EN 302 567 v2.2.0, "Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU", ETSI

- TR 36.889, “Study on Licensed-Assisted Access to Unlicensed Spectrum", 3GPP

- BRAN230026v003, “Wideband Data Transmission Systems (WDTS) for Mobile and Fixed Radio Equipment operating in the 57 - 71 GHz band; Harmonised Standard for access to radio spectrum”

- R1-2006909, “Simulation Results for NR from 52.6 GHz to 71 GHz”, Nokia, Nokia Shanghai Bell

- R1-2005193, “Summary #3 of email discussions for [101-e-Post-NR-52_71_GHz]”, Intel

- BRAN(20)108042, “60GHz beamforming coexistence”, Nokia

- ECC Report 288, “Conditions for the coexistence between Fixed Service and other envisaged outdoor uses/applications in the 57-66 GHz range”, CEPT

- EN 303 722 v0.0.11, “Wideband Data Transmission System (WDTS) for Fixed Network Radio Equipment operating in the 57 - 71 GHz band; Harmonised Standard for access to radio spectrum”, ETSI

- “Draft Report of 3GPP TSG RAN WG1 #103-e v0.2.0”, 3GPP

- R1-2106193, “Feature lead summary of channel access mechanism for 52.6GHz-71GHz band, ver03”, Qualcomm

- “Draft Report of 3GPP TSG RAN WG1 #104-e v0.3.0”, 3GPP

- “Draft Report of 3GPP TSG RAN WG1 #104bis-e v0.1.0”, 3GPP

- “Draft Report of 3GPP TSG RAN WG1 #105-e v0.2.0”, 3GPP

- R1-2102563, “Channel access mechanism”, Nokia, Nokia Shanghai Bell

- R1-2107104, “Initial Access Aspects”, Nokia, Nokia Shanghai Bell

- R1-2107110, “Simulation results”, Nokia, Nokia Shanghai Bell

# Appendix 1

In RAN1#104e meeting [13], following agreements were made on the channel access mechanism:

Agreement:

The baseline ED threshold can be computed as

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS definition of Operating Channel BW

- FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi.

- FFS: EDT when the COT has time varying transmission beams and varying EIRP

Agreement:

For LBT for single carrier transmission, consider the following alternatives

- Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth)

- Alt SC.2. gNB/UE performs LBT over the transmission bandwidth (from the lowest RB to the highest RB used for the transmission)

- Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth

For LBT for multi-carrier transmission in intra-band CA, consider the following alternatives

- Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately

- Alt CA.2. gNB/UE performs single LBT over all CCs

- Alt CA.3. gNB/UE performs multiple LBT, one for each CC over the transmission bandwidth (from the lowest RB in to the highest RB used for the transmission in the CC)

- Alt CA.4. gNB/UE performs LBT over the transmission bandwidth over all CCs (from the lowest RB in the lowest CC to the highest RB in the highest CC used for the transmission)

- Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

Note: supporting more than one alternative for at least multi-carrier transmission in intra-band CA is not precluded.

Agreement:

For energy measurement in 8us deferral period, down-select from the following:

- Alt 1. Two energy measurements are required

- Alt 2. One measurement is required

- Alt 3. Extend the 8us to 10us and perform two measurements, one in each 5us segment

For energy measurement in 5us observation slot, perform single measurement

- FFS minimum duration of the measurement

- FFS location of the measurement

Agreement:

On maximum gap within a COT to allow COT sharing without LBT, down-select from

- Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration

- Alt 2. Define a maximum gap X, such that a later transmission can share the COT without LBT only if the later transmission starts within X from the end of the earlier transmission

  - FFS: Value for X

- Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT

  - FFS: Value for Y

  - FFS:  How to define the one-shot LBT

Agreement:

For Cat 2 LBT, down-select from the following alternatives

- Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation

- Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation

Agreement:

If Cat 2 LBT is introduced, the following use cases can be further studied:

- Resume transmission after a gap Y:  Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y)

- COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT

- Multi-Beam LBT:  Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z)

- Rx-Assistance:  Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling

Other use cases not precluded.

FFS if Cat 2 LBT is mandated for each use case or not.

Agreement:

For receiver to provide assistance, channel sensing and reporting need to be performed. The following set of tools can be considered for further discussion

- Alt 1. Legacy RSSI measurement and reporting with possible enhancements

- Alt 2. AP-CSI report with possible enhancements

- Alt 3. LBT at receiver

  - Alt 3.1 eCCA

  - Alt 3.2 Cat2 LBT

Agreement:

For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both)

- Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

Agreement:

Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations

- Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

  - FFS: Details on the definition of "cover"

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

- Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

Agreement:

Define Type A and Type B multi-channel channel access as:

- Type A: Perform independent eCCA for each channel

- Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot

Down-selection between

- Alt1: Support Type A multi-channel channel access only

- Alt2: Support both Type A and Type B multi-channel channel access.

Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

Agreement:

- SSB transmission with LBT is supported, at least when the conditions for contention exempt short control signalling based SSB transmission is not met

  - Note the channel access for SSB with LBT may not be different from a normal COT with multiple beams

  - FFS: If any difference from a multi-beam COT LBT needs to be introduced

In RAN1#104bis-e meeting [14], following agreements and working assumptions were made on the channel access mechanisms:

Working assumption:

For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

Agreement:

- Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH.

  - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule

  - FFS: Whether this can be applied to all supported SCS or specific SCS.

  - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH

  - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

- FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc

Working assumption:

For energy measurement in 5us observation slot, when performing single measurement, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us.

Agreement:

For LBT for single carrier transmission, continue down selection between

- Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth)

- Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth

For LBT for multi-carrier transmission in intra-band CA, continue down selection between

- Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately

- Alt CA.2. gNB/UE performs single LBT over all CCs

- Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

Agreement:

For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed in TDM fashion

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Agreement:

Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed one after another in time domain

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode. Down-select between

- Alt 1. Support cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) gNB indication

- Alt 2. Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

- FFS: Whether the indication of the decision on applying LBT mode or no-LBT  mode is per beam (can be different for different UEs in different beams or can be different for different beam pairs between gNB and the UE) or per cell (can be different for different cells for a UE in carrier aggregation)

- FFS: Whether a gNB and its UE(s) can have different mode

- FFS: Whether L1 signalling can be used for both Alt 1 and Alt 2 for gNB indication

Agreement:

For contention exemption short control signalling based DL transmission of SS/PBCH, further consider if the following signals/channels can be multiplexed with SS/PBCH block transmission.

- RMSI PDCCH and RMSI PDSCH

- Other broadcast PDSCH

- PDSCH without user-plane data

- PDCCH

- CSI-RS

- PRS

- Other signals/channels contained in Discovery Burst (i.e., exemption applies to Discovery Burst)

Note: Total exempted signals/channels should meet the restriction of 10% over any 100ms interval.

FFS: If contention exemption short control signalling based DL transmission is allowed when not multiplexed with SS/PBCH block transmission.

In RAN1#105-e meeting [15], following agreements and working assumptions were made on the channel access mechanisms:

Agreement: For energy measurement in 8us deferral period, continue down-selection between the following alternatives

- Alt 1. Two energy measurements are required, with one measurement in the first 3us and one measurement in the last 5us

- Alt 2. One measurement is required

  - FFS where the measurement is located

Note: By implementation, it is possible to support longer than 8us deferral period (Intend to cover Alt 3 as implementation choice for either Alt 1 or Alt 2)

Agreement:

On maximum gap within a COT to allow COT sharing without LBT, down-select or support both of the following two alternatives

- Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration

- Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT

Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

Agreement: Contention Exempt Short Control Signalling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS.

  - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals)

  - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell

  - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective

- FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signalling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

# Appendix 2

Simulation parameters are provided in this appendix.

Figure 7. Half size office scenario.

Table 1. Simulation Parameters

|  | Base Station - BS | User Equipment - UE |
| --- | --- | --- |
| Carrier Frequency [GHz] | 60 GHz |  |
| Subcarrier Spacing [kHz] | 960 kHz |  |
| Bandwidth [MHz] | 2000 MHz |  |
| Number of RB | 160 |  |
| Deployment Scenario | Scenario indoor-A, half size |  |
| UE distribution | 100% indoor |  |
| Channel Model | - gNB-to-gNB and gNB-to-UE links: InH – office channel & PL model from TR38.901 - UE-to-UE links: [InH – office channel & PL model from TR38.901] |  |
| Mobility | 3 km/hr |  |
| Antenna Configuration | (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH) | (Mg,Ng,M,N,P) = (1,2,2,2,2) with (0.5 dv, 0.5 dH) |
| Antenna Pattern | Table A.2.1-7 of TR38.802 for ceiling mount | Table A.2.1-8 of TR38.802 |
| Antenna Element Gain | 5 dBi | 5 dBi |
| Power Limitation | 40 dBm EIRP | 25 dBm EIRP |
| Noise figure | 7 dB | 10 dB |
| Transmission Rank | Rank adaptative transmission between Rank 1 and 2 |  |
| TDD DL/UL Ratio | Dynamic TDD |  |
| CSI Feedback | Ideal feedback |  |
| Traffic Model | FTP Model 3 (27 Mbyte file) Num packets per second per user: 2 (low), 3.5 (medium), 5 (high) (100% DL) Num packets per second per user: 1.5 (low), 2.75 (medium), 4 (high) (50/50%, split between UL/DL) |  |
| UE Receiver | MMSE-IRC |  |
| Cell selection criteria | Random select from strongest RSRP with 1 dB HO Margin |  |
| DL/UL Traffic Ratio | 100% DL, 50/50% DL/UL |  |