3GPP TSG RAN WG1 #106-e	R1-2107180

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.3.1.1

Source: 	Lenovo, Motorola Mobility

Title: 	HARQ-ACK feedback enhancement for IIoT/URLLC

Document for:	Discussion and Decision

# Introduction

During RAN1 #104bis-e and #105-e meetings, RAN1 made following agreements regarding UE HARQ-ACK feedback enhancement for Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) [1], [2]:

Agreements: For SPS HARQ-ACK deferral, for the determination of valid symbols in the target slot/sub-slot a collision with semi-static DL symbols, SSB and CORESET#0 is regarded as ‘invalid’ or ‘no symbols for UL transmission’.

Agreements: For SPS HARQ-ACK deferral, support a limit on the maximum deferral of SPS HARQ in terms of k1def  or k1+ k1def

  - FFS: limitation given by a maximum value of k1def or a maximum of k1eff =k1+ k1def

  - FFS how the limitation is determined (e.g. by K1 set(s) or RRC configured limit)

Agreements: For SPS HARQ-ACK deferral, there is no lower limit defined for k1def

Agreement: Restrict the further discussions on the initial slot handling for SPS HARQ-ACK deferral to the identified alternatives Alt. 1, Alt. 1A and 2.

Agreement: For SPS HARQ-ACK deferral, the limit on the maximum deferral of SPS HARQ is defined in terms of k1eff =k1+ k1def.

Working assumption: To handle the collision for the same HARQ process due to deferred SPS HARQ-ACK the following behaviour is to be specified:

- In case the UE receives PDSCH of a certain HARQ Process ID, the deferred SPS HARQ bit(s) for this HARQ Process ID are dropped.

Agreement: For SPS HARQ-ACK deferral, the initial HARQ-ACK transmission occasion is considered to determine the out-of-order HARQ condition

Agreement: Support Type-1 HARQ-ACK codebook for sub-slot based PUCCH configuration in Rel-17.

- The properties of the Type-1 HARQ-ACK codebook for sub-slot PUCCH at least includes that a PDSCH TDRA is associated with a UL /PUCCH sub-slot if the end of the PDSCH overlaps with the associated sub-slot determined by a k1 in the set of sub-slot timing values K1.

- FFS: whether the PDSCH TDRA grouping is performed per DL slot or sub-slot

  - Decide between PDSCH TDRA grouping per DL slot and sub-slot during RAN1#105-e

Agreement: Support PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH and semi-static configuration

- Details are FFS (including applicability of dynamic and/or semi-static means)

- Aim for minimum specification impact

- Dynamic indication and/or semi-static configuration are subject to separate UE capabilities

- The semi-static PUCCH carrier switching configuration operation is based on RRC configured PUCCH cell timing pattern of applicable PUCCH cells and supports PUCCH carrier switching across cells with different numerologies.

  - FFS whether additional rules are needed to support PUCCH carrier switching across cells with different numerologies

- FFS the maximum number of PUCCH cells

- FFS whether and how to support joint operation of dynamic and semi-static carrier switching for a UE

- FFS whether and how to support joint operation of PUCCH carrier switching and SPS HARQ-ACK deferral

Working Assumption: For at least HARQ-ACK re-transmission:

- Support at least one enhanced Type 3 HARQ-ACK CB with smaller size (compared to Rel-16) in Rel-17

  - Definition of enhanced Type 3 CB:

    - The codebook size of a single triggered enhanced Type 3 HARQ-ACK codebook at least determined by RRC configuration

    - The codebook construction uses HARQ processes as a bases (i.e. ordered according to HARQ-IDs and serving cells)

- Support one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB (i.e. Alt. 3) in Rel-17

  - Details are FFS

- Enhanced Type 3 HARQ-ACK CB and/or one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB are subject to separate UE capabilities

Agreement: For PUCCH carrier switching, the PUCCH resource configuration is per UL BWP (i.e. per candidate cell and UL BWP of that specific candidate cell).

Agreement: For PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH (i.e. Alt. 1), the PDSCH to HARQ-ACK offset k1 is interpreted based on the numerology of the dynamically indicated target PUCCH cell.

In this document, we discuss remaining details on SPS HARQ-ACK deferral and re-transmission mechanisms of a cancelled HARQ-ACK transmission for HARQ-ACK feedback enhancement in IIoT/URLLC.

# SPS HARQ-ACK Deferral

With dynamic TDD operations in non-paired spectrum, it may be difficult to guarantee uplink regions for configured SPS PDSCH HARQ-ACK transmission occasions, especially when SPS PDSCH HARQ-ACK feedback occurs very frequently.

When a UE can defer SPS PDSCH HARQ-ACK transmission?

In Rel-15/16 NR, when a UE would transmit dynamic HARQ-ACK (i.e. HARQ-ACK in response to a corresponding PDCCH) and SPS PDSCH HARQ-ACK without a corresponding PDCCH in a slot/sub-slot, a PUCCH resource for transmitting the dynamic HARQ-ACK and the SPS PDSCH HARQ-ACK is determined by the last DCI format among DCI formats indicating the slot/sub-slot for the PUCCH transmission. Also, the UE can perform the PUCCH transmission indicated by DCI on semi-statically configured flexible symbols (if not configured to monitor PDCCH for DCI format 2_0) and does not expect that the PUCCH transmission indicated by DCI overlaps with semi-statically configured DL symbols or dynamically indicated DL symbols. Thus, deferring HARQ-ACK feedback is necessary, only if the PUCCH transmission for the HARQ-ACK feedback does not include dynamic HARQ-ACK. In case that the UE misses the last DCI format determining the PUCCH resource for the HARQ-ACK feedback, gNB can determine UE’s DTX (e.g. based on DM RS detection on the PUCCH resource) on the PUCCH resource and can further determine a PUCCH resource where the SPS PDSCH HARQ-ACK is potentially transmitted by the UE.

Proposal 1: A UE defers HARQ-ACK, if a corresponding HARQ-ACK codebook consists of only HARQ-ACK information for SPS PDSCH(s) without a corresponding PDCCH(s).

HARQ-ACK codebook construction with deferred HARQ-ACK

In Rel-16 NR, for a HARQ-ACK codebook for a SPS PDSCH reception(s) without a corresponding PDCCH(s), HARQ-ACK bits are ordered such that in ascending order of DL slot per SPS configuration index per serving cell index, and then in ascending order of SPS configuration index per serving cell index, and then in ascending order of serving cell index.

If a UE is configured with a Type-1 (i.e. semi-static) HARQ-ACK codebook, for a HARQ-ACK codebook consisting of at least one HARQ-ACK bit for a PDSCH or SPS PDSCH reception(s) with a corresponding PDCCH(s) or a PDCCH indicating a SPS PDSCH release, a HARQ-ACK bit location for a SPS PDSCH reception is derived based on a row index of an applicable time domain resource allocation (TDRA) table and a K1 value indicated by a PDSCH-to-HARQ feedback timing indicator field in a DCI format of SPS PDSCH activation. A HARQ-ACK bit location for a SPS PDSCH release with a separate release DCI format is derived based on the row index of the TDRA table indicated in the activation DCI format and a K1 value indicated by a PDSCH-to-HARQ feedback timing indicator field in the separate release DCI format. A HARQ-ACK bit location for a SPS PDSCH release with a joint release DCI format is derived based on a row index of the TDRA table indicated in an activation DCI format for SPS PDSCH with the lowest SPS configuration index among jointly released configurations and a K1 value indicated by a PDSCH-to-HARQ feedback timing indicator field in the joint release DCI format. If a DCI format does not include a PDSCH-to-HARQ feedback timing indicator field, the UE determines the K1 value based on the high-layer parameter dl-DataToUL-ACK or dl-DataToUL-ACKForDCIFormat1_2.

If a UE is configured with Type-2 (i.e. dynamic) HARQ-ACK codebook, for a HARQ-ACK codebook consisting of at least one HARQ-ACK bit for a PDSCH or SPS PDSCH reception(s) with a corresponding PDCCH(s) or a PDCCH indicating a SPS PDSCH release, a HARQ-ACK bit order for a SPS PDSCH release with a separate/joint release DCI format is derived based on a downlink assignment index (DAI) and a K1 value indicated by a PDSCH-to-HARQ feedback timing indicator field in the release DCI format. A HARQ-ACK bit order for a SPS PDSCH with a corresponding PDCCH is derived based on a DAI and a K1 value indicated in the PDCCH (i.e. PDCCH including an activation DCI format). A HARQ-ACK bit(s) for a SPS PDSCH reception(s) without a corresponding PDCCH is appended after HARQ-ACK bits for dynamically scheduled PDSCHs and/or for a SPS PDSCH release(s). The HARQ-ACK bit(s) for SPS PDSCH receptions without corresponding PDCCHs are ordered such that in ascending order of DL slot per SPS configuration index per serving cell index, and then in ascending order of SPS configuration index per serving cell index, and then in ascending order of serving cell index.

Observation 1: For Type-1 (i.e. semi-static) HARQ-ACK codebook, the Rel-15/16 codebook construction method is not directly applicable to deferred HARQ-ACK (i.e. K1 value for PDSCH-to-HARQ feedback timing needs to be redefined).

Observation 2: For Type-2 (i.e. dynamic) HARQ-ACK codebook, deferred SPS PDSCH HARQ-ACK bits may need to be re-ordered, if additional SPS PDSCH HARQ-ACK bits are multiplexed in a newly determined PUCCH resource.

If a UE has a delayed HARQ-ACK codebook associated with a previous PUCCH occasion (i.e. a previous HARQ-ACK feedback occasion) and if the UE is scheduled to transmit HARQ-ACK information on a current PUCCH occasion (i.e. a current HARQ-ACK feedback occasion), the UE can construct an aggregated HARQ-ACK codebook by concatenating a current HARQ-ACK codebook scheduled on the current PUCCH occasion and the delayed HARQ-ACK codebook. Constructing the current HARQ-ACK codebook only based on the HARQ-ACK information scheduled on the current PUCCH occasion may reduce UE complexity, since the UE does not have to reorder HARQ-ACK information scheduled on the previous PUCCH occasion. The UE can simply attach (e.g. prepend or append) the delayed HARQ-ACK codebook to the current HARQ-ACK codebook to generate the aggregated HARQ-ACK codebook. Since each codebook (i.e. the current and delayed codebooks) is constructed according to a configured codebook type (i.e. semi-static vs dynamic codebook), specification efforts may be minimal.

In order to maintain a proper payload size for deferred HARQ-ACK, a UE may further receive information of a subset of SPS PDSCH configurations of a serving cell and/or a subset of serving cells for which the UE can defer HARQ-ACK feedback, when a PUCCH occasion for the HARQ-ACK feedback is not available for transmission. When the UE constructs an aggregated HARQ-ACK codebook based on a delayed HARQ-ACK codebook and a current HARQ-ACK codebook, the UE may include the current HARQ-ACK codebook and only HARQ-ACK bits corresponding to the subset of SPS PDSCH configurations of the serving cell and/or HARQ-ACK bits corresponding to the subset of serving cells from the delayed HARQ-ACK codebook in the aggregated HARQ-ACK codebook.

Proposal 2: Support deferred HARQ-ACK transmission with concatenation of a delayed HARQ-ACK codebook and a current scheduled HARQ-ACK codebook to construct an aggregated HARQ-ACK codebook.

# Re-transmission of cancelled HARQ-ACK transmission

UE may have to cancel HARQ-ACK transmission due to collision between PUCCH/PUSCH resources of a high priority index and PUCCH/PUSCH resources of a low priority index, urgent DL transmissions from gNB, and/or reception of an uplink cancellation indication. Instead of incurring the system performance loss due to dropping of cancelled HARQ-ACK transmission, it is beneficial to support mechanisms to re-transmit the cancelled HARQ-ACK transmission. In this section, three different mechanisms to enable retransmission of cancelled HARQ-ACK transmission are discussed.

## DCI based one-shot triggering of HARQ-ACK re-transmission

For DCI based one-shot triggering of HARQ-ACK re-transmission, UE may detect a DCI format with DCI including information of HARQ-ACK retransmission, construct a HARQ-ACK codebook for retransmission based on the information of HARQ-ACK retransmission, and transmit the HARQ-ACK codebook in a PUCCH or a PUSCH. The information of HARQ-ACK retransmission may include information of one or multiple cancelled HARQ-ACK transmission occasions for retransmission, e.g. a number of cancelled HARQ-ACK codebooks to be included for retransmission and timing information of the cancelled HARQ-ACK transmission occasions (e.g. in terms of slot/sub-slot offset with respect to a DL slot where the DCI format triggering retransmission is detected). This information allows the UE to have a common understanding with gNB in terms of cancelled HARQ-ACK transmission occasions, and errors due to missed DCI and corresponding missed HARQ-ACK feedback by the UE can be avoided.

For example, a DCI format triggering HARQ-ACK retransmission may include a bit field, e.g. ‘HARQ-ACK retransmission indicator’, indicating HARQ-ACK information triggered for retransmission. as follows:

-	HARQ-ACK retransmission indicator – The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer harq-ackRetransmissionList if the higher layer parameter is configured; otherwise I is the number of entries in the default table shown in Table 1.

Table 1 Default table for the HARQ-ACK retransmission indicator field

| Row index | Slot/sub-slot offset(s) of cancelled HARQ-ACK transmission occasions with respect to a DL slot where a DCI format triggering HARQ-ACK retransmission is detected | Number of cancelled HARQ-ACK transmission occasions for retransmission |
| --- | --- | --- |
| 1 | N/A | 0 |
| 2 | 1 slot | 1 |
| 3 | 2 slots | 1 |
| 4 | 4 slots (for 1st occasion) and 2 slots (for 2nd occasion) | 2 |

Proposal 3: DCI triggering HARQ-ACK retransmission includes information of cancelled HARQ-ACK transmission occasions for retransmission, e.g. a number of cancelled HARQ-ACK codebooks to be included for retransmission and timing information of the cancelled HARQ-ACK transmission occasions.

## Implicit triggering of HARQ-ACK re-transmission

If a UE receives an implicit or explicit indication of two PUCCH resources of different slots/sub-slots (i.e. different DL-to-HARQ feedback delay values) in DCI and if the UE cannot perform HARQ-ACK transmission on an earlier HARQ-ACK transmission occasion, the UE can perform retransmission of cancelled HARQ-ACK transmission on a later HARQ-ACK transmission occasion, without receiving a separate triggering DCI format. gNB may indicate implicit triggering of HARQ-ACK retransmission to UE, when an UL transmission by the UE at the earlier HARQ-ACK transmission occasion may not be guaranteed for various reasons (e.g. dynamic TDD, UL cancellation indication, LBT failure in unlicensed spectrum).

For example, a codepoint of a PUCCH resource indicator (PRI) field in DCI can indicate one or multiple PUCCH resources. Furthermore, a codepoint of a PDSCH-to-HARQ-timing-indicator field in DCI can indicate multiple values for the number of slots (or sub-slots).

In another example, an offset value of a second PDSCH-to-HARQ-timing indication value with respect to a first PDSCH-to-HARQ-timing indication value can be dynamically indicated in DCI. The UE may assume a PUCCH resource index of a second PUCCH resource is same as a PUCCH resource index of a first PUCCH resource.

Proposal 4: Support implicit triggering of HARQ-ACK retransmission based on indication of two HARQ-ACK transmission occasions in DCI.

## Autonomous one-shot HARQ-ACK re-transmission

With dynamic TDD operations in non-paired spectrum and/or operation in unlicensed spectrum, it may be difficult to guarantee uplink regions or channel occupancy for configured SPS PDSCH HARQ-ACK transmission occasions, especially when SPS PDSCH HARQ-ACK feedback occurs very frequently. As another solution approach to SPS HARQ-ACK dropping in TDD or in unlicensed spectrum, potential enhancement of Rel-16 Type-3 HARQ-ACK codebook has been discussed.

Since Rel-17 URLLC/IIoT will support UE-initiated COT in a semi-static channel access mode, if a UE is configured with configured grant (CG) PUSCH, the UE can autonomously use a CG-PUSCH resource for HARQ-ACK retransmission with Type-3 HARQ-ACK codebook. That is, when a configured PUCCH resource for HARQ-ACK transmission is not available or when a configured PUCCH transmission for HARQ-ACK feedback fails due to listen-before-talk (LBT) operation, the UE may multiplex the corresponding HARQ-ACK information in the next earliest CG-PUSCH (i.e. where the CG-PUSCH starts earlier than the earliest PUCCH resource for the HARQ-ACK feedback, and the UE performs multiplexing even when the CG-PUSCH does not overlap with a PUCCH for HARQ-ACK). The multiplexed HARQ-ACK information is determined based on the maximum allowed time duration of delaying HARQ-ACK feedbacks. The max allowed duration for delaying may be determined based on a latency requirement of a traffic (e.g. specific to an IIoT/URLLC application). A HARQ-ACK report for all HARQ processes may be transmitted in one shot in a CG-PUSCH resource so that there is no ambiguity in terms of HARQ-ACK codebook. For HARQ-ACK codebook size reduction, the UE may further be configured via RRC to include HARQ-ACK information for a subset of HARQ processes and/or a subset of component carriers of a PUCCH group.

Proposal 5:  Support autonomous one-shot HARQ-ACK re-transmission for all or a subset of HARQ processes in a CG-PUSCH resource, where the CG-PUSCH is available in an earlier slot/sub-slot than a slot/sub-slot where the earliest available PUCCH resource for HARQ-ACK is.

# Conclusion

In summary, we observe and propose the followings for enhancements to HARQ-ACK feedback in Rel-17 URLLC/IIoT:

Proposal 1: A UE defers HARQ-ACK, if a corresponding HARQ-ACK codebook consists of only HARQ-ACK information for SPS PDSCH(s) without a corresponding PDCCH(s).

Observation 1: For Type-1 (i.e. semi-static) HARQ-ACK codebook, the Rel-15/16 codebook construction method is not directly applicable to deferred HARQ-ACK (i.e. K1 value for PDSCH-to-HARQ feedback timing needs to be redefined).

Observation 2: For Type-2 (i.e. dynamic) HARQ-ACK codebook, deferred SPS PDSCH HARQ-ACK bits may need to be re-ordered, if additional SPS PDSCH HARQ-ACK bits are multiplexed in a newly determined PUCCH resource.

Proposal 2: Support deferred HARQ-ACK transmission with concatenation of a delayed HARQ-ACK codebook and a current scheduled HARQ-ACK codebook to construct an aggregated HARQ-ACK codebook.

Proposal 3: DCI triggering HARQ-ACK retransmission includes information of cancelled HARQ-ACK transmission occasions for retransmission, e.g. a number of cancelled HARQ-ACK codebooks to be included for retransmission and timing information of the cancelled HARQ-ACK transmission occasions.

Proposal 4: Support implicit triggering of HARQ-ACK retransmission based on indication of two HARQ-ACK transmission occasions in DCI.

Proposal 5:  Support autonomous one-shot HARQ-ACK re-transmission for all or a subset of HARQ processes in a CG-PUSCH resource, where the CG-PUSCH is available in an earlier slot/sub-slot than a slot/sub-slot where the earliest available PUCCH resource for HARQ-ACK is.

# References

- “RAN1 Chairman’s Notes”, RAN1 105-e meeting, May 10th – 27th, 2021.

- “RAN1 Chairman’s Notes”, RAN1 104bis-e meeting, April 12th – 20th, 2021.