3GPP TSG RAN WG1 #106-e			R1-2107194

e-Meeting, Aug 16th – 27th, 2021

Agenda item:	8.4.1

Source: 	Hyundai Motors

Title:	Discussion on timing relationship enhancements for NTN

Document for:	Discussion and Decision

Introduction

In the Rel-17 WID [1], timing relationship enhancements for NTN is one of the objectives to be developed by RAN1. In NTN, it is important to solve the problems which are caused from the propagation delay much larger than the propagation delay in the conventional territorial networks. The timing relationship enhancement is introduced to compensate the huge scale of propagation delay. Topics on timing relationship enhancements were discussed [2] in RAN1 #105-e meeting.

In this contribution, we discuss K_offset update regarding timing relationship enhancements for NTN.

Discussion

In RAN1 #102-e, there was an agreement regarding K_offset delivery in initial access as follows:

Agreement: For Koffset used in initial access, the information of Koffset is carried in system information. - FFS implicit and/or explicit signaling of Koffset in system information. - FFS a cell specific Koffset value used in all beams of a cell and/or each beam in a cell uses a beam-specific Koffset value. - FFS whether/how to update Koffset after initial access.

In RAN1 #104bis-e, there was an agreement regarding K_offset update as follows:

Agreement: For updating K_offset after initial access, at least one of the following options is supported: - Option 1: RRC reconfiguration - Option 2: MAC CE FFS: Other options

During RAN1 #105-e meeting there were no conclusions about the options and Moderator suggested two options to be down-selected in the next meeting [2] as follows:

Moderator recommendation on K_offset update option: Companies are encouraged to provide input on this issue at the next RAN1 meeting, taking the following into account. - Option 1: support MAC CE option - Option 2: support both MAC CE option and RRC reconfiguration option

RAN1 considers three types of satellites: GEO, LEO with earth-moving cells and LEO with earth-fixed cells. We can expect that a period for updating K_offset depends on the type of satellites and their ephemeris. For example, GEO would not require updates of K_offset after initial access if we do not consider UE’s mobility. Meanwhile, LEO with earth-fixed cell requires higher update rate than GEO. From the examples, we expect that it would be beneficial to control update rate of K_offset, so we propose a new parameter as follows:

Proposal 1: A new RRC signalling should be considered to provide update frequency of K_offset.

Assume there are some UEs and EFC (earth-fixed cells) LEO satellite serving the UEs. When the angle of elevation is small, the distance between the satellite and every UE changes quickly. Thus, gNB needs to update K_offset for most of UEs in the cell via MAC-CE. In the case, even if not initial access, renewing K_offset value with cell-specific K_offset via a new SIB periodically or aperiodically would be more efficient than updating K_offset provided by MAC-CE.

Proposal 2: Even after the initial access, cell-specific or beam-specific K_offset values provided by SIBs can be used to update K_offset.

Considering a situation where several UEs move from the the serving cell (to be disconnected during handover procedure) to the target cell (to be connected during handover procedure), the UEs should be provided with cell-specific K_offset of the target cell. Regarding transmission of K_offset values for the target cell, we propose two options as described in Proposal 3 and Proposal 4. Proposal 3 is simple and almost the same as the K_offset reception during initial access. However, reception and decoding of SIB seems to lead significant latency. In addition, the large period of SIB broadcasting may cause latency also. To overcome the inefficiency regarding latency, Proposal 4 states that the serving cell can be informed of the target cell’s K_offset via inter-cell or inter-satellite networks signalling and utilizes the serving cell in order to obtain the target cell’s K_offset during handover procedure.

Proposal 3: SIB of the target cell provides the handover UE with its cell-specific K_offset after connection to the target cell.

Proposal 4: The serving cell provides the handover UE with the cell-specific K_offset of the target cell using RRC reconfiguration during handover procedure.

Conclusion

In this contribution, the following conclusions were made:

Proposal 1: A new RRC signalling should be considered to provide update frequency of K_offset.

Proposal 2: Even after the initial access, cell-specific or beam-specific K_offset values provided by SIBs can be used to update K_offset.

Proposal 3: SIB of the target cell provides the handover UE with its cell-specific K_offset after connection to the target cell.

Proposal 4: The serving cell provides the handover UE with the cell-specific K_offset of the target cell using RRC reconfiguration during handover procedure.

Reference

[1] RP-210908, “Solutions for NR to support non-terrestrial networks (NTN),” Mar. 2021.

[2] R1-2106254, “FL summary#4 on NTN timing relationships,” RAN1 #105-e, Moderator (Ericsson).

[3] Chairman’s Note, 3GPP TSG RAN1 #105-e Meeting, May 2021.