3GPP TSG RAN WG1 Meeting #106-e                                     R1-2107199

e-Meeting, 16th August – 27th August, 2021

Source:               TCL Communication

Title:                    Discussion on joint channel estimation for PUSCH

Agenda item:      8.8.1.3

Document for:    Discussion and Decision

- Introduction

In the RAN1#105-e meeting, the discussions focused on use cases, time domain window. Some issues had achieved agreements or working assumptions [1]. However some issues are still under discussion. This contribution discusses use cases, time domain window.

- Discussion

  - Use cases

In the RAN1#104-e meeting, the following potential use cases are agreed for joint channel estimation for PUSCH:

  - Use case 1: back-to-back PUSCH transmissions within one slot.

  - Use case 2: non-back-to-back PUSCH transmissions within one slot.

  - Use case 3: back-to-back PUSCH transmissions across consecutive slots.

  - Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

  - Use case 5: PUSCH transmissions across non-consecutive slots.

In the RAN1#105-e meeting, the back-to-back PUSCH transmissions within one slot for use case 1 was supported, and the non-back-to-back PUSCH transmissions within one slot (use case 2) was not supported.

Agreement: - Joint channel estimation over non-back-to-back PUSCH transmissions within one slot is not supported. Agreement: - For back-to-back PUSCH transmissions within one slot, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases: - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A with consecutive slots - FFS: additional specification enhancements on top of that defined to support repetition Type A - Only for single layer transmissions - Subject to UE capability - Joint channel estimation over back-to-back PUSCH transmissions with different TBs within one slot is not supported.

The following cases are still under discussion.

#1：Back-to-back PUSCH transmissions across consecutive slots with different TBs for use case 3

For back-to-back PUSCH transmissions across consecutive slots, PUSCH transmissions with different TBs can improve coverage from JCE. The key requirements are to keep power consistency and phase continuity. These requirements would have the same impact on specifications for back-to-back PUSCH transmissions with the same TB and different TBs. We propose joint channel estimation should support back-to-back PUSCH transmissions across consecutive slots with different TBs.

Proposal 1: Joint channel estimation should support back-to-back PUSCH transmission across consecutive slots with different TBs.

#2：Non-back-to-back PUSCH transmissions across consecutive slots for use case 4

Another use case to discuss is that the non-back-to-back PUSCH transmissions across consecutive slots.  In RAN1#105-e, the following working assumption related to it has been made [1].

Working assumption: - For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases: - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant. - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A. - FFS: additional specification enhancements on top of that defined to support repetition Type A - Only for single layer transmissions - Subject to UE capability - FFS: Over non-back-to-back PUSCH transmissions with different TBs - FFS: Over non-back-to-back PUSCH transmissions for TBoMS - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.) - FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot.

From the RAN4 LS reply, if the gap is less than 14 symbols, the feasibility of maintaining phase and power condition has been confirmed when UE is not required to meet the existing off power requirements. The working assumption can be confirmed. For the FFS part of working assumption about TBoMS and different TBs, we also propose to confirm this.

Proposal 2: Confirm the following working assumption:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - Over non-back-to-back PUSCH transmissions with different TBs

  - Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot

  - Time domain window

#1: TDW enablement

In the last meeting, the issue of “whether the time domain window is implicitly enabled or disabled from joint channel estimation” was discussed. Joint channel estimation is based on the conditions to keep power consistency and phase continuity. The TDW is a time window in which the UE meets the conditions of JCE. As we can see from the current discussion, the TDW is configured to UE in order to execute JCE at gNB. If TDW is not configured by gNB, JCE is not executed at gNB. So we propose that JCE is implicitly enabled or disabled from TDW.

Proposal 3: Joint channel estimation is implicitly enabled or disabled from TDW.

#2: TDW for PUSCH repletion type A

The time domain window for PUSCH repletion type A has been discussed in last RAN#1 meeting. Two alternatives are agreed for down selection.

Agreement: For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, down select one of the following alternatives for the time domain window. - Alt 1: All the repetitions are covered by one single time domain window - The start of the window is the first PUSCH transmission - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum - FFS: frequency hopping and precoder cycling - Alt 2: All the repetitions are covered by one or multiple time domain windows - For the start of each window, - The start of the first window is the first PUSCH transmission. - FFS: how to determine the start of other windows, e.g., whether multiple windows are consecutive or non-consecutive, whether the start of the window depends on DL/UL configuration for unpaired spectrum - For the length of each window, - FFS Each window consists of at least two adjacent physical slots for UL transmission. - The length of each window is no longer than the maximum duration. - FFS: how to determine the length of each window - FFS: whether the length of each window depends on DL/UL configuration for unpaired spectrum - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum. - FFS: frequency hopping and precoder cycling - Other alternatives are not precluded.

The motivation for introducing time domain window is that UE is expected to maintain power consistency and phase continuity among PUSCH transmissions during the window. From RAN4 LS Reply, we know that modulation order, frequency resources, and transmission power level should not be changed to maintain power consistency and phase continuity. For non-back-to-back transmission with non-zero gap in-between adjacent transmissions, the gap is no more than 14 un-scheduled OFDM symbols and no downlink reception in the gap.

In this agreement, Alt 1 has only one time domain window for all repetitions. UE should meet the requirements from RAN4 LS among the whole transmission. It is difficult in TDD case and frequency hopping. Alt 2 has multiple time domain slots, and each window can be configured in continuous slots or continuous symbols. Alt 2 is easy to meet the requirements in both paired spectrum and unpaired spectrum.

Proposal 4: Alt 2 is preferred.

#3: The unit of time domain window

For paired spectrum and unpaired spectrum, each PUSCH transmission pattern should be used the same set of TDW unit. We don’t support that the time domain window is implicitly determined by DL/UL configuration.

Proposal 5: A uniform set of TDW unit for each PUSCH transmission pattern should be used for paired spectrum and unpaired spectrum.

In our view, the selection of unit should be based on the transmission pattern. For different PUSCH transmissions, it is not recommended to use uniform TDW units other than slot. We suggest the TDW may be specified using units of:

- For PUSCH repetition type A:  slot, repetition

- For PUSCH repetition type B:  slot, nominal repetition and/or actual repetition

- For different TBs: 		      slot, TB

- For TBoMS: 	                   slot, or a single time domain window across the entire TB

Proposal 6: Different TDW units are applied to each PUSCH transmission pattern.

- Conclusion

In this contribution the following proposals have been made:

Proposal 1: Joint channel estimation should support back-to-back PUSCH transmission across consecutive slots with different TBs.

Proposal 2: Confirm the following working assumption:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - Over non-back-to-back PUSCH transmissions with different TBs

  - Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot

Proposal 3: Joint channel estimation is implicitly enabled or disabled from TDW.

Proposal 4: Alt 2 is preferred.

Proposal 5: A uniform unit for each PUSCH transmission pattern should be used for paired spectrum and unpaired spectrum.

Proposal 6: Different TDW units are applied to each PUSCH transmission pattern.

References

[1] 3GPP RAN1 Chair’s Notes RAN1#105-e

[2] R1-2106152 Summary of email discussion on joint channel estimation for PUSCH

[3] 3GPP RAN1#104b-e, R1-2104119, Reply LS on PUCCH and PUSCH repetition, RAN1

[4] 3GPP RAN1#105-e, R1-2104168, Reply LS on PUCCH and PUSCH repetition, RAN4

[5] 3GPP RAN1#104b-e, R1-2102298, Reply on LS on PUCCH and PUSCH repetition, RAN4