3GPP TSG RAN WG1#106-e                                	R1-2107287

e-Meeting, August 16th – August 27th, 2021

Source: 	FGI, Asia Pacific Telecom, III, ITRI

Title: 	Timing relationship enhancements in NTN

Agenda item:	8.4.1

Document for:	Discussion and Decision

# Introduction

In RAN1#105-e, the following agreements were made.

Agreement in RAN1#105-e If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH. - Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized. Agreement in RAN1#105-e The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT. - The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac. Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by NTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×TcTTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of NTA,common×Tc and K_mac. How to treat NTA and NTA,offset can be further discussed. Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed. Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed. Agreement in RAN1#105-e The K_offset value signaled in system information is always used for - The transmission timing of RAR / fallbackRAR grant scheduled PUSCH - The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI - The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI - FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI - The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI - FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI FFS: how to treat additional transmission timings related to fallback DCI formats FFS: how to update this formulation with beam-specific K_offset if beam-specific K_offset is agreed to be supported

Based on the FFS in the above agreements, the following discussions were needed.

- K_mac signaling

- Accuracy of the estimated UE-gNB RTT

- FFS: Transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH and MsgB

- FFS: Transmission timings related to fallback DCI formats

- FFS: Beam-specific K_offset update if supported.

Based on the feature lead summary R1-2106325, the following discussions were encouraged.

- Issue #1: TA reporting

- Issue #10: PDCCH ordered PRACH

# Discussion

## K_mac signaling

The intention to introduce K_mac is to prevent UE activates a MAC CE command before gNB confirms the command by receiving HARQ feedback of the command. K_mac is needed If DL and UL frame timing are not aligned at gNB.

In RAN1#105-e, companies have agreed to use K_mac to estimate UE-gNB RTT at least for the start of RAR windows. Regarding the estimated UE-gNB RTT is needed before RRC_CONNECTED, K_mac shall be carried in SIB1.

K_mac is needed in initial access, which can be carried explicitly or implicitly in SIB.

For K_mac used in initial access, the information of K_mac shall be carried explicitly in SIB1, if NTN-SIB is not supported.

However, the estimated UE-gNB RTT would be used for DRX operation based on the RAN2 agreements, see below.

Agreements via email (from offline 103) in RAN2#114-e: RAN2 working assumption: Offset for drx-HARQ-RTT-TimerUL is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it). Agreements - via email (from offline [103]) in RAN2#113b-e: RAN2 has agreed to use UE-gNB RTT as the offset to start some UP timers (e.g., drx-HARQ-RTT-TimerDL). Ask RAN1 to provide inputs on (i) how UE acquires UE-gNB RTT and (ii) what additional information needs to be broadcasted other than that for TA pre-compensation, if any.

For quasi-earth-fixed cells, i.e., earth-fixed cells provided by an LEO-based NW, gNB-satellite RTT would change few microseconds per second. Therefore, if updating K_mac after initial access is not supported, the estimate of gNB-satellite RTT equal to the sum of NTA,common×Tc and K_mac will be degraded as time goes by.

Updating K_mac after initial access shall be supported to benefit DRX operation, at least for quasi-earth-fixed cells.

For updating K_mac after initial access, singling can be either via RRC reconfiguration or via MAC CE. To prevent an ambiguous period between UE receives the RRC reconfiguration and UE applies the RRC, updating K_mac after initial access via MAC CE shall be supported.

MAC CE shall be considered to update K_mac after initial access to prevent an ambiguous period due to an unclear action time for applying an RRC reconfiguration.

K_mac description has been added in RAN1#103-e, and its high-level description is made in RAN1#105-e.  See below.

Agreement in RAN1#103-e Denote by K_mac a scheduling offset other than K_offset: - If downlink and uplink frame timing are aligned at gNB: - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed. - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed. - If downlink and uplink frame timing are not aligned at gNB: - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is needed. - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed. - Note: This does not preclude identifying exceptional MAC CE timing relationship(s) that may or may not require K_mac. Agreement in RAN1#104b-e UE can be provided by network with a K_mac value. - When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. Agreement in RAN1#105-e If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH. Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

However, as we highlight above, it is unclear for UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, if a UE is provided with a K_mac value.

If a UE is provided with a K_mac value, it is unclear for UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH.

Thus, we then propose to make it complete.

For an uplink configuration, if a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on the uplink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ, where µ is the SCS configuration for the PUCCH.

Note that sometimes it may be difficult to define a MAC CE common is on a DL configuration or a UL configuration, e.g., if K_offset or K_mac is updated via a MAC CE command, then they would be used for different procedures.

## Accuracy of the estimated UE-gNB RTT

The intention to estimate UE-gNB RTT is used to enhance 1) the start of the RAR window, 2) drx-HARQ-RTT-TimerDL, 3) drx-HARQ-RTT-TimerUL, and 4) sr-ProhibitTimer, if agreed. However, the required granularities are quite different.

For the RAR window, no offset exists in the current release of NR. However, considering the RAR window length is given in slots, see TS 38.331 below, the required granularity for UE-gNB RTT could be in slots.

ra-ResponseWindow                   ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

RACH-ConfigGeneric field descriptions

ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 is signalled, UE shall ignore the ra-ResponseWindow (without suffix).

For drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL, the current values are in symbols of the BWP where the TB was received, see TS 38.331 below. To minimize implementation impact, the required granularity for UE-gNB RTT shall follow the legacy as the values in symbols of the BWP where the TB was received.

drx-HARQ-RTT-TimerDL                INTEGER (0..56),

drx-HARQ-RTT-TimerUL                INTEGER (0..56),

DRX-Config field descriptions

drx-HARQ-RTT-TimerDL Value in number of symbols of the BWP where the transport block was received.

drx-HARQ-RTT-TimerUL Value in number of symbols of the BWP where the transport block was transmitted.

For sr-ProhibitTimer, the current values are given in milliseconds, see TS 38.331 below. To minimize implementation impact, the required granularity for UE-gNB RTT shall be in milliseconds.

sr-ProhibitTimer				ENUMERATED {ms1, ms2, …, ms64, ms128}		OPTIONAL, -- Need S

SchedulingRequestToAddMod field descriptions

sr-ProhibitTimer Timer for SR transmission on PUCCH in TS 38.321 [3]. Value is in ms. Value ms1 corresponds to 1ms, value ms2 corresponds to 2ms, and so on. When the field is absent, the UE applies the value 0.

The required granularities are ms, slots, and symbols. To meet the minimum granularity, the accuracy of the estimated UE-gNB RTT shall reach a symbol level.

The accuracy requirement of the estimated UE-gNB RTT must be within a symbol.

However, the estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac, where UE’s TA can be accurate within the initial transmit timing requirement of Te or a new requirement of Te_NTN being around 20% to 40% of the CP length or few symbols, and K_mac can be accurate to a slot level.

The estimated UE-gNB RTT cannot fit the accuracy requirement at least for DRX operations.

RAN2 shall be informed that the estimated UE-gNB RTT can only provide a slot-level accuracy, and from the RAN1 perspective it may not be worth providing a symbol-level accuracy to support DRX operations.

LS to RAN2 that the estimated UE-gNB RTT provides a slot-level accuracy and may have issues to support DRX operations, e.g., offset for drx-HARQ-RTT-TimerUL is equal to UE-gNB RTT.

## Transmission timings for K_offset usage

The intention to always use the K_offset value signaled in system information (SI) is to simplify the Random Access (RA) procedure among UEs, which seems problematic if some UEs use the UE-specific K_offset and the others use the cell-specific K_offset during the RA.

Since the RA includes contention-based RA (CBRA) and contention-free RA (CFRA), to our best understanding they shall follow the same principle by always using the K_offset value signaled in SI.

Support the transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI.

Support the transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI.

The intention to differentiate fallback DCI formats and non-fallback DCI formats for K_offset is to prevent that during the update of K_offset, UE and network (NW) may not have the same understanding about the K_offset value used in timing relationships that do not use the one signaled in system information.

However, this misunderstanding between UE and NW only happens if the K_offsest update is via RRC reconfiguration. To our best understanding, the support of K_offset update via MAC CE can ensure UE and NW have the same understanding about K_offset value by a proper MAC CE action time well defined by RAN1 in this release.

No additional transmission timings related to fallback DCI formats are needed if the MAC CE option is supported for updating K_offset after initial access.

The intention to discuss beam-specific K_offset is to further enhance its accuracy by considering the use of a small satellite beam. However, the association between satellite beams and BWPs is unclear. Propose to postpone this.

Postpone the discussion on beam-specific K_offset update for waiting for more progress on the association between satellite beams and BWPs.

## TA reporting

The intention to use TA reporting is to update UE-specific K_offset for a better scheduling efficiency and also build a common understanding between UE and NW about UE-gNB RTT.

As shown in R1-2106325, companies have reached a proposal about TA reporting. See below.

Moderator recommendation on TA reporting:

Companies are encouraged to provide input on this issue at the next RAN1 meeting, taking into account the questions asked by RAN2 LS:

- At least for uplink scheduling adaptation, what is the exact content of UE reporting of information about the UE-specific TA pre-compensation?

  - Option 1: UE specific TA: [Apple, MediaTek, CATT, Lenovo/MM(?)]

  - Option 2: Full TA: [ZTE, Lenovo/MM(?)]

  - Option 3: UE location: [Panasonic]

  - Option 4: Difference between UE-specific K_offset and cell-specific K_offset: [CMCC]

  - Other option(s)?

- At least for uplink scheduling adaptation, how frequent is the UE reporting of information about the UE-specific TA pre-compensation?

  - Option 1: Event-triggered: [Apple, ZTE, Panasonic, LG, Qualcomm, Xiaomi, CMCC, CATT]

  - Option 2: Network request: [ZTE, Qualcomm]

  - Option 3: Periodic: [Xiaomi, Lenovo/MM]

  - Other option(s)?

For 1) the exact content of UE reporting of information, Option 3: UE location may have a privacy issue and Option 4 seems further optimization, which may not be essential to the current stage. However, for both Option 1: UE-specific TA and Option 2: Full TA, RAN1 may need to consider the impact of a new gradual timing adjustment (GTA) requirements for the NTN network introduced by RAN4.

The new GTA will allow UE to update UL timing autonomously in a larger step comparing to the legacy GTA. The goal is to track the DL reception timing drift and allow UE to adjust UL timing accordingly. Therefore, if the TA reporting lacks the UL timing adjustment contributed by the GTA, then UE and NW have a misunderstanding on UE-gNB RTT.

TA reporting at least for Option 1: UE-specific TA and for Option 2: Full TA shall include UL timing adjustment contributed by the gradual timing adjustment requirements.

For 2) at least for uplink scheduling adaptation, how frequent is the UE reporting of information, considering TA reporting may be only useful for quasi-earth-fixed cells, i.e., earth fixed cells for LEO-based NW, Option 3: periodic shall not be supported. Option 1: event-triggered seems to be an optimization for Option 2: NW request.

TA reporting shall at least support Option 2: NW request as a baseline, and the support of Option 1: event-triggered shall be considered if further enhancement is needed.

## PDCCH ordered PRACH

The intention to introduce K_offset for PDCCH ordered PRACH is to reduce unnecessary network blind detection. As shown in R1-2106325, companies seem to reach a consensus. See below.

In the first round of email discussions, 18 companies provided views.

- 15 companies support / can accept Option 1: [Intel, Samsung, APT, ZTE, Nokia/NSB, Spreadtrum, Panasonic, LG, CAICT, Huawei/HiSi, MediaTek, Sony, InterDigital, CMCC, CATT]

- 3 companies support Option 2: [NTT Docomo, OPPO, Lenovo/MM]

However, Moderator has decided to wait for [CAICT] to report back the offline discussion outcome and then make a proposal. Before any progress is made by CAICT, we would like to repeat that introducing K_offset would be beneficial for relaxing the processing time requirement at the UE side and reducing blind detection at the NW side.

On the timing relationship of PDCCH ordered PRACH, Option 1: Introduce K_offset to enhance the timing relationship of PDCCH ordered PRACH shall be supported.

# Conclusion

In this contribution, we have the following observations

Observation 1	K_mac is needed in initial access, which can be carried explicitly or implicitly in SIB.

Observation 2	If a UE is provided with a K_mac value, it is unclear for UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH.

Observation 3	The accuracy requirement of the estimated UE-gNB RTT must be within a symbol.

Observation 4	The estimated UE-gNB RTT cannot fit the accuracy requirement at least for DRX operations.

Based on observations, the following proposals are made

Proposal 1	For K_mac used in initial access, the information of K_mac shall be carried explicitly in SIB1, if NTN-SIB is not supported.

Proposal 2	Updating K_mac after initial access shall be supported to benefit DRX operation, at least for quasi-earth-fixed cells.

Proposal 3	MAC CE shall be considered to update K_mac after initial access to prevent an ambiguous period due to an unclear action time for applying an RRC reconfiguration.

Proposal 4	For an uplink configuration, if a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on the uplink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ, where µ is the SCS configuration for the PUCCH.

Proposal 5	LS to RAN2 that the estimated UE-gNB RTT provides a slot-level accuracy and may have issues to support DRX operations, e.g., offset for drx-HARQ-RTT-TimerUL is equal to UE-gNB RTT.

Proposal 6	Support the transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI.

Proposal 7	Support the transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI.

Proposal 8	No additional transmission timings related to fallback DCI formats are needed if the MAC CE option is supported for updating K_offset after initial access.

Proposal 9	Postpone the discussion on beam-specific K_offset update for waiting for more progress on the association between satellite beams and BWPs.

Proposal 10	TA reporting at least for Option 1: UE-specific TA and for Option 2: Full TA shall include UL timing adjustment contributed by the gradual timing adjustment requirements.

Proposal 11	TA reporting shall at least support Option 2: NW request as a baseline, and the support of Option 1: event-triggered shall be considered if further enhancement is needed.

Proposal 12	On the timing relationship of PDCCH ordered PRACH, Option 1: Introduce K_offset to enhance the timing relationship of PDCCH ordered PRACH shall be supported.

# Reference

Chairman’s Notes, 3GPP TSG RAN WG1 #105 e-meeting

Chairman’s Notes, 3GPP TSG RAN WG2 #114 e-meeting

R1-2106325, Feature lead summary#5 on timing relationship enhancements, Ericsson