3GPP TSG-RAN WG1 #106-e	R1-2107331

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.2.2

Source: 	Qualcomm Incorporated

Title: 	PDCCH monitoring enhancements

Document for:	Discussion/Decision

# Introduction

In RAN1 #103-e, the RAN1-part study on the NR operation in a frequency regime between 52.6GHz and 71GHz was finalized. During the study, potential issues with physical signal and channel designs related to the adoption of new high numerologies, i.e., 480kHz and 960kHz, have been identified and summarized in TR 38.808 [1]. In RAN #90-e and RAN #92-e, the WID was revised based on the outcomes of the RAN1 study [2].

For PDCCH monitoring aspect, the following enhancements have been suggested for the WI:

- Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

In this contribution, we present our views on the multi-slot PDCCH monitoring capability design and further PDCCH monitoring enhancement for NR operation in the frequency regime between 52.6GHz and 71GHz, i.e., FR2-2.

# Discussion

## Multi-slot PDCCH monitoring capability

In RAN1 #104bis-e, the following agreement, in relation to the multi-slot PDCCH monitoring capability, has been made:

Agreement: Previous agreement is modified as follows: Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability - Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability. - Each slot group consists of X slots - Slot groups are consecutive and non-overlapping - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g. the Y [symbols or slots] always start at the first slot within a slot group - FFS: Further definition of capabilities - Alt 2: Use an (X, Y) span as the baseline to define the new capability - X is the minimum time separation between the start of two consecutive spans - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots] - Y <= X - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary. - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots. - FFS: Further definition of capabilities - Alt 3: Use a sliding window of X slots as the baseline to define the new capability. - The capability indicates the BD/CCE budget within the sliding window - The sliding unit of the sliding window is [1] slot. - FFS: Further definition of capabilities - Specific numbers for X, Y may depend on UE capability and gNB configuration - Examples: - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS

In this section, we present our views on the multi-slot PDCCH monitoring capability in relation to the above agreement.

### Multi-slot PDCCH monitoring support for different SCSs

For high SCSs, it would be necessary to support both per-slot and multi-slot PDCCH monitoring capabilities; although the maximum number of BD/CCE is limited, per-slot PDCCH monitoring may be useful in some use cases, e.g., for serving a low-latency traffic or for channel monitoring outside a COT. As pointed out in earlier meetings, per-slot PDCCH monitoring is a special case of multi-slot PDCCH monitoring and can be handled in the same framework. If both per-slot and multi-slot capabilities are supported, the default capability, which is assumed when there is no dedicated RRC configuration, should be determined, e.g., when 480 kHz SCS is used for initial access. Noting that per-slot PDCCH monitoring would be more demanding for the high SCSs, having multi-slot PDCCH monitoring as the default capability is desirable.

Proposal 1: For 480 kHz and 960 kHz SCSs, multi-slot PDCCH monitoring is the default capability, and assumed during the idle/inactive mode operation (e.g., for ANR detection) and initial access procedure.

Proposal 2: For 480 kHz and 960 kHz SCSs, per-slot PDCCH monitoring (i.e., X = 1 slot) is supported as an optional UE capability during a connected mode operation.

If a UE supports both per-slot and multi-slot PDCCH monitoring (i.e., supporting more than one X values), switching between the two different PDCCH monitoring behaviors may be required. For instance, for the operation in FR2-2 unlicensed band, more frequent PDCCH occasions (e.g., every slot) would be necessary so that the gNB can start transmitting PDCCH as early as possible after LBT success. On the other hand, for data transmission and reception during a COT, per-multi-slot PDCCH monitoring may be assumed at least for the power efficient operation. In Rel-16, for FR1, each BWP can be configured with either per-slot or per-span PDCCH monitoring (i.e., by configuration of monitoringCapabilityConfig-r16) and, thus, the switching between two different PDCCH monitoring behaviors is through BWP switching. Thus, if the per-span PDCCH monitoring capability is extended for high SCSs as will be discussed in Section 2.1.3, the same BWP-based mechanism would directly be applied.

As an alternative switching mechanism, particularly for the unlicensed band operation, search space set group switching can be considered. In this case, each search space set group may be configured for either per-slot or multi-slot PDCCH monitoring. For example, search space set group 0 (i.e., the default group) can be configured with per-slot PDCCH monitoring and used when the UE is outside the channel occupancy time. On the other hand, search space set group 1 can be configured with multi-slot PDCCH monitoring and used during a COT. Although search space set group switching has dedicatedly been used for NR-U operation in Rel-16, the discussion on the extension for licensed band operation is in progress in Rel-17 UE power saving WI. Therefore, if supported for the licensed band operation, search space set group switching will provide more dynamic transition between per-slot and multi-slot PDCCH monitoring, both for unlicensed and licensed band operation.

Proposal 3: For UEs supporting both per-slot and multi-slot PDCCH monitoring capabilities, support a dynamic switching mechanism between per-slot and multi-slot PDCCH monitoring capabilities.

Observation 1: Bandwidth part switching and search space set group switching mechanisms can be considered as candidate switching mechanism between per-slot and multi-slot PDCCH monitoring.

### Supported value(s) of X for multi-slot PDCCH monitoring

In determining the value(s) of X, the existing per-slot PDCCH monitoring capability for 120 kHz can be the baseline. The new capability should achieve at least a similar extent of scheduling flexibility and power consumption as the 120 kHz SCS. Therefore, as stated in the FL’s proposal A1-2 in RAN1 #104bis-e, X = 4 slots for 480 kHz SCS and X = 8 slots for 960 kHz SCS should be the considered as default values that are supported by all UEs. Larger values of X than those may adversely affect the performance compared to that of 120 kHz SCS and should not be supported. On the other hand, based on the UE capability, additional values smaller than X = 4 slots for 480 kHz SCS and X = 8 slots for 960 kHz SCS, e.g., {1, 2} for 480 kHz and {1, 4} for 960 kHz, may optionally be supported. In particular, X = 1 corresponds to the per-slot PDCCH monitoring capability.

Proposal 4: For the value of X in the multi-slot PDCCH monitoring capability, the following sets are considered:

- 480 kHz SCS: X = {1, 2, 4} slots, where 4 is the default value (supported by all UEs), while X=1 and X=2 are per UE capability,

- 960 kHz SCS: X = {1, 4, 8} slots, where 8 is the default value (supported by all UEs), while X=1 and X=4 are per UE capability.

In the same vein, not to harm the performance compared to that of 120 kHz SCS, the same number of BD/CCE limit should be considered as the starting point with X = 4 slots for 480 kHz SCS and X = 8 slots for 960 kHz SCS.

Proposal 5: For the multi-slot PDCCH monitoring capability with X = 4 slots for 480 kHz SCS and X = 8 slots for 960 kHz SCS, at least the same maximum numbers of PDCCH candidates and non-overlapped CCEs as 120 kHz SCS are supported (i.e., 20 BDs and 32 CCEs).

### Definition of multi-slot PDCCH monitoring capability

As we discussed, one of the key motivations of multi-slot PDCCH monitoring is the improved scheduling flexibility by not reducing the maximum number of BD/CCEs per monitoring occasion (MO). Another important motivation, which should not be overlooked, is the enhanced power efficiency. In general, by separating two PDCCH MOs far enough, the micro-sleep opportunity increases. For example, if there is only one PDCCH MO or one span within X = 4 slots for 480 kHz SCS and X = 8 slots for 960 kHz SCS, at least the same micro-sleep opportunity as that of 120 kHz SCS is guaranteed. Since 480 kHz or 960 kHz SCSs can support a larger bandwidth (i.e., larger portion of RF power in overall power consumption) compared to 120 kHz SCS, the same micro-sleep opportunity may be translated to a higher power saving gain. On the contrary, a larger number of MOs dispersed within X slots may adversely impact the power efficiency by requiring UE

- to keep the RF frontend blocks active for a longer time,

- to repeat ramping up and down its frontend blocks more often, and

- to perform more FFT operation across MOs.

Observation 2: More than one PDCCH monitoring occasion or span dispersed within a X-slot duration may adversely impact the power efficiency.

Among different alternatives agreed in RAN1 #104bis-e, Alt 1 and Alt 3 do not restrict the position and number of MOs within a X-slot window, particularly when Y is large, and the gain of multi-slot scheduling is diluted. Thus, to improve power efficiency, one can pose additional restriction on the position and number of MO within the window, and they boil down to Alt 2. As such, due to the clear advantage in power efficiency, it is desirable to support Alt 2.

For further details of Alt 2, the first up to 3 symbols within a slot can be used for MOs, at least for UE-specific search space sets, Type 1 common search space set with dedicated RRC configuration, and Type 3 common search space set; unlike the cases of SCSs smaller than or equal to 120 kHz, the motivation and benefit of sub-slot-level PDCCH monitoring is not clear. Furthermore, in the agreement for Alt 2, it is FFS whether the repetition of the same span pattern over a number of slots is needed. The FFS is based on the view that Alt 2 is an extension of Rel-15 PDCCH monitoring capability, i.e., FG 3-5b (pdcch-MonitoringAnyOccasionsWithSpanGap). However, in our view, Alt 2 should be regarded as an extension of Rel-16 per-span PDCCH monitoring capability, i.e., FG 11-2 (pdcch-Monitoring-r16), and the notion of the repeated span pattern is not relevant. To clarify, the same definition of span in Rel-16 should be used (Section 10 in TS 38.213):

- A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH.

- A span starts at a first symbol where a PDCCH MO starts and ends at a last symbol where a PDCCH MO ends, where the number of symbols of the span is up to Y.

Proposal 6: For the definition of multi-slot PDCCH monitoring capability, Alt 2 is supported with the following modification:

- Alt 2: Use an (X, Y) span as the baseline to define the new capability

  - X is the minimum time separation in symbols between the start of two consecutive spans

  - The capability indicates the BD/CCE budget within a span of at most Y consecutive symbols

    - A span is placed within the first 3 OFDM symbols of a slot for USS and CSS with dedicated RRC configuration

  - The following combinations of (X, Y) are supported:

    - 480 kHz SCS: (14, 3), (28, 3), (56, 3)

    - 960 kHz SCS: (14, 3), (56, 3), (112, 3)

### Common search space set enhancement

In Proposal 1, it is proposed to assume multi-slot PDCCH monitoring as the default capability for 480kHz and 960kHz SCSs. This implies that, while the UE is monitoring common search spaces during idle/inactive mode operation (e.g., for ANR detection) or initial access procedure, multi-slot PDCCH monitoring should be applied. Further, in Proposal 6, it is proposed to adopt Alt 2 definition of the multi-slot PDCCH monitoring. These two proposals may lead to the following potential issues:

Issue 1: The current design of search space set #0 (i.e., Type0 PDCCH CSS) for SSB-CORESET multiplexing pattern 1 requires the UE to monitor PDCCH over two consecutive slots. This requirement is not compliant with the default multi-slot PDCCH monitoring capability and a new design is needed.

Issue 2: When the UE monitors CSSs (e.g., Type0/0A/1/2 PDCCH CSSs) during the connected mode operation, Alt 2 definition requires that the MOs for CSS and USS should either be aligned in the same Y-symbol span, or be separated by at least X symbols. As identified in earlier meetings, this would be too restrictive in some cases. For example, the network may prefer to stagger different UEs’ USS MOs over X slots as shown in Figure 1 to distribute the control channel overhead. However, since the CSS MO is common for all UEs, it may not be possible to align the CSS MO with all UEs’ span. In the example in Figure 1, where the USS MOs are configured per (X, Y) = (56, 3), UE1’s CSS and USS MOs coincide in the same span and it can monitor both the CSS and USS. However, UE2, UE3, and UE4 cannot monitor both CSS and USS MOs since the minimum separation of X = 56 symbols is not satisfied between CSS and USS MOs.

Figure 1: Configuration example of USS and CSS MOs.

To address the issues, some enhancement and modification for the CSS design is necessary. The enhancement should be applied to Type0/0A PDCCH CSSs, as well as Type1/2 PDCCH CSSs with or without dedicated configuration. Different candidate approaches would be considered to address the issues.

Alt 1: New search space set #0 (Type0 CSS) design

As discussed in Issue 1 above, for SSB-CORESET multiplexing pattern 1, the existing Type0 CSS requires the UE to monitor two consecutive slots, slot n0 and slot n0+1, which is not compliant with multi-slot PDCCH monitoring. Thus, to address the issue, a new design of Type0 CSS may require the UE to monitor two non-consecutive slots, i.e., slot n0 and slot n0+X0, where X0 satisfies the default UE capability, e.g., X0=4 for 480 kHz and X0=8 for 960 kHz, as discussed in Proposal 4. Similar design enhancement can further be discussed for other SSB-CORESET multiplexing pattens.

To address Issue 2, the PDCCH transmission in a CSS MO would be repeated over multiple consecutive slots. For example, in addition to the enhancement discussed above, the same or equivalent PDCCH (and the associated PDSCH) may be repeated over slots n0, n0+1, …, and n0+X0-1. It should be noted that the repetition assumes the same QCL-TypeD property, and different from the existing beam sweeping transmission of CSS. Thus, the network can configure different UE’s USS so that at least one occasion of the CSS repetition overlaps with the USS MO, and the UE only monitors CSS MO that overlap with its USS MO. Furthermore, once the baseline design of the CSS repetition is introduced, it may be enhanced to support extended coverage for FR2-2 in later NR releases.

Alt 2: New CSS prioritization rule

In Rel-15, when the MOs of different CORESETs overlap in time, a prioritization rule is applied. For example, when a CSS MO overlaps with a USS MO with a different QCL-TypeD property, monitoring of the CSS MO is prioritized. To address Issue 2, the CSS prioritization can be extended so that the UE prioritize CSS over USS, not only when they overlap, but also when they are non-overlapping but closely located.

For instance, the prioritization rule may be augmented with a notion of CSS zone. As illustrated in Figure 2, windows of X1 and X2 slots (or symbols) may be placed before and after the CSS MO, respectively, to define a CSS zone, where the values of X1 and X2 may be up to UE capability. If a USS MO falls within the CSS zone, the UE may be expected to prioritize CSS MO and drop the USS MO. From UE’s perspective, among the multiple CSS MOs with different QCL-TypeD properties (i.e., up to Lmax=64 different beams), only one or a few of them are actually monitored and associated with CSS zones. The UE and network may agree on the CSS MOs that are actually monitored based on another rule or signaling, which may include:

- A MAC CE activation command indicating a TCI state for the CORESET associated with the CSS (i.e., CORESET #0),

- An SSB identified by a recent random access procedure by the UE, which is not initiated by a PDCCH order,

- Active TCI states of the active BWP, which includes CSI-RSs quasi-co-located with SSBs, or

- Dedicated configuration of Type1/2 PDCCH CSS.

During the connected mode operation, monitoring of CSS is relatively infrequent and thus the actual blockage event of PDCCH transmission in USS would be rare. Also, even though the USS is cancelled by the aforementioned CSS prioritization, the UE can still receive a scheduling grant with C-RNTI within the CSS MO via DCI format 0_0/1_0. Therefore, the impact of the extended CSS prioritization rule can be kept marginal.

Since there could be many different alternatives than the two discussed above, it would be desirable to extend the discussion in RAN1 and specify any enhancement of the common search space design.

Proposal 7: If 480 kHz or 960 kHz SCS is used for ANR detection or initial access in the SPCell, common search space set design should be enhanced to address multi-slot-based CSS monitoring and multiplexing with USS; Enhancement of search space set #0 (i.e., Type0 PDCCH CSS) for SSB-CORESET multiplexing pattern 1 is prioritized.

Figure 2: CSS prioritization with CSS zone.

## Other PDCCH-related issues

### Cross-carrier scheduling

In RAN1 #104-e, the following draft proposal has been captured in the FL’s summary for further discussion [3]:

Modified Feature Lead Proposal A1-4: - Cross-carrier scheduling of a cell within 52.6-71 GHz from/[to] a cell outside 52.6-71 GHz is supported. - FFS: potential limitations on the applicable SCS(s) of the scheduling and scheduled cells/BWPs.

To support both SA and NSA operations efficiently for cells in 52.6-71 GHz, extending the use of cross-carrier scheduling seems necessary. However, when the SCS difference between the scheduling and scheduled cells are very large (e.g., scheduling from 15 kHz SCS to 960 kHz SCS, and vice versa), the gain of cross-carrier scheduling may be harmed, while the design (e.g., timeline design) would be complicated. Therefore, it would be fair to put some restriction on the selection of SCSs. Since Rel-15 already supports cross-carrier scheduling between 15 kHz and 120 kHz SCSs as the extreme case, the same ratio of SCSs may be assumed for 51.6-71 GHz.

Proposal 8: Cross-carrier scheduling of a cell within 52.6-71 GHz from/to a cell outside 52.6-71 GHz is supported, at least for |μPDCCH − μPDSCH | ≤ 3.

### Multi-carrier PDCCH monitoring capability and overbooking

If a UE is configured with multiple downlink cells of 480 kHz or 960 kHz SCSs, and each of the multiple cells is configured with multi-slot PDCCH monitoring, the distribution of the number of monitored PDCCH candidates and non-overlapped CCEs across the carriers should be considered. A clear benefit of adopting Alt 2 definition of the multi-slot PDCCH monitoring in Proposal 6 is that the existing rules associated with Rel-16 per-span PDCCH monitoring capability (pdcch-MonitoringCA-r16) can directly be applied (Section 10.1 in TS 38.213). That is, the distribution rules depend on the SCSs of active BWPs of the active cells, and on whether the spans of Y symbols are aligned or not aligned across cells.

Observation 3: For multi-slot PDCCH monitoring capability Alt 2, the existing rules for Rel-16 per-span PDCCH monitoring can directly be applied to determine multi-carrier PDCCH monitoring capability.

Similarly, for PDCCH overbooking, the existing rules associated with Rel-16 per-span PDCCH monitoring capability can directly be applied. That is, the overbooking is allowed only on a primary cell, and dropping of PDCCH candidates and non-overlapped CCEs is performed on a span-basis.

Observation 4: For multi-slot PDCCH monitoring capability Alt 2, the existing rules for Rel-16 per-span PDCCH monitoring can directly be applied for PDCCH overbooking and dropping.

# Conclusion

Proposal 1: For 480 kHz and 960 kHz SCSs, multi-slot PDCCH monitoring is the default capability, and assumed during the idle/inactive mode operation (e.g., for ANR detection) and initial access procedure.

Proposal 2: For 480 kHz and 960 kHz SCSs, per-slot PDCCH monitoring (i.e., X = 1 slot) is supported as an optional UE capability during a connected mode operation.

Proposal 3: For UEs supporting both per-slot and multi-slot PDCCH monitoring capabilities, support a dynamic switching mechanism between per-slot and multi-slot PDCCH monitoring capabilities.

Proposal 4: For the value of X in the multi-slot PDCCH monitoring capability, the following sets are considered:

Proposal 5: For the multi-slot PDCCH monitoring capability with X = 4 slots for 480 kHz SCS and X = 8 slots for 960 kHz SCS, at least the same maximum numbers of PDCCH candidates and non-overlapped CCEs as 120 kHz SCS are supported (i.e., 20 BDs and 32 CCEs).

Proposal 6: For the definition of multi-slot PDCCH monitoring capability, Alt 2 is supported with the following modification:

Proposal 7: If 480 kHz or 960 kHz SCS is used for ANR detection or initial access in the SPCell, common search space set design should be enhanced to address multi-slot-based CSS monitoring and multiplexing with USS; Enhancement of search space set #0 (i.e., Type0 PDCCH CSS) for SSB-CORESET multiplexing pattern 1 is prioritized.

Proposal 8: Cross-carrier scheduling of a cell within 52.6-71 GHz from/to a cell outside 52.6-71 GHz is supported, at least for |μPDCCH − μPDSCH | ≤ 3.

Observation 1: Bandwidth part switching and search space set group switching mechanisms can be considered as candidate switching mechanism between per-slot and multi-slot PDCCH monitoring.

Observation 2: More than one PDCCH monitoring occasion or span dispersed within a X-slot duration may adversely impact the power efficiency.

Observation 3: For multi-slot PDCCH monitoring capability Alt 2, the existing rules for Rel-16 per-span PDCCH monitoring can directly be applied to determine multi-carrier PDCCH monitoring capability.

Observation 4: For multi-slot PDCCH monitoring capability Alt 2, the existing rules for Rel-16 per-span PDCCH monitoring can directly be applied for PDCCH overbooking and dropping.

# References

- TR 38.808, “Study on supporting NR from 52.6 GHz to 71 GHz (Release 17)”

- RP-211584, “Revised WID: Extending current NR operation to 71GHz”, RAN #92-e, Qualcomm, Intel

- R1-2102242, “Feature lead summary#2 for [104-e-NR-52-71GHz-02] on PDCCH monitoring enhancements”, RAN1 #104-e, Lenovo