| 3GPP TSG-RAN WG1 Meeting #106-e e-Meeting, August 16th – 27th, 2021 | R1-2107338 |
| --- | --- |

Agenda item:	8.3.2

Source: 	Qualcomm Incorporated

Title: 	Uplink enhancements for URLLC in unlicensed controlled environments

Document for:	Discussion/Decision

# Introduction

In RAN#88e, the WID of “Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR” was revised with following objectives [1]:

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering

    - UE feedback enhancements for HARQ-ACK [RAN1]

    - CSI feedback enhancements to allow for more accurate MCS selection [RAN1]

Note: DMRS-based CSI feedback is not in scope of this WI

- Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]:

  - Specify support for UE-initiated COT for FBE with minimum specification effort

  - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

- Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]:

- Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH.

- Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline

- Enhancements for support of time synchronization:

- RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2]

- Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4]

- RAN enhancements based on new QoS related parameters if any, e.g. survival time, burst spread, decided in SA2. [RAN2, RAN3]

In this contribution, we will focus on the second item to discuss uplink enhancements for URLLC in unlicensed controlled environments.

# UE-initiated COT for FBE

In RAN1#102e and RAN1#103e, we agreed the following:

Agreements:

- The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s).

  - Note: UE initiated FBE configuration is configured per serving cell

Agreements:

- A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst.

Based on the agreements above, UE can initiate a COT for UL transmission and may share it to gNB for FBE mode in general. In this section, we will discuss some detailed designs to support this feature.

## Determination of COT for scheduled UL transmission

In RAN1#104e, we got the following agreements for UE how to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT. In RAN1#105e, Alt-a is taken in the following agreement.

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

- Alt-b: Determination based on the rules applied for a configured UL transmission

In the agreement, UE will determine whether a scheduled UL transmission is based on UE-initiated COT or sharing gNB-initiated COT through the corresponding content field in the scheduled DCI. In this section, we will handle the two FFS items in the agreement.

### Content field for UE-initiated COT

It is agreed that the determination is based on the content in the scheduling DCI. For FBE mode, the UE is configured with a FFP configuration. gNB may dynamically enable/disable the UL transmission to initiate the COT with the configured FFP. The indication bits can be either new field or the combination/single existing content fields included in DCIs for both DL and UL grant. For instance, UE initiates a COT if the starting point of configure FFP  is aligned with the starting point of the first symbol of the UL transmission and the COT is enabled by the bit in DCI. Otherwise, UE does not initiate a COT. In addition, gNB controls the UE FFP structure does not change frequently (200ms regulation requirement). Specifically, we can use the reserved entry in the following table to indicate, i. e., if CAPC field is mapped to index 0/1/2, the UE will not initiate its own COT and do the scheduled UL transmission based on sharing gNB-initiated COT; and if CAPC field is mapped to index 3, the UE will initiate its own COT when the scheduled UL transmission is aligned with UE FFP starting point. For DCI format 0_2/1_2, need to introduce a same field as CAPC field in DCI format 0_1/1_1.

Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided

Figure 2-2-1 gNB indicates UE to initiate a COT Based on ChannelAccess-CPext field in DCI

Proposal 1: gNB indicates UE to initiate a COT for UL transmission based on ChannelAccess-CPext field in DCI.

### Scheduled UL transmission in next gNB FFP period

In this section, we will address the FFS of whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period.

In next gNB’s FFP period, if gNB could always pass LBT, initiate the COT and share it to UEs, it should be no any problem, i. e., UE determines whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT based on the content in the scheduling DCI. However, if gNB’s LBT for next FFP period fails, how UE response to this is a problem. Therefore, the behavors for UE in the next FFP period should be defined.

First of all, as the agreement in last meeting, UE determines whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT based on the content in the scheduling DCI.

Case 1: If DCI indicates the UE to initiate UE COT,

The UE assume the scheduled UL transmission corresponds to UE-initiated COT

Case 2: If DCI indicates the UE to share gNB-initiated COT,

Case 2.1: If the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the scheduled UL transmission corresponds to gNB-initiated COT

Case 2.2; UE does not detect any DL signal/Channel. There are two options

Option 2.2.1: UE doesn’t transmit UL in the later g-FFP

Option 2.2.2: UE assumes that the scheduled UL transmission corresponds to UE-initiated COT

To provide more flexibility for UE,  a new RRC parameter can be introduced to switch between option 2.2.1 and 2.2.2.

Proposal 2: gNB indicates UE to initiate a COT for UL transmission in next FFP based on ChannelAccess-CPext field in DCI and define the behavors of UE if gNB’s LTB fails

Another related topic is the HARQ ACK/NACK feedback in the next FFP period for DL transmission in previous FFP period. To be robust, if the UE fails to decode the DL transmission in a later FFP, the UE assumes HARQ ACK/NACK shall be transmitted for the DL transmission(s) regardless of whether DL signal is detected within the gNB COT. Then, the follow-up question is whether the PUCCH for HARQ-ACK will be transmitted based on sharing gNB-initiated COT or UE-initiated COT. In fact, we can follow the agreement above, UE determines whether a HARQ ACK/NACK transmission is based on UE-initiated COT or sharing a gNB-initiated COT based on the content in the scheduling DCI. If DCI indicates the UE to initiate UE COT, the UE assume the scheduled UL transmission corresponds to UE-initiated COT. However,  if DCI indicates the UE to share gNB-initiated COT but gNB fails on LBT, considering the high priority of HARQ ACK/NACK, UE may assume that the transmission corresponds to UE-initiated COT.

Proposal 3: UE assumes that the transmission of HARQ ACK/NACK corresponds to UE-initiated COT when gNB does not initiate a COT

## UE-initiated COT sharing

In RAN1#102e, we got the following agreements to support UE-to-gNB sharing.

Agreements:

- UE-to- gNB COT sharing in semi-static channel access mode is supported.

  - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP.

    - FFS details

  - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP.

    - FFS whether/how UE to gNB COT sharing when the gap is >16us

In this section, we will discuss the indication of gNB sharing UE-initiated COT and the ED threshold selection for UE to initiate the COT.

### Indication of gNB sharing UE-initiated COT

Besides UE to gNB COT sharing, it is also interested to allow UE to share its COT to the other UEs through gNB. For LBE mode in Rel.16 NR-U, when UE initiated a COT and share back to the gNB, gNB is not allowed to share the COT with other UEs. However, the agreement is applicable to LBE only. It is preferable to allow the COT to be shared for other UEs, but it is not likely for the two cases. The first one is that if controlled environment is assumed, WiFi should not complain. The other is that other UEs transmit data even they detect some signal/channel from the gNB. By Rel.16 NR-U FBE UE behavior, the UEs cannot tell this is gNB initiated COT (though the signal/channel detected is not at the beginning of the gNB FFP), so naturally the UEs can share. More likely, we will follow LBE case, and do not allow COT to be shared with other UEs for data transmission. For DL transmission, this can be enforced by gNB scheduler. For UL transmission, the other UEs need to be able to tell the COT is not initiated by the gNB. There are three options as follows.

- Option 1: add a bit in COT-SI

After detecting the DL signal/channel, UE needs to obtain information that the gNB is not the owner of the FFP, so other UEs should not transmit. We can add a bit in DCI 2_0 to differentiate the transmission is in a gNB initiated COT or in a shared COT from a UE.

- Option 2: Use different locations of DL signal/channel

Define the behavior that if the DL signal/channel detection is at the beginning of the COT (or more generally a set of time locations within the FFP), this is gNB COT, and if the signal/channel detected is not at the beginning of the COT (or more specifically at another set of time locations within the FFP), this is a shared COT.

- Option 3: Consideration for Rel-16 UE

The problem of option 1 and option 2 above is that the solutions does not work for Rel-16 UE because they can not understand the indications in option 1 and 2. For example, when a Rel-16 UE receives any DL signal/channel, it will share the COT no matter the COT is initiated by gNB itself or by UE. To solve this problem, we can provide separated GC-PDCCHs for Rel-16 UE and Rel-17 UE, respectively. There are several solutions for this purposes.

- Configure different CORESET scrambling IDs for Rel.16 and Rel.17 UEs, respectively

- Configure different PDCCH monitoring occasions for Rel.16 and Rel.17 UEs, respectively

- Configure different RNTI (e.g., SFI-RNTI) for Rel.16 and Rel.17 UEs, respectively

- Use different DCI length for Rel.16 and Rel.17 UEs, i.e., longer DCI size for Rel.17 UEs

Proposal 4: Study the scheme of indication of gNB sharing UE-initiated COT for DL transmission to disable UE sharing the COT.

### ED threshold selection for UE to initiate the COT

Based on the agreements in NR-U, when COT sharing ED threshold is configured, UE will always use the configured ED threshold for LBT and the COT will always be shared to gNB. There are several drawbacks, the channel access probability of the UE is reduced since configure COT sharing ED threshold is more sensitive than the non-COT sharing ED threshold value calculated by the UE transmission power. In case that gNB does not have any DL data to send after the end of UL transmission, UE does not need to share its COT with gNB. Therefore, It should be good to allow a UE to decide which ED threshold should be used. For CG-UL, it is possible for UE to select an ED threshold other than the configured one for LBT and tell gNB via UCI. In fact, this is already supported as there is an entry in the COT sharing index table in CG-UCI indicating no sharing. However, there is no solution for DG-PUSCH (Dynamic Grant) on how to allow UE to select different ED threshold when COT sharing ED threshold is configured. We have two alternatives for this as follows.

- Select ED threshold based on TX priority

The ED threshold control is associated with priority of DG-PUSCH, either hard coded, or configurable. gNB may set both high and low priority traffic to use higher ED threshold for certain UE to allow the UE to transmit UL easily if the PUSCH of the UE is more important than the DL. On the other hand, gNB may set both high and low priority traffic to use lower ED threshold for certain UE if the PUSCH of the UE is less important than the DL sharing opportunity. Another option is that high priority traffic uses higher ED threshold and low priority use lower ED threshold. In this case, high priority traffic prioritize higher chance for channel access, and lower priority traffic prioritize UL to DL COT sharing

- Select ED threshold based on CG/DG

Traffic priority aside, there is a use case that the gNB needs separate control for the ED threshold for CG and DG. For example, the gNB may want the CG to share COT, but does not require DG to share COT

Proposal 5: Study ED thresholds selection when UE share its COT to gNB

## UE-initiated COT by PRACH transmission in idle mode

In RAN1#102e, we got the following agreements for any UL transmission to initiate a COT by a UE in RRC_CONNECTED mode and FFS the case of IDLE/INACTIVE mode.

Agreements:

- For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode

  - FFS the case when the UE is IDLE/INACTIVE mode

Actually, the FFS part is related to the design whether PRACH or msgA in 2-step RACH is allowed to initiate a COT for transmission.

In NR-U, it was concluded that “for semi-static channel access, an idle UE may transmit PRACH only within FFPs for which SSB, SIBs or paging transmissions are detected”. This implies that PRACH can only be transmitted by detecting some DL signals with SI-RNTI masking. This restricts the chance that a UE can send PRACH even when configured PRACH is available, as for a FFP with PRACH configured, if the gNB does not send SSB/SIB/paging, the UE cannot take advantage of the PRACH occasion. Besides, it was also agreed that “A PRACH resource is considered invalid if it overlaps with the IDLE period of a fixed frame period (FFP) when FBE operation is indicated.” This further restrict the chance that a UE can send PRACH.

To increase the chance of PRACH transmissions, one way is to support using the PRACH transmission to initiate a COT by UE in IDLE/INACTIVE mode. In this case, an idle UE can transmit PRACH even when the UE doesn’t detect any DL signals within the FFP. As shown in Figure 2-1-1, the UE detects SSB in the first gNB FFP, so the UE can transmit PRACH in the first FFP. While the UE cannot transmit PRACH in the second gNB FFP since UE doesn’t detect SSB, SIBs or paging transmission in that FFP. However, if UE initiated COT is supported for idle UE, the UE may initiate a COT using the PRACH transmission at the beginning of UE FFP. Obviously, this provides more chances that the UE can send PRACH.

Figure 2-5-1: An example of UE initiated COT in IDLE/INACTIVE mode

Another way to increase the probability of PRACH transmission is to allow the PRACH transmission in the idle period of an FFP. As shown in Figure 2-1-1, some PRACH occasions in the second PRACH slot of the first PRACH period are overlapped with idle period of gNB FFP. These PRACH occasions are invalid in Rel.16. Such restriction can be removed in Rel.17 in order to increase the chance for PRACH transmission.

Observation 1: Either supporting UE initiated COT in IDLE/INACTIVE mode or allowing PRACH transmission in idle period can provide more chances for the UE to send PRACH.

Proposal 6: Study the following alternatives for PRACH transmission in idle mode:

- Alt.1: Supporting UE initiated COT by PRACH transmission in idle mode;

- Alt.2: Allowing PRACH transmission in idle period of an FFP.

For PRACH transmission, one thing needs to be considered is the SSB to PRACH occasion mapping. In Rel.16, the UE needs first determine the valid PRACH occasions and then map the SSB to the valid PRACH occasions based on some predefined rule. Based on Alt2 above, valid PRACH occasions may be different from Rel.16 which may lead to different SSB to PRACH occasion mapping. In order to not impact the SSB to PRACH mapping for legacy UEs, some new SSB to PRACH occasion mapping need to be considered.

For the sake of discussion, we consider the SSB to PRACH mapping enhancement for PRACH transmission in idle period in the following. Two alternatives can be considered for SSB to PRACH mapping:

- Alt.1: Divide PRACH occasions into two groups and SSB is mapped to PRACH occasion per group.

- Alt.2: Introduce two PRACH configurations and SSB is mapped to PRACH occasions per PRACH configuration.

For Alt.1, the PRACH occasions can be divided into two groups based on whether it is overlapped with idle period or not. Then the UE performs two rounds SSB to PRACH occasion mapping.  For example, the first group includes PRACH occasions outside the idle period; the second group includes PRACH occasions overlapped with the idle period. Then the UE perform SSB to PRACH occasion mapping for the first group of PRACH occasions and the second group of PRACH occasions based on Rel.16 SSB to PRACH occasion mapping, respectively. Figure 2-1-2 shows an example for option 1.

Figure 2-5-2 An example of  Alt.1

For Alt.2, the UE needs to be configured with two PRACH configurations, one is the same as Rel.16. Another is a new PRACH configuration which need to be designed to accurately match with the idle period. Then the UE performs SSB to PRACH occasion mapping per PRACH configuration based on Rel.16 mapping rule.

Comparing Alt.1 and Alt. 2, Alt.1 is simple but increase of PRACH transmission occasion may be limited by the existing PRACH configuration. Alt.2 can provide more flexibility on the new PRACH transmission occasions but may have more specification impact since different FFP periodicity, different PRACH periodicity, and different PRACH preamble may need different PRACH configurations.

Proposal 7: Study the following two alternatives for SSB to PRACH mapping:

- Alt.1 Divide PRACH occasions into two groups and SSB is mapped to PRACH occasion per group;

- Alt.2: Introduce two PRACH configurations and SSB is mapped to PRACH occasions per PRACH configuration.

For two-step RACH, the same issue may happen for MsgA transmission, since UE cannot transmit MsgA if UE doesn’t detect any DL signal in the gNB FFP. This restricts the chance that a UE can send MsgA even when configured PUSCH is available. Besides, a PUSCH resource is considered invalid if it overlaps with the IDLE period of a fixed frame period (FFP) when FBE operation is indicated. So MsgA cannot be transmitted in the idle period of an FFP which further restricts the chance that a UE can send MsgA. To increase the transmission probability of MsgA, one way is to support using MsgA to initiate a UE COT, another way is to remove the restriction that MsgA is invalid in idle period.

For MsgA transmission, one thing needs to be considered is the PRACH occasion (RO) to PUSCH occasion (PO) mapping. In Rel.16, the UE needs first determine the valid ROs and valid POs and then map the valid ROs to the valid POs based on some predefined rule. When ROs and POs in idle period are considered as valid, valid ROs and valid POs may be different from Rel.16 which may lead to different RO-to-PO mapping. In order to not impact the RO-to-PO mapping for legacy UEs, some new RO-to-PO mapping need to be considered.

For the sake of discussion, we consider the RO-to-PO mapping enhancement for MsgA transmission in idle period in the following. Two alternatives can be considered:

- Alt.1: Divide POs into two groups and RO is mapped to PO per group;

- Alt.2: Introduce two sets of PUSCH configurations and each PUSCH configuration is associated with one PRACH configuration.

Figure 2-1-3 shows an example for Alt.1. In this example, the ROs and POs are divided into two groups, respectively. The first group include ROs/POs outside the idle period, the second group include ROs/POs overlapped with the idle period. Then the valid ROs in the first group is mapped to the valid POs in the first group and the valid ROs in the second group is mapped to the valid POs in the second group.

For Alt.2, the UE needs to be configured with two sets of PUSCH configurations. One is the same as legacy PUSCH configuration and another is new PUSCH configuration which is configured to accurately match with the idle period. Then the UE performs PO-to-RO mapping per PUSCH configuration based on Rel.16 mapping rule.

Comparing Alt.1 and Alt.2, Alt.1 is simple but increase of PUSCH occasions is limited by the existing PUSCH configuration. Alt.2 have more flexibility on PUSCH occasions but may increase the RRC overhead since different RRC parameters need to be configured for different set of PUSCH configurations.

Figure 2-5-3 An example of Alt.1

Proposal 8: Study the following alternatives for MsgA transmission in idle mode:

- Alt.1: Supporting UE initiated COT by MsgA transmission in idle mode;

- Alt.2: Allowing MsgA transmission in idle period of an FFP.

Proposal 7: Study the following for RO-to-PO mapping:

- Alt.1: Divide PUSCH occasions into two groups and PRACH occasion is mapped to PUSCH occasion per group;

- Alt.2: Introduce two sets of PUSCH configurations and each PUSCH configuration is associated with one PRACH configuration.

# Harmonizing CG-UL in NR-U and URLLC in Rel-16

In this section, we will analyze the CG features for NR-U and URLLC in Rel-16 first and then introduce the harmonization of HARQ mechanism and multiple CG configuration.

## Existing CG-UL in Rel-16

We listed the CG features and the corresponding information in Table 3-1-1. We discussed if the CG features is supported for URLLC and NR-U in Rel-16, bundled with Cg-timer and possible harmonization. After analysis, we think multiple CG configurations and multiple TB can be harmonized for URLLC and NR-U. We will discuss this in next section.

Table 3-1-1 CG features for NR-U and URLLC in Rel-16

| CG features | Rel-16 URLLC supportable | Rel-16 NR-U supportable | Cg-timer bundled | Possible harmonization |
| --- | --- | --- | --- | --- |
| Multiple CG configurations | Yes | Yes | No | Yes |
| HARQ Process ID determination | Equation with first TO | In CG-UCI | No | Including HARQ-ID in CG-UCI for flexibility for transmission of different packet with both DG and CG |
| Sharing HARQ ID among multiple CG configurations | No | Yes | No | Yes |
| RV determination | {0,0,0,0} {0,3,0,3}{0,2,3,1}with TO | In CG-UCI | No | Including RV in CG-UCI for flexibility |
| Flexible initial transmission occasion (TO) | Yes | Yes | No | Using Rel-16 NR-U solution |
| Repetition scheme(s) | Type A/B | Multiple TB | No | Supporting multiple TB |
| CG-Downlink feedback information (DFI) | No | Yes | Yes | No for FBE |
| CG Re-transmission timer | No | Yes | Yes | No for FBE |
| CG automatic Re-transmission scheme | No | Yes | Yes | No for FBE |

In RAN1#102e, we got the following agreements about cg-RetransmissionTimer for FBE mode.

Agreements:

At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

In fact, URLLC can also work in LBE mode in unlicensed controlled environments. In this case, we think configuration of (cg-RetransmissionTimer) should not be mandated.

Proposal 10: For LBE, configuration of (cg-RetransmissionTimer) should be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

## Harmonization for multiple CG configuration

Based on the discussion above, we can harmonize the designs as follows.

- Type A PUSCH repetition for NR-U CG-PUSCH

We can start from the current NR-U CG-PUSCH resource definition

  - Reuse SLIV for the first PUSCH in the first slot

  - Reuse # of PUSCH in a slot in NR-U fashion (continuous L after the initial SLIV in the slot)

  - Reuse # of slot in NR-U fashion (the continuous slot the CG-PUSCH resource will be defined in)

    - The above defines the resource available for CG-PUSCH

    - These are exactly the same as in Rel.16 NR-U

For Type A PUSCH repetition, UE repeats the TBs in K consecutive slots where K is the # of repetitions. However, for NR-U CG, the UE repeats the TBs in repK consecutive transmission occasions. So the problem is how to determine the number of repetitions for each TB for Type A PUSCH repetition for NR-U CG. A simple way is to reinterpret the # of repetitions in consecutive slots as the # of repetitions in consecutive transmission occasions. If the # of repetitions is provided by activation DCI for Type-2 CG or configured by RRC for Type-1 CG, the UE repeats the TBs in # of repetitions consecutive transmission occasions; Otherwise, the UE repeats the TBs in repK consecutive transmission occasions. gNB handles the gap between transmissions by selecting proper parameters, such as SLIV. But in general, there can be gaps between repetitions or different TBs, and the UE will need to do separate LBT for the transmission of each burst.

Figure 3-2-1 Harmonization for Type A PUSCH repetition

Proposal 11: NR-U CG-PUSCH shall support type A PUSCH repetition introduced in Rel.16 URLLC by reinterpreting the # of repetitions in consecutive slots as the # of repetitions in consecutive transmission occasions.

- Type B PUSCH repetition for NR-U CG-PUSCH with light harmonization

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG

  - FFS whether/how to enhance

The key idea of Type B repetition is back to back repetition pattern. It is very important for URLLC service to reduce the latency. In addition, multiple TB (Transport Block) supporting in Rel-16 NR-U can further reduce the latency. Therefore, it is a good enhancement that multi-TBs transmission can be supported for PUSCH repetition type B.

To enable PUSCH repetition type B to support multi-TBs transmission, we just need to release the single TB restriction in NR IIoT Rel-16 based CG. First, the number of repetitions (denoted by K) in Rel.16 URLLC is reinterpret as the total number of PUSCH repetitions across multiple TBs. repK in Rel-16 NR-U is used to determine the number of TBs, where the number of TBs (denoted by L) can be determined as L=K/repK. If K is not multiple of repK, L can be deduced based on one of the following options:

- Option 1: N = floor(K/repK) to strive for better performance

The number of nominal repetitions for the first TB is repK + (K - N*repK), the number of repetitions for the remaining (N-1) TBs is repK.

- Option 2: N = ceil(K/repK) to support more TBs

The number of repetitions for the last TB is repK - (N*repK – K), the number of repetitions for the first (N-1) TBs is repK.

Further study is needed to investigate how to design the HARQ processing ID to support multi-TBs transmission for PUSCH repetition type B.

Proposal 12: NR-U CG-PUSCH shall support type B PUSCH repetition introduced in Rel.16 URLLC with supporting multiple TBS

- Type B PUSCH repetition for NR-U CG-PUSCH with deep harmonization

More complicated than Type A case, given Type B repetitions are back to back with segmentations over slot boundaries excluding other DL symbols and invalid symbols. Our design goal is to keep the back to back repetition feature and segmentation feature.

We can reuse the parameter of number of consecutive slots which is allocated within a configured grant period and configured in Rel-16 NR-U by RRC. The first SLIV for the first CG-PUSCH is configured by RRC for Type-1 CG or indicated by activation DCI for Type-2 CG, and the the remaining PUSCH allocations have the same length and PUSCH mapping type, and are appended following the previous allocations without any gaps up to # of slots. The # of PUSCH in slot is not used (for type B). If the # of repetitions is provided by activation DCI for Type-2 CG or configured by RRC for Type-1 CG, the UE repeats the TBs in # of repetitions consecutive transmission occasions; Otherwise, the UE repeats the TBs in repK consecutive transmission occasions.

Figure 3-2-2 Harmonization for Type B PUSCH repetition

Proposal 13: NR-U CG-PUSCH shall support type B PUSCH repetition introduced in Rel.16 URLLC with the proposal in this contribution.

- Orphan symbol handling for Type B PUSCH repetition for NR-U CG-PUSCH

In Rel.16 URLLC, for type B PUSCH repetition, due to segmentation, there is a chance that actual repetition is consist of one symbol. The Rel.16 URLLC decision is not to transmit the actual repetition with single symbol (also called orphan symbol), as there is no one symbol PUSCH format to support that. But for NR-U, leaving one symbol gap will cause the UE to loss the channel. Some companies suggested to handle this issue. We have several solutions to solve this issue.

First of all, if there is actual repetition transmitted after the orphan symbol, i.e., single-symbol actual repetition, use CP extension from the next actual repetition to fill the one symbol gap. The length of the CP extension can be single symbol duration or single symbol duration – 16us. However, if there is no actual repetition transmitted after the orphan symbol, i.e., single-symbol actual repetition, drop the orphan symbol. In this case, the orphan symbol is not a gap. If the one symbol is at the end of a burst, since both gNB and UE know about this, the gNB can schedule the next transmission accordingly, with the cancellation of this symbol in mind.

Proposal 14: CP extension can be used to handle the non-transmission of orphan symbol for Tpye B PUSCH repetition.

# Conclusions

In this contribution, we discuss the uplink enhancements for URLLC in unlicensed controlled environments. We have the following proposals:

Proposal 1: gNB indicates UE to initiate a COT for UL transmission based on ChannelAccess-CPext field in DCI.

Proposal 2: gNB indicates UE to initiate a COT for UL transmission in next FFP based on ChannelAccess-CPext field in DCI and define the behavors of UE if gNB’s LTB fails.

Proposal 3: UE assumes that the transmission of HARQ ACK/NACK corresponds to UE-initiated COT when gNB does not initiate a COT.

Proposal 4: Study the scheme of indication of gNB sharing UE-initiated COT for DL transmission to disable UE sharing the COT.

Proposal 5: Study ED thresholds selection when UE share its COT to gNB.

Observation 1: Either supporting UE initiated COT in IDLE/INACTIVE mode or allowing PRACH transmission in idle period can provide more chances for the UE to send PRACH.

Proposal 6: Study the following alternatives for PRACH transmission in idle mode:

- Alt.1: Supporting UE initiated COT by PRACH transmission in idle mode;

- Alt.2: Allowing PRACH transmission in idle period of an FFP.

Proposal 7: Study the following two alternatives for SSB to PRACH mapping:

- Alt.1 Divide PRACH occasions into two groups and SSB is mapped to PRACH occasion per group;

- Alt.2: Introduce two PRACH configurations and SSB is mapped to PRACH occasions per PRACH configuration.

Proposal 8: Study the following alternatives for MsgA transmission in idle mode:

- Alt.1: Supporting UE initiated COT by MsgA transmission in idle mode;

- Alt.2: Allowing MsgA transmission in idle period of an FFP.

Proposal 9: Study the following for RO-to-PO mapping:

- Alt.1: Divide PUSCH occasions into two groups and PRACH occasion is mapped to PUSCH occasion per group;

- Alt.2: Introduce two sets of PUSCH configurations and each PUSCH configuration is associated with one PRACH configuration.

Proposal 10: For LBE, configuration of (cg-RetransmissionTimer) should be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Proposal 11: NR-U CG-PUSCH shall support type A PUSCH repetition introduced in Rel.16 URLLC by reinterpreting the # of repetitions in consecutive slots as the # of repetitions in consecutive transmission occasions.

Proposal 12: NR-U CG-PUSCH shall support type B PUSCH repetition introduced in Rel.16 URLLC with the proposal in this contribution.

Proposal 13: NR-U CG-PUSCH shall support type B PUSCH repetition introduced in Rel.16 URLLC with the proposal in this contribution.

Proposal 14: CP extension can be used to handle the non-transmission of orphan symbol for Tpye B PUSCH repetition

# References

- RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, 3GPP TSG RAN Meeting #88e, June 29th – July 3rd, 2020

- R1-1912938, Channel access procedures for NR unlicensed, Qualcomm, 3GPP TSG RAN WG1#99, Reno, USA, November 18-22, 2019