3GPP TSG RAN WG1 #106-e						R1-2107447

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.3.4

Source: 	LG Electronics

Title: 	Discussion on propagation delay compensation enhancements

Document for:	Discussion and decision

Introduction

In the last e-meeting, following working assumption was made [1]:

Agreements: If downlink frame timing detection error needs to be considered separately from propagation delay estimation error, take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RX) at the UE for evaluation of the overall time synchronization error for RTT based propagation delay compensation Agreements: - Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario. - Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837. Agreement: Existing DL reference signal(s) are used for Rx – Tx time difference estimation at UE side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - FFS whether PRS can be used for UE Rx – Tx time difference estimation or not - FFS which DL reference signal(s) to be used if/when PRS is not used

In this contribution, we discuss and provide our views on propagation delay compensation enhancements and necessity of RAN1 involvement.

Discussion

  - Uu interface budget among 5GS synchronicity budget requirement

From previous RAN2 email discussion, synchronicity budget per Uu interface has been agreed as following:

| Scenario | Single Uu interface Budget |
| --- | --- |
| Control-to-Control | ±145ns to ±275ns |
| Smart Grid | ±795ns to ±845ns |

According to description in [2], these values are determined with assumptions such that network-side synchronization for Control-to-control and Smart Grid is based on gPTP and GNSS, respectively. The error caused by the limited granularity of referenceTimeInfo-r16 IE (±5ns) is already included in the other part budget, so that error doesn’t need to be considered.

According to the maximum timing synchronization error results with UE propagation delay compensation summarized in the NR Industrial Internet of Things (IoT) study item, it could meet Uu interface synchronization error budget for Smart Grid scenarios.

Table 1: Summary of maximum timing synchronization error results 
with UE propagation delay compensation from IIoT SI [3].

|  | 15kHz SCS | 30kHz SCS | 60kHz SCS |
| --- | --- | --- | --- |
| Source R1-1900156 [21] | 488ns | 357.5ns | 276.5ns |
| Source R1-1901334 [22] | 505ns | 371ns | 287.5ns |
| Source R1-1900935 [23] | 472.5ns | 338.5ns |  |
| Source R1-1901252 [24] | 536ns | 438ns | 357ns |

  - Simple analysis of PD compensation

In the RAN#92-e, following recommendation was made for this AI from RP-211569, in order to foucs RTT-based mechanism.

- Recommendation3: Provide the following RAN guidance on Propagation delay compensation enhancements [RAN2, RAN1, RAN3, RAN4] - Support TA-based propagation delay compensation based on the Rel-15/16 timing advance procedure in Rel-17 without changes on existing TA requirements/procedures. - RAN1/2/4 to focus on RTT-based propagation delay compensation enhancements in Rel-17.

For PD compensation, TA mechanism and RTT based mechanism have been discussed. Figure 1 and 2 show brief synchronization procedure of each mechanism.

| Figure 1. Synchronization procedure of TA mechanism. | Figure 2. Synchronization procedure of RTT mechanism |
| --- | --- |

For RTT-based mechanism, the total synchronicity error would be simple as following

errortotal, RTT≤errorBS, DL, TX+errorUE, DL, RX+errorUE, UL, TX+errorBS, UL, RX+errorRTT_indication2

- BS transmit timing error (errorBS, DL, TX):

  - For control-to-control, FFS whether to use ±32.5 ns or 65 ns for the evalation.

  - For smart grid, it was agreed to use 65ns or 200ns for the evaluation

  - We assume errorBS, DL, TX is 65 ns

- BS detecting error (errorBS, UL, RX) : 100 ns

- Downlink frame timing error (errorUE, DL, RX)

- UE Initial transmit timing error (errorUE, DL,TX) : from 260 ns to 325 ns ((from 8 to 10)*64*Tc )

  - According to RAN4 reply LS (R1-2104171), errorUE, DL, RX is already included in Te,

  - Thus, we assume errorUE, DL,TX + errorUE, DL, RX = Te

- errorRTT_indication: 8*64*Tc/2 , 130ns or 260ns depending on SCS.

  - Set TA granularity as minimum boundary.

As characteristic of RTT-based mechanism, errorRTT_indication is added. When UE reports its time stamp to gNB, the accuracy of value is limited to the reporting granularity. Thus, errorRTT_indication refers the reporting granularity of time stamp. In the above analysis, total error budget of RTT-based mechanism is close to 275ns. In other words, RTT-based mechanism could achieve Control-to-Control Uu interface budget with any small enhancement even on RTT indication granularity.

Currently, RAN1 sent LS to RAN4 for possibility of enhancement on timing errors. As we can see, some error component take dominant potion in error budget. If it is possible to reduce such error component, the result can change totally. It is highly recommended to re-consider after RAN4 reply LS for PD compensation scheme.

Observation #1: Up to value of errorUE, UL, TX and errorRTT_indication, RTT-based mechanism could meet error budget under 275ns.

Proposal #1: Take following equation for evaluations of the overall time synchronization error for RTT-based PD compensation.

- errorRTT_indication means time granularity of estimated Rx – Tx time difference

errortotal, RTT≤errorBS, DL, TX+errorUE, DL, RX+errorUE, UL, TX+errorBS, UL, RX+errorRTT_indication2

Proposal #2: It is necessary to discuss again after RAN4 reply LS is sent if variables in assumption are able to be changed by RAN4.

One remaining issue is reference signal for RTT-based mechanism. Considering required accuracy of Rx – Tx time difference measurement, we think PRS-like reference signaling would be safer than other signaling. On the other hand, existing RS should be able to be used at least complementally. It is necessary to discuss further which existing RS can be used.

Conclusions

In this contribution, propagation delay compensation enhancements and necessity of RAN1 involvement were discussed, and the followings are proposed.

Observation #1: Up to value of errorUE, UL, TX and errorRTT_indication, RTT-based mechanism could meet error budget under 275ns.

Proposal #1: Take following equation for evaluations of the overall time synchronization error for RTT-based PD compensation.

- errorRTT_indication means time granularity of estimated Rx – Tx time difference

errortotal, RTT≤errorBS, DL, TX+errorUE, DL, RX+errorUE, UL, TX+errorBS, UL, RX+errorRTT_indication2

Proposal #2: It is necessary to discuss again after RAN4 reply LS is sent if variables in assumption are able to be changed by RAN4.

References

RAN1 Chairman notes, RAN1#104bis-e

TS 22.104 v17.4.0, Service requirements for cyber-physical control applications in vertical domains; Stage 1 (Release 17)

TR 38.825 v16.0.0, Study on NR Industrial Internet of Things (IoT)