3GPP TSG RAN WG1 #106-e	R1- 2107469

E-meeting, August 16th – 27th, 2021

Source:	Panasonic

Title: 	HARQ enhancement for NTN

Agenda Item:		8.4.3

Document for:	Discussion and decision

Introduction

A work item on Non Terrestrial Network (NTN) [1] has been started, and several agreements related to HARQ enhancement have been made as listed in Annex C. In this contribution, we discuss various issues related to HARQ enhancements for NTN.

Signalling of HARQ process ID

It was agreed that the maximal supported HARQ process number is up to 32. In the past RAN1 meetings, HARQ process ID indication methods were discussed and the following was agreed.

Agreement: (RAN1#103-e) - Enhanced HARQ process ID indication is supported for DCI 0-2/1-2 and DCI 0-1/1-1 by at least one of following: - Option 1: Slot index as the MSB - Option 1-a:Slot index as the LSB - Option 2: Reusing one bit from other bit field - Option 3: Extending the HARQ process ID field up to 5 bits - FFS: DCI 0-0/1-0 Note: 32 is taken as maximal supported HARQ processes number for both UL and DL Agreement: (RAN1#105-e) For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-2/1-2 Agreement: (RAN1#105-e) For enhancement on the HARQ process indication at least for DCI 0-1/1-1, the Option-1 and Option-1a are lower priority for further discussion.

In Rel.15/16 NR, HARQ process ID is indicated in 4-bit field to point up to 16 process IDs in the DCI. For DCI 0_2/1_2, it was agreed to extend the HARQ process ID field up to 5 bits in order to support up to 32 HARQ process IDs. It would be sufficient to simply add 1 bit for the HARQ process ID field in DCI format 0_1/1_1 as well.. Note that this 1 bit should not be added in DCI format 0_0 and 1_0 because the extension of the HARQ process ID field should be based on RRC configuration.

If there are unnecessary bits in DCI for NTN, to reuse 1 bit from other field (i.e. option 2 in the agreement) would be one possibility. On the other hand, bit reduction of other fields requires further discussions and should be separated from HARQ process ID discussion.

Proposal 1: 1 bit is added for HARQ process ID indication in DCI format 0_1/1_1.

HARQ-ACK codebook for feedback-disabled HARQ process

The following is the agreement on HARQ-ACK codebook enhancement in the previous RAN1 meetings.

Agreement: (RAN1#103-e) HARQ codebook enhancement is supported as: - For Type-2 HARQ codebook: - Option-1: Reduce codebook size with: - HARQ-ACK codebook only includes HARQ-ACK of PDSCH with feedback-enabled HARQ processes - FFS: the details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes - FFS: at least DCI for SPS release/SPS PDSCH - Option-2: No enhancement - Other options are not precluded. - For Type-1 HARQ codebook, further discuss is needed with down selection among following options: - Option-1: No enhancement; - Option-2: Report NACK on disabled process - Option-3: Reduce codebook size with criteria FFS: Enhancements for Type-3 HARQ codebook Agreement: (RAN1#104-e) For Type-2 HARQ codebook in NTN: Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes - FFS: The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes Agreement: (RAN1#105-e) For Type-2 HARQ codebook in NTN, - For the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes - FFS: Whether DCI for SPS release and any other DCIs are included in counting of C-DAI and T-DAI

For type 1 HARQ-ACK codebook, the codebook size is semi-statically determined. It would not be preferable to support dynamic change of the codebook size depending on which of feedback enabled or disabled process is used for the scheduling. In our view, no enhancement (i.e Option-1 in the agreement) would be sufficient. Because UE processing timeline should be the same for feedback enabled process and disabled process, to include HARQ-ACK for feedback disabled process would not be a problem. Note that from the discussion in the previous meetings, our understanding is that “no enhancement” means no change of UE behavior from Rel.15/16, i.e. generate HARQ-ACK even for feedback disabled process and include it in the HARQ-ACK codebook. How to treat the reported HARQ-ACK for the feedback disabled process is up to gNB implementation. On the other hand, if it is not desired to generate HARQ-ACK for HARQ-feedback disabled process, to report NACK on disabled process (i.e. Option-2 in the agreement) would also be acceptable as this is the same behavior as no scheduling case in Rel.15/16.

Proposal 2: No enhancement for type 1 HARQ-ACK codebook

For type 2 HARQ-ACK codebook, the codebook size is dynamically determined depending on the scheduling results. Therefore, it would be reasonable to include only HARQ-ACK of PDSCH with feedback enabled processes as agreed in RAN1#104e. For type 2 HARQ-ACK codebook, downlink assignment indicators (C-DAI and T-DAI) are used to determine the HARQ-ACK codebook size. C-DAI indicates the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH or SPS release is present, up to the current serving cell and current PDCCH monitoring occasion. T-DAI indicates the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH or SPS release is present, up to the current PDCCH monitoring occasion and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion.

Obviously, feedback-disabled HARQ processes should not be counted in both C-DAI and T-DAI because HARQ-ACK bits for feedback-disabled process is not included in the HARQ-ACK codebook. In RAN1#105-e, it was agreed that for the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes. On the other hand, the DAI values in the DCI of PDSCH with feedback-disabled HARQ processes would need discussion. The following options could be considered.

Options for DAI fields in the DCI of PDSCH with feedback-disabled HARQ processes

Option 1: C-DAI and T-DAI field indicates the respective DAI values.

In this option, for the determination of HARQ-ACK codebook size UE can use the DAIs in the DCIs that schedule feedback-disabled processes in addition to DAIs for feedback-enabled processes. Probability of HARQ-ACK codebook size misalignment between gNB and UE due to UE’s misdetection of the DCI can be alleviated because UE can determine the total number of HARQ-ACK bits up to the current slot from the DAI indicated in the DCI that schedules feedback-disabled process even if the UE misdetects the other DCIs in the slot. On the other hand, C-DAI would be less useful compared to T-DAI because UE can know the number of missing DCIs from the T-DAI value.

Option 2: T-DAI field indicates the total DAI value. C-DAI field is reserved.

Similar to Option 1, probability of HARQ-ACK codebook size misalignment between gNB and UE due to UE’s misdetection of the DCI can be alleviated because UE can determine the total number of HARQ-ACK bits up to the current slot from the T-DAI indicated in the DCI that schedules feedback-disabled process even if the UE misdetects the other DCIs in the slot. C-DAI field (2-4 bits) in the DCI that schedules feedback-disabled process is reserved and can be used for other purpose in Rel.17 or later release.

Option 3: Both C-DAI and T-DAI fields are reserved.

In this option, UE determines the HARQ-ACK codebook size only from the DAIs in the DCI that schedules feedback-enabled process. C-DAI and T-DAI fields in the DCI that schedules feedback-disabled process are reserved and can be used for other purpose in Rel.17 or later release.

Above three options are illustrated in Figure 1. Either options are workable, and the specification effort looks marginal. In our view, option 2 would be most reasonable option because T-DAI is useful information for the UE to avoid potential misalignment of HARQ-ACK codebook size between gNB and UE.

Figure 1 C-DAI and T-DAI indication options for feedback disabled process

In the current specification (TS38.213 Table 9.1.3-1, copied below), there are no entries to indicate the number of {serving cell, PDCCH monitoring occasion}-pair(s) is zero. Therefore, for DAI field in the DCI of PDSCH with feedback-disabled process, DAI indication when there are no DCI of PDSCH with feedback-enabled process before the reception of DCI of PDSCH with feedback-disabled process (i.e. when the number of HARQ-ACKs is zero) would need to be discussed.

Proposal 3: For type 2 HARQ-ACK codebook, PDSCH for feedback-disabled HARQ processes should not be counted in both C-DAI and T-DAI.

Proposal 4: T-DAI field in the DCI of PDSCH with feedback-disabled process indicates the total DAI value while C-DAI field is reserved.

Proposal 5: DAI indication in the DCI of PDSCH with feedback-disabled process when the number of HARQ-ACKs is zero should be discussed.

Necessity of restriction on feedback disabled HARQ process

It was discussed whether to restrict the transmission of MAC CE and SPS PDSCH to HARQ-feedback enabled process in RAN1#104-e. Some companies are supportive for the restriction to ensure the reliability of SPS PDSCH. Some companies have a concern on the specification effort to support HARQ-feedback disabled process for MAC CE transmission because current specification is written based on HARQ-feedback timing. Our view is that such restrictions on feedback disabled HARQ process is not necessary. SPS PDSCH and MAC CE cases are elaborated respectively as follows.

- SPS PDSCH

In the current specification, UE transmit HARQ-ACK when UE receives DCI for SPS release, while UE does not transmit HARQ-ACK feedback when UE receives DCI for SPS activation. Therefore, it may be preferable to configure feedback enabled HARQ processes for SPS PDSCH to allow gNB to detect the potential DCI reception error by the UE. On the other hand, waiting for HARQ-ACK may cause unacceptable delay due to the long propagation delay, and therefore, gNB may guarantee the DCI reception by using robust MCS (i.e. higher number of CCE aggregation level). Furthermore, if feedback enabled process need to be used for SPS PDSCH, many HARQ processes might need to be configured for SPS PDSCH because separate HARQ processes need to be used within RTT. For example, the maximum RTT for GEO is around 540ms [TR38.821]. For 10ms SPS periodicity, 54 HARQ processes are necessary, but it’s not possible with the maximum 32 HARQ processes. Furthermore, depending on the scenarios, it may be preferable to configure feedback disabled process for SPS PDSCH in order to use more feedback-enable processes for dynamic scheduling considering the number of HARQ processes is restrictive compared to RTT in NTN. For example, it should be allowed to use SPS with feedback-disabled process for delay sensitive and no-strict error rate requirement traffic like voice traffic and to use feedback-enabled process for more strict error rate requirement traffic.

In our view, whether to use feedback enabled process or disabled process for SPS PDSCH should be purely network implementation matter.

Proposal 6: Whether to use feedback-disabled process or enabled process for SPS PDSCH is up to network implementation.

- MAC CE

There was a proposal to mandate to use feedback-enabled process for MAC CE whose activation/deactivation time is coupled to the transmission time of the associated HARQ-ACK. But, how to guarantees the reliability of the MAC CE transmission should be network responsibility in our view. One way would be to use feedback enabled HARQ process and one way would be to use feedback disabled process with a robust MCS. Furthermore, if HARQ-feedback enabled process needs to be used for MAC CE whose activation/deactivation time is coupled to the transmission time of the associated HARQ-ACK, we see the following issues.

Firstly, the TB from MAC layer does not necessarily contain only MAC CE, but it can contain the other user data as shown below figure. Physical layer does not know whether the TB contains the MAC CE which is coupled to the transmission time of the associated HARQ-ACK. Physical layer specification would need to specify which MAC CE is coupled to the transmission time of the associated HARQ-ACK and corresponding operation to determination of HARQ-feedback enabled or disabled process. This requires more interaction between MAC layer and physical layer, which increase the complexity in the specification and the implementation.

TS38.321 Figure 6.1.2-4 Example of a DL MAC PDU

Secondly, it might cause a significant delay of MAC CE delivery in some scenarios. For example, for an operation with SCS=120kHz and RTT=42ms (max RTT for LEO 1200km), within the RTT period (42ms=335 slots) only up to 32 slots are available for feedback-enabled transmissions. gNB would easily use up all feedback enabled HARQ processes and would have to use feedback disabled HARQ processes until (re)transmission of one of the HARQ-enabled process is completed (i.e. ACK is received) although it is not efficient due to the low MCS transmission to achieve lower BLER. When MAC CE is generated, gNB has to wait the MAC CE transmission until a feedback enabled process becomes available if the MAC CE transmission is restricted to feedback enabled process. This might create significant delay for MAC CE transmission. On the other hand, it is possible for gNB to override an ongoing feedback enabled HARQ process for the MAC CE transmission. But in this case RLC retransmission is triggered when the “stolen” HARQ process is NACK. This further increase the delay of the TB.

A concern raised in RAN1#104e meeting was specification impact related to the MAC CE action timing for feedback disabled HARQ process. But, in our view, for feedback disabled HARQ process, the MAC CE action timing can be determined in the same way as for feedback enabled HARQ process. Actually, the current specification text for the MAC CE action timing is valid irrespective of whether or not the UE actually transmits the HARQ-ACK because it is described as “the UE applies the activation command in the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH”. The HARQ-ACK timing can be defined based on K1 value indicated in the DCI similar to the feedback enabled case. The UE processing timeline is the same as the case with feedback enabled process, and the difference is just not to transmit HARQ-ACK in case of feedback disabled process. Therefore, no or limited specification impact is expected to support MAC CE transmission using feedback disabled process.

Proposal 7: Whether to use feedback-disabled process or enabled process for MAC CE transmission is up to network implementation. MAC CE action timing when feedback-disabled process is used is well covered by the current specification text.

Enhancement of PDSCH/PUSCH transmissions to improve user throughput

In RAN1#105-e, it was agreed that discussion of enhancement(s) on the aggregated transmission (including repetition) is prioritized to improve the performance in NTN. In this section, we discuss throughput improvement utilizing repetition under the limited number of HARQ processes in NTN.

As shown in our simulation results in [4], even though user throughput is improved by using 32 HARQ processes compared to 16 HARQ processes, the user throughput for 32 HARQ processes is significantly degraded compared to larger number of HARQ processes (e.g. 64, 256) especially for Ka-band where slot length is shorter (0.25ms or 0.125ms) because only 32 slots (8ms or 4ms) during the long RTT period (e.g. 42ms for LEO 1200km) can be used or inefficient low BLER operation of HARQ feedback disabling. There would be enough room to improve the user throughput by enhancement of PDSCH/PUSCH transmissions.

Proposal 8: Enhancement of PDSCH/PUSCH transmission to improve user throughput without further increasing the number of HARQ processes should be discussed.

A solution to improve user throughput without further increasing the number of HARQ processes is discussed below.

The degradation of the user throughput comes from the fact that transmission opportunity (i.e. the number of slots that can be used for transmission within RTT) is too restrictive due to lack of HARQ processes, i.e. the amount of data which can be transmitted within the time period of RTT is restrictive. If transmission with one HARQ process can use multiple slots, more slots within RTT can be used for transmission without increasing the number of HARQ processes. In NR Rel.15/16, transmission with multiple slots is supported by repetition that can be configured by pdsch-AggregationFactor or repetitionNumber in PDSCH-TimeDmainResourceAllocationList-r16 for PDSCH and by pusch-AggregationFactor or repetitionNumber in PUSCH-TimeDmainResourceAllocationList-r16 for PUSCH. The target of repetition in Rel.15/16 is coverage enhancement and reliability improvement. In these target usages, the overall coding rate after the repetition is reduced compared with non-repetition case. On the other hand, in order to enhance the user throughput, transport block (TB) size needs to be increased in accordance with the number of repetitions because the overall coding rate after the repetition is not intended to be reduced compared with non-repetition case. Figure 2 illustrates the repetition with increased TB size. In Figure 2, 4x TB size is used for 4x repetition and 4 times more slots within RTT are used for the transmission compared to no repetition case. In this case, instead of the reliability, the throughput is improved.

Figure 2 Repetition with increased TB size

We conducted system level simulations to evaluate the performance for repetition with increased TB size using the simulation assumptions shown in Annex A. In addition to the number of HARQ processes 16, 32, 64, and 256, 32 HARQ processes with 2x repetition with 2x TB size and 32 HARQ processes with 4x repetition with 4x TB size are evaluated. The simulation results for target RU=20% and 50% are shown in Figure 3. 32 HARQ processes with 2x repetition with 2x TB size achieves similar user throughput performance with 64 HARQ processes, and 32 HARQ processes with 4x repetition with 4x TB size achieves similar user throughput performance with 256 HARQ processes.

(a) target RU=20%                                                                 (b) target RU=50%

Figure 3 user throughput performance for repetition with scaled TB size (LEO1200km, Ka-band)

Instead of scaling factor for TB size, use of higher MCS level may be considered, e.g. use MCS with 4x larger SE for 4x repetition. The resulting TB size can be scaled with the number of repetitions, but the efficiency is worse than TB size scaling because higher order modulation needs to be selected. Furthermore, throughput is limited by the highest MCS level even if large number of repetition is used. SLS results are shown in Annex B. Saturation of the throughput is seen in case of 4x repetition with 4x SE.

For the specification implementation, one way would be to add a scaling factor S’ to enlarge TB size in the equation for Ninfo in TS38.214 section 5.1.3.2, e.g. Ninfo=NRE·R·Qm·υ·S', similar to the scaling factor used for PDSCH scheduled with RA-RNTI and P-RNTI. The scaling factor can be identical to the number of repetitions or independently configured/indicated.

On the other hand, in coverage enhancement WI, support of TB processing over multi-slot PUSCH was agreed and TBS determination for multi-slot PUSCH is one of the work item objectives. The related texts in the WID [6] of coverage enhancement WI is copied below.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

TB size determination for multi-slot PUSCH is discussed in our contribution for coverage enhancement [7]. For the multi-slot PUSCH, TB size is determined based on the number of resource element within multiple slots used for the PUSCH transmission. Thus, the TB size is roughly scaled with the number of slots used for the PUSCH transmission. Therefore, the same effect as TB size calculation using scaling factor explained above is obtained. Simulation results shown above are valid for multi-slot PUSCH transmission as well. It is preferable to apply the same or similar solution to NTN. Therefore, after some progress in coverage enhancement, it can be considered to apply the same solution as the multi-slot PUSCH, and apply it to PDSCH as well.

Above solution can achieve sufficiently high user throughput with a little specification impact. Thus, we make the following proposal.

Proposal 9: Transport block size scaling in case of repetition should be considered to improve user throughput with a limited number of HARQ processes. Alternatively, it should be considered to apply the same solution as multi-slot PUSCH in coverage enhancement WI for both PUSCH and PDSCH in NTN.

Conclusion

We discussed issues related to HARQ enhancement in NTN. We propose the following.

Proposal 1: 1 bit is added for HARQ process ID indication in DCI format 0_1/1_1.

Proposal 2: No enhancement for type 1 HARQ-ACK codebook

Proposal 3: For type 2 HARQ-ACK codebook, PDSCH for feedback-disabled HARQ processes should not be counted in both C-DAI and T-DAI.

Proposal 4: T-DAI field in the DCI of PDSCH with feedback-disabled process indicates the total DAI value while C-DAI field is reserved.

Proposal 5: DAI indication in the DCI of PDSCH with feedback-disabled process when the number of HARQ-ACKs is zero should be discussed.

Proposal 6: Whether to use feedback-disabled process or enabled process for SPS PDSCH is up to network implementation.

Proposal 7: Whether to use feedback-disabled process or enabled process for MAC CE transmission is up to network implementation. MAC CE action timing when feedback-disabled process is used is well covered by the current specification text.

Proposal 8: Enhancement of PDSCH/PUSCH transmission to improve user throughput without further increasing the number of HARQ processes should be discussed.

Proposal 9: Transport block size scaling in case of repetition should be considered to improve user throughput with a limited number of HARQ processes. Alternatively, it should be considered to apply the same solution as multi-slot PUSCH in coverage enhancement WI for both PUSCH and PDSCH in NTN.

- Reference

- RP-201256, “WID: Solutions for NR to support non-terrestrial networks (NTN)”, Thales

- TR38.811 V15.1.0, “Study on New Radio (NR) to support non terrestrial networks”

- TR38.821 V16.0.0, “Solutions for NR to support non-terrestrial networks (NTN)”

- R1-2006327, “HARQ enhancement for NTN”, Panasonic

- TR38.913 V15.0.0, section 7.15

- RP-202928, “New WID on NR coverage enhancement,” China Telecom, RAN#90e, December 2020.

- R1-2101018, “Discussion on TB processing over multi-slot PUSCH”, Panasonic

- Annex A: Simulation assumptions for SLS

Table 1 simulation assumptions

| Frequency band | Ka-band (20GHz) |
| --- | --- |
| Satellite type | LEO 1200km |
| System bandwidth | 400MHz |
| Subcarrier spacing | 120kHz |
| Slot length | 0.125ms |
| Frequency reuse | 1 |
| Polarization reuse | 1 |
| Deployment scenario | Rural, UE velocity=3km/h |
| Number of UEs per satellite beam | 10 |
| RTT | 41.77ms (335 slots) |
| CQI feedback period | 20 slots |
| CQI feedback delay | 20.89ms (168 slots) |
| Number of HARQ processes | 16, 32, 64, 256 |
| Max. number of HARQ (re)transmissions | 4 |
| Target iBLER (initial BLER) | 10% |
| Scheduler | Proportional fair |
| Traffic model | FTP3, packet size 0.5Mbyte Interval 4500ms (for RU=20%) Interval 2500ms (for RU=50%) |
| Target RU (Resource Utilization) | 20%, 50% |

User throughput definition is based on [5] as follows.

User throughput (during active time) is defined as the size of a burst divided by the time between the arrival of the first packet of a burst and the reception of the last packet of the burst.

- Annex B: SLS results for repetition with higher SE (MCS level)

(a) target RU=20%                                                                 (b) target RU=50%

Figure 4 user throughput performance for repetition with scaled TB size (LEO1200km, Ka-band)

- Annex C: list of the agreements

RAN1#102-e meeting

Agreement:

Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signalling

Agreement:

The extension of maximal HARQ process number can be considered with following assumptions:

- The maximal supported HARQ process number is up to 32.

- FFS: Support on the maximal HARQ process number is up to UE capability

Minimizing the impacts on specification and scheduling

RAN1#103-e meeting

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until [X] after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- FFS: value of X and units in which it is defined.

- FFS: Whether TB of the two PDSCHs needs to be different

Agreement:

- Enhanced HARQ process ID indication is supported for DCI 0-2/1-2 and DCI 0-1/1-1 by at least one of following:

  - Option 1: Slot index as the MSB

  - Option 1-a:Slot index as the LSB

  - Option 2: Reusing one bit from other bit field

  - Option 3: Extending the HARQ process ID field up to 5 bits

- FFS: DCI 0-0/1-0

- Note: 32 is taken as maximal supported HARQ processes number for both UL and DL

Agreement:

HARQ codebook enhancement is supported as:

- For Type-2 HARQ codebook:

  - Option-1: Reduce codebook size with:

    - HARQ-ACK codebook only includes HARQ-ACK of PDSCH with feedback-enabled HARQ processes

      - FFS: the details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

    - FFS: at least DCI for SPS release/SPS PDSCH

  - Option-2: No enhancement

  - Other options are not precluded.

- For Type-1 HARQ codebook, further discuss is needed with down selection among following options:

  - Option-1: No enhancement;

  - Option-2: Report NACK on disabled process

  - Option-3: Reduce codebook size with criteria

- FFS: Enhancements for Type-3 HARQ codebook

RAN1#104-e meeting

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- Working assumption: X = T_proc,1

- FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0

- Note: The TB of the two PDSCHs can be either same or different

Agreement:

For Type-2 HARQ codebook in NTN: Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes

- FFS: The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

RAN1#105-e meeting

Agreement:

For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-2/1-2

Agreement:

For Type-2 HARQ codebook in NTN,

- For the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes

- FFS: Whether DCI for SPS release and any other DCIs are included in counting of C-DAI and T-DAI

Agreement:

Confirm the previous working assumption for  X = T_proc,1 where X is defined from the end of the reception of the last PDSCH or slot-aggregated PDSCH for a given HARQ process with disabled feedback to the start of the PDCCH carrying the DCI scheduling another PDSCH or set of slot-aggregated PDSCH for the given HARQ process.

Agreement:

For enhancement on the HARQ process indication at least for DCI 0-1/1-1, the Option-1 and Option-1a are lower priority for further discussion.

Agreement:

Discussion of enhancement(s) on the aggregated transmission (including repetition) is prioritized to improve the performance in NTN.