3GPP TSG RAN WG1 Meeting #106-e 	R1-2107503

e-Meeting, Aug. 16th – Aug. 27th, 2021

Agenda Item: 7.1

Source: MediaTek Inc.

Title: Discussion on PUSCH/PUCCH power control adjustment state and UL BWP switch

Document for: Discussion and decision

Introduction

In this contribution, we discuss the issue about PUSCH/PUCCH power control adjustment state in case of UL BWP switch. A recommended solution is provided and text proposal for the proposed solution can be found in a companion CR document [1].

Restriction on the number of PUSCH/PUCCH power control adjustment state

In RAN1 #90-bis [2], it is agreed that up to 2 closed-loop power control processes is maintained for PUSCH:

Agreement: For N closed-loop power control processes, i.e., fc(i,l), for NR PUSCH power control for serving cell c, the following working assumption is confirmed:

N is up to 2

In RAN1 #90-bis [2], it is also agreed as a working assumption that up to 2 closed-loop power control processes is maintained for PUCCH:

Working Assumption:

- Support Pcmax,c(i), P0_PUCCH(F), PLc(k), g(i) for NR PUCCH power control in slot i for serving cell c.

- Support up to 2 closed-loop power control processes, i.e., l

However, in current 38.213 spec [3] 7.1.1/7.2.1 as shown below, the PUSCH/PUCCH power control adjustment state /, which corresponds to the closed-loop power control process, is define per UL BWP.  Since each UL BWP can be configured with up to two power control adjustment states, this is not aligned with the agreements in RAN1#90-bis on the number of power control adjustment states simultaneously maintained (up to 2 for PUSCH, and up to 2 for PUCCH, per serving cell).

Observation 1: The current specification in 38.213 [3] 7.1.1/7.2.1 imply UE maintains PUSCH/PUCCH power control adjustment state (/) in a per UL BWP basis. Since each UL BWP can be configured with up to two power control adjustment states, this is not aligned with the agreements in RAN1#90-bis on the number of power control adjustment states simultaneously maintained at the UE (up to 2 for PUSCH, and up to 2 for PUCCH, per serving cell).

Figure 1 below gives an illustrative example of “per BWP power state control” v.s. “per cell power state control”. It can be seen that “per cell power state control” offers benefits for transmit power continuity over “per BWP power state control” when BWP changes.

Observation 2: According to the illustration in Fig. 1, it can be seen that “per cell power state control” offers benefits for transmit power continuity over “per BWP power state control” when BWP changes.

Figure 1. An illustrative example of “per BWP power state control” v.s. “per cell power state control”

With Observation 1 and Observation 2, we have the following proposal:

Proposal 1: With Observation 1 & 2, adopt the “per cell power state control” instead of “per BWP power state control” implied in current spec (38.213 [3] 7.1.1/7.2.1) for PUSCH/PUCCH using the text in the companion CR document [1] as appended below

- 38.213 7.1.1: For each l, UE uses the same PUSCH power control adjustment state  before and after UL BWP change. A UE is not expected to maintain more than two PUSCH power control adjustment states per serving cell

- 38.213 7.2.1: For each l, UE uses the same PUCCH power control adjustment state  before and after UL BWP change.  A UE is not expected to maintain more than two PUCCH power control adjustment states per serving cell

- 7.1.1	UE behaviour

If a UE transmits a PUSCH on active UL BWP  of carrier  of serving cell  using parameter set configuration with index  and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power  in PUSCH transmission occasion  as

[dBm]

where,

……

- For the PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  in PUSCH transmission occasion

-	 is a TPC command value included in a DCI format that schedules the PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in clause 11.3

-	 if the UE is configured with twoPUSCH-PC-AdjustmentStates and  if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in clause 8.3

- 7.2.1	UE behaviour

If a UE transmits a PUCCH on active UL BWP  of carrier  in the primary cell  using PUCCH power control adjustment state with index , the UE determines the PUCCH transmission power  in PUCCH transmission occasion  as

[dBm]

where

……

-	For the PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion

-	 is a TPC command value included in a DCI format scheduling a PDSCH reception for active UL BWP  of carrier  of the primary cell  that the UE detects for PUCCH transmission occasion , or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 38.212], as described in clause 11.3

-	 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and  if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo

Conclusion

In this contribution, we discuss the issue about PUSCH/PUCCH power control adjustment state in the case of UL BWP switch and have the following observations and proposals:

Observation 1: The current specification in 38.213 [3] 7.1.1/7.2.1 imply UE maintains PUSCH/PUCCH power control adjustment state (/) in a per UL BWP basis. Since each UL BWP can be configured with up to two power control adjustment states, this is not aligned with the agreements in RAN1#90-bis on the number of power control adjustment states simultaneously maintained at the UE (up to 2 for PUSCH, and up to 2 for PUCCH, per serving cell).

Observation 2: According to the illustration in Fig. 1, it can be seen that “per cell power state control” offers benefits for transmit power continuity over “per BWP power state control” when BWP changes.

Proposal 1: With Observation 1 & 2, adopt the “per cell power state control” instead of “per BWP power state control” implied in current spec (38.213 [3] 7.1.1/7.2.1) for PUSCH/PUCCH using the text in the companion CR document [1] as appended below

- 38.213 7.1.1: For each l, UE uses the same PUSCH power control adjustment state  before and after UL BWP change. A UE is not expected to maintain more than two PUSCH power control adjustment states per serving cell

- 38.213 7.2.1: For each l, UE uses the same PUCCH power control adjustment state  before and after UL BWP change.  A UE is not expected to maintain more than two PUCCH power control adjustment states per serving cell

Reference

R1-2107503, “Draft 38.213 CR on PUSCH/PUCCH power control adjustment state”, MediaTek, RAN1 #106e

RAN1#90-bis Final Minutes report

3GPP TS 38.213 g60, ”NR; Physical layer procedures for control”