3GPP TSG-RAN WG1 Meeting #106-e	R1-2107503

# E-meeting, August 16th – 27th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 00xx | rev | - | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft 38.213 CR on PUSCH/PUCCH power control adjustment state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current specification can be interpreted to imply UE maintains PUSCH/PUCCH control adjustment state (/) in a per UL BWP basis. Since each UL BWP can be configured with up to two power control adjustment states, this is not aligned with the agreements in RAN1#90-bis on the number of power control adjustment states simultaneously maintained at the UE (up to 2 for PUSCH, and up to 2 for PUCCH, per serving cell). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on PUSCH/PUCCH power control adjustment states that the same power control adjustment state (/) for each index l is used before and after UL BWP change. Clarification on the number of PUSCH/PUCCH power control adjustment states simultaneously maintained at the UE to capture the agreements in RAN1#90-bis of up to 2 for PUSCH, and up to 2 for PUCCH, per serving cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current spec is in confilict with previous RAN1 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1, 7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: The CR aims to clarify the UE behavior on PUSCH/PUCCH power control adjustment state (/) during UL BWP switch. The proposed clarification is beneficial for transmit power continuity when BWP changes, and aligns current spec with RAN1#90-Bis agreement on the number of power control adjustment states simultaneously maintained at the UE (up to 2 for PUSCH, and up to 2 for PUCCH, per serving cell). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

7.1.1	UE behaviour

<Unchanged parts are omitted>

For the PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  in PUSCH transmission occasion

-	 is a TPC command value included in a DCI format 0_0 or DCI format 0_1 that schedules the PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in Clause 11.3

-	 if the UE is configured with twoPUSCH-PC-AdjustmentStates and  if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in Clause 8.3

-	For each l, UE uses the same PUSCH power control adjustment state   before and after UL BWP change. A UE is not expected to maintain more than two PUSCH power control adjustment states per serving cell

<Unchanged parts are omitted>

7.2.1	UE behaviour

<Unchanged parts are omitted>

For the PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion

-	 is a TPC command value and is included in a DCI format 1_0 or DCI format 1_1 for active UL BWP  of carrier  of the primary cell  that the UE detects for PUCCH transmission occasion  or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 38.212], as described in Clause 11.3

-	 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and  if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo

-	For each l, UE uses the same PUCCH power control adjustment state   before and after UL BWP change.  A UE is not expected to maintain more than two PUCCH power control adjustment states per serving cell

<Unchanged parts are omitted>