3GPP TSG RAN WG1 #106-e	R1-2107551

e-Meeting, August 16th – August 27th, 2021

Agenda Item:	8.8.2

Source: 	LG Electronics

Title: 	PUCCH enhancements

Document for:	Discussion and decision

Introduction

In the last meeting, an agreement, a working assumption and a conclusion were made regarding on PUCCH enhancement. In this contribution, we discuss about dynamic PUCCH repetition factor indication.

Dynamic PUCCH repetition factor indication

In Rel-16 NR, since the number of PUCCH repetitions is indicated by RRC for the PUCCH Format, it is currently possible only after RRC connection. That is, if the UE is in an uplink coverage shortage situation when transmitting acknowledge for Msg 4, PUCCH also needs repetition when transmitting Msg3 PUSCH with repetition. However, since it was decided to apply dynamic PUCCH repetition without DCI enhancement, it is natural to be indicated without DCI enhancement in the same way for PUCCH without dedicated PUCCH resource configuration.

PUCCH repetition without dedicated PUCCH resource configuration

The UE without a dedicated PUCCH resource configuration receives a PUCCH transmission resource set configured with PRI and CCE index, and there is no repetition number in this PUCCH resource set. Therefore, in the case of a coverage limited UE, there is no way to overcome the channel until the RRC connection is established. Therefore, discussion for supporting the repetition of PUCCH for the UE who does not have dedicated PUCCH resource configuration is needed. The following three alternatives can be studied.

- The number of PUCCH repetition is tied with the repetition number of PUSCH.

- The PUCCH repetition number is indicated by new rule. (e.g., indicated by PRI, system information)

- A new PUCCH resource set including the repetition number is defined.

First, as a method of relating the repetition number of PUCCH with the repetition number of PUSCH, it can be considered to introduce a mapping relationship between the repetition number and the PUCCH repetition number and apply it when the repetition of Msg3 PUSCH is configured. Second, adding a rule indicating the number of PUCCH repetitions by the combination of PRI and CCE index, or a method of indicating the number of PUCCH repetitions in SIB1 or other system information may be considered. Finally, a method of defining a multiple PUCCH resource sets which repetition is added may be considered. The pros and cons of each method should be discussed.

Proposal 1: The following methods to configure PUCCH repetition for the UE without dedicated PUCCH resource configuration should be studied.

- PUCCH repetition is indicated by using repetition number of PUSCH.

- PUCCH repetition is indicated by PRI and/or system information.

- Introduce a PUCCH resource set with repetition number.

With dedicated PUCCH resource configuration

In the previous meeting, it was agreed that the dynamic PUCCH repetition factor indication without DCI enhancement is supported. For this, the simplest way for RRC enhancement would be to add a repetition number to the information element of PUCCH-resource. Since the CCE index is used for the resource selection rule of the existing PUCCH format 0 and PUCCH format 1, adding a new rule using the CCE index should not overlap with the existing rule. This may limit the scheduling flexibility of gNB, so it is better to avoid it. That is, it is preferable to add a repetition number to the PUCCH resource and indicate it only with PRI.

The enhancement to put the number of repetitions in the PUCCH resource implies the number of PUCCH repetitions is indicated in the following two information elements: the number of repetitions indicated in the PUCCH-resource, and the number of repetitions set in the existing PUCCH-FormatConfig. Therefore, a rule is needed between the two to avoid misunderstanding of UE, which means the relationship between them needs to be discussed. Whatever the enhancement method, such a rule is required between the number of PUCCH repetitions which is dynamically indicated and the number of repetitions which is indicated in the PUCCH format.

Proposal 2: The repetition number of PUCCH is added in the information element of PUCCH-resource.

Proposal 3: It is necessary to discuss the relationship between the PUCCH repetition number indicated dynamically and the PUCCH repetition number indicated in the PUCCH-FormatConfig.

Conclusion

- In this contribution, we discuss on specifications of PUCCH enhancement. From the discussion, we obtained following proposals:

Proposal 1: The following methods to configure PUCCH repetition for the UE without dedicated PUCCH resource configuration should be studied.

- PUCCH repetition is indicated by using repetition number of PUSCH.

- PUCCH repetition is indicated by PRI and/or system information.

- Introduce a PUCCH resource set with repetition number.

Proposal 2: The repetition number of PUCCH is added in the information element of PUCCH-resource.

Proposal 3: It is necessary to discuss the relationship between the PUCCH repetition number indicated dynamically and the PUCCH repetition number indicated in the PUCCH-FormatConfig.

Reference

- RAN1 chairman’s notes, RAN1 #105-e, e-Meeting, May 10th – 27th, 2021

Agreements in RAN1#105-e

Agreement: For DMRS bundling for PUCCH repetitions, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH repetitions subject to power consistency and phase continuity requirements. - Strive for common design of the time domain window for PUSCH/PUCCH with DMRS bundling as much as possible. Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication. - Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor. - FFS: RRC signaling enhancement details Conclusion: For PUCCH repetitions, the following use cases are deprioritized in RAN1 work on PUCCH DMRS bundling  Use case 1: back-to-back PUCCH repetitions within one slot.  Use case 2: non-back-to-back PUCCH repetitions within one slot. ‐ Use case 2a: no uplink transmission in the middle of two PUCCH repetitions - ‐ Use case 2b: other uplink transmissions in the middle of two PUCCH repetitions