3GPP TSG RAN WG1 #106-e	R1-2107555

e-Meeting, August 16th – 27th, 2021

Agenda item:		7.2.5

Source:	Nokia, Nokia Shanghai Bell

Title:	UCI enhancements maintenance: Conflict between the first PUCCH repetition and semi-static configuration in Rel-15/16

Document for:		Discussion and Decision

# Introduction

This contribution contains discussions on conflicts between the first PUCCH repetition and semi-static configuration, as earlier discussed as part of the Rel-16 URLLC UCI enhancements maintenance in RAN1#105-e in [105-e-NR-L1enh-URLLC-02].

# Conflict between the first PUCCH repetition and semi-static configuration in Rel-15/16

The group discussed during RAN1#105-e postponing of a PUCCH repetition that collides with SSB symbols or symbols indicated as DL. A consensus was reached how this should be handled in Rel-16, but it was noted that the same question was discussed for Rel-15 and then different implementation choices were allowed. A proposal was to include in the meeting minutes a note that concerns only Rel-16. Because the problem was discussed for Rel-15 and one approach was agreed then, it seems not possible to change that decision if there are objections. Therefore, we support adding the note in the form proposed in RAN1 #105-e. An objection was based on thinking that, as specifications are identical for Rel-15 and Rel-16, we should not make a note only for Rel-16. However, without a note, ambiguity would remain also with Rel-16 which seems unfortunate.

Proposal: Include in the Chairman’s Notes the clarification
“It is clarified that, according to the running R16 specification, a PUCCH repetition in case NPUCCHrepeat>1 (including the first PUCCH repetition) is postponed to the next available slot if the PUCCH repetition collides with SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.”

We would also fine with specifically mentioning that different interpretations are allowed in Rel-15 as proposed by CATT in RAN1 #105-e.