3GPP TSG RAN WG1 #106-e			                                                                     R1-2107566

e-Meeting, August 16th – 27th, 2021

Source: 	Intel Corporation

Title:	Discussion on physical layer aspects of small data transmission

Agenda item:	5.2

Document for:	Discussion and Decision

# Introduction

At the RAN1#105-e meeting, the following agreements were made for CG-SDT [1].

Agreement:

- The SSB-to-PUSCH resource mapping within the CG configuration is implicitly defined.

- The ordering of the SSB and CG PUSCH resources are to be captured in RAN1 spec.

  - A PUSCH resource refers to a transmission occasion and a DMRS resource used for PUSCH transmission

  - The ordering of the SSB can reuse from the SSB-to-RO mapping

  - The ordering of CG PUSCH resources can reuse from that of MsgA PUSCH as much as possible

- FFS determination of mapping ratio and association period, e.g., explicitly signaled or implicitly derived

- FFS any limitation on the combination of the parameters for CG resources

Agreement:

- The SSB subset for RSRP based TA validation is determined at least based on a configured absolute RSRP threshold.

- FFS the SSB subset which could be

  - within a set of SSBs configured per CG configuration

  - or within a set of SSBs configured for all CG configurations

  - or within a set of all SSBs actually transmitted as indicated in SIB1.

  - or highest N SSBs that are measured to derive the subset for a UE across all CG configurations

Further, RAN1 received a reply LS from RAN2, which includes the following questions [2]:

Furthermore, RAN2 discussed the details of both RA-SDT and CG-SDT leading to the following agreements with potential RAN1 impacts. Agreement: For RA-SDT - CFRA is not supported for RA-SDT For CG-SDT - Working assumption: UE-specific search space is configured for UEs performing CG-SDT. RAN2 asks RAN1 whether this working assumption can be confirmed - CG-SDT resource can be configured on either initial BWP or separate SDT BWP. Ask RAN1 to confirm - RAN2 thinks that some feedback may be beneficial in case CG is used for subsequent transmission. RAN2 assumes that existing mechanism can be used.. For RA-SDT (for both 2-step RA and 4-step RA), in particular, RAN2 would like to request RAN1 to provide configuration parameters for the PRACH resource configuration when PRACH occasions are shared between SDT and non-SDT and when PRACH occasions are separately configured for SDT and non-SDT. For CG-SDT, RAN2 respectfully requests RAN1 if the agreements 3/4 mentioned above can be confirmed and provides feedback on whether there is any issue with the assumption of reusing the existing L1 mechanism for agreement 5. # 2. Actions: RAN2 respectfully asks RAN1 to provide feedbacks on the physical layer aspects of small data transmission as requested above.

In the contribution, we present our views on physical layer aspects of small data transmission.

# Discussion on RA-SDT

For RA-SDT, separate PRACH occasions or separate PRACH preamble in case of shared PRACH occasions can be used to differentiate SDT and non-SDT operation. Note that based on RAN2 agreement, contention free random access is not supported for RA-SDT [2].

Note that the existing PRACH configuration parameters can be reused for RA-SDT operation. For instance, when separate PRACH occasions are configured for SDT and non-SDT operation, the following parameters may need to be configured for SDT operation for 4-step RACH:

- prach-ConfigurationIndex

- totalNumberOfRA-Preambles

- prach-RootSequenceIndex

- zeroCorrelationZoneConfig

- restrictedSetConfig

- msg1-FDM

- msg1-FrequencyStart

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB

Similar design principle can be applied for 2-step RACH based SDT operation when separate PRACH occasions are configured for SDT and non-SDT operation, i.e., existing PRACH configurations can be reused for 2-step RACH based SDT operation. Note that if some of these parameters are not configured, default values from CBRA 4-step RACH or 2-step RACH may be applied.

Proposal 1

- In case of separate PRACH occasions for non-SDT and SDT, existing PRACH configuration can be reused for 4-step RACH and 2-step RACH based SDT operation.

When separate PRACH preambles with shared ROs are used to differentiate SDT and non-SDT operation, CB-PreamblesPerSSB can be configured, which is used to indicate number of preambles allocated for the SDT operation. In addition, this parameter can be separately configured for 2-step and 4-step RACH based SDT, if both 2-step RACH and 4-step RACH based SDT are used.

Further, in case of shared ROs, preambles for 4-step RA-SDT can be allocated after 4-step and 2-step CBRA RACH associated with a same SSB index, followed by the preambles for 2-step RA-SDT. Figure 1 illustrates PRACH preambles allocated for 4-step and 2-step RA-SDT. Note that when 2-step RACH is not configured, preambles for 4-step RA-SDT can be allocated after 4-step CBRA RACH associated with the same SSB index.

Figure 1. Preambles allocated for 4-step and 2-step RA-SDT

In order to further minimize the impact on the legacy system, when one SSB is associated with more than one ROs, a subset of ROs may be configured and utilized for RA-SDT operation, which is similar to what was defined in Rel-16 2-step RACH. In this regard, PRACH mask can be defined to indicate a subset of ROs associated with same SSB index for RA-SDT.

Proposal 2

- In case of separate PRACH preambles with shared PRACH occasions for non-SDT and SDT,

  - Number of preambles is separately configured for 4-step and 2-step RACH based RA-SDT, respectively.

  - Preambles for 4-step RA-SDT can be allocated after 4-step and 2-step CBRA RACH associated with a same SSB index, followed by the preambles for 2-step RA-SDT.

  - PRACH mask is defined to indicate a subset of ROs associated with same SSB index for RA-SDT.

# Discussion on CG-SDT

## Association between SSB and CG-PUSCH resource

For CG-SDT, association between CG resource and SSB is needed for proper multi-beam operation. At the RAN1#105-e meeting, it was agreed that the SSB-to-PUSCH resource mapping within the CG configuration is implicitly defined. Further, the ordering of the SSB can reuse from the SSB-to-RO mapping and the ordering of CG PUSCH resources can reuse from that of MsgA PUSCH as much as possible [1].

To provide more flexibility on the association between SSB and CG resource, mapping ratio and association period can be explicitly configured by the network, which is similar to SSB to RO association. In this case, more flexible mapping ratio, including many to one mapping, or one to one mapping can be supported for association between SSB to CG-PUSCH occasion.

Figure 2 illustrates many-to-one and one-to-one mapping between SSB and CG-PUSCH occasion. For many-to-one mapping, if gNB is equipped with multiple panels, the gNB may attempt to decode CG-PUSCH transmissions from different beam directions simultaneously. In this case, multiple DMRS resources including DMRS APs and/or sequences can be defined so as to allow one SSB to be associated with at least one DMRS resource. Further, for one-to-one mapping, different SSBs are associated with different CG-PUSCH occasions in different times, and only one DMRS resource may be configured for a CG-PUSCH configuration.

Figure 2. Many-to-one and one-to-one mapping between SSB and CG-PUSCH occasion

Proposal 3

- For the association between SSBs and CG-PUSCH resources

  - Multiple DMRS resources can be configured within a CG-PUSCH occasion.

  - Mapping ratio and association period are explicitly configured

  - Many-to-one and one-to-one mapping between SSB and CG-PUSCH occasion are supported.

As defined for 2-step RACH in Rel-16, MsgA PUSCH occasion is considered as valid only when certain conditions are satisfied, including overlapping between RO in 2-step and 4-step RACH, semi-static DL/UL configurations and SSB transmission, etc. For CG-PUSCH occasions for CG-SDT, similar mechanisms can be reused to ensure that UE transmits uplink data on the valid CG-PUSCH resources. Note that further validation rule on handling potential overlapping between CG-PUSCH occasions for CG-SDT and MsgA PUSCH occasions for 2-step RACH may need to be considered.

Proposal 4

- CG-PUSCH occasion validation rule for CG-SDT follows that was defined for MsgA PUSCH occasion for 2-step RACH.

  - FFS: potential overlapping between CG-PUSCH occasions for CG-SDT and MsgA PUSCH occasions for 2-step RACH.

## Subsequent transmission for CG-SDT

As indicated in the reply LS from RAN2, it was agreed as working assumption that UE specific search space is configured for CG-SDT. Given that CG-SDT operation is mainly UE specific, e.g., including configuration of SSB indexes, and CG-PUSCH configurations, it is straightforward to assume that UE specific search space is configured for CG-SDT operation. Hence, in our view, RAN1 can confirm the working assumption that UE specific search space is configured for CG-SDT.

Further, it was agreed in RAN2 that CG-SDT resource can be configured on either initial BWP or separate SDT BWP. This is mainly targeted to avoid potential congestion in the initial UL BWP, especially considering the case when relatively large number of UEs perform SDT in the system. In this case, gNB may configure different SDT BWPs for different group of UEs so as to balance the traffic load and minimize the impact to the legacy system.

Proposal 5

- RAN1 to confirm the working assumption that UE-specific search space is configured for UEs performing CG-SDT.

- RAN1 to confirm that CG-SDT resource can be configured on either initial BWP or separate SDT BWP.

In addition, as mentioned in reply LS from RAN2, it was agreed that some feedback may be beneficial in case CG is used for subsequent transmission and RAN2 assumes that existing mechanism as defined in RAN1 can be reused. For instance, DFI based mechanism as introduced in Rel-16 NR-U may be reused and applied as explicit HARQ-ACK feedback for CG-PUSCH transmission during CG-SDT operation. In addition, DG-PUSCH scheduling with same HARQ process ID as CG-PUSCH and toggled/non-toggled NDI can be used for new transmission and retransmission during CG-SDT operation, respectively. Hence, in our view, RAN1 can confirm that existing L1 mechanism can be reused for feedback of CG-PUSCH transmission during CG-SDT.

Proposal 6

- RAN1 to confirm that existing L1 mechanism can be reused for feedback of CG-PUSCH transmission during CG-SDT.

# Conclusions

In this contribution, we presented our views on the physical layer aspects of small data transmission. Further, we summarize the proposals as follows:

Proposal 1

- In case of separate PRACH occasions for non-SDT and SDT, existing PRACH configurations can be reused for 4-step RACH and 2-step RACH based SDT operation.

Proposal 2

- In case of separate PRACH preambles with shared PRACH occasions for non-SDT and SDT,

  - Number of preambles is configured for 4-step and 2-step RACH based RA-SDT, respectively.

  - Preambles for 4-step RA-SDT can be allocated after 4-step and 2-step CBRA RACH associated with a same SSB index, followed by the preambles for 2-step RA-SDT.

  - PRACH mask is defined to indicate a subset of ROs associated with same SSB index for RA-SDT.

Proposal 3

- For the association between SSBs and CG-PUSCH resources

  - Multiple DMRS resources can be configured within a CG-PUSCH occasion.

  - Mapping ratio and association period are explicitly configured

  - Many-to-one and one-to-one mapping between SSB and CG-PUSCH occasion are supported.

Proposal 4

- CG-PUSCH occasion validation rule for CG-SDT follows that was defined for MsgA PUSCH occasion for 2-step RACH.

  - FFS: potential overlapping between CG-PUSCH occasions for CG-SDT and MsgA PUSCH occasions for 2-step RACH.

Proposal 5

- RAN1 to confirm the working assumption that UE-specific search space is configured for UEs performing CG-SDT.

- RAN1 to confirm that CG-SDT resource can be configured on either initial BWP or separate SDT BWP.

Proposal 6

- RAN1 to confirm that existing L1 mechanism can be reused for feedback of CG-PUSCH transmission during CG-SDT.

# References

- Chairman’s notes, RAN1 #105 e-Meeting, May 2021

- R2-2106561, “Reply LS to RAN1 on physical layer aspects of small data transmission”, e-Meeting, May 2021