3GPP TSG RAN WG1 #106-e			                                                                    R1-2107578

e-Meeting, August 16th – 27th, 2021

Source:	Intel Corporation

Title:	Discussion on PDCCH monitoring enhancements for extending NR up to 71 GHz

Agenda item:	8.2.2

Document for:	Discussion/Decision

# Introduction

At RAN plenary meeting #90-e, a revised work item (WI) to extend the current NR operation to 71GHz was agreed [1]. Enhancements on PDCCH monitoring is one aspect which is captured by the following objective.

- In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

  - Limited sync raster entry numbers

    - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

  - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS.

  - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

  - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17.

  - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

  - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access)

  - Note: Dependency or lack thereof for a UE supporting 480kHz and/or 960kHz numerology for data and control to also support 480kHz SSB numerology for initial access is to be tackled as part of UE capability discussion.

- Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB

  - FFS: additional method(s) to enable support to obtain neighbour cell SIB1 contents related to CGI reporting

  - Only 1 CORESET#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (120, 120), (480, 480) and (960, 960).

  - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

  - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

  - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB.

  - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

- Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

In last meetings, the following agreements were made. However, further elaboration on the alternatives was discussed but not agreed.

Agreement: Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability - Alt 1: A fixed pattern of N slots. - Alt 2: Use the Rel-16 capability (pdcch-Monitoring-r16, (X, Y) span) as the baseline to define the new capability - FFS: Values of X and Y and units in which they are defined - FFS: Whether number of slots within which the number of monitoring occasions is counted is needed and if needed, the value of the number of slots - Alt 3: A sliding window of N slots for defining multi-slot PDCCH monitoring capability. - FFS: Increments in which sliding occurs - Specific numbers for X, Y and N may depend on UE capability and gNB configuration - Examples: - N = [4] slots for 480 kHz SCS and N = [8] slots for 960 kHz SCS - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS Agreement: Previous agreement is modifed as follows: Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability - Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability. - Each slot group consists of X slots - Slot groups are consecutive and non-overlapping - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g. the Y [symbols or slots] always start at the first slot within a slot group - FFS: Further definition of capabilities - Alt 2: Use an (X, Y) span as the baseline to define the new capability - X is the minimum time separation between the start of two consecutive spans - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots] - Y <= X - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary. - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots. - FFS: Further definition of capabilities - Alt 3: Use a sliding window of X slots as the baseline to define the new capability. - The capability indicates the BD/CCE budget within the sliding window - The sliding unit of the sliding window is [1] slot. - FFS: Further definition of capabilities - Specific numbers for X, Y may depend on UE capability and gNB configuration - Examples: - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS Conclusion: For 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed. Agreement: For 120 kHz SCS in 52.6-71 GHz, the BD/CCE budget is the same as that for 120 kHz in FR2.

Therefore, in this contribution, we provide our further views on PDCCH monitoring capability and search space set configuration for the PDCCH monitoring.

# Multi-slot PDCCH monitoring capability

In the agreement in RAN1#104bis_e, three alternatives were listed for the definition of multi-slot PDCCH monitoring capability. Though there was no explicit further agreement, the companies compromised to further work on Alt 1 or Alt 2, with the understanding that some key design points of Alt 3 should be supported by Alt 1 or Alt 2. A first point is to avoid back-to-back PDCCH monitoring occasions in adjacent X-slot groups [2]. This is to avoid the increased number of configured BDs/CCEs which results in high UE complexity. A second point is to allow the flexibility to configure the USS sets and CSS sets. It is preferrable to allow the configuration of USS sets and CSS sets in different slots/symbols. The updated Alt 1 and Alt 2 in the final moderator summary are cited below,

- Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability. - Each slot group consists of X slots - Slot groups are consecutive and non-overlapping - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g. the Y [symbols or slots] always start at the first slot within a slot group - FFS: Further definition of capabilities - Alt 2: Use an (X, Y) span as the baseline to define the new capability - X is the minimum time separation between the start of two consecutive spans - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots] - Y <= X - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary. - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots. - FFS: Further definition of capabilities

Regarding value X, there is no objection that X can be 4 for SCS 480kHz and 8 for SCS 960kHz. Further, additional smaller values are not excluded. Having smaller value X allows gNB to configure PDCCH monitoring more frequently, which can enable quick response of certain indication. For example, if LBT is used for the channel access, the frequency PDCCH monitoring enables gNB to start effective DL transmission right after the successful of DL LBT. Therefore, it is beneficial to support X equals to 2 for SCS 480kHz. Further, X can be 2 or 4 for SCS 960kHz.

There was an email discussion on the potential value Y in the end of RAN1#104bis_e meeting. However, no agreement was reached. By observing the discussions, it seems most companies agree to support a maximum value of Y as X/2, which avoids back-to-back configuration of PDCCH monitoring occasions. Regarding the minimum value of Y, the candidates include 3 symbols, one slot or more than one slots. According to the understanding that the above two points should be addressed in Alt1/2 design, 3 symbols as minimum value of Y should be ruled out. Since the CSS sets are shared by the UEs, restricting Y of 3 symbols results in that the USS/CSS sets of the UEs are configured in same monitoring occasions, which reduces the scheduling flexibility of gNB. Therefore, it is preferred to restrict a minimum Y of two slots, which allows gNB to configure USS sets and CSS sets in same or different symbols in same or different slots. As an exception, if X equals to two slots, Y can be just one slot. In this case, it only supports the gNB to configure USS sets and CSS sets in same or different symbols in the same slot.

One open issue is whether per-slot PDCCH monitoring capability can be supported for SCS 480/960kHz. It is expected that the number of BDs/CCEs per slot that can be monitored by a UE is reduced for the high SCSs. As a result, a UE may not support the PDCCH monitoring of high aggregation level, e.g., 16 or 8. Otherwise, if the number of BDs/CCEs per slot is increased, the overall complexity for PDCCH monitoring of a UE becomes quite high. Therefore, we prefer that per-slot PDCCH monitoring capability is not supported for SCS 480/960kHz.

Proposal 1:

- X can be 2 and 4 for SCS 480kHz, 2, 4 and 8 for SCS 960kHz

  - Per-slot PDCCH monitoring capability is not supported for SCS 480/960kHz

- The maximum value Y is X/2.

- The minimum value of Y is 2, except the case that X equals to 2 and Y equals to 1 for SCS 480kHz.

According to the revised WID [1], initial access is supported for SCS 480kHz with support of Type0-PDCCH CSS set. Further, for ANR and PCI confusion detection, Type0-PDCCH SCS set is supported for SCS 480kHz and 960kHz. Therefore, for SS/PBCH block and CORESET multiplexing pattern 1 in NR, UE needs to monitor Type 0/0A/1/2 CSS sets with search space set 0 in two consecutive slots. The time position of the two slots depends on the position of the associated with a SS/PBCH block. On the other hand, other CSS/USS sets are configured by high layer parameters monitoringSlotPeriodicityAndOffset and duration. The combined pattern of slots configured with Type 0/0A/1/2 CSS sets and other CSS/USS sets may satisfy the pattern of the multi-slot PDCCH monitoring capability for a SS/PBCH block of the UE. However, when the SS/PBCH block of the UE changes, the combined pattern of slots may become invalid for the multi-slot PDCCH monitoring capability.

Instead of reconfiguration of all CSS/USS sets when the SS/PBCH block changes, dropping the PDCCH MOs in a slot can be considered to make the combined pattern valid. However, dropping Type 0/0A/1/2 CSS sets has negative impact on the scheduling of broadcast information. On the other hand, dropping other CSS/USS sets is not desirable for PDSCH/PUSCH scheduling. Since Type 0/0A/1/2 CSS sets only occurs in long cycle, i.e., 20ms, it can be considered to temporarily allow the violation of slot pattern of the multi-slot PDCCH monitoring capability near to a slot carrying Type 0/0A/1/2 CSS sets.

Proposal 2:

- If Type0/0A/1/2 CSS sets are monitored in a slot, configured PDCCH monitoring occasions around the slot can temporarily violate the multi-slot PDCCH monitoring capability, with the limitation of maximum number of BDs/CCEs.

In NR Rel-15, according to the UE capability on the maximum number of BDs/CCEs in a slot,

- For PCell or PSCell, it is allowed that the configured number of BDs/CCEs in a slot by the configuration of SS set(s) is larger than the corresponding maximum numbers. Certain dropping rule is defined so that the actual number in the slot doesn’t exceed the corresponding maximum numbers.

- For a SCell, the gNB should guarantee that the configured numbers of BDs/CCEs in a slot by the configuration of SS set(s) do not exceed the corresponding maximum numbers.

The similar rules could be extended to multi-slot PDCCH monitoring capability,

- For PCell or PSCell, it is allowed that the configured number of BDs/CCEs in a X-slot group by the configuration of SS set(s) is larger than the corresponding maximum numbers. Certain dropping rule is defined so that the actual number in the X-slot group doesn’t exceed the corresponding maximum numbers.

- For a SCell, the gNB should guarantee that the configured numbers of BDs/CCEs in a X-slot group by the configuration of SS set(s) do not exceed the corresponding maximum numbers.

Proposal 3:

- When multi-slot PDCCH monitoring capability is supported,

  - PDCCH overbooking is supported for PCell or PSCell

  - For a SCell, the configured BDs/CCEs do not exceed the corresponding maximum numbers.

As in Rel-15, it is desired there is no dropping for CSS sets even for PCell/PSCell. Therefore, it is up to gNB to guarantee that CSS sets are properly configured. One thing to note is that multiple slots in a X-slot group may contain MOs for a CSS set subject to gNB configuration. In this case, the total numbers of BDs/CCEs in the multiple slots for the USS set are multiple times of that configured in single slot.  Consequently, the numbers of available BDs/CCEs for USS sets are reduced. The UE capability on maximum numbers of BDs/CCEs needs to consider the increase of BDs/CCEs in the X-slot group for a CSS set.

Regarding handling USS sets if total number of BDs/CCEs exceed the corresponding maximum numbers, a same principle as in Rel-15 can be reused, i.e. a USS set with high SS set index is dropped. Further, since the PDCCH MOs of the USS set may be configured in multiple slots in the X-slot group, a discussion point is whether the USS set in all the multiple slots is dropped as a whole or dropped slot by slot. The latter option is preferred since it allows more capacity for PDCCH monitoring without exceeding UE capability.

Proposal 4:

- To handling USS dropping in PDCCH overbooking

  - A USS set with largest SS set index is dropped

  - If the PDCCH MOs of a USS set are configured in multiple slots in the X slots, the USS set in the multiple slots is dropped slot by slot.

The search space set group (SSSG) switching as defined NR-U is shown in Figure 1. The frequent PDCCH monitoring is assumed before gNB gets the channel occupation. For example, search space set with mini-slot level or slot-level PDCCH monitoring can be configured, which is the first search space set group (SSSG). The frequent PDCCH monitoring reduces the delay for gNB to start DL transmission immediately after the LBT is successful. On the other hand, after gNB starts a COT, a second SSSG is configured, which may contain infrequent PDCCH monitoring for power saving. The DL performance for the second SSSG is guaranteed by the configuration of more PDCCH candidates with same or different DCI format in each MO of the second SSSG.

Figure 1: SSSG switching in NR-U

Dynamic SSSG switching provides a means to balance the fast channel access right after LBT is successful and effective scheduling and power saving after the COT is obtained. The feature is useful for high frequency too. The configured search space configuration in the two SSSGs has different requirements on the PDCCH monitoring capability. In general, two options can be considered.

- Option 1: switching between per-slot PDCCH monitoring capability and multi-slot PDCCH monitoring capabilities

- Option 2: switching between two multi-slot PDCCH monitoring capabilities

As discussed in Proposal 1, per-slot PDCCH monitoring capability is not preferred due to the limitation of PDCCH monitoring or UE complexity for high SCSs. Therefore, the search space set configuration of the two SSSGs can be configured based on Option 2. It is expected that both X and Y are small values to still allow frequent MOs in the first SSSG. On the other hand, the second SSSG relies on a large gap between MOs for the power saving.

Based on the above analysis, UE needs to dynamically change its PDCCH monitoring capability together with SSSG switching.

Proposal 5:

- Dynamic SSSG switching is supported for all SCSs 120, 240 and 960kHz.

- The search space set configurations of the two SSSG can correspond to two different combinations (X, Y) of the multi-slot PDCCH monitoring capabilities

# PDCCH monitoring occasion(s) in a slot

In NR, a search space (SS) set could be configured for the UE to monitor PDCCH. Up to 10 SS sets can be configured for each DL BWP in a serving cell. The time domain pattern of a SS set is configured by the following RRC parameters

-	a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset

-	a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot

-	a duration of Ts<ks slots indicating a number of slots that the search space set s exists by duration

Specifically, 3 cases for SS set configuration within a slot are supported by parameter monitoringSymbolsWithinSlot,

- Case 1: PDCCH monitoring of all SS sets monitored in a slot occurs within 3 consecutive OFDM symbols that have fixed positions in each slot

  - Case 1-1: PDCCH monitoring limited to within first three OFDM symbols of a slot

  - Case 1-2: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot

    - For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot

- Case 2: PDCCH monitoring cases other than Case 1

Case 1-1 is the basic PDCCH monitoring occasion(s) in the beginning of a slot, which should be supported for high SCS. Case 2 is to configure more frequent PDCCH MOs within a slot, which is targeted to reduce scheduling latency. This is important especially for low SCS, e.g. 15kHz or 30kHz. On the other hand, it is not necessary for a high SCS, e.g. 480kHz or 960kHz, given that the slot length is quite short, i.e. 1/32ms or 1/64ms. In this case, there is no clear motivation to allow full flexibility on the positions of PDCCH MO(s) in a slot, i.e. Case 2. Therefore, restriction on PDCCH MOs in a slot can simplify UE implementation without performance degradation.

Proposal 6:

- On the PDCCH monitoring occasion in a slot

  - Case 1-1 is supported for all SCS 120kHz, 480kHz and 960kHz

  - Case 2 is supported for SCS 120kHz

  - Case 2 is not supported for SCS 480/960kHz

# Search space set configuration

The configuration of PDCCH MO in all slots may not be necessary for high SCS in a real deployment. As shown in Figure 4, if the TDD UL/DL configuration is to align with LTE TDD configuration of 5ms switching periodicity, there exists up to 40 DL slots if SCS 120kHz is used. The specification supports gNB to configure MOs in each slot using a single SS set configuration. That is, the periodicity is 5ms with slot offset 0, which can be configured by monitoringSymbolsWithinSlot. Meanwhile, the parameter duration can be set to a value that equals to the number of DL slots in the 5ms period. However, if the SCS is increased to 960kHz, the current specification cannot support the configuration of a SS set with same periodicity and same location of MOs as SCS 120kHz. Note that the necessary number/frequency of MOs in time is mainly determined by the traffic property, e.g. the latency requirement, especially considering that multi-slot PDSCH/PUSCH is also supported. Therefore, it is a big drawback if the MO pattern for SCS 960kHz in Figure 2 cannot be supported.

Figure 2: Time pattern of MOs with SCS 120kHz and 960kHz

With the existing SS set configuration, up to 40 SS sets need to be configured to achieve the MO pattern in Figure 1. On the other hand, considering different DCI formats (fallback DCI or normal DCI) and different type of SS set (USS, CSS type0/0A/1/2, CSS type3 with different DCI formats), the required number of SS sets must be much higher than 40. The main drawback of the current SS set configuration comes from the parameter ‘duration’ which is defined as a number of consecutive slots. To support the MO pattern for SCS 960kHz in Figure 1, a simple extension is to allocate a MO in every N slot, instead of consecutive slot allocation. The parameter ‘duration’ is still needed but can be reinterpreted as the window that MOs may be allocated, e.g. the DL period in a TDD period. Denote the number of slots that are configured with MOs of the USS set as M, then duration=N∙M.

Proposal 7:

- Within a period of a SS set configuration

  - The parameter ‘duration’ is reinterpreted as a window on which MOs may be configured.

  - One slot in every N slots within the window is configured with PDCCH MOs

Cross-carrier scheduling is a quite useful feature for NR. Therefore, it is expected that cross-carrier scheduling between serving cells using SCS 120/480/960kHz can be supported. On the other hand, one more discussion point is the carrier aggregation (CA) between a cell with 52.6-71GHz frequency and a cell in FR2 or even FR1. From specification completeness point of view, such CA scenario could be supported, especially considering a PCell in lower frequency than 52.6-71GHz is more appropriate for coverage/robustness. As discussed in MR-DC in Rel-16, the minimum PDSCH scheduling delay and minimum A-CSI RS triggering offset applicable to SCS 480kHz and 960kHz should be discussed. On the other hand, if such kind of CA is supported and cross-carrier scheduling is considered, an extreme case could be that, a slot with SCS 15kHz is used to schedule up to 64 slots with SCS 960kHz. Without a clear motivation, we prefer to avoid unnecessary optimization.

Proposal 8:

- Cross-carrier scheduling of cell with 52.6-71GHz frequency from/to a cell of FR1 and FR2 is allowed by specification

  - The minimum PDSCH scheduling delay and the minimum A-CSI RS triggering offset applicable to SCS 480kHz and 960kHz needs to be discussed.

  - Additional enhancements are deprioritized unless a clear motivation is identified.

# Conclusions

In this contribution, we discussed the potential enhancements on PDCCH monitoring in 52.6-71GHz frequency. The following is a summary of the proposals:

Proposal 1:

- X can be 2 and 4 for SCS 480kHz, 2, 4 and 8 for SCS 960kHz

  - Per-slot PDCCH monitoring capability is not supported for SCS 480/960kHz

- The maximum value Y is X/2.

- The minimum value of Y is 2, except the case that X equals to 2 and Y equals to 1 for SCS 480kHz.

Proposal 2:

- If Type0/0A/1/2 CSS sets is monitored in a slot, the configuration PDCCH monitoring occasions around the slot can temporarily violate the multi-slot PDCCH monitoring capability, with the limitation of maximum number of BDs/CCEs.

Proposal 3:

- When multi-slot PDCCH monitoring capability is supported,

  - PDCCH overbooking is supported for PCell or PSCell

  - For a SCell, the configured BDs/CCEs do not exceed the corresponding maximum numbers.

Proposal 4:

- To handling USS dropping in PDCCH overbooking

  - A USS set with largest SS set index is dropped

  - If the PDCCH MOs of a USS set are configured in multiple slots in the X slots, the USS set in the multiple slots is dropped slot by slot.

Proposal 5:

- Dynamic SSSG switching is supported for all SCSs 120, 240 and 960kHz.

- The search space set configurations of the two SSSG can correspond to two different combinations (X, Y) of the multi-slot PDCCH monitoring capabilities

Proposal 6:

- On the PDCCH monitoring occasion in a slot

  - Case 1-1 is supported for all SCS 120kHz, 480kHz and 960kHz

  - Case 2 is supported for SCS 120kHz

  - Case 2 is not supported for SCS 480/960kHz

Proposal 7:

- Within a period of a SS set configuration

  - The parameter ‘duration’ is reinterpreted as a window on which MOs may be configured.

  - One slot in every N slots within the window is configured with PDCCH MOs

Proposal 8:

- Cross-carrier scheduling of cell with 52.6-71GHz frequency from/to a cell of FR1 and FR2 is allowed by specification

  - The minimum PDSCH scheduling delay and the minimum A-CSI RS triggering offset applicable to SCS 480kHz and 960kHz needs to be discussed.

  - Additional enhancements are deprioritized unless a clear motivation is identified.

# References

- RP-211584, “Revised WID: Extending current NR operation to 71GHz,” Qualcomm, Intel

- R1-2100644, “Discussion on PDCCH monitoring enhancements for extending NR up to 71 GHz” Intel