3GPP TSG RAN WG1 #106-e			R1- 2107634

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.8.1.1 Enhancements on PUSCH repetition type A

Source: 	Sierra Wireless

Title: 	Design considerations for PUSCH repetition Type A Enhancements

Document for: 	Discussion and decision

# Introduction

This tdoc discusses design considerations for PUSCH repetition Type A Enhancements for the following work item [1] objectives:

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

Significant agreements made in previous meetings:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

FFS details

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or Configured Grant Config.

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

Down-selection in RAN1#106-e:

Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method,

Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

In addition to {1, 2, 3, 4, 7, 8, 12, 16} and {32}, the following additional value set for repetition factor is supported in Rel-17.

- {20, 24, 28}

Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition.

RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not.

If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot.

Select one from the following (further refinement of the alternatives can be further discussed), for the procedure of Rel-17 PUSCH repetition Type A (other alternatives are not precluded)

Alt 1-B consisting of two steps

Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Alt 1-B’ consisting of two steps

Step 1: Determine K repetitions based on available slots, where the available slot is the UL slot and flexible slot indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability

Alt 2-A consisting of a single step

Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic signaling (e.g. SFI, UL CI, DCI for high priority channel) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

Alt 2-B consisting of two steps

Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic SFI in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

FFS timeline for the dynamic signalling

Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

# Type A Repetitions

The following agreement was made last meeting:

Down-selection in RAN1#106-e:

Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method,

Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

Alt 2 will not provide an adequate amount of coverage improvement for the VoIP cases in unpaired spectrum. Also, Alt 2 will add undue complexity to the specification and implementation in the UE and gNB. Thus, the simplified Alt 1 should be taken.

Select Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method

# Determination of available slots

The following agreement was made last meeting:

Select one from the following (further refinement of the alternatives can be further discussed), for the procedure of Rel-17 PUSCH repetition Type A (other alternatives are not precluded)

Alt 1-B consisting of two steps

Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Alt 1-B’ consisting of two steps

Step 1: Determine K repetitions based on available slots, where the available slot is the UL slot and flexible slot indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability

Alt 2-A consisting of a single step

Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic signaling (e.g. SFI, UL CI, DCI for high priority channel) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

Alt 2-B consisting of two steps

Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic SFI in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

FFS timeline for the dynamic signalling

Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

The main difference between Alt 1 and 2 is whether dynamic signaling is used to determine available slots. This aspect is an open issue where the following conclusion was made in RAN1#104-e.

Conclusion: Discuss further to select one of the following alternatives: - Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions. - Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

The proponents of Alt-a have already expressed several reasons to support it, which at least include:

Alt-a simplifies UE implementation in terms of processing timeline

Alt-a resolves different understandings of available slots between gNB and UE due to detection failure of the dynamic signaling at the UE side.

A third reason to support Alt-a is for support of HD-FDD Redcap UEs. If Alt-b is chosen, in some FDD scenarios the UE will require FD-FDD to be able to decode DL dynamic control information (e.g. SFI, CI, PUSCH priority) while transmitting, which is not possible for RedCap HD-FDD UEs. It may be possible for the gNB to avoid this situation for HD-FDD RedCap UEs but this would require complex scheduling algorithms or complex specification changes.

Given this, the following proposal is made:

Alt-1B is chosen.

Alt 1-B consisting of two steps

Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability

WRT this FFS in Alt 1-B’: “FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability”. The highlighted text  seems contradictory and should be deleted. Obviously, there shall be no requirement for a UE without CI capability to support dropping based on UL CI.

# Applicability to Paired and Unpaired Spectrum

The following proposal was made by the FL but not handled in the last meeting:

- For PUSCH Type A repetitions, counting based on available slots is only applicable to unpaired spectrum.

The available slot counting based on RRC is mostly only applicable to unpaired spectrum. Invalid slot configurations are also supported for paired spectrum, but networks tend to allocate invalid slots infrequently, so the concern of miscounting slots is not as prominent.

That said,32 repeats should still be supported in paired spectrum. Thus, if 32 repeats are supported, this should provide enough coverage for paired spectrum if counting is based on physical slots. This will simply the UE procedure.

For PUSCH Type A repetitions, counting based on available slots is only applicable to unpaired spectrum.

For PUSCH Type A repetitions, the maximum of 32 repeats is supported in paired and unpaired spectrum

# Conclusions

- Select Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method

Alt-1B is chosen.

Alt 1-B consisting of two steps

Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability

For PUSCH Type A repetitions, counting based on available slots is only applicable to unpaired spectrum.

For PUSCH Type A repetitions, the maximum of 32 repeats is supported in paired and unpaired spectrum

# References

RP-202928 China Telecom, “New WID on NR coverage enhancements”

RP-210918 WI “Support of reduced capability NR devices”