3GPP TSG RAN WG1 #106-e		                                    		   R1-2107647

e-Meeting, Aug. 16th – Aug. 27th, 2021

Agenda Item:	8.5.4

Source:	InterDigital, Inc.

Title:	Discussion on latency improvements for DL and DL+UL positioning methods

Document for:	Discussion and Decision

# Introduction

In the RAN#105e meeting, agreements related to latency reduction were made [1]. In this contribution we discuss need for measurement-gap less reception of PRS and fast configuration of measurement gap. Both schemes have merits, and we describe latency reduction techniques for both cases. Particularly, prioritization among different PRS signals, priority association with measurement report, fast measurement gap activation and dynamic muting are discussed in this contribution.

# Latency reduction

Measurement gap less reception and processing of PRS

In RAN1#105e, the following agreements are made related to measurement gap less configuration.

Agreement: - Further study the following options (with the same numerology) to support PRS measurement without MGs for latency reduction in Rel-17 - Option 1: The PRS is from the serving cell and UE measurement is inside the active DL BWP - Option 2: The PRS can be from the serving cell and non-serving cell, and UE measurement is inside the active DL BWP - Option 3: The PRS (from the serving cell or non-serving cell) used for UE measurement may extend outside or be completely outside the active DL BWP (including with potentially a different numerology) - Note: RAN1 strives not to increase the PRS measurement time compared with Rel-16 MG-based measurement - The following aspects are FFS - PRS processing prioritization window - Mechanism to trigger UE DL PRS measurements and report - UE/gNB assumptions on processing of DL PRS and other DL physical channels / signals - UE DL PRS processing capabilities - Note: Companies are encouraged to compare the latency benefits of introducing MG-less PRS measurements over MG-based PRS measurements - Note: Depending on the comparison of latency benefits (and other considerations such as complexity) between introducing MG-less PRS measurements and MG-based PRS measurements, none/one/multiple of the above options should be adopted in Rel-17.

Rel-17 positioning services includes stringent latency requirements. To meet the Rel-17 positioning requirements, enhancements of the existing positioning methods, including the signalling aspects and PRS configuration, are needed. In Rel. 16, the UE is expected to make measurements on the PRS received when the measurement gap is configured. The measurement gap is configured per request from the UE and it is configured through RRC. It has been shown during Rel. 17 SI that configuration of measurement gap takes nearly 13ms [2]. Thus, latency reduction can be achieved by eliminating measurement gap configuration to receive the PRS.

Elimination of measurement gap requires some considerations for scheduling since PRS and DL channels neeed to coexist. For PRS resources are transmitted within the serving cell, the gNB can schedule channels according to prioritization rules associated with PRS. Thus, measurement gap-less configuration should be supported at least for PRS transmitted from the serving cell.

Proposal 1: Measurements and processing of PRS without measurement gap should be supported with at least Option 1 (The PRS is from the serving cell and UE measurement is inside the active DL BWP)

For PRS is transmitted from neighobring cells, the serving gNB and neighobring gNBs need to coordinate over scheduling. For example, PDCCH transmission schedule from the serving or neighobring gNBs may need to be conveyed to LMF to schedule PRS transmission. Thus further analysis is required for Option 2 or Option 3. For the above reasons, measurement gap less PRS transmission may be restricted to PRS transmitted from the serving gNBs only.

On-demand PRS, can be used to change PRS configuration based on the UE’s operating conditions. Therefore, if on-demand PRS is activated without a measurement gap, the LMF should prioritize on-demand PRS over other channels in case of an overlapping transmission.

Observation 1: On-demand PRS should be supported flexibly with priorities associated with them.

Proposal 2: In the presence of no measurement gap, the UE is expected to receive PRS with higher priority, associated with on-demand PRS, over other channels if the PRS overlaps with other channels in the time domain.

Measurement reporting

Similar to overlapping PRS transmissions, the UE can have multiple measurements reports overlapping in the same reporting time. For example, the UE can have an overlapping measurement reports related to different cell IDs or different TRP IDs. Another example is the UE having multiple measurement reports corresponding to different type of PRS (semi-persistent or aperiodic PRS). Therefore, the UE should prioritize among the overlapping measurements reports. The UE can associate a priority to a measurement report.

As priorities may be assigned to different types of PRS, similar priorities should be assigned to corresponding measurement reports. Another option for the UE to determine the priority of a measurement report could be based on the content of the report. For example, the UE assigns high priority to the measurement report which include report of on-demand PRS measurements.

Proposal 3: Support priorities related to measurement reports and priority depends on types of PRS (e.g., on-demand PRS) that is associated with the report.

Fast measurement gap configuration

In RAN1#105e, the following agreement was made

Agreement: RAN1 to further study at least the following aspects for MG enhancement with regards to MG requesting and configuration/activation/triggering for the purpose of latency reduction for positioning: - Preconfiguration of multiple MGs - Triggering/activation of MG(s) with lower layer signalings (DCI or DL MAC CE) - Request of MG(s) with lower layer signaling by the UE to the gNB - Request/determination of MG(s) by LMF indication to the gNB/UE - Note: The combination of the above items is possible.

While the aforementioned measurement gap-less configuration allows flexible transmission controlled by associated priority, measurement gap configuration allows controlled operation of positioning, dedicating interference-free measurement time for the UE. Such configuration may be useful in IIoT scenarios where precise positioning is required. Measurement gaps enables the UE to monitor and process PRS measurements without being required to receive any downlink transmission or transmit uplink transmission. In Rel-16, MG are requested by the UE semi-statically using RRC signalling.

With the support of on-demand PRS, the PRS configuration, including the transmission time, may not be known beforehand and the measurement gap required by the UE will depend on the selected PRS configuration. Only RRC signalling to request the MG can have an impact on the latency performance and consequently can lead to not meet the requirements. Lower layer signalling can be used to request a measurement gap and activate it. For example, the UE can request a specific preconfigured measurement gap pattern using a MAC CE. To reduce the MAC CE overhead, the MAC CE can point to one of a preconfigured MG patterns to the UE.

Proposal 4: Support fast activation of preconfigured measurement gap via MAC-CE

Preconfigured measurement gap patterns can be defined by different measurement gap repetition period, length, offset. Using a lower layer signaling, the UE can indicate desired measurement gap parameters to the gNB.

Proposal 5 : Preconfigured measurement gap patterns are characterized by repetition period, length and offset

Priorities among measurement gaps

As mentioned above, different types of PRS (e.g., on-demand) may be associated with different priority level, depending on urgency of PRS. Thus, measurement gaps configured to allow the UE to make measurements on the PRS may be associated with the priority level of each type of PRS. In such a case, measurement gap configuration may overlap with reception of DL channels and the UE may determine to receive PRS or DL channel transmission depending on the associated priority. Priority associated with measurement gap allows flexibility in scheduling for the gNB.

Proposal 6: Support priority indication for the measurement gap associated with PRS.

DL-PRS muting:

According to R16, the UE is not expected to receive DL-PRS and other data simultaneously. DL-PRS can be deprioritized for another downlink reception (e.g., URLLC like data). Therefore, the number of DL-PRS measurement occasions for positioning measurement may be reduced especially for the DL-PRS muting configuration with long periodicity. Consequently, it may affect the positioning measurement of the UE and the availability of the measurement report. To address this issue, additional DL-PRS measurement occasions can be provided to supplement DL-PRS measurement if the DL-PRS is muted and it is dropped due to collision with other channels. This objective can be achieved by a configuration of multiple muting patterns. The UE is then allowed to switch between different patterns if a DL-PRS resource is deprioritized/muted.

Proposal 7: Support dynamic muting of PRS.

# Conclusion

The following observation and proposals are made in this contribution:

Observation 1: On-demand PRS should be supported flexibly with priorities associated with them.

Proposal 1: Measurements and processing of PRS without measurement gap should be supported with at least Option 1 (The PRS is from the serving cell and UE measurement is inside the active DL BWP)

Proposal 2: In the presence of no measurement gap, the UE is expected to receive PRS with higher priority, associated with on-demand PRS, over other channels if the PRS overlaps with other channels in the time domain.

Proposal 3: Support priorities related to measurement reports and priority depends on types of PRS (e.g., on-demand PRS) that is associated with the report.

Proposal 4: Support fast activation of preconfigured measurement gap via MAC-CE

Proposal 5 : Preconfigured measurement gap patterns are characterized by repetition period, length and offset

Proposal 6: Support priority indication for the measurement gap associated with PRS.

Proposal 7: Support dynamic muting of PRS.

# Reference

- 3GPP RAN1 Chairman’s notes, RAN1#105e, May. 2021.

- 3GPP, “Study on NR Positioning Enhancements, ” TR 38.857, v.1.1.0, March 2021.