3GPP TSG RAN WG1 #106-e		                                    		  R1-2107654

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.8.3

Source:	InterDigital, Inc.

Title:	Type A PUSCH repetitions for Msg3

Document for:	Discussion and Decision

# Introduction

This contribution addresses the following issues for Type A PUSCH repetitions for Msg3:

- Determination of RSRP threshold for requesting msg3 repetitions

- Indication of the number of repetitions of Msg3 initial transmission

- Indication of the number of repetitions of Msg3 retransmission

# RSRP threshold

RAN1 made the following agreement at RAN1#105-e:

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold. - FFS the determination of the RSRP threshold.

For the determination of the RSRP threshold, options discussed at RAN1#105-e include reuse of legacy RSRP threshold, new (single) SIB1 configured RSRP threshold, or a set of UE power-class dependent thresholds.

Reusing legacy RSRP threshold would greatly reduce flexibility of the feature since the legacy threshold is not used for the same purpose. Utilizing different thresholds for different UE power classes could enable fewer repetitions for UEs capable of higher transmission power. However, the benefit seems limited considering the additional complexity of configuring the multiple thresholds and it would also not be consistent with design of legacy RACH (without msg3 repetition). For these reasons, a new SIB1 configured RSRP threshold is preferred.

Proposal 1: UE requests msg3 PUSCH repetition when RSRP of the downlink pathloss reference is lower than a new SIB1 configured RSRP threshold.

# Indication of the number of repetitions for msg3 initial transmission

RAN1 made the following working assumption at RAN1#105-e:

Working assumption: - Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission. - TDRA information field with introducing a new TDRA table including the repetition factors. - MCS information field - TPC information field - CSI request information field - FDRA information field - The total size of RAR UL grant does not change. - Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not. - FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc.

Utilizing the TDRA information field is a straightforward option reusing the same mechanism as PUSCH repetition. In RAN1#105-e, several companies expressed concerns about loss of scheduling flexibility since the number of codepoints available for indicating a time allocation for a given number of repetitions would be reduced. However, other alternatives also suffer from either loss of scheduling flexibility (e.g. MCS, FDRA) or possible forward-compatibility issue (CSI request) as the total size of the RAR UL grant is constrained not to change. Utilizing a subset of bits TPC information field can also be considered an attractive option because the penalty would only be loss of granularity of the TPC command for the msg3 transmission which could be corrected in subsequent transmissions.

Proposal 2: Downselect from the following fields in RAR UL grant for indication of the number of msg3 initial transmission:

- TDRA information field

- TPC information field

# Indication of the number of repetitions for msg3 retransmissions

RAN1 made the following agreement at RAN1#105-e:

Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options. - Option 1: Use the same mechanism as supported for Msg3 initial transmission. - Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

If RAN1 agrees to using TDRA information field for the initial transmission, Option 1 is preferred because the same TDRA table can be applicable to both initial transmission and retransmission and there is no need to specify use the reserved bits. If RAN1 agrees on using another field such as MCS or TPC, with Option 1 the network would lose some flexibility for adapting the link for the retransmission and Option 2 may become relatively more attractive.

Proposal 3: If RAN1 agrees on using TDRA field of RAR for initial transmission of msg3, TDRA field of DCI format 0_0 indicates repetitions for retransmission of msg3.

# Conclusion.

This contribution proposes the following:

Proposal 1: UE requests msg3 PUSCH repetition when RSRP of the downlink pathloss reference is lower than a new SIB1 configured RSRP threshold.

Proposal 2: Downselect from the following fields in RAR UL grant for indication of the number of msg3 initial transmission:

- TDRA information field

- TPC information field

Proposal 3: If RAN1 agrees on using TDRA field of RAR for initial transmission of msg3, TDRA field of DCI format 0_0 indicates repetitions for retransmission of msg3.

# References

[1] RAN1 Chairman’s notes, RAN1#104-bis-e, Apr. 2021.

# Appendix

Agreements from RAN1#105-e

R1-2106007

Update on 5/22:

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold.

- FFS the determination of the RSRP threshold.

Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options.

- Option 1: Use the same mechanism as supported for Msg3 initial transmission.

- Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI.

Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission.

- The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214.

- The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214.

- FFS: The RV cycling for Msg3 is based on transmission occasions on available slot.

Agreement:

- For requesting Msg3 PUSCH repetition, support the following:

  - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs.

    - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE.

    - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH).

  - FFS whether or not to additionally support one (& only one) more option:

    - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs

    - E.g., Option 3: Use separate RO, which include

      - the separate RO configured by a separate RACH configuration index from legacy UE, and

      - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated.

Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon.

- A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols.

  - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition.

  - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

Working assumption:

- Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission

  - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission.

    - TDRA information field with introducing a new TDRA table including the repetition factors.

    - MCS information field

    - TPC information field

    - CSI request information field

    - FDRA information field

- The total size of RAR UL grant does not change.

- Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not.

- FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc.

Conclusion:

- Companies are encouraged to perform additional evaluations regarding intra-slot frequency hopping for Msg 3 with repetition. Aim to conclude whether or not to support this feature in RAN1#106-e (note: if supported, the intention is to not configure intra- and inter-slot frequency hopping simultaneously)

Final summary in R1-2106247

Agreements from RAN1#104b-e

R1-2103829

Agreement: For Msg3 PUSCH repetition,  support the following modified Option 2-1.

- Option 2-1: For UE requested triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

- A UE can request trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.).

- Whether a UE would request trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- If Msg3 PUSCH repetition is requested triggered by UE, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission.

- FFS the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure as usual

- FFS details if any.

Update on 4/16

Agreements: For the determination of RV for Msg3 PUSCH repetition,

- RV of the first repetition is determined in the same way as legacy.

  - Use RV 0 for the first repetition of Msg3 PUSCH initial transmission.

  - Use a dynamically indicated RV id via DCI 0_0 with CRC scrambled by TC-RNTI for the first repetition of Msg3 PUSCH re-transmission.

- FFS determination of the RV sequence.

Agreements: For indication of the number of repetitions for Msg3 initial transmission, Option 1 (i.e., using UL grant scheduling Msg3) is adopted.

- FFS additionally using MAC RAR for indication.

Agreements: For indication of the number of repetitions for Msg3 re-transmission, Option 1 (i.e., using DCI format 0_0 with CRC scrambled by TC-RNTI) is adopted.

Working assumption: The number of repetitions is counted on the basis of available slots for Type A PUSCH repetitions for Msg3.

- FFS: the determination of available slots.

Agreements from RAN1#104-e

Agreements:

- For indication of the number of repetitions for Msg3 initial transmission, down-select one option from the options below.

  - Option1: UL grant scheduling Msg3.

    - FFS details.

    - FFS fallbackRAR UL grant.

    - Note: Optimization specific for fallbackRAR UL grant in 2-step RACH is not considered in Rel-17 CovEnh WI, if supported.

  - Option2: DCI format 1_0 with CRC scrambled by RA-RNTI

    - FFS details.

  - Option3: SIB1 only

- Any modifications of RAR UL grant or DCI format 1_0 with CRC scrambled by RA-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the RAR or DCI format 1_0 with CRC scrambled by RA-RNTI respectively

Agreements:

- For indication of the number of repetitions for Msg3 re-transmission, down-select one option from the options below.

Option1: DCI format 0_0 with CRC scrambled by TC-RNTI.

FFS details.

Any modifications of DCI format 0_0 with CRC scrambled by TC-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the DCI format 0_0 with CRC scrambled by TC-RNTI.

Option2: Can be determined based on the repetition number  for  Msg3 initial transmission

Agreements:

Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission.

FFS details, e.g., signaling etc.

Agreements:

For Msg3 PUSCH repetition,  the following options are considered, aiming for down-selection in RAN1#104b-e:

- Option 1-1: For gNB scheduled Msg3 PUSCH repetition without UE request,

- A UE indicates to support of Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions.

- For a UE supporting Msg3 PUSCH repetition, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

- FFS details if any.

- Option 1-2: For gNB scheduled Msg3 PUSCH repetition without UE request,

- gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

- For UE does not support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH without repetition

- For UE does support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH with repetition as indicated by gNB and UE uses, e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways)

- Note: e.g., this can be for differentiation between UEs not supporting Msg3 PUSCH repetition and Rel-17 CE UEs supporting Msg3 PUSCH repetition or between RACH procedure with Msg3 PUSCH repetition and Msg3 PUSCH without repetition, etc.

- gNB blindly decodes Msg3 PUSCH with two different assumptions, w/ and w/o repetition.

- FFS details if any.

- Option 2-1: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

- A UE can trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions.

- Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- If Msg3 PUSCH repetition is triggered by UE, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission.

- FFS details if any.

- Option 2-2: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

  - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

  - If Msg3 PUSCH repetition is scheduled, UE transmits Msg3 PUSCH with or without repetition. If UE transmits Msg3 PUSCH repetition, the number of repetition follows the indication of gNB and UE uses e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways)

- Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

  - FFS details if any.

- Other options are not precluded.