3GPP TSG RAN WG1 #106-e	R1-2107657

e-Meeting, August 16th – August 27th, 2021

Agenda item:      8.14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Performance results in indoor hotspot and dense urban deployments of CG and VR/AR applications

Document for:     Discussion

# Introduction

RAN#91 approved a revised SID on XR Evaluations for NR [1]. The XR SI is now ongoing with focus on related system-level performance evaluation to assess how well the currently standardized NR system can support such services. In this document, we present our preliminary results of XR and CG services in 5G NR. Two deployment scenarios are considered for evaluating the performance of XR and CG services, as agreed in [2]: (1) Indoor Hotspot (InH) and (2) Dense Urban (UMa). In these two deployments, we evaluate DL and UL system capacity of Cloud Gaming (CG) and Virtual Reality/Augmented Reality (VR/AR) applications according to the DL traffic models discussed in [3].

# Statistical Models and KPI for XR Traffic

In this section, we discuss the parameters we used in our performance evaluation for the statistical model for XR traffic. The parameters have been set according to the discussion in [2,4,5]. Table 1 illustrates the parameters of the traffic models used for generating DL and UL traffic. Table 2 and Table 3 illustrate the traffic models of XR applications as well as their QoS constraints used to evaluate user satisfaction and system capacity for DL and UL, respectively. Note that the maximum packet size of the transport layer is fixed to the maximum frame size to avoid segmentation at transport layer.

Table 1 – Parameters of DL and UL traffic models for XR applications.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Type of content | Video (DL) | Video (DL) | Video (UL) | Pose (UL) |
| Traffic model | Single stream for dual-eye buffer | Single stream for dual-eye buffer | Single stream for dual-eye buffer | Single stream |
| Number of streams | 1 | 1 | 1 | 1 |
| Packet size distribution | Truncated Gaussian |  |  | Constant |
| Bitrate | 30 Mbps | 45 Mbps | 10 Mbps | 200 kbps |
| Packet size (avg) | 62500 Bytes | 93750 Bytes | 20833 Bytes | 100 Bytes |
| Packet size (std) | 6563 Bytes (10.5% of mean value) | 9844 Bytes (10.5% of mean value) | 2188 Bytes (10.5% of mean value) | 0 |
| Packet size (max) | 93750 Bytes (150% of mean value) | 140625 Bytes (150% of mean value) | 31250 Bytes (150% of mean value) | 100 Bytes |
| Packet size (min) | 31250 Bytes (50% of mean value) | 46875 Bytes (50% of mean value) | 10417 Bytes (50% of mean value) | 100 Bytes |
| Transport protocol | UDP |  |  |  |
| Maximum transport packet size | 93750 Bytes 1 packet = 1 PDU | 140625Bytes 1 packet = 1 PDU | 31250 Bytes 1 packet = 1 PDU | 100 Bytes 1 packet = 1 PDU |
| Packet Rate | 60 fps | 60 fps | 60 fps | 250 pps |
| Inter-arrival time distribution | Truncated Gaussian |  |  | Periodic |
| Interarrival time (avg) | 16.67 ms |  |  | 4 ms |
| Interarrival time (std) | 2 ms |  |  | 0 |
| Interarrival time (max) | 20.67 ms |  |  | 4 ms |
| Interarrival time (min) | 12.67 ms |  |  | 4 ms |

Table 2 – Traffic models and QoS constraints used to evaluate XR applications in DL direction.

| Application | CG | VR | AR |
| --- | --- | --- | --- |
| Traffic model | Video single-stream | Video single-stream | Video single-stream |
| Bitrate | 30 Mbps | {30, 45} Mbps | {30, 45} Mbps |
| Packet rate | 60 fps | 60 fps | 60 fps |
| Packet Delay Budget (PDB) | 15 ms | 10 ms | 10 ms |
| Number of streams | 1 | 1 | 1 |

Table 3 – Traffic models and QoS constraints used to evaluate XR applications in UL direction.

| Application | CG | VR | AR |
| --- | --- | --- | --- |
| Traffic model | Pose single-stream | Pose single-stream | Video single-stream |
| Bitrate | 200 kbps | 200 kbps | 10 Mbps |
| Packet rate | 250 pps | 250 pps | 60 fps |
| Packet Delay Budget (PDB) | 15 ms | 10 ms | 30 ms |
| Number of streams | 1 | 1 | 1 |

As agreed in [2,4,5], the system capacity is measured in terms on “number of satisfied UEs”. More specifically, a UE is marked as “satisfied UE” if more than X% of packets are successfully transmitted over the air interface within the PDB. The system capacity is defined as the maximum number of UEs per cell with at least Y% of UEs marked as satisfied UEs. In our evaluation, we adopt X=99% as the percentage of frames to be successfully received within the PDB to identify satisfied UEs, while Y=90% as the percentage of satisfied UEs to measure the system capacity.

# Capacity Evaluation of Indoor Hotspot

## Simulation settings

The carrier frequency is set to 4 GHz and 30 GHz for FR1 and FR2, respectively. System bandwidth is assumed to be 100 MHz for FR1 and for FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30 kHz and 120 kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set as follows: 31dBm with 100MHz (24dBm per 20MHz) in FR1, and 24dBm with (23dBm per 80MHz) in FR2. Table 4 lists the main parameters of the Indoor Hotspot deployment that are considered in this study.

Table 4 – Main parameters for Indoor Hotspot (InH) deployment

| Parameter | Value |  |
| --- | --- | --- |
| Layout | - 120m x 50m, Single layer (indoor floor, open office) - 12 cells/TRPs - ISD: 20m |  |
| Channel model | InH |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |
| System bandwidth | FR1: 100 MHz | FR2: 100 MHz |
| BS height | 3 m |  |
| UE height | 1.5 m |  |
| BS noise figure | FR1: 5 dB | FR2: 7 dB |
| UE noise figure | FR1: 9 dB | FR2: 13 dB |
| BS receiver | LMMSE-IRC |  |
| UE receiver | LMMSE-IRC |  |
| Channel estimation | Realistic (with ideal CSI) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS Tx power | FR1: 31dBm (24dBm per 20MHz) | FR2: 24 dBm (23dBm per 80MHz) |
| UE Tx max power | 23 dBm |  |
| TDD Frame structure | DDDSU |  |
| Cell Selection | RSRP Slow Fading |  |
| BS antenna configuration | - Pattern: Ceiling-mount antenna radiation pattern - Gain: 5 dBi - Downtilt: 90° |  |
|  | Configuration in FR1: - 32 TxRU - (M, N, P, Mg, Ng) = (4,4,2,1,1) - (dH, dV) = (0.5, 0.5)λ - (Mp, Np) = (4,4) | Configuration in FR2: - 2 TxRU - (M, N, P, Mg, Ng) = (16, 8, 2,1,1) - (dH, dV) = (0.5, 0.5) λ - (Mp, Np) = (1,1) Grid of Beams: - Azimuth angles (degrees): {90, 90, 90, 112.5, 112.5, 112.5, 67.5, 67.5, 67.5, 140, 140, 140, 40, 40, 40} - Elevation angles (degrees): {-30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30} |
| UE antenna configuration | - Pattern : Omni-directional, - Gain : 0 dBi, - Configuration : 2T/4R (M, N, P, Mg, Ng) = (1,2,2,1,1) (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,2) | - Pattern: UE radiation pattern model 1 (TR 38.901) - Gain: 5 dBi - Configuration (Option 1): (M, N, P) = (1, 4, 2), (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,1) 3 panels (left, right, top) |
| Scheduler | SU-MIMO, Proportional Fairness |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |
| PDCCH overhead | Modelled |  |
| Target BLER | 10% for first transmission |  |
| Max HARQ transmission | 3 |  |
| HARQ scheme | Chase combining |  |

## Simulation results

In the following, we illustrate the results for capacity evaluation obtained with our system level simulator in Indoor Hotspot deployment scenario presented in the previous section. In our evaluation, we independently evaluate the DL and UL capacity.  We first illustrate the DL system capacity for “Video Single-Stream” traffic model with parameters for CG (Section 3.2.1) and VR/AR (Section 3.2.2) applications. Then, we illustrate the UL system capacity for “Pose Single Stream” traffic model for CG/VR (Section 3.2.3). We selected the even load random placement of UEs in the simulation area to be sure that all cells get the same number of connected UEs. Specifically, UEs are randomly dropped in the simulated area one by one according to a spatial uniform distribution. This process is repeated for all UEs until each cell served exactly the same number of UEs. Note that cell selection for a UE is based on RSRP.

### Cloud Gaming (Video Single-Stream): DL Capacity

Figure 1 shows the ratio of satisfied UEs for CG computed with 99% of frames received within the PDB of 15ms as a function of the average cell load, which is measured in number of UEs per cell. Results correspond to a system bandwidth of 100MHz for both FR1 and FR2, and a for data rate of 30Mbps. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we conclude that in the Indoor Hotspot deployment we can support up to 5 UEs per cell (i.e., 60 UEs in the network) in FR1, with the capacity showing a significant increase in FR2 with more than 10 satisfied UEs per cell (i.e., more than 120 UEs in the network). The higher capacity for FR2 is mainly due to the use of the gNB GoB and the multi panel UE.

| (a) CG in FR1 at 30Mbps with X=99%. | (b) CG in FR2 at 30Mbps with X=99%. |
| --- | --- |

Figure 1 – DL capacity evaluation of CG traffic in Indoor Hotspot in FR1 and FR2 with X=99% of frames received within PDB, 100MHz system bandwidth and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 1: When we consider CG traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 5.

Table 5 – System capacity in DL for CG (PDB=15ms) at 30Mbps [#UEs/cell] in an InH deployment

|  | 30Mbps |
| --- | --- |
| FR1 | 5 |
| FR2 | >10 |

### Virtual/Augmented Reality (Video Single-Stream): DL Capacity

In this section, we illustrate the results of the system capacity obtained for VR/AR applications with PDB=10ms. Figure 2 shows the ratio of satisfied UEs computed with 99% of frames received within the PDB as a function of the average cell load, which is measured in number of UEs; results correspond to a system bandwidth of 100MHz for both FR1 and FR2, and data rates of 30Mbps and 45Mbps. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we can conclude that in the Indoor Hotspot deployment in FR1 we can support 5 UEs per cell running VR/AR applications at 30Mbps (i.e., 60 UEs in the network), with such capacity increasing to at least 10 UEs per cell (i.e., at least 120 UEs in the network) for the same data rate when using FR2. For a 45Mbps data rate, in FR1 we can support 3 UEs per cell (i.e., 36 UEs in the network) whereas in FR2 up to 5 UEs per cell (i.e., 60 UEs in the network) can be supported.

| (a) VR/AR in FR1 at 30Mbps with X=99%. | (b) VR/AR in FR2 at 30Mbps with X=99%. |
| --- | --- |
| (c) VR/AR in FR1 at 45Mbps with X=99%. | (d) VR/AR in FR2 at 45Mbps with X=99%. |

Figure 2 –  DL capacity evaluation of VR/AR traffic in Indoor Hotspot in FR1 and FR2 with X=99% of frames received within PDB, 100MHz system bandwidth and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 2: When we consider VR/AR traffic model at 30Mbps and 45Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 6.

Table 6 – System capacity in DL for VR/AR (PDB=10ms) at 30Mbps [#UEs/cell] in an InH deployment

|  | Rate: 30Mbps | Rate: 45Mbps |
| --- | --- | --- |
| FR1 | 5 | 3 |
| FR2 | ≥10 | 6 |

### Cloud Gaming and Virtual Reality (Pose Single-Stream): UL Capacity

In this section, we illustrate the results of the UL system capacity obtained considering the “Pose Single-Stream” traffic model for CG and VR applications at 200kbps, with PDB=10ms. Figure 3 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of packets containing pose information received within the PDB for an FR1 configurations as a function of the average cell load, which is measured in number of satisfied UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that more than 10 UEs per cell  running XR services and generating pose information can be supported in UL in FR1.

Figure 3 – UL capacity evaluation of CG and VR traffic (Pose Single-Stream) in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 3: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Indoor Hotspot deployment, we can support a number of UEs per cell indicated in Table 7.

Table 7 – System capacity in UL for CG and VR (Pose Single-Stream) at 200kbps [#UEs/cell]

|  | Rate: 200 Kbps |
| --- | --- |
| FR1 | > 10 |

# Power Saving Evaluation in Indoor Hotspot

## Simulation settings

The same configuration of the Indoor Hotspot deployment described in Section 3.1 has been considered for the evaluation of UE power consumption (please refer to Table 4 for a detailed description of main simulation parameters). Only DL traffic for CG and VR/AR applications at 30Mbps has been considered in this analysis (baseline data rate for both applications); both FR1 and FR2 are considered. Furthermore, the following three CDRX configurations have been considered to evaluate the trade-off between power saving and achievable capacity:

- CDRX 1: (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2)

- CDRX 2: (DRX long cycle, inactivity timer value, On duration timer value)=(8, 4, 4)

- CDRX 3: (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8)

## Simulation results

In the following, we illustrate the measured power saving and capacity in Indoor Hotspot deployment when CDRX is enabled. The three proposed configurations are compared to the results with no DRX shown in Section 3 (labeled “Always ON” in this section) in terms of average power saving gain and capacity loss.

### Cloud Gaming: DL Capacity vs Power Consumption in FR1

In this section, we present the system capacity results obtained for CG application at 30Mbps in FR1, with PDB=15ms.  Figure 4 illustrates the absolute values of capacity and mean power consumption (across all UEs in the system) considering UEs always ON (no DRX is used) and the three CDRX configurations described above. Table 8 reports the average power gain and system capacity loss of the three considered CDRX configurations with respect to Always ON policy.

| - Capacity - Always ON. | (b) UE Mean Power - Always ON. |
| --- | --- |
| - Capacity - DRX (4,2,2). | (d) UE Mean Power - DRX (4,2,2). |
| (e) Capacity - DRX (8,4,4). | (f) UE Mean Power - DRX (8,4,4). |
| (g) Capacity - DRX (16,8,8). | (h) UE Mean Power - DRX (16,8,8). |

Figure 4 – Capacity and UE mean power consumption in Indoor Hotspot in FR1 for CG at 30Mbps with X=99%           (DL only).

Table 8 – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR1}

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Always ON | - | - | - | - | 4.95 / 5 |
| (4, 2, 2) | 27 % | 28 % | 28 % | 22 % | 4.85 / 5 |
| (8, 4, 4) | 24 % | 26 % | 24 % | 18 % | 4.8 / 5 |
| (16, 8, 8) | 15 % | 17 % | 16 % | 11 % | 4.7 / 5 |

Observation 4: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 8.

Observation 5: CDRX reduces UE mean power consumption by up to 27%, and although it seems not to affect the DL system capacity much for CG applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 6: The CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for CG applications in Indoor Hotspot deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

### Cloud Gaming: DL Capacity vs Power Consumption in FR2

In this section, we present the system capacity results obtained for CG application at 30Mbps in FR2, with PDB=15ms.  Figure 5 illustrates the absolute values of capacity and mean power consumption (across all UEs in the system) considering UEs always ON (no DRX is used) and the three CDRX configurations described above. Table 9 reports the average power gain and system capacity loss of the three considered CDRX configurations with respect to Always ON policy.

| - Capacity - Always ON. | (b) UE Mean Power - Always ON. |
| --- | --- |
| - Capacity - DRX (4,2,2). | (d) UE Mean Power - DRX (4,2,2). |
| (e) Capacity - DRX (8,4,4). | (f) UE Mean Power - DRX (8,4,4). |
| (g) Capacity - DRX (16,8,8). | (h) UE Mean Power - DRX (16,8,8). |

Figure 5 – Capacity and UE mean power consumption in Indoor Hotspot in FR2 for CG at 30Mbps with X=99%           (DL only).

Table 9  – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR2}

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Always ON | - | - | - | - | 10 / 10 |
| (4, 2, 2) | 26 % | 26 % | 26 % | 25 % | 10 / 10 |
| (8, 4, 4) | 22 % | 23 % | 22 % | 21 % | 10 / 10 |
| (16, 8, 8) | 13 % | 15 % | 13 % | 12 % | 9.783 / 10 |

Observation 7: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR2 are indicated in Table 9.

Observation 8: CDRX reduces UE mean power consumption by up to 26%, and although it seems not to affect the DL system capacity much for CG applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly lower for the case (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8).

Observation 9: The CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for CG applications in Indoor Hotspot deployment in FR2 is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

### Virtual/Augmented Reality: DL Capacity vs Power Consumption in FR1

In this section, we illustrate the results for the system capacity and power consumption obtained for VR/AR applications at 30Mbps and 45Mbps in FR1, with PDB=10ms. For 30Mbps, Figure 6 shows the absolute values of capacity and mean power consumption (across all UEs in the system) considering UEs always ON (no DRX) and the three CDRX configurations described above. Also, for 30Mbps, Table 7 reports the average power gain and system capacity loss of the three considered CDRX configurations with respect to Always ON policy.

| - Capacity - Always ON. | (b) UE Mean Power - Always ON. |
| --- | --- |
| (c) Capacity - DRX (4,2,2). | (d) UE Mean Power - DRX (4,2,2). |
| (e) Capacity - DRX (8,4,4). | (f) UE Mean Power - DRX (8,4,4). |
| (g) Capacity - DRX (16,8,8). | (h) UE Mean Power - DRX (16,8,8). |

Figure 6 – Capacity and UE mean power consumption in Indoor Hotspot in FR1 for VR/AR at 30Mbps with X=99% (DL-only).

Table 10 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 30Mbps, FR1}

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Always ON | - | - | - | - | 4.71 / 5 |
| (4, 2, 2) | 27 % | 28 % | 28 % | 22 % | 4.46 / 5 |
| (8, 4, 4) | 24 % | 26 % | 24 % | 18 % | 4.2 / 5 |
| (16, 8, 8) | 15 % | 17 % | 16 % | 11 % | 0.025 / 5 |

For 45Mbps, Figure 7 shows the absolute values of capacity and mean power consumption (across all UEs in the system) considering UEs always ON (no DRX) and the three CDRX configurations described above. Also, for 45Mbps, Table 11 reports the average power gain and system capacity loss of the three considered CDRX configurations with respect to Always ON policy.

| (a) Capacity - Always ON. | (b) UE Mean Power - Always ON. |
| --- | --- |

| (c) Capacity - DRX (4,2,2). | (d) UE Mean Power - DRX (4,2,2). |
| --- | --- |
| (e) Capacity - DRX (8,4,4). | (f) UE Mean Power - DRX (8,4,4). |
| (g) Capacity - DRX (16,8,8). | (h) UE Mean Power - DRX (16,8,8). |

Figure 7 – Capacity and UE mean power consumption in Indoor Hotspot in FR1 for VR/AR at 45Mbps with X=99% (DL-only).

Table 11 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 45Mbps, FR1}

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Always ON | - | - | - | - | 2.93 / 3 |
| (4, 2, 2) | 25 % | 26 % | 26 % | 23 % | 2.84 / 3 |
| (8, 4, 4) | 21 % | 23 % | 21 % | 18 % | 2.51 / 3 |
| (16, 8, 8) | 13 % | 15 % | 14 % | 9 % | 0 / 3 |

Observation 10: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications in Indoor Hotspot deployment in FR1 are indicated in Table 10 for 30Mbps and in Table 11 for 45Mbps.

Observation 11: CDRX reduces UE mean power consumption  for VR/AR applications by up to 27% for 30Mbps and up to 25% for 45Mbps but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 12: The CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for VR/AR applications considering both 30Mbps and 45Mbps in Indoor Hotspot deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

### Virtual/Augmented Reality: DL Capacity vs Power Consumption in FR2

In this section, we illustrate the results for the system capacity and power consumption obtained for VR/AR applications at 30Mbps and 45Mbps in FR2, with PDB=10ms. For 30Mbps, Figure 8 shows the absolute values of capacity and mean power consumption (across all UEs in the system) considering UEs always ON (no DRX) and the three CDRX configurations described above. Also, for 30Mbps, Table 12 reports the average power gain and system capacity loss of the three considered CDRX configurations with respect to Always ON policy.

| (a) Capacity - Always ON. | - UE Mean Power - Always ON. |
| --- | --- |
| (c) Capacity - DRX (4,2,2). | (d) UE Mean Power - DRX (4,2,2). |

| (e) Capacity - DRX (8,4,4). | (f) UE Mean Power - DRX (8,4,4). |
| --- | --- |
| (g) Capacity - DRX (16,8,8). | (h) UE Mean Power - DRX (16,8,8). |

Figure 8 – Capacity and UE mean power consumption in Indoor Hotspot in FR2 for VR/AR at 30Mbps with X=99% (DL-only).

Table 12 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 30Mbps, FR2}

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Always ON | - | - | - | - | 10 / 10 |
| (4, 2, 2) | 26 % | 26 % | 26 % | 25 % | 9.25 / 10 |
| (8, 4, 4) | 22 % | 23 % | 22 % | 21 % | 2.4 / 10 |
| (16, 8, 8) | 13 % | 15 % | 13 % | 12 % | 0 / 10 |

For 45Mbps, Figure 9 shows the absolute values of capacity and mean power consumption (across all UEs in the system) considering UEs always ON (no DRX) and the three CDRX configurations described above. Also, for 45Mbps, Table 13 reports the average power gain and system capacity loss of the three considered CDRX configurations with respect to Always ON policy.

| (a) Capacity - Always ON. | (b) UE Mean Power - Always ON. |
| --- | --- |
| (c) Capacity - DRX (4,2,2). | (d) UE Mean Power - DRX (4,2,2). |
| (e) Capacity - DRX (8,4,4). | (f) UE Mean Power - DRX (8,4,4). |
| (g) Capacity - DRX (16,8,8). | (h) UE Mean Power - DRX (16,8,8). |

Figure 9 – Capacity and UE mean power consumption in Indoor Hotspot in FR2 for VR/AR at 45Mbps with X=99% (DL-only).

Table 13 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR/AR, DL Only, 45Mbps, FR2}

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Always ON | - | - | - | - | 5.92 / 6 |
| (4, 2, 2) | 24 % | 24 % | 24 % | 23 % | 4.92 / 6 |
| (8, 4, 4) | 20 % | 21 % | 20 % | 19 % | 0.58 / 6 |
| (16, 8, 8) | 11 % | 13 % | 12 % | 10 % | 0 / 6 |

Observation 13: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications in Indoor Hotspot deployment in FR2 are indicated in Table 12  for 30Mbps and in Table 13  for 45Mbps.

Observation 14: CDRX reduces UE mean power consumption  for VR/AR applications by up to 26% for 30Mbps and up to 24% for 45Mbps but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 15: The CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for VR/AR applications considering both 30Mbps and 45Mbps in Indoor Hotspot deployment in FR2  is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

# Capacity Evaluation in Dense Urban

## Simulation settings

The carrier frequency is set to 4 GHz and 30GHz for FR1 and FR2, respectively. System bandwidth has been fixed to 100MHz for both FR1 and FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRB) of 12 subcarriers, each of 30kHz and 120kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmissions before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set to 51 dBm (i.e., 44dBm for 20MHz). Table 8 lists the main parameters of the Dense Urban deployment that are considered in this study.

Table 14– Main parameters for Dense Urban (UMa) deployment

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Layout | 21 cells with wraparound (ISD: 200m) |  |  |
| Channel model | Uma |  |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |  |
| System bandwidth | FR1: Option 1: 100 MHz | FR2: Option 1: 100 MHz |  |
| BS height | 25m |  |  |
| UE height | hUT = 3(nfl – 1) + 1.5 |  |  |
|  | Outdoor: nfl = 1 | Indoor: - nfl ~ uniform(1,Nfl) - Nfl ~ uniform(4,8) |  |
| BS noise figure | FR1: 5 dB |  | FR2: 7 dB |
| UE noise figure | FR1: 9 dB |  | FR2: 13 dB |
| BS receiver | MMSE-IRC |  |  |
| UE receiver | MMSE-IRC |  |  |
| Channel estimation | Realistic (with ideal CSI) |  |  |
| UE speed | 3 km/h |  |  |
| MCS | Up to 256QAM |  |  |
| BS Tx power | 44 dBm per 20 MHz 51 dBm per 100 MHz |  |  |
| UE Tx max power | FR1: 23 dBm |  | FR2: 23 dBm, maximum EIRP 43 dBm |
| TDD Frame structure | Option 1: DDDSU |  |  |
| Mechanical Downtilt | Baseline: 12 degrees |  |  |
| Cell Selection | RSRP Slow Fading |  |  |
| BS antenna configuration | FR1: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2), (dH, dV) = (0.5λ, 0.5λ) | FR2: 2TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1), (dH, dV) = (0.5λ, 0.5λ) Grid of Beams - Azimuth angles: {33.75, 56.25, 78.75, 101.25, 123.75, 146.25, 33.75, 56.25, 78.75, 101.25, 123.75, 146.25} degrees - Elevation angles: {-12.5, -12.5, -12.5, -12.5, -12.5, -12.5, -57.5, -57.5, -57.5, -57.5, -57.5, -57.5} degrees |  |
| UE antenna configuration | FR1: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, -N/Aλ) | FR2: (M, N, P)=(1, 4, 2), 3 panels (left, right, top) (Mp, Np)=(1, 1) |  |
| Power control parameter | Open loop, Alpha = 1, P0 = -106 dBm |  |  |
| Scheduler | SU-MIMO, Proportional Fairness |  |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |  |
| PDCCH overhead | Modelled |  |  |
| Target BLER | 10% for first transmission |  |  |
| Max HARQ transmission | 3 |  |  |
| HARQ scheme | Chase combining |  |  |

## Simulation results

In the following, we illustrate the results for capacity evaluation obtained with our system level simulator in Dense Urban deployment scenario presented in the previous section. In our evaluation, we independently evaluate the DL and UL capacity.  We first illustrate the DL system capacity for “Video Single-Stream” traffic model with parameters for CG (Section 5.2.1) and VR/AR (Section 5.2.2) applications. Then, we illustrate the UL system capacity for “Pose Single Stream” traffic model for CG/VR. We selected the even load random placement of UEs in the simulation area to be sure that all cells get the same number of connected UEs. Specifically, UEs are randomly dropped in the simulated area one by one according to a spatial uniform distribution. This process is repeated for all UEs until each cell served exactly the same number of UEs. Note that cell selection for a UE is based on RSRP.

### Cloud Gaming (Video Single-Stream): DL Capacity

In this section, we illustrate the results on the DL system capacity obtained considering the “Video Single-Stream” traffic model for CG application at 30Mbps, with PDB=15ms. Figure shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that up to 8 UEs per cell running CG services can be supported in DL in both FR1 and FR2.

| (a) CG in FR1 at 30Mbps with X=99%. | (b) CG in FR2 at 30Mbps with X=99%. |
| --- | --- |

Figure 10 – DL Capacity evaluation of CG traffic (Video Single-Stream) for Dense Urban deployment in FR1 and FR2 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 16: When we consider baseline CG traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support the number of UEs per cell indicated in Table 15.

Table 15 – System capacity in DL for CG (PDB=15ms) at 30Mbps [#UEs/cell]

|  | Rate: 30Mbps |
| --- | --- |
| FR1 | 8 |
| FR2 | 8 |

### Virtual/Augmented Reality (Video Single-Stream): DL Capacity

In this section, we illustrate the results on the DL system capacity obtained considering the “Video Single-Stream” traffic model for VR/AR application, namely with PDB=10ms. As in the previous analysis, UE are placed in the simulation area to obtain evenly loaded cells in terms of connected UEs. Figure 11 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. Given the smaller PDB, we can observe that up to 6 UEs per cell running VR/AR services at 30Mbps can be supported in DL in FR1 and FR2. Furthermore, we can observe that up to 4 and 3 UEs per cell running VR/AR services at 45Mbps can be supported in DL in FR1 and FR2, respectively.

| (a) VR/AR in FR1 at 30Mbps with X=99%. | (b) VR/AR in FR2 at 30Mbps with X=99%. |
| --- | --- |
| (a) VR/AR in FR1 at 45Mbps with X=99%. | (b) VR/AR in FR2 at 45Mbps with X=99%. |

Figure 11 – DL capacity evaluation of VR/AR traffic (Video Single-Stream) for Dense Urban deployment in FR1 and FR2 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 17: When we consider baseline VR/AR traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 16.

Table 16 – System capacity in DL for VR/AR (PDB=10ms) at 30Mbps [#UEs/cell]

|  | Rate: 30Mbps | Rate: 45Mbps |
| --- | --- | --- |
| FR1 | 6 | 4 |
| FR2 | 6 | 3 |

### Cloud Gaming and Virtual Reality (Pose Single-Stream): UL Capacity

In this section, we illustrate the results of the UL system capacity obtained considering the “Pose Single-Stream” traffic model for CG and VR applications in FR1 at 200kbps, with PDB=10ms. Figure 12 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of packets containing pose information received within the PDB as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that at least up to 10 UEs per cell running XR services generating Pose information can be supported in UL in FR1.

Figure 12 – UL capacity evaluation of CG and VR traffic (Pose Single-Stream) for Dense Urban deployment in FR1 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 18: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 17.

Table 17 – System capacity in UL for CG and VR (Pose Single-Stream) at 200kbps [#UEs/cell]

|  | Rate: 200Kbps |
| --- | --- |
| FR1 | ≥10 |

# Summary

In this contribution we have discussed performance results in indoor and outdoor deployments and bottlenecks that could be improved to increase 5G NR system capacity and UE power saving. Those naturally require further studies and discussions. In summary, following observations have been made for system capacity evaluation:

The following observations have been made for system capacity evaluation in Indoor Hotspot:

Observation 1: When we consider CG traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 5.

Observation 2: When we consider VR/AR  traffic model at 30Mbps and 45Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 6.

Observation 3: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Indoor Hotspot deployment, we can support a number of UEs per cell indicated in Table 7.

The following observations have been made for UE power saving evaluation in Indoor Hotspot:

Observation 4: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 8.

Observation 5: CDRX reduces UE mean power consumption by up to 27%, and although it seems not to affect the DL system capacity much for CG applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly different in each case.

Observation 6: The CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for CG applications in Indoor Hotspot deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

Observation 7: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR2 are indicated in Table 9.

Observation 8: CDRX reduces UE mean power consumption by up to 26%, and although it seems not to affect the DL system capacity much for CG applications for any of the considered DRX configurations, the fractional number of supported UEs is slightly lower for the case (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8).

Observation 9: The CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for CG applications in Indoor Hotspot deployment in FR2 is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

Observation 10: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications in Indoor Hotspot deployment in FR1 are indicated in Table 10 for 30Mbps and in Table 11 for 45Mbps.

Observation 11: CDRX reduces UE mean power consumption  for VR/AR applications in FR1 by up to 27% for 30Mbps and up to 25% for 45Mbps but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 12: The CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for VR/AR applications considering both 30Mbps and 45Mbps in Indoor Hotspot deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

Observation 13: Power saving gain and DL capacity loss with different CDRX configurations for VR/AR applications in Indoor Hotspot deployment in FR2 are indicated in Table 12  for 30Mbps and in Table 13  for 45Mbps.

Observation 14: CDRX reduces UE mean power consumption  for VR/AR applications in FR2  by up to 26% for 30Mbps and up to 24% for 45Mbps but it also strongly affects the DL system capacity, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 15: The CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for VR/AR applications considering both 30Mbps and 45Mbps in Indoor Hotspot deployment in FR2  is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

The following observations have been made for system capacity evaluation in Dense Urban:

Observation 16: When we consider baseline CG traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support the number of UEs per cell indicated in Table 15.

Observation 17: When we consider baseline VR/AR traffic model for DL (Video Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 16.

Observation 18: When we consider baseline CG and VR traffic model for UL (Pose Single-Stream) for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 17.

All previous observations on system capacity and UE power saving gains are captured and summarized by the following tables:

|  |  | Cloud Gaming (DL) | Virtual/Augmented Reality (DL) |  | Cloud Gaming (UL – Pose) |
| --- | --- | --- | --- | --- | --- |
| Deployment | Frequency Range | Rate: 30Mbps | Rate: 30Mbps | Rate: 45Mbps | Rate: 200Kbps |
| Indoor Hotspot (DL) | FR1 | 5 | 5 | 3 | >10 |
|  | FR2 | ≥10 | ≥10 | 6 | - |
| Dense Urban (DL) | FR1 | 8 | 6 | 4 | >10 |
|  | FR2 | 8 | 6 | 3 | - |

|  |  | Cloud Gaming (30Mbps) |  | Virtual/Augmented Reality (30Mbps) |  | Virtual/Augmented Reality (45Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Deployment | PS Scheme | Mean PS gain | Capacity | Mean PS gain | Capacity | Mean PS gain | Capacity |
| Indoor Hotspot (FR1) | Always ON | - | 4.95 / 5 | - | 4.71 / 5 | - | 2.93 / 3 |
|  | CDRX (4, 2, 2) | 27 % | 4.85 / 5 | 27 % | 4.46 / 5 | 25 % | 2.84 / 3 |
|  | CDRX (8, 4, 4) | 24 % | 4.8 / 5 | 24 % | 4.2 / 5 | 21 % | 2.51 / 3 |
|  | CDRX (16, 8, 8) | 15 % | 4.7 / 5 | 15 % | 0.025 / 5 | 13 % | 0 / 3 |
| Indoor Hotspot (FR2) | Always ON | - | 10 / 10 | - | 10 / 10 | - | 5.92 / 6 |
|  | CDRX (4, 2, 2) | 26 % | 10 / 10 | 26 % | 9.25 / 10 | 24 % | 4.92 / 6 |
|  | CDRX (8, 4, 4) | 22 % | 10 / 10 | 22 % | 2.4 / 10 | 20 % | 0.58 / 6 |
|  | CDRX (16, 8, 8) | 13 % | 9.783 / 10 | 13 % | 0 / 10 | 11 % | 0 / 6 |

# References

- RP-210460 Revised SID on XR Evaluations for NR, RAN#91

- R1-2009812 FL Summary of RAN1 103-e agreements and email discussions on Rel-17 SI on XR evaluations for NR

- RAN1 Chairman’s Notes (v018) 3GPP TSG RAN WG1 Meeting #104-e e-Meeting, January 25th – February 5th, 2021

- R1-2101866 Summary of [104-e-NR-XR-02]_Other Eval Methodology.

- Chair notes RAN1, 104-bis-e Chair_notes_104b-e