3GPP TSG RAN WG1 Meeting #106-e	R1-2107663

E-meeting, August 16 –August 27, 2021

Agenda Item:	8.2.7

Source:	Huawei, HiSilicon

Title:	LLS results for enhancement reference signals

Document for:	Discussion and decision

Introduction

Since RAN1#101-e meeting, the evaluation assumptions of Link Level Simulation have been discussed, and more specific conditions of the detailed parameters was determined in RAN1#104b-e meeting [1]. In RAN1#104b-e meeting, an agreement was made such that the simulation results under not-optional should be discussed first. To this end, simulation results under non-optional assumptions, including the TDL channels, are additionally provided.

Evaluation assumptions and results

Evaluation assumptions

Table 1. Evaluation assumptions of PDSCH/PUSCH simulation

| Assumptions | Value |  |
| --- | --- | --- |
|  | CP-OFDM | DFT-s-OFDM |
| Carrier frequency | 60 GHz |  |
| Duplexing | TDD |  |
| Numerology | [120 kHz, 480 kHz, 960 kHz] |  |
| Channel | TDL-A@10ns, CDL-B@20ns, CDL-B@50ns, CDL-D@20ns, CDL-D@30ns |  |
| Antenna Configuration (Mg,Ng,M,N,P) | TDL: 2*2 CDL: Configuration 2 - (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) - (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) |  |
| PN model | Example 2 phase noise model scaling to 60 GHz in 38.803 |  |
| Channel coding | LDPC code |  |
| frame structure | 2(PDCCH)/1(DMRS)/11(data+PTRS) |  |
| System BW | 120 kHz: 400MHz@4096 FFT size 480 kHz: 1600MHz@4096 FFT size 960 kHz: 1960MHz@2048 FFT size |  |
| Transmission BW | 120 kHz & 480 kHz: 256RB, 128RB, 64RB 960 kHz: 160RB, 128RB, 64RB | 120 kHz & 480 kHz: 256RB, 64RB 960 kHz: 160RB, 64RB |
| TTI | 1 slot |  |
| DMRS | 1/2 in frequency for each port 1 in frequency domain for each port |  |
| PTRS | TD: L=1 FD: - Rel-15 PTRS: K=4; - Block PTRS: (CN,CS) = (1,64),(1,41),(1,33),(1,32),(1,17) | TD among DFT-s-OFDM symbols: L=1 PTRS pattern within DFT-s-OFDM symbol: (NgroupPT-RS, Nsamplegroup)=(8,4), (16,2), (16,4) for 256RB and 160RB (NgroupPT-RS, Nsamplegroup)=(4,4), (8,4) for 64RB |
| Receiver | CPE compensation or ICI compensation | Interpolation among PTRS groups |
| Channel estimation | Realistic |  |

Simulation results

DMRS

For the new settings of the SCS value, the symbol duration becomes shorter, and hence there might be a degradation in accuracy of channel estimation. To address this issue, we propose the following DMRS pattern for a multi-slot PDSCH: more than one or even all DMRS symbols of the given multi-slot PDSCH can be sent at the beginning of the multi-slot PDSCH. The proposed scheme is referred to as the bundling pattern. Note that there will be no extra time-domain overhead of the bundling pattern compared with that of the traditional DMRS pattern. Evaluation results, as shown in Figure 1, are the BLER performance comparison between the traditional DMRS pattern and the bundling pattern. More specifically, for the traditional DMRS pattern, there is one and only one FL DMRS symbol in each slot, and the channel is estimated independently among slots. Meanwhile, for the bundling pattern, 4 DMRS symbols are mapped on the four consecutive symbols of the first slot per 4 slots of 480 kHz, and 8 DMRS symbols are mapped on the eight consecutive symbols of the first slot per 8 slots of 960 kHz. In this case, the channel is estimated based on all the consecutive DMRS symbols per 4 (or 8) slots jointly. In the figures, traditional DMRS pattern are denoted by “Pat(a)+independent”, while bundling DMRS is denoted by “Pat(b)+joint”.

From Figure 1, we can see that the bundling DMRS improves the BLER performance of all the simulation cases, with 0.2~0.6dB gain.

Figure 1. BLER performance comparison of different DMRS patterns in time domain for 480 kHz and 960 kHz in different channel scenarios

Observation 1: Bundling DMRS, such that the DMRS symbols of a multi-slot PDSCH are mapped to consecutive symbols at the beginning of the first PDSCH slot, can improve channel estimation accuracy and in turn system performance with no extra overhead compared to  current DMRS pattern defined in FR2-1, such that DMRS are mapped per slot.

PTRS

In this section, PTRS of CP-OFDM in PDSCH and PTRS of DFT-S OFDM in PUSCH are discussed.

PTRS for CP-OFDM

For ICI compensation, block PTRS with a new sequence is proposed in [3], especially for the case in which power boosting is enabled. As analyzed in [3], power boosting cannot work effectively for the distributed PTRS. This is because that most of the received signal used to estimate ICI coefficients are from unknown data signal whose power are not boosted. However, it can be addressed by applying block PTRS, due to the fact that most of the received signal used to estimate ICI coefficients are from PTRS with power boosted.

To evaluate the performance benefit of block PTRS for ICI estimation, simulations with different settings of MCS, SCS, and scheduled bandwidth are presented. In the evaluations, every K-th (K=4) RB is used as a baseline (i.e., Rel-15 PTRS). In addition, (CN, CS) is applied to block PTRS, where CN denotes the number of blocks per OFDM symbol, and CS denotes the PTRS block size. The detailed design of (CN, CS) can be found in Table 2. Furthermore, the block is mapped to the middle of each sub-band, whose size is determined by the block number. Algorithm 1 (also referred to as Alg-1 for short), the direct algorithm discussed in [2], is applied to Rel-15 PTRS. For block PTRS, both Algorithm 1 and Algorithm 2 (proposed in [3] with lower complexity, and denoted by Alg-2) are evaluated.

Table 2. Length of base sequence and circular sequence for block PTRS

| Block size M | 65 | 65 | 64 | 41 | 41 | 33 | 33 | 17 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ICI order estimated | 7 | 5 | 5 | 5 | 3 | 5 | 3 | 5 | 3 |
| Length of base sequence L | 59 | 61 | 59 | 37 | 37 | 29 | 31 | 13 | 13 |
| Length of head circular sequence q | 3 | 2 | 2 | 2 | 2 | 2 | 1 | 2 | 2 |
| Length of tail circular sequence p | 3 | 2 | 3 | 2 | 2 | 2 | 1 | 2 | 2 |

Power boosting is applied to both PTRS patterns. Therefore, the average power of an OFDM symbol with block PTRS is a little higher than that of Rel-15 PTRS. When taking an assumption that the average power per RE is 1, then the total power of an OFDM symbol with two PTRS patterns are shown in the Table 3. From the table, we can see that the power difference between two PTRS patterns are so small to assume that the average power per OFDM symbol are same.

Table 3. Average power of an OFDM symbol with two PTRS patterns when power boosting value is 3dB

| RB | Average power of an OFDM symbol with Rel-15 PTRS (P1) | Average power of an OFDM symbol with block PTRS (P2) | P2/P1*100% |
| --- | --- | --- | --- |
| 64 | 784 | 785 | 100.13% |
| 128 | 1568 | 1569 | 100.06% |
| 160 | 1960 | 1961 | 100.05% |
| 256 | 3136 | 3137 | 100.03% |

The results are shown from Figure 2 to Figure 10. As can be observed from the results, with power boosting, the demodulation SNR of BLER at 1% and 10% for block PTRS are lower than that of Rel-15 PTRS for almost all cases, especially when the scheduled bandwidth is 64RB:

(1) With 256RB and 120 kHz or 480 kHz, the block PTRS using either algorithm performs slightly better than the distributed PTRS. The gains are about {0.3~0.5dB, 0.2dB, 0.1dB} for TDL-A, CDL-B, and CDL-D, respectively;

(2) With 160RB and 960 kHz, the block PTRS provides about {0.4~0.5dB, 0.5dB, 0.2~0.3dB} gains for TDL-A, CDL-B, and CDL-D, respectively;

(3) With 128RB, the gains of the block PTRS are {0.6~0.8dB, 0.4~0.6dB, 0.2~0.4dB} for TDL-A, CDL-B, and CDL-D, respectively;

(4) With 64RB, the gains of the block PTRS are uniformly larger than those with other settings, especially for high MCS or for TDL-A. Specifically, for TDL-A and MCS22, the gains for 120 kHz, 480 kHz, and 960 kHz are about {3dB, 1.5dB, 1.8dB}, respectively. For MCS26 and CDL channels, the gains are about {0.8~1.6dB, 1~1.8dB} for 480 kHz and 960 kHz, respectively. In addition, for MCS22 and 120 kHz, the gains are also considerable for CDL channels, which are about 0.8~1.5dB.

Figure 2. BLER and SE performance comparison of different PTRS patterns for 120 kHz with 256RB, power boosting and same average power per OFDM symbol

Figure 3. BLER and SE performance comparison of different PTRS patterns for 120 kHz with 128RB, power boosting and same average power per OFDM symbol

Figure 4. BLER and SE performance comparison of different PTRS patterns for 120 kHz with 64RB, power boosting and same average power per OFDM symbol

Figure 5. BLER and SE performance comparison of different PTRS patterns for 480 kHz with 256RB, power boosting and same average power per OFDM symbol

\

Figure 6. BLER and SE performance comparison of different PTRS patterns for 480 kHz with 128RB, power boosting and same average power per OFDM symbol

Figure 7. BLER and SE performance comparison of different PTRS patterns for 480 kHz with 64RB, power boosting and same average power per OFDM symbol

Figure 8. BLER and SE performance comparison of different PTRS patterns for 960 kHz with 160RB, power boosting and same average power per OFDM symbol

Figure 9. BLER and SE performance comparison of different PTRS patterns for 960 kHz with 128RB, power boosting and same average power per OFDM symbol

Figure 10. BLER and SE performance comparison of different PTRS patterns for 960 kHz with 64RB,  power boosting and same average power per OFDM symbol

Observation 2: With power boosting, the BLER and SE performance is significantly improved by block PTRS. The demodulation SNR of BLER at 10% and 1% of the block PTRS are lower than those of Rel-15 PTRS for almost all cases. The improvement is especially significant when the scheduled bandwidth is 64RB.

Since power boosting doesn’t provide any performance benefit for distributed PTRS, additional evaluations with normalized power per OFDM symbol between two PTRS patterns are simulated. In these simulations, average power per RE for both PTRS patterns are “1”. Power boosting is only set for block PTRS, and the power of corresponding data is reduced to keep the average power per OFDM symbol same as that of distributed PTRS. From the results shown in Figure 11 to Figure 13, we can see, block PTRS performs better than distributed PTRS under almost all the cases, except the case with 120 kHz and 256RB.

Figure 11. BLER and SE performance comparison of different PTRS patterns for 120 kHz and normalized power per OFDM symbol

Figure 12. BLER and SE performance comparison of different PTRS patterns for 480 kHz and normalized power per OFDM symbol

Figure 13. BLER and SE performance comparison of different PTRS patterns for 960 kHz and normalized power per OFDM symbol

PTRS for DFT-s-OFDM

The evaluation results of PTRS for DFT-s-OFDM are shown in details in this section. As discussed in [4], the PTRS pattern of DFT-s-OFDM defined in Rel-15 is insufficient when a wide scheduled bandwidth and a high scheduled MCS are applied due to the long interpolation range between two adjacent PTRS groups. As a solution to this problem, a new PTRS pattern is proposed. To clarify the influence of PTRS samples per PTRS group, we set (CN, CS) = (8, 4) as a baseline, set  (CN, CS) = (16, 2) with no extra overhead which has more PTRS groups and fewer PTRS samples per PTRS group, and set (CN, CS) = (16, 4) for the case with more PTRS groups and same PTRS samples per PTRS group. Denote NgroupPT-RS and Nsamplegroup by the CN and the CS, respectively.

Figure 14. BLER performance comparison of different PTRS patterns for 120 kHz

Figure 15. BLER performance comparison of different PTRS patterns for 480 kHz

Figure 16. BLER performance comparison of different PTRS patterns for 960 kHz

From Figure 14 to Figure 16, we can see that the new pattern provides significant gains for high MCS for different SCSs and different channels. More specifically, compared with half of the interpolation range of pattern (CN, CS) = (8, 4), the new pattern (CN, CS) = (16, 2) improves the BLER floor at least one order of magnitude. For medium MCS, the performance of the new pattern is almost the same as that of pattern (CN, CS) = (8, 4). This is because of the worse noise reduction performance with only 2 PTRS samples per PTRS group at medium SNR region, which counteracts the gain introduced by shorter interpolation range.

Observation 3: Simulation results for DFT-s-OFDM with different PTRS patterns show that, if the pattern defined in Rel-15 is used, the BLER performances of 120 kHz, 480 kHz, and 960 kHz for MCS26 reach floors above 10-2 due to longer interpolation range. BLER performance can be improved by using a new pattern with more PTRS groups and the same overhead.

As discussed in [3], due to the Rx timing shift, (at least part of) a PTRS group placed at the tail of the transmitter’s DFT-s-OFDM symbol, may wrap-around to the head of the symbol from the receiver’s perspective. As a result, there might be an unnecessarily increase in Rx complexity, as well as a deterioration in PN compensation performance. The BLER performances of DFT-s-OFDM PTRS with different Rx advance shifts when the DFT-s-OFDM’s tail includes PTRS groups are shown as followed.

The location 0 denotes the existed PTRS location defined in Rel.15. Z% indicates the Rx advance shift percentage of the CP duration. CN and CS denote NgroupPT-RS and Nsamplegroup, respectively.

Figure 17. BLER performance comparison of different Rx advance shifts (0%, 10%, 20%) for 120 kHz

Figure 18. BLER performance comparison of different Rx advance shifts (0%, 10%, 20%) for 480 kHz

Figure 19. BLER performance comparison of different Rx advance shifts (0%, 10%, 20%) for 960 kHz

From Figure 17 to Figure 19, the system performance decreases with the Rx advance shift for different SCS and different Channel, especially with high MCS. As aforementioned, the Rx advance shift leads to PTRS group placed at the tail of the transmitter’s DFT-s-OFDM symbol wrapping-around to the head of the symbol from the receiver’s perspective, and thus deteriorates Phase Noise compensation performance of PTRS.

Observation 4: Simulation results for DFT-s-OFDM with different Rx advance shifts show that, if the PTRS location defined in Rel-15 is used, BLER performance of 120 kHz, 480 kHz and 960 kHz are decreased when Nsamplegroup equals to 4, especially for high MCS, due to a worse PN compensation caused by the Rx timing shift.

To solve the problem, a new PTRS location, such that all PTRS groups are allocated in the middle of each interval, is presented for the case in which Nsamplegroup equals to 4. . By doing so, even if there is an Rx advance shift, the PTRS group will not wrap-around to the head of the symbol and hence influence Phase Noise compensation performance. The BLER performance comparison of different PTRS locations with different Rx advance shifts for 120 kHz, 480 kHz, and 960 kHz are shown as followed. The location 0, again, denotes the existed PTRS location defined in Rel.15, while location 1 denotes the presented PTRS location.

From the results of 120 kHz shown in Figure 17, we can see that [CN, CS] = [4, 4] is insufficient for high MCS such as MCS26. The simulation adopts [CN, CS] = [8, 4] for MCS26 and 120 kHz and [CN, CS] = [4, 4] for other settings.

Figure 20. BLER performance comparison of different PTRS locations with different Rx advance shifts (0%, 10%) for 120 kHz

Figure 21. BLER performance comparison of different PTRS locations with different Rx advance shifts (0%, 10%) for 480 kHz

Figure 22. BLER performance comparison of different PTRS locations with different Rx advance shifts (0%, 10%) for 960 kHz

Figure 23. BLER performance comparison of different PTRS locations with different Rx advance shifts (0%, 10%, 20%) in different Channels for 120 kHz @ MCS22

From Figure 20 to Figure 23, we can see that, for different SCS and different Channel with different PTRS location and different Rx advance shift, the proposed PTRS location outperforms the existed PTRS location with considerable performance gains. For example, the BLER performance of the presented PTRS location for120 kHz, 480 kHz and 960 kHz with MCS26 reaches a floor above 10-2 due to 10% Rx advance shift. On the contrary, the BLER performance of the existed PTRS location with 10% Rx advance shift cannot even reach 10-1.

Observation 5: Simulation results for DFT-s-OFDM with different PTRS locations and different Rx advance shifts show that, the proposed PTRS location, in the middle of each interval, can mitigate the influence on BLER performance caused by Rx advance timing shift.

Conclusions

In this contribution, we have provided link level and system level simulation results for both licensed and unlicensed bands. Based on the discussion, we have the following proposals and observations:

Observation 1: Bundling DMRS, such that the DMRS symbols of a multi-slot PDSCH are mapped to consecutive symbols at the beginning of the first PDSCH slot, can improve channel estimation accuracy and in turn system performance with no extra overhead compared to  current DMRS pattern defined in FR2-1, such that DMRS are mapped per slot.

Observation 2: With power boosting, the BLER and SE performance is significantly improved by block PTRS. The demodulation SNR of BLER at 10% and 1% of the block PTRS are lower than those of Rel-15 PTRS for almost all cases. The improvement is especially significant when the scheduled bandwidth is 64RB.

Observation 3: Simulation results for DFT-s-OFDM with different PTRS patterns show that, if the pattern defined in Rel-15 is used, the BLER performances of 120 kHz, 480 kHz, and 960 kHz for MCS26 reach floors above 10-2 due to longer interpolation range. BLER performance can be improved by using a new pattern with more PTRS groups and the same overhead.

Observation 4: Simulation results for DFT-s-OFDM with different Rx advance shifts show that, if the PTRS location defined in Rel-15 is used, BLER performance of 120 kHz, 480 kHz and 960 kHz are decreased when Nsamplegroup equals to 4, especially for high MCS, due to a worse PN compensation caused by the Rx timing shift.

Observation 5: Simulation results for DFT-s-OFDM with different PTRS locations and different Rx advance shifts show that, the proposed PTRS location, in the middle of each interval, can mitigate the influence on BLER performance caused by Rx advance timing shift.

- References

- Chairman’s Notes RAN1#104b-e

- P. Mathecken, T. Riihonen, S. Werner, and R. Wichman, “Constrained phase noise estimation in OFDM
using scattered pilots without decision feedback,” IEEE Transactions on Signal Processing, vol. 65, no. 9, pp. 2348–2362, May 2017

- R1-2102331. PDSCH/PUSCH enhancements for 52-71GHz spectrum. Huawei, HiSilicon.

- R1-2007604 PHY design in 52.6-71 GHz using NR waveform, Huawei, HiSilicon.

- Appendix A.

Table 4. SINR in dB achieving PDSCH BLER of 10% /1% with different DMRS pattern in time domain

| SCS,RB | Channel | DS | MCS | MCS7 |  | MCS16 |  | MCS22 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DMRS pattern | Per slot | bund | Per slot | bund | Per slot | bund |
| 480kHz, 256RB | CDL-B | 20ns | 10%th | 3.111 | 2.522 | 11.28 | 10.948 | 16.733 | 16.414 |
|  |  |  | 1%th | 4.645 | 4.071 | 12.654 | 12.44 | 18.063 | 17.775 |
|  | CDL-D | 20ns | 10%th | -0.874 | -1.291 | 6.61 | 6.355 | 11.683 | 11.383 |
|  |  |  | 1%th | -0.012 | -0.797 | 7.281 | 7.057 | 12.133 | 11.864 |
|  |  | 30ns | 10%th | -0.873 | -1.291 | 6.611 | 6.356 | 11.676 | 11.383 |
|  |  |  | 1%th | -0.024 | -0.794 | 7.303 | 7.047 | 12.15 | 11.872 |
| 960kHz, 160RB | CDL-B | 20ns | 10%th | 3.991 | 3.166 | 11.606 | 11.188 | 17.566 | 16.998 |
|  |  |  | 1%th | 6.0 | 7.0 | 15 | 14.262 | 20.472 | 19.911 |
|  | CDL-D | 20ns | 10%th | 0.367 | -0.227 | 7.55 | 7.197 | 12.995 | 12.528 |
|  |  |  | 1%th | 1.336 | 0.59 | 8.184 | 7.83 | 13.667 | 13.16 |
|  |  | 30ns | 10%th | 0.379 | -0.189 | 7.548 | 7.201 | 12.992 | 12.528 |
|  |  |  | 1%th | 1.321 | 0.616 | 8.184 | 7.84 | 13.663 | 13.164 |

Table 5. SINR in dB achieving PDSCH BLER of 10% /1% with different PTRS pattern and 256RB with power boosting

| SCS | channel | BLER | MCS22 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | K=4, ICI1 | [1,64] or [1,65], ICI1 | [1,64], ICI2 |
| 120k | TDL-A, 10ns | 0.1 | 24.949 | 24.63 | -- |
|  |  | 0.01 | 28.633 | 28.21 | -- |
|  | CDL-B, 20ns | 0.1 | 18.018 | 17.802 | 17.822 |
|  |  | 0.01 | 19.695 | 19.317 | 19.338 |
|  | CDL-B, 50ns | 0.1 | 17.272 | 17.009 | 17.026 |
|  |  | 0.01 | 18.823 | 18.39 | 18.472 |
|  | CDL-D, 20ns | 0.1 | 12.244 | 12.123 | 12.139 |
|  |  | 0.01 | 13 | 12.867 | 12.946 |
|  | CDL-D, 30ns | 0.1 | 12.246 | 12.116 | 12.133 |
|  |  | 0.01 | 12.991 | 12.892 | 12.978 |
| 480k | TDL-A, 10ns | 0.1 | 23.208 | 22.955 | -- |
|  |  | 0.01 | 25.852 | 25.312 | -- |
|  | CDL-B, 20ns | 0.1 | 16.343 | 16.086 | 16.094 |
|  |  | 0.01 | 17.634 | 17.345 | 17.372 |
|  | CDL-B, 50ns | 0.1 | 16.567 | 16.264 | 16.293 |
|  |  | 0.01 | 17.809 | 17.498 | 17.531 |
|  | CDL-D, 20ns | 0.1 | 11.894 | 11.797 | 11.803 |
|  |  | 0.01 | 12.404 | 12.282 | 12.282 |
|  | CDL-D, 30ns | 0.1 | 11.897 | 11.792 | 11.808 |
|  |  | 0.01 | 12.398 | 12.269 | 12.294 |

Table 6. SINR in dB achieving PDSCH BLER of 10% /1% with different PTRS pattern and 160RB with power boosting

| SCS | channel | BLER | MCS22 |  |  | MCS26 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | K=4, ICI | [1,41], ICI1 | [1,41], ICI2 | K=4, ICI | [1,41], ICI1 | [1,41], ICI2 |
| 960k | TDL-A, 10ns | 0.1 | 22.818 | 22.385 | -- | -- | -- | -- |
|  |  | 0.01 | 25.516 | 24.964 | -- | -- | -- | -- |
|  | CDL-B,20ns | 0.1 | 18.582 | 17.904 | 17.93 | 23.861 | 23.317 | 23.313 |
|  |  | 0.01 | 22 | 21.453 | 21.449 | 28.8 | 27.431 | 27.365 |
|  | CDL-D,20ns | 0.1 | 13.446 | 13.234 | 13.265 | 17.979 | 17.628 | 17.65 |
|  |  | 0.01 | 14.247 | 14.044 | 14.082 | 19.004 | 18.53 | 18.611 |
|  | CDL-D,30ns | 0.1 | 13.446 | 13.234 | 13.269 | 17.977 | 17.633 | 17.653 |
|  |  | 0.01 | 14.242 | 14.042 | 14.072 | 19.014 | 18.506 | 18.619 |

Table 7. SINR in dB achieving PDSCH BLER of 10% /1% with different PTRS pattern and 128RB with power boosting

| SCS | channel | BLER | MCS22 |  |  | MCS26 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | K=4, ICI | [1,33], ICI1 | [1,33], ICI2 | K=4, ICI | [1,33], ICI1 | [1,33], ICI2 |
| 120k | TDL-A, 10ns | 0.1 | 26.066 | 24.956 | -- | -- | -- | -- |
|  |  | 0.01 | 31.549 | 30.573 | -- | -- | -- | -- |
|  | CDL-B, 20ns | 0.1 | 17.402 | 17.037 | 17.139 | -- | -- | -- |
|  |  | 0.01 | 21.253 | 20.912 | 20.897 | -- | -- | -- |
|  | CDL-B, 50ns | 0.1 | 17.189 | 16.689 | 16.849 | -- | -- | -- |
|  |  | 0.01 | 20.598 | 19.857 | 19.929 | -- | -- | -- |
|  | CDL-D, 20ns | 0.1 | 13.024 | 12.744 | 12.83 | -- | -- | -- |
|  |  | 0.01 | 13.84 | 13.558 | 13.669 | -- | -- | -- |
|  | CDL-D, 30ns | 0.1 | 13.023 | 12.741 | 12.827 | -- | -- | -- |
|  |  | 0.01 | 13.84 | 13.544 | 13.653 | -- | -- | -- |
| 480k | TDL-A, 10ns | 0.1 | 23.641 | 23.055 | -- | -- | -- | -- |
|  |  | 0.01 | 27.692 | 26.111 | -- | -- | -- | -- |
|  | CDL-B, 20ns | 0.1 | 16.485 | 16.083 | 16.258 | -- | -- | -- |
|  |  | 0.01 | 19.668 | 19.143 | 19.318 | -- | -- | -- |
|  | CDL-B, 50ns | 0.1 | 16.6 | 16.286 | 16.459 | -- | -- | -- |
|  |  | 0.01 | 19.118 | 18.527 | 18.794 | -- | -- | -- |
|  | CDL-D, 20ns | 0.1 | 12.78 | 12.566 | 12.665 | -- | -- | -- |
|  |  | 0.01 | 13.507 | 13.316 | 13.453 | -- | -- | -- |
|  | CDL-D, 30ns | 0.1 | 12.778 | 12.56 | 12.67 | -- | -- | -- |
|  |  | 0.01 | 13.507 | 13.31 | 13.454 | -- | -- | -- |
| 960k | TDL-A, 10ns | 0.1 | 22.747 | 22.207 | -- |  |  |  |
|  |  | 0.01 | 25.251 | 24.774 | -- |  |  |  |
|  | CDL-B,20ns | 0.1 | 16.313 | 16.19 | 16.209 | 21.793 | 21.21 | 21.297 |
|  |  | 0.01 | 19.238 | 18.931 | 18.939 | 25.414 | 24.54 | 24.54 |
|  | CDL-D,20ns | 0.1 | 12.778 | 12.722 | 12.743 | 17.663 | 17.207 | 17.26 |
|  |  | 0.01 | 13.517 | 13.495 | 13.531 | 18.582 | 18.069 | 18.122 |
|  | CDL-D,30ns | 0.1 | 12.786 | 12.72 | 12.746 | 17.677 | 17.21 | 17.257 |
|  |  | 0.01 | 13.51 | 13.489 | 13.537 | 18.555 | 18.082 | 18.133 |

Table 8. SINR in dB achieving PDSCH BLER of 10% /1% with different PTRS pattern and 64RB with power boosting

| SCS | channel | BLER | MCS22 |  |  | MCS26 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | K=4, ICI | [1,17], ICI1 | [1,17], ICI2 | K=4, ICI | [1,17], ICI1 | [1,17], ICI2 |
| 120k | TDL-A, 10ns | 0.1 | 29.345 | 25.507 | -- | -- | -- | -- |
|  |  | 0.01 | NAN | 33.929 | -- | -- | -- | -- |
|  | CDL-B, 20ns | 0.1 | 17.661 | 16.72 | 16.887 | -- | -- | -- |
|  |  | 0.01 | 23.2 | 20.749 | 21.066 | -- | -- | -- |
|  | CDL-B, 50ns | 0.1 | 18.541 | 16.988 | 17.2 | -- | -- | -- |
|  |  | 0.01 | 22 | 20.113 | 20.676 | -- | -- | -- |
|  | CDL-D, 20ns | 0.1 | 13.896 | 13.144 | 13.312 | -- | -- | -- |
|  |  | 0.01 | 14.837 | 14.067 | 14.286 | -- | -- | -- |
|  | CDL-D, 30ns | 0.1 | 13.903 | 13.146 | 13.313 | -- | -- | -- |
|  |  | 0.01 | 14.843 | 14.076 | 14.256 | -- | -- | -- |
| 480k | TDL-A, 10ns | 0.1 | 24.739 | 23.163 | -- | -- | -- | -- |
|  |  | 0.01 | 29.798 | 28.665 | -- | -- | -- | -- |
|  | CDL-B, 20ns | 0.1 | 16.585 | 16.266 | 16.464 | 23.15 | 21.574 | 21.762 |
|  |  | 0.01 | 19.143 | 18.923 | 19.358 | 28 | 24.685 | 24.982 |
|  | CDL-B, 50ns | 0.1 | 16.555 | 16.382 | 16.629 | 22.993 | 21.65 | 21.866 |
|  |  | 0.01 | 19.386 | 19.296 | 19.829 | 27.071 | 25.06 | 25.155 |
|  | CDL-D, 20ns | 0.1 | 13.16 | 13.001 | 13.139 | 18.282 | 17.476 | 17.651 |
|  |  | 0.01 | 13.927 | 13.814 | 14.054 | 19.419 | 18.595 | 18.898 |
|  | CDL-D, 30ns | 0.1 | 13.156 | 12.999 | 13.134 | 18.278 | 17.482 | 17.653 |
|  |  | 0.01 | 13.938 | 13.831 | 14.045 | 19.438 | 18.594 | 18.916 |
| 960k | TDL-A, 10ns | 0.1 | 23.822 | 22.346 | -- | -- | -- | -- |
|  |  | 0.01 | 29.468 | 26.098 | -- | -- | -- | -- |
|  | CDL-B,20ns | 0.1 | 16.764 | 16.496 | 16.722 | 23.483 | 21.636 | 21.879 |
|  |  | 0.01 | 20.446 | 19.368 | 19.64 | 28.4 | 24.679 | 24.854 |
|  | CDL-D,20ns | 0.1 | 13.216 | 13.041 | 13.197 | 18.527 | 17.51 | 17.678 |
|  |  | 0.01 | 13.986 | 13.857 | 14.212 | 19.676 | 18.506 | 18.74 |
|  | CDL-D,30ns | 0.1 | 13.216 | 13.044 | 13.198 | 18.521 | 17.513 | 17.686 |
|  |  | 0.01 | 13.992 | 13.875 | 14.2 | 19.649 | 18.481 | 18.744 |

Table 8. SINR in dB achieving PDSCH BLER of 10% /1% with different PTRS pattern and normalized power per OFDM symbol and TDL-A channel

| SCS | BLER | 64RB |  | 128RB |  | 160RB |  | 256RB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | K=4 | [1,17] | K=4 | [1,33] | K=4 | [1,41] | K=4 | [1,65] |
| 120k | 0.1 | 29.372 | 25.369 | 26.122 | 24.633 | -- | -- | 24.878 | 25.035 |
|  | 0.01 | NAN | 33.693 | 31.409 | 30.784 | -- | -- | 28.571 | 29.071 |
| 480k | 0.1 | 24.896 | 23.498 | 23.674 | 22.999 | -- | -- | 23.254 | 23.121 |
|  | 0.01 | 30.479 | 29.643 | 27.916 | 26.258 | -- | -- | 25.852 | 25.846 |
| 960k | 0.1 | 23.996 | 22.577 | 22.831 | 22.24 | 22.847 | 22.456 | -- | -- |
|  | 0.01 | 28.222 | 26.991 | 25.357 | 24.903 | 25.556 | 25.145 | -- | -- |

Table 9. SINR in dB achieving PUSCH BLER of 10% /1% for DFT-s-OFDM and different PTRS patterns

| MCS | Channel | 120 kHz,256RB |  |  | 480 kHz,256RB |  |  | 960 kHz,160RB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (CN, CS) = 8*4 | (CN, CS) = 16*2 | (CN, CS) = 16*4 | (CN, CS) = 8*4 | (CN, CS) = 16*2 | (CN, CS) = 16*4 | (CN, CS) = 4*4 | (CN, CS) = 8*4 | (CN, CS) = 16*2 |
| 16 | CDL-B, 20ns | 12.03 /13.38 | 12.10 /13.39 | 11.69 /12.99 | 11.95 /13.21 | 12.07 /12.99 | 11.80 /12.92 | 13.60 /16.97 | 13.37 /16.81 | 13.56 /16.76 |
|  | CDL-D, 20ns | 6.54 /6.94 | 6.61 /6.97 | 6.15 /6.62 | 6.42 /6.82 | 6.61 /7.05 | 6.21 /6.55 | 7.93 /8.77 | 7.58 /8.30 | 7.82 /8.58 |
|  | CDL-D, 30ns | 6.54 /6.95 | 6.61 /9.97 | 6.15 /6.62 | 6.42 /6.83 | 6.62 /7.05 | 6.21 /6.52 | 7.94 /8.78 | 7.59 /8.33 | 7.82 /8.59 |
| 22 | CDL-B, 20ns | 20.46 /NAN | 17.64 /19.46 | 17.78 /19.75 | 19.14 /27.12 | 17.72 /19.16 | 17.82 /19.44 | 20.95 /29.10 | 19.60 /23.34 | 19.36 /22.83 |
|  | CDL-D, 20ns | 14.24 /17.89 | 11.73 /12.21 | 11.84 /12.56 | 12.66 /14.83 | 11.64 /12.07 | 11.63 /12.28 | 14.42 /20.40 | 13.20 /14.16 | 12.83 /13.61 |
|  | CDL-D, 30ns | 14.26 /17.87 | 11.73 /12.22 | 11.83 /12.56 | 12.66 /14.78 | 11.64 /12.06 | 11.63 /12.26 | 14.44 /20.40 | 13.19 /14.16 | 12.83 /13.62 |
| 26 | CDL-B, 20ns | NAN /NAN | 22.46 /24.65 | NAN /NAN | NAN /NAN | 22.22 /24.10 | 29.40 /NAN | NAN /NAN | 29.73 /NAN | 23.89 /27.46 |
|  | CDL-D, 20ns | NAN /NAN | 16.54 /17.24 | 21.38 /NAN | NAN /NAN | 15.95 /16.40 | 18.31 /NAN | NAN /NAN | 22.79 /NAN | 17.07 /17.89 |
|  | CDL-D, 30ns | NAN /NAN | 16.54 /17.25 | 21.42 /NAN | NAN /NAN | 15.96 /16.40 | 18.33 /NAN | NAN /NAN | 22.77 /NAN | 17.08 /17.89 |

Table 10. SINR in dB achieving PUSCH BLER of 10% /1% for DFT-s-OFDM with different PTRS location and Rx advance shift

| MCS | Channel | 120 kHz,64RB |  |  | 480 kHz,64RB@1600 MHz |  |  | 960 kHz,64RB@1960 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | old_location+0%shift | old_location+10%shift | new_location+10%shift | old_location+0%shift | old_location+10%shift | new_location+10%shift | old_location+0%shift | old_location+10%shift | new_location+10%shift |
| 16 | CDL-B, 20ns | 11.17 /14.66 | 11.35 /14.82 | 11.03 /14.61 | 11.28 /13.97 | 11.56 /14.11 | 11.28 /13.88 | 11.48 /14.94 | 11.55 /14.94 | 11.43 /14.93 |
|  | CDL-D, 20ns | 7.32 /8.05 | 7.52 /8.31 | 7.21 /7.88 | 6.93 /7.57 | 7.14 /7.85 | 6.93 /7.48 | 6.93 /7.54 | 7.05 /7.65 | 6.94 /7.55 |
|  | CDL-D, 30ns | 7.31 /8.05 | 7.51 /8.25 | 7.21 /7.88 | 6.93 /7.54 | 7.15 /7.82 | 6.94 /7.46 | 6.92 /7.56 | 7.06 /7.70 | 6.92 /7.55 |
| 22 | CDL-B, 20ns | 18.10 /22.16 | 20.27 /28.22 | 17.32 /21.02 | 17.50 /20.29 | 18.73 /21.96 | 17.36 /20.18 | 17.86 /20.98 | 18.27 /21.45 | 17.69 /20.79 |
|  | CDL-D, 20ns | 14.35 /16.45 | 17.21 /25.05 | 13.37 /14.37 | 12.22 /12.95 | 13.82 /16.58 | 12.16 /12.87 | 12.15 /12.77 | 12.78 /13.72 | 12.10 /12.73 |
|  | CDL-D, 30ns | 14.36 /16.43 | 17.19 /25.05 | 13.37 /14.34 | 12.21 /12.97 | 13.80 /16.50 | 12.16 /12.83 | 12.14 /12.76 | 12.78 /13.74 | 12.11 /12.72 |
| 26 | CDL-B, 20ns | 22.02 /26.25 | NAN /NAN | 21.52 /25.53 | 23.06 /26.85 | NAN /NAN | 22.31 /25.37 | 23.67 /NAN | NAN /NAN | 22.25 /25.32 |
|  | CDL-D, 20ns | 18.12 /20.28 | NAN /NAN | 17.55 /18.53 | 17.38 /18.62 | NAN /NAN | 16.86 /17.65 | 16.65 /17.43 | NAN /NAN | 16.40 /17.09 |
|  | CDL-D, 30ns | 18.12/ 20.11 | NAN /NAN | 17.61 /18.55 | 17.38 /18.62 | NAN /NAN | 16.86 /17.65 | 16.64 /17.43 | NAN /NAN | 16.40 /17.08 |