3GPP TSG RAN WG1 Meeting #106-e	R1-2107673

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.7.1.3

Source:	Huawei, HiSilicon

Title:	Analysis on power consumption for IDLE mode UE

Document for:	Discussion and Decision

# Introduction

In RAN#86, it is approved to further specify enhancements for UE power saving in Rel-17 on top of Rel-16 schemes. For idle/inactive mode, enhancements for paging procedure and assistance RS will be specified.

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

In this contribution, analysis on power consumption of legacy idle mode UE is provided to facilitate the discussion of power saving mechanism for NR idle mode UE, and the relationship between paging early indication and assistance RS is also discussed.

# Analysis on power consumption of idle mode UE

## Contribution of power consumption in idle mode

According to our investigation, the time and power distribution of typical wearable devices (smart watch) are shown in Figure 1. It is observed that idle mode contributes about 60% power consumption of the daily use. Therefore, it is strongly desired to reduce the RedCap UE power consumption in idle mode. In our view, the paging enhancement and assistance TRS should be both specified to improve the power consumption of eMBB UE and RedCap UE, as claimed in WID.

- The time and power distribution of typical wearable devices

- Idle mode power consumption contributes significant power share of wearable devices, for which the power consumption is also intended to be reduced in the WID.

For eMBB UE (e.g. smart phone), the energy consumption of IDLE mode also occupies share of the whole energy consumption. This is also the motivation for the introduction of RRM relaxation for IDLE mode in Rel-16 power saving WI. In Rel-17, paging enhancement for IDLE mode is also important for the power consumption of eMBB UE.

- Paging enhancement for IDLE mode is also important for the power consumption of eMBB UE.

## Analysis on power consumption of LTE and NR legacy idle mode UE

- Illustration for UE behaviors in idle/inactive mode

In idle mode, UE usually stays in deep sleep state in most of the time and Figure 2 illustrates a typical UE processing procedure on one paging occasion to monitor the paging message and do RRM measurements. Before UE receives paging, UE needs to wake up, and perform pre-sync procedures including AGC adjustment and T/F synchronization. In addition to paging, UE needs to perform RRM measurement for serving cell and neighbor cells when measurement criterion is met.

In LTE, UE uses CRS for pre-sync procedures. Since the CRS is an always-transmitted reference signal, an idle mode UE needs to wake up just one or two slots before its paging occasion to perform T/F tracking and AGC adjustment. However, in NR Rel-15/16, the only RS available for idle mode UE is SSB and the periodicity of SSB (typically 20ms) can be much larger than that of CRS (1ms), UE has to pre-wake up by several SSB bursts periods before its paging occasion. Figure 3 illustrates typical paging reception procedures in FR1 for both LTE and NR Rel-15/16 idle mode UE. The power consumption is analyzed based on the power model specified in [2] and agreed in RAN1#102-e.

The time gap from the start of a paging occasion to the closest SSB burst before the PO would be from 0ms to 18ms, considering the SSB periodicity is 20ms and SSB burst duration is 2ms. For simplicity, we only analyze the average case, that is, the paging occasion occurs 10ms after a SSB assuming the periodicity of SSB is 20ms. For paging reception, we assume that UE receives all the SSB before a PO and only receives the paging PDCCH in the monitoring occasion which corresponds to the SSB with best reception quality. Therefore the PO reception occurs 0.5ms in 30kHz SCS when Paging PDSCH is scheduled in the same slot of Paging PDCCH.

In RAN1#102, the power model used for IDLE mode power evaluation was agreed. The analysis results in Figure 4 is based on the agreed power model, which is listed in Table 2 in Appendix. For LTE case, the power consumption to receive CRS, PDCCH and PDSCH is assumed the same as that of NR.

- Illustration of default association paging monitoring occasions

The power consumption distribution of each processing operation in paging procedure in idle mode for legacy LTE and NR with RRM relaxation is shown in Figure 4.

- The relative power consumption for LTE and NR

The light grey bars in Figure 4 correspond to the deep sleep mode and related state transition energy overhead. Considering the time duration in which an idle mode UE stays in deep sleep mode occupies the most of the time in a paging DRX cycle (1.28s in this evaluation). Therefore, the absolute energy consumption of the grey bar should be similar for LTE and NR in the figure. From the detailed power consumption, it can be seen that compared with LTE the main reason of power consumption increment for NR comes from the light sleep and power state transition between light sleep and active state, especially in the low SINR scenario.

The NR idle mode UE needs 1~3 SS bursts to perform T/F tracking and AGC adjustment, and additional SMTC window for neighbor cell measurements. However, the 1~3 SS bursts and the PO are transmitted in non-continuous slots, which is quite different from LTE. Therefore a NR idle mode UE has to pre-wake-up, and then transitions to light sleep to wait for a next signal arrival, which consumes much power of the UE. However, the LTE idle mode UE can finish all the operation without any light sleep and power state transition.

- The light sleep state and power state transition between light sleep and active state in NR consume more power compared to LTE system, which is the main reason for the high power consumption of NR Rel-15/16.

# The relationship between paging early indication and assistance RS

In RAN1#103, paging early indication (PEI) was agreed to be supported. Assistance TRS shall be also supported in Rel-17. This section, the relationship between PEI and assistance RS are discussed.

Firstly, PEI and assistance RS can be used separately and can provide the power saving gain independently. As analyzed in the companion contributions [3][4].

- Either paging early indication or assistance RS can provide power saving gain when the other scheme is not configured.

Furthermore, PEI and assistance TRS can be configured simultaneously and utilized by UE to obtain better power saving gains. Considering the power consumption of NR Rel-16 UE, according to our analysis in section 2.2, the more power consumption mainly comes from unnecessary ‘state transition’ and ‘light sleep’ in the unpaged DRX cycles and long pre-wake time duration needed for time/frequency tracking relying on serval number of SS bursts.

- Illustration of utilization of PEI and assistance TRS simultaneously by UE.

For those DRX cycles that idle/inactive mode UEs are not paged, the power consumption of ‘light sleep’, ‘light sleep state transition’ and the PO reception are actually not needed, which just waste the power consumption. If the network could transmit the paging information carried by the paging DCI earlier, the UE could know whether a PO reception is needed so that unnecessary ‘state transition’ and ‘light sleep’ in the unpaged DRX cycles can be avoided. Therefore, the UE can go to deep sleep immediately after the PEI reception if there is no paging message for the UE, i.e. Procedure A in Figure 5. More analysis for PEI can be found in our companion paper [3].

For those DRX cycles in which the concerned idle/inactive mode UE is paged, the UE shall be indicated to receive the corresponding PO. In this case, the UE needs to perform time/frequency tracking and AGC in order to reliably receive the paging PDSCH. The assistance TRS/CSI-RS can reduce the power consumption for the AGC and time/frequency tracking depending on the assistance TRS/CSI-RS position. As an example in Procedure B1 in Figure 5, if the TRS occasion at certain position is available, the UE can reduce the number of received SS bursts, therefore, additional power saving gain can be obtained.

According to the analysis in Figure 5, if PEI and assistance TRS are utilized by UE simultaneously, further power saving can be obtained.

- UE can utilize both paging early indication and assistance RS for better power saving gain if both features are configured.

The early paging information can be used to inform the availability of TRS/CSI-RS for idle/inactive mode UE. In RAN1#105-e, it was agreed to support at least L1 signaling for availability indication of assistance TRS.

Agreement: Confirm the following working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB

According to the procedures in Figure 5, PEI is the most straight forward way for the availability indication of assistance TRS. Also, it provides the lowest resource overhead, and can support frequent change of the availability status. Based on the analysis in our companion paper [4], it is proposed to confirm the working assumption and use PEI to support availability of assistance TRS.

- Paging early indication is the straight forward and best way to inform the availability of TRS/CSI-RS for idle/inactive mode UE.

# Conclusions

In this contribution, we analyze the power consumption of legacy idle/inactive mode UE and discuss the relationship between the paging early indication and assistance RS. Based on the analysis, we have the following observations:

- Idle mode power consumption contributes significant power share of wearable devices, for which the power consumption is also intended to be reduced in the WID.

- Paging enhancement for IDLE mode is also important for the power consumption of eMBB UE.

- The light sleep state and power state transition between light sleep and active state in NR consume more power compared to LTE system, which is the main reason for the high power consumption of NR Rel-15/16.

- Either paging early indication or assistance RS can provide power saving gain when the other scheme is not configured.

- UE can utilize both paging early indication and assistance RS for better power saving gain if both features are configured.

- Paging early indication is the straight forward and best way to inform the availability of TRS/CSI-RS for idle/inactive mode UE.

# References

RP-193239, “New WID: UE Power Saving Enhancements”, MediaTek

TR 38.840, Study on UE power saving.

R1-2106479, " Paging enhancements for UE power saving in IDLE/inactive mode", Huawei, Hisilicon

R1-2106480, " Assistance RS occasions for IDLE/inactive mode", Huawei, Hisilicon

# Appendix

- Basic evaluation assumptions

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4 GHz (FR1) |
| Subcarrier spacing | 30kHz |
| CC number | 1 |
| Bandwidth of BWP | 20MHz |
| DRX cycle | 1280ms |
| Per-UE paging rate | 1% |
| NR of UE per paging occasion | 11 |
| Paging rate per PO | 10.47% |

- Power consumption model for eMBB UE

| Power State | Relative Power (FR1 reference from TR 38.840) | Relative Power (Idle/inactive-mode operation with reception bandwidth 20 MHz) |
| --- | --- | --- |
| Deep Sleep (PDS) | 1 | 1 |
| Light Sleep (PLS) | 20 | 20 |
| Micro sleep (PMS) | 45 | 45 |
| PDCCH-only (PPDCCH) | 100 | 50Note |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 300 | 120 |
| PDSCH-only (PPDSCH) | 280 | 112 |
| SSB/CSI-RS proc. (PSSB) | 100 (synchronization or serving cell measurement) | 50 |
| Intra-frequency RRM measurement (Pintra) | · 150 (synchronous case, N=8, measurement only; Pintra, meas-only) · 200 (combined search and measurement; Pintra, search+meas) | · [60] (synchronous case, N=8, measurement only; Pintra, meas-only) · [80] (combined search and measurement; Pintra, search+meas) |
| Inter-frequency RRM measurement (Pinter) | · 150 (measurement only per freq. layer; Pinter, meas-only) · 150 (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer | · [60] (measurement only per freq. layer; Pinter, meas-only) · [150] (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer |
| Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}. |  |  |