3GPP TSG RAN WG1 #106-e			R1-2107692

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.10.1

Source:	AT&T

Title:	Enhancements for IAB resource multiplexing

Document for:	Discussion/Approval

# Introduction

As part of the Rel-17 Work Item on Enhancements for Integrated Access and Backhaul for NR [1], 3GPP has agreed to the following RAN1 led objectives:

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

This contribution discusses different physical layer enhancements to support simultaneous operation of child and parent links including aspects impacting resource allocation and RRM for IAB nodes.

# Resource Multiplexing Enhancements for IAB

During Rel-16, for in-band operation IAB nodes are assumed to operate with a half-duplex constraint as shown in Figure 1, which means they can only do the following at any given time:

- Receive on the access link (UE to IAB node) and/or backhaul link (IAB node to IAB node)

- Transmit on the access link and/or backhaul link

Figure 1: Half-duplex constraint for IAB in Rel-16

As a result, the frame structure, timing alignment, and initial access/RRM procedures for IAB nodes in Rel-16 focused on so-called TDM operation where either the access link or backhaul link was active in a given time/frequency resources. However in Rel-17 enhancements are considered which extend the resource multiplexing to additional scenarios such as spatial division multiplexing (SDM) and multi-panel Tx/Rx (MPTR) as shown in Figure 2. MPTR is a subset of potential “full duplex IAB” operation where the different panels are used for access and backhaul links in a given slot. Depending on the scenario, MPTR could be extended to support simultaneous operation within a panel, however potentially with stricter interference requirements.

SDM

MPTR

SDM

MPTR

Figure 2: SDM and MPTR for IAB in Rel-17

## Support for Dynamic multiplexing capability indications

The following agreements were reached in RAN1#104bis-e:

Agreement

Adaptation of an IAB-node’s multiplexing operation is supported. The adaptation may be based on multiple factors, for example (not necessary to support all of the following):

- Resource type (D/U/F) at the IAB-DU and IAB-MT

- Specific sets of time/frequency resources

- Certain conditions being met (e.g. supported timing modes, power control enhancements (if supported), etc.)

FFS:  Mechanisms for informing/coordination the change in multiplexing operation(s) between child and parent nodes (including whether the adaptation is dynamic or semi-static)

FFS: Need for explicit linkage between indicated multiplexing operations and other features/enhancements – e.g. number of required guard symbols, supported timing modes, and power control enhancements (if supported)

Rel-16 allows for the parent and child to exchange information about the supported multiplexing capabilities and IAB-specific guard symbols which can be used to enable more efficient utilization of resources, including guard symbols and especially soft resources at the child IAB-DU. However, multiple factors may influence whether a given multiplexing case is more or less appropriate for a given time/frequency resource for example a desire to avoid certain DL or UL slots used by access UEs or TDM-only IAB nodes, and also other factors such as self or cross-link interference, single or multi-parent operation, transmit or receive panel selection, and device capabilities. As a result, in order for the dynamic indication of availability of soft resources to be utilized effectively the parent and node should be able to take this potential variability of the multiplexing operation and guard symbols into account. Enhancements to DCI Format 2_5 and the MAC CE for indicating the number of desired/available guard symbols should be considered to support this dynamic indication.

Proposal 1: Support dynamic adaptation of multiplexing capability for a subset of time/frequency resources of a given backhaul link via enhancements to DCI Format 2_5 and adaptation of the desired/available guard symbols per multiplexing capability via enhancements to MAC-based signaling between an IAB node and its parent node.

## Extension of frequency resources to H/S/NA resource attributes

In order to extend the H/S/NA framework in case of FDM operation beyond inter-carrier granularity, the following agreements were made in RAN1#105-e:

Agreement

For frequency domain multiplexing, H/S/NA configurations for an IAB-node are provided separately in addition to the Rel-16 H/S/NA

Agreement

DCI Format 2_5 is reused to support soft resource availability indications for frequency-domain resources

- FFS: If additional enhancements are necessary

Agreement

If an IAB node is configured with a frequency-domain H/S/NA configuration down select between the following options:

- Alt. 1 Either the Rel-16 H/S/NA configuration or frequency domain configuration is applied for a given resource

  - FFS: Whether configurations are switched with per-slot, per-resource type within a slot, or per-symbol granularity

- Alt. 2 The Rel-16 H/S/NA configuration and frequency domain configuration are jointly applied

Agreement

The minimum resource size for configuring the frequency domain granularity is a set of N RBs:

- Candidate values for N: {4, 8, 16, other values TBD}

- N is at least the # PRBs that are corresponding to the MT’s # PRBs of an RBG).

- FFS: Scaling or configuration of N based on system BW or size of IAB-MT BWP

In order to make an efficient tradeoff between overhead of the signaling and the flexibility to support FDM within a carrier between access and backhaul links, using the same RBG-level frequency granularity as the IAB-MT for the indication with per-resource type within a slot, can be taken as the granularity for the resource configuration. This enables the network to avoid TDM-only H/S/NA configurations and RBG-level granularity for the frequency domain can coexist well with access UEs and IAB-MTs.

Proposal 2: If an IAB node is configured with a frequency-domain H/S/NA configuration, either the Rel-16 H/S/NA configuration or frequency domain configuration is applied for a given resource with per-resource type within a slot granularity.

Proposal 3: The configured resource size for an IAB-DU’s frequency domain H/S/NA granularity is a set of N RBs, where N is exactly the number of PRBs that are configured for the corresponding MT’s RBG.

## Extension of spatial resources to H/S/NA resource attributes

In RAN1#105-e the following agreements was reached:

Agreement

In case of simultaneous MT/DU operation,

- the parent node can dynamically indicate to the child node at least a set of restricted beams at the IAB-DU of the child node

- the child node can dynamically report to the parent node a set of recommended beams, not preferred beams, or both recommended and not preferred beams of the IAB-MT of the child node

  - FFS: Whether the specification supports all reporting combinations.

- FFS: Applicability to specific multiplexing cases or specific time-frequency resources

- FFS: Additional semi-static signaling

- FFS: Per-panel granularity in addition to per-beam granularity

- FFS: Relationship between child IAB-MT beam indication and parent IAB-DU beam indication

- Note: This does not preclude any enhancements for either DU or MT-based CLI measurement and reports

In addition, using the beam-management framework defined for UEs, IAB nodes which support multiple panels or even multiple beams within a same panel with sufficient isolation could conceivably be simultaneously utilized on different access and backhaul links. Similar to time/frequency subsets, the existing multiplexing capability signaling framework does not have sufficient granularity to allow differentiation based on spatial information. As a result, there may be a benefit to extend the H/S/NA resource indication to the spatial domain at the panel/beam level. RAN1 should further investigate mechanisms to associate a given H/S/NA resource at the IAB-DU with different QCL assumptions of a co-located IAB-MT using the existing beam-management framework (e.g. on the basis of different TCI states configured at the IAB-MT).

Proposal 4: The parent node can dynamically indicate to the child node a set of restricted beams at the IAB-DU of the child node which are associated with a given set of H/S/NA resource(s) and multiplexing capabilities at the IAB-DU.

Proposal 5: The child node can dynamically report to the parent node a set of recommended beams, not preferred beams, or both recommended and not preferred beams of the IAB-MT of the child node which are associated with different QCL assumptions of the IAB-MT using the existing beam-management framework and potential extensions to support panel-level granularity (if supported by the Rel-17 MIMO WI).

## Multi-parent scenarios

In addition, in Rel-16, the resource allocation and multiplexing framework focused on single-parent scenarios for a given IAB-node. However, to support redundancy and load-balancing, multi-parent scenarios are also possible.

Furthermore, during RAN1#105-e the following agreement was reached for multi-parent scenarios:

Agreement

In case of intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios the following are supported:

- Reusing the Rel-16 CA TDD prioritization rules in case of UL/DL conflict when applicable

  - FFS: Whether all prioritization rules apply in case of NR-DC

  - FFS: Need of new prioritization rules in case of NR-DC

- Coordinating the IAB-MT’s TDD configurations to avoid conflicts from different parent nodes in case the child IAB-MT does not support simultaneous TX and RX on different carries

  - FFS: Coordination for scheduling conflicts involving at least DCI Format 2_0 usage (e.g. usage of flexible symbols)

- Exchanging H/S/NA configurations between parent nodes/donors

In order to address multi-parent operation, signaling for the IAB-DU resource configuration may need enhancements in Rel-17, since to allow multiplexing which is more efficient than strict TDM requires awareness of the configured resources at the multiple parents to avoid scheduling conflicts (e.g. in case of the use of flexible symbols via DCI Format 2_0 from multiple parent nodes). In this case, per-link resource configurations exchanged between the parents can be used to override the per-DU configurations to avoid severe resource restrictions and overhead.

Proposal 6: Per-backhaul link resource configurations and multiplexing capability indications independent of the per-IAB-DU configuration and multiplexing capability indication are supported in Rel-17. Coordination signaling to exchange the per-link resource configurations and multiplexing capability indications between up to two parent IAB-nodes/donors should be additionally supported in Rel-17.

# Conclusion

This contribution analyzed potential physical layer enhancements for IAB resource allocation and multiplexing in Rel-17. The following proposals were made:

Proposal 1: Support dynamic adaptation of multiplexing capability for a subset of time/frequency resources of a given backhaul link via enhancements to DCI Format 2_5 and adaptation of the desired/available guard symbols per multiplexing capability via enhancements to MAC-based signaling between an IAB node and its parent node.

Proposal 2: If an IAB node is configured with a frequency-domain H/S/NA configuration, either the Rel-16 H/S/NA configuration or frequency domain configuration is applied for a given resource with per-resource type within a slot granularity.

Proposal 3: The configured resource size for an IAB-DU’s frequency domain H/S/NA granularity is a set of N RBs, where N is exactly the number of PRBs that are configured for the corresponding MT’s RBG.

Proposal 4: The parent node can dynamically indicate to the child node a set of restricted beams at the IAB-DU of the child node which are associated with a given set of H/S/NA resource(s) and multiplexing capabilities at the IAB-DU.

Proposal 5: The child node can dynamically report to the parent node a set of recommended beams, not preferred beams, or both recommended and not preferred beams of the IAB-MT of the child node which are associated with different QCL assumptions of the IAB-MT using the existing beam-management framework and potential extensions to support panel-level granularity (if supported by the Rel-17 MIMO WI).

Proposal 6: Per-backhaul link resource configurations and multiplexing capability indications independent of the per-IAB-DU configuration and multiplexing capability indication are supported in Rel-17. Coordination signaling to exchange the per-link resource configurations and multiplexing capability indications between up to two parent IAB-nodes/donors should be additionally supported in Rel-17.

# Reference

[1] RP-201293 New WID on Enhancements to Integrated Access and Backhaul, Qualcomm

[2] R1-2100777 Enhancements for IAB interference management, AT&T