3GPP TSG- RAN WG1 Meeting #106-e                                                 R1-2107719

# e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.2.1

Source:	Apple Inc.

Title:	On Multi-TRP Reliability Enhancement

Document for:	Discussion/Decision

# Introduction

In RAN1 #104b and #105 meeting, the following agreements of multi-TRP reliability enhancement have been achieved.

Agreement When DL DCI is transmitted via PDCCH repetition, for PUCCH resource determination for HARQ-Ack when the corresponding PUCCH resource set has a size larger than eight, starting CCE index and number of CCEs in the CORESET of one of the linked PDCCH candidates is applied, and option 2 is supported - Option 2: The one with the lowest SS set ID is applied. - FFS: Support of Option 2 does not mean PDCCH repetition based on two linked search space set within one CORESET is supported Agreement For PDSCH rate matching around the scheduling DCI in the case of PDCCH repetition, the previous agreement for FR1 also applies to FR2. Agreement For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition, support - UE reports one [or more] number(s) as required number of BDs for the two PDCCH candidates - Candidate values: 2, 3. - FFS: Default behaviour - FFS: Whether one of the candidate values imply that UE supports soft combining - FFS: Whether additional candidate values are supported (e.g. non-integer numbers) - FFS: RRC configuration based on reported UE capability Agreement If a PDSCH with mapping Type B is scheduled by a DCI in PDCCH candidates that are linked for repetition - For the purpose of the earliest time that the PDSCH can be scheduled as well as for the purpose of the reference symbol for SLIV (when UE is configured with ReferenceofSLIV-ForDCIFormat1_2, and when receiving the PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0), a reference candidate is used. Select one among the following: - Alt1: The candidate that starts later in time - Alt3: The candidate that starts earlier in time - FFS: How to define d1,1 for PDSCH processing time in this case Agreement If a PDSCH is scheduled by a DCI in PDCCH candidates (the first PDCCH candidate associated with a first CORESET and the second PDCCH candidate associated with a second CORESET) that are linked for repetition, - Working assumption: The UE expects the same configuration for the first and second CORESETs wrt presence of TCI field in DCI. - If the TCI field is not present in the DCI, and the scheduling offset is equal to or larger than timeDurationForQCL if applicable, PDSCH QCL assumption is based on the CORESET with lower ID among the first and second CORESETs - FFS: Whether additional options are needed (e.g. to enable SDM/FDM/TDM PDSCH schemes w/o TCI field in the DCI) Agreement For a UE supporting reception with two different beams, support identifying two QCL-TypeD properties for multiple overlapping CORESETs - FFS: How to enhance existing QCL-TypeD priority rules for overlapping CORESETs - Note: The primary goal of this enhancement for the purpose of this sub-AI is to support time-overlapping PDCCH repetitions in FR2. Agreement When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET, for the purpose of BD counting and interpretation of a detected DCI, select one option among the following in RAN1#105-e: - Option 1: The individual candidate is not counted for monitoring - Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate). - Option 2: The candidate in a higher SS set ID is not counted for monitoring - Interpretation of the detected DCI depends on which candidate is not counted (either based on Rel. 15/16 rules or based on Rel. 17 PDCCH repetition rules). - FFS: Impact to the other linked PDCCH candidate - Option 3: The candidate associated with SS set(s) with lower priority is not counted for monitoring, where for two linked SS sets, the priority is according to one of the two SS sets with a lower SS set ID - Interpretation of the detected DCI depends on which candidate is not counted (either based on Rel. 15/16 rules or based on Rel. 17 PDCCH repetition rules). - FFS: Impact to the other linked PDCCH candidate - FFS: Whether a max limit on number of such overlaps is needed. Additional specification support may be introduced for the purpose of resolving ambiguity (if any) for interpretation of the detected DCI. For example, - Distinguished by different RNTIs defined for the linked candidate versus the individual candidate - Distinguished by aggregation level restrictions that can be expected by the UE in the case of overlap Agreement For PDCCH repetition with two linked candidates, if due to Rel. 15/16 procedures, one of the linked candidates is not monitored (is dropped), select one option from Options 1 and 2 in RAN1#105-e: - Option 1: UE still monitors the linked candidate that is not dropped and interprets the DCI based on Rel. 17 PDCCH rules (wrt reference PDCCH candidate) - Option 2: Even the candidate that is not dropped is not monitored (Both linked candidates are dropped if at least one of them is dropped) - FFS: Which of the following Rel. 15/16 rules are applicable for this purpose: - Case 1: Overlap with SSB - Case 2: Overlap with rate matching resources: RateMatchPattern, lte-CRS-ToMatchAround, or LTE-CRS-PatternList-r16, availableRB-SetPerCell-r16 - Case 3: Due to TDD DL/UL related conflicts: Overlap with semi-static / dynamic UL symbols or overlap with PRACH - Case 4: QCL-TypeD prioritization rule among CORESETs result in one of the linked candidates not being monitored - Case 5: Overbooking results in one of the linked candidates not being monitored - Case 6: Overlap with reserved PRB(s) and OFDM symbol(s) indicated by DCI format 2_1 where UE may assume no transmission intended for the UE - Other cases are not precluded - FFS: Whether there is an impact to BD count Agreement For the case of multi-TRP, to support per-TRP power control in FR1, the linking of PUCCH resource with [one or] two power control parameter sets, the following is supported - MAC-CE indicates RRC IE that configures power control parameter sets (p0, pathloss RS ID, and a closed-loop index). - The exact design of RRC IE is up to RAN2 but from RAN1 point of view, one possible example is to reuse PUCCH-SpatialRelationInfo except for the referenceSignal Note: It is common understanding in RAN1 that one PUCCH resource can be linked to one power control parameter set. Conclusion With reference to the normative work on NR-feMIMO: Related to the support of switching gap between UL transmissions towards two TRPs in RAN1 specifications, there is no consensus in RAN1 to specify symbol gap(s) for the following cases - PUSCH Type A - PUCCH scheme 1 - PUSCH Type B - PUCCH scheme 3 The above applies for the case included in the LS from RAN4 in R1-2102297. Agreement When inter-slot frequency hopping is configured with Scheme 1, decide one from the below options in RAN1#105-e meeting, - Option 1 - If sequential mapping pattern is configured, frequency hopping is performed on slot level (as in Rel-15). - If cyclical mapping pattern is configured, frequency hopping is performed among the repetitions with the same beam. - Option 2: - gNB always configures sequential mapping pattern and frequency hopping is performed on slot level. (no spec impact) - Option 3: - Frequency hopping is performed on slot level as in Rel-15 (no spec impact). Agreement When SRS resources from two SRS resource sets indicated in DCI format 0_1/0_2, for linking SRI fields to two power control parameters, it is up to RAN2 to finalize the RRC details related to linking. RAN1 identified that the following options could be used. - Alt. 1: Add second sri-PUSCH-MappingToAddModList, and select two SRI-PUSCH-PowerControl from two sri-PUSCH-MappingToAddModList - Alt. 2: Add SRS resource set ID in SRI-PUSCH-PowerControl, and select SRI-PUSCH-PowerControl from sri-PUSCH-MappingToAddModList considering the SRS resource set ID Agreement For PHR reporting related to M-TRP PUSCH repetition, select one from the following options in RAN1 #105-e meeting. - Option 1: Calculate one PHR associated with the first PUSCH occasion (earliest repetition that overlaps with the first slot in which the PUSCH that carries the PHR MAC-CE is transmitted) - Option 2: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, but report one of them - FFS: How to select the PHR for reporting. - Option 4: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, and report two PHRs - Option 5: No changes to legacy PHR reporting Agreement When MAC-CE indicates a PL-RS ID for one or more SRI IDs, it also indicates whether the SRI IDs are associated with the first or the second SRS resource set. Agreement For multiplexing A-CSI on two PUSCH repetitions in the case of multi-TRP PUSCH repetition, - For S-DCI based multi-TRP PUSCH repetition Type B, support multiplexing A-CSI on the first PUSCH repetition corresponding to the first beam and the first (X = 1) PUSCH repetition corresponding to the second beam. - The UE is expected to follow the above operation for multiplexing A-CSI on two PUSCH repetitions only if - the first actual repetition corresponding to the first beam and the first actual repetition corresponding to the second beam have the same number of symbols, and - UCIs other than the A-CSI are not multiplexed on any of the two PUSCH repetitions. - When the UE does not follow the above operation, UE multiplexes A-CSI only on the first PUSCH repetition similar to Rel. 15/16. - The content for the two A-CSI should be the same - Note: RAN1 has the assumption on CSI timelines are followed as rel-15/16, including UE shall expect the timeline for the first A-CSI meets Z and Z’ requirement - FFS: For s-DCI based multi-TRP PUSCH repetition Type A and B, support multiplexing of A-CSI on the first PUSCH repetition corresponding to the first beam and the first PUSCH repetition corresponding to the second beam when there is no TB carried in the PUSCH. - The UE assumes that the number of repetitions is 2 regardless of the indicated number of repetitions. - For PUSCH repetition Type B, the first and second nominal repetitions are expected to be the same as the first and second actual repetitions, respectively (no segmentation). Working Assumption For indicating STRP/MTRP dynamic switching for non-CB/CB based MTRP PUSCH repetition, - Introduce a new field in DCI to indicate at least the S-TRP or M-TRP operation - FFS: Whether the new field is 1 bit or 2 bits Working Assumption For non-codebook based multi-TRP PUSCH, the first SRI field is used to determine the entry of the second SRI field which only contains the SRI(s) combinations corresponding to the indicated rank (number of layers) of the first SRI field. The number of bits, N2, for the second SRI field is determined by the maximum number of codepoint(s) per rank among all ranks associated with the first SRI field. For each rank x, the first Kx codepoint(s) are mapped to Kx SRIs of rank x associated with the first SRS field, the remaining (2N2-Kx) codepoint(s) are reserved. Agreement For the indication of open-loop power control parameter (OLPC) in DCI format 0_1/0_2, support enhanced open-loop power control parameter (OLPC) set indication by indicating per-TRP OLPC set. - FFS: Details of indication. Agreement For CB based M-TRP PUSCH repetition, the first TPMI field is used to determine the entry of the second TPMI field which only contains TPMIs corresponding to the indicated rank (number of layers) of the first TPMI field. The second TPMI field’s bit width, M2, is determined by the maximum number of TPMIs per rank among all ranks associated with the first TPMI field. For each rank y, the first Ky codepoint(s) of the second TPMI field are mapped to Ky TPMI(s) of rank y associated with the first TPMI field in increasing order codepoint index, the remaining (2M2-Ky) codepoint(s) are reserved. - How to describe/capture this in 38.212 is up to the editor. Agreement Confirm the following Working Assumption: For PUCCH multi-TRP enhancements in Scheme 1, it is possible to configure either cyclic mapping or sequential mapping of spatial relation info’s over PUCCH repetitions. - FFS: Applicability of mapping patterns for different beam switching gaps - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - Note: For Scheme 1, cyclical mapping pattern and sequential mapping pattern are as follows, - Cyclical mapping pattern: the first and second beam are applied to the first and second PUCCH repetition, respectively, and the same beam mapping pattern continues to the remaining PUCCH repetitions. - Sequential mapping pattern: the first beam is applied to the first and second PUCCH repetitions, and the second beam is applied to the third and fourth PUCCH repetitions, and the same beam mapping pattern continues to the remaining PUCCH repetitions. Agreement Confirm the following Working Assumption (with small correction of typo and clarification on UE capability in RED): - For beam mapping /power control parameter set mapping for PUCCH repetitions, - For M-TRP PUCCH Scheme 1 in FR1, it is possible to configure either cyclic mapping or sequential mapping of power control parameter sets over PUCCH repetitions (similar to spatial relation info’s over PUCCH repetitions). - For M-TRP PUCCH Scheme 3, reuse the same methods as Scheme 1 (by replacing slots with sub-slots) for beam mapping or power control resource parameter set mapping to sub-slots. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. Agreement Confirm the following working assumption (with removing the last bullet): For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - FFS: Support of half-half mapping. - FFS: Additional considerations on mapping patterns (including required beam switching gaps) Agreement For single DCI based M-TRP PUSCH Type B repetition, the indication of PTRS-DMRS association for maxRank > 2 is supported, down select one of the following options in RAN1 #105-e meeting, - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - Option 1 (4 bits): with a second PTRS-DMRS association field (similar to the existing field), and each field separately indicating the association between PTRS port and DMRS port for two TRPs. - Option 2 (2 bits): using the existing PTRS-DMRS association field in DCI for the first TRP, and using reserved entries/bits in DM-RS port indication field for the second TRP. - Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports. - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port. Agreement For type 1 or type 2 CG based multi-TRP PUSCH repetition, - Introduce the second fields of 'p0-PUSCH-Alpha' and 'powerControlLoopToUse' in 'ConfiguredGrantConfig’ - For type 1 CG based m-TRP PUSCH repetition, introduce the second fields of ‘pathlossReferenceIndex’, 'srs-ResourceIndicator' and 'precodingAndNumberOfLayers' in 'rrc-ConfiguredUplinkGrant'. - For type 2 CG based M-TRP PUSCH, two SRIs/TPMIs are indicated via the activating DCI. - FFS1: UL PT-RS port(s) and DM-RS port(s) for CG type 1 - FFS3: Details on RV mapping. - FFS4: Possible transmission occasion for initial transmission - FFS5: Other TRP specific parameters in 'rrc-ConfiguredUplinkGrant', e.g., 'dmrs-SeqInitialization'. Agreement For indicating per-TRP OLPC set in DCI format 0_1/0_2, if two SRI fields present in the DCI, - Use the existing field (1 bit) for OLPC set indication and a second p0-PUSCH-SetList-r16. - if value of the field equals to ‘0’, the UE determine value of P0 from SRI-PUSCH-PowerControl with a sri-PUSCH-PowerControlId value mapped to the SRI field value corresponding to each TRP. - if value of the field equals to ‘1’, the UE determine value of P0 from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value corresponding to each TRP. Agreement For s-DCI based multi-TRP PUSCH repetition Type A and B, support transmitting A-CSI on the first PUSCH repetition corresponding to the first beam and the first PUSCH repetition corresponding to the second beam when there is no TB carried in the PUSCH. - The UE assumes that the number of repetitions is 2 regardless of the indicated number of repetitions. - The UE is expected to follow the above operation for transmitting A-CSI on two PUSCH repetitions only if - For PUSCH repetition Type B, the first and second nominal repetitions are expected to be the same as the first and second actual repetitions, respectively (no segmentation). - For PUSCH repetition Type A and B, UCIs other than the A-CSI are not multiplexed on any of the two PUSCH repetitions. - When the UE does not follow the above operation, UE transmits A-CSI only on the first PUSCH repetition similar to Rel. 15/16. - Note: The scheduling offset for the first A-CSI should meet the Z and Z’ requirement Agreement For s-DCI based multi-TRP PUSCH repetition Type A, the UE is expected to multiplex A-CSI on two PUSCH repetitions only if UCIs other than the A-CSI are not multiplexed on any of the two PUSCH repetitions. - When the UE does not follow the above operation, UE multiplexes A-CSI only on the first PUSCH repetition similar to Rel. 15/16. Agreement For multi-TRP PUCCH (scheme 1 and 3) and PUSCH (Type A and B) repetition, when the number of repetitions is equal to two, the first and second transmission occasion shall be associated with two TRPs, respectively (two UL beams or Power control parameter sets), regardless of the configured mapping pattern. - Note: For M-TRP PUSCH type B, the number of repetitions refers to ‘nominal’ repetition. Agreement The following working assumption is confirmed. For non-codebook based multi-TRP PUSCH, the first SRI field is used to determine the entry of the second SRI field which only contains the SRI(s) combinations corresponding to the indicated rank (number of layers) of the first SRI field. The number of bits, N2, for the second SRI field is determined by the maximum number of codepoint(s) per rank among all ranks associated with the first SRI field. For each rank x, the first Kx codepoint(s) are mapped to Kx SRIs of rank x associated with the first SRS field, the remaining (2N2-Kx) codepoint(s) are reserved. Agreement For type 2 CG based multi-TRP PUSCH repetition: - The first (legacy) RRC-configured fields ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’ are associated with the first SRS resource set. - The second (new) RRC-configured fields ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’ are associated with the second SRS resource set. - Applying the first, second, or both first and second RRC-configured fields ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’ is determined from the new DCI field (for dynamic switching) of the activating DCI similar to the case of DG-PUSCH. Agreement Confirm the Working Assumption (with supporting two bits for the new field). - For indicating STRP/MTRP dynamic switching for non-CB/CB based MTRP PUSCH repetition, - Introduce a new field in DCI to indicate at least the S-TRP or M-TRP operation. - The new field is 2 bits Agreement For the new field in the DCI for dynamic switching, support Alt.1 (modified). Alt.1 - Support 2 bits with the following combinations. Codepoint; SRS resource set(s); SRI (for both CB and NCB)/TPMI (CB only) field(s). 00; s-TRP mode with 1st SRS resource set (TRP1); 1st SRI/TPMI field (2nd field is unused). 01; s-TRP mode with 2nd SRS resource set (TRP2); 1st SRI/TPMI field (2nd field is unused). 10; m-TRP mode with (TRP1,TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set; Both 1st and 2nd SRI/TPMI fields. 11; m-TRP mode with (TRP2,TRP1 order) 1st SRI/TPMI field: FFS 2nd SRI/TPMI field: FFS; Both 1st and 2nd SRI/TPMI fields. - The SRS resource set with lower ID is the first SRS resource set, and the other SRS resource set is the second SRS resource set. - For codebook and non-codebook usage, respectively - The same number of SRS resource shall be configured in the two SRS resource sets. Agreement For SP-CSI report on mTRP PUSCH repetition Type A and B activated by a DCI, further study the use of a similar mechanism to A-CSI multiplexing on M-TRP PUSCH without a TB, which includes the following, - When SP-CSI multiplexed on m-TRP PUSCH, SP-CSI multiplexed on the two repetitions associated with the two TRPs, and the number of repetitions is always assumed to be 2, regardless of the value indicated. - Reuse similar conditions (e.g. UCIs other than the A-CSI are not multiplexed, same number for first actual repetitions, the content of the CSI is the same) to support SP-CSI multiplexing on m-TRP PUSCH as defined in A-CSI multiplexing on M-TRP PUSCH. Agreement Confirm the working assumption with removing brackets on [consecutive] and adding UE capability. - For PUCCH reliability enhancement, support multi-TRP intra-slot repetition (Scheme 3) for all PUCCH formats. - The same PUCCH resource carrying UCI is repeated for X = 2 [consecutive] sub-slots within a slot. - Refer the design details related to sub-slot configurations (e.g. other values of X) to Rel-17 eIIoT - Note1: The decision of supporting scheme 3 is only applicable for multi-TRP operation. - This feature is optional. Conclusion For multi-TRP PUCCH schemes, only one ‘twoPUCCH-PC-AdjustmentStates’ parameter is configured for both TRPs, and the parameter is shared across both TRPs, which means there will be two closed loops in total (no RAN1 spec impact). For future meetings: Further study the enhancements needed on grouping of PUCCH resources for Rel-17 multi-TRP PUCCH repetition Agreement - To support per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2, a second TPC field can be configured via RRC. - When the second field is configured by RRC, a second TPC field (similar to the existing TPC field) is added in DCI formats 1_1 / 1_2 (option 3). - Each TPC field is for each closed-loop index value respectively - FFS: Whether or not the mapping between the TPC field and the PUCCH transmissions is needed - When the second field is not configured by RRC, a single TPC field (the existing TPC field) is used in DCI formats 1_1 / 1_2, and the TPC value applied for the closed loop index(es) for the scheduled PUCCH - To support per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2, adopt the same solution as with M-TRP PUCCH schemes. - FFS: any additional considerations - Support UE to report the capability on whether it supports the second TPC field - Note1: Per TRP closed-loop power control is only applicable when the “closedLoopIndex” values are not the same for TRPs. Agreement For single-DCI based M-TRP PUSCH repetition schemes, when one SRS resource per SRS resource set is configured (i.e., when two SRI fields are absent in DCI formats 0_1 / 0_2), default P0, alpha, PL-RS, and closed loop index is defined per TRP. Select one from the following in RAN1 #106-e meeting, - Alt.1 - The first P0/alpha, PL-RS, and closed loop index are determined by sri-PUSCH-PathlossReferenceRS-Id, sri-P0-PUSCH-AlphaSetId, and sri-PUSCH-ClosedLoopIndex mapped to the first sri-PUSCH-PowerControl associated with the first SRS resource set. - The second P0/alpha, PL-RS, and closed loop index are determined by sri-PUSCH-PathlossReferenceRS-Id, sri-P0-PUSCH-AlphaSetId, and sri-PUSCH-ClosedLoopIndex mapped to the first sri-PUSCH-PowerControl associated with the second SRS resource set. - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2. - Alt.2 - The first set of values {the first value in P0-AlphaSet, the PL-RS corresponded to PUSCH-PathlossReferenceRS-Id = 0 and closed-loop index l = 0} can be used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS corresponded to PUSCH-PathlossReferenceRS-Id = 1 and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise } can be used for TRP2. - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2. - Alt.3 - If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, the first set of values {the first value in P0-AlphaSet, the PL-RS corresponding to the first sri-PUSCH-PowerControl associated with the first SRS resource set and closed-loop index l = 0} is used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS corresponding to the first sri-PUSCH-PowerControlassociated with the second SRS resource set and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise} is used for TRP2. - Otherwise, the first set of values {the first value in P0-AlphaSet, the PL-RS with PUSCH-PathlossReferenceRS-Id=0 and closed-loop index l = 0} can be used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS with PUSCH-PathlossReferenceRS-Id = 1 and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise } can be used for TRP2. - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2. For further study in future meetings: For PHR reporting related to M-TRP PUSCH repetition, study following aspects related to option 4, - Option 4: Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs. - FFS1: How the PHRs are calculated for reporting (actual PHR or virtual PHR) - FFS2: How the PHRs are calculated for reporting for other CCs if the multi-cell PHR MAC CE is applied. - FFS3: Required changes to triggering conditions including the required higher layer parameters (e.g.,’phr-PeriodicTimer’, ‘phr-ProhibitTimer’, ‘phr-Tx-PowerFactorChange’ as TRP specific). - FFS4: Report P-MPR and MPE per TRP within the same MAC-CE extension. Note: Down-selection between Options 1-5 will be based on this study as well as the trade-off between benefit versus UE complexity.

In this contribution, we provide some discussion on PDCCH/PUSCH/PUCCH reliability enhancement.

# PDCCH reliability enhancement

## QCL-TypeD collision handling

In RAN1 #104b, the following agreement on QCL-TypeD collision handling enhancement was reached.

Agreement For a UE supporting reception with two different beams, support identifying two QCL-TypeD properties for multiple overlapping CORESETs - FFS: How to enhance existing QCL-TypeD priority rules for overlapping CORESETs - Note: The primary goal of this enhancement for the purpose of this sub-AI is to support time-overlapping PDCCH repetitions in FR2.

In Rel-15/Rel-16, a priority rule for PDCCH reception is defined when QCL-TypeD collision happens, which is based on the type of SS, CORESET ID and CC ID. However, to support multi-TRP operation, UE may be able to receive PDCCH repetitions with different QCL-TypeD. Then this priority rule needs to be enhanced. The outcome for the QCL-TypeD collision handling is to identify two beams that can be received by UE simultaneously.

In Rel-17, group-based beam reporting has been enhanced and UE can report N groups of beams where each group includes 2 beams that can be received simultaneously. So the priority should be selected per group of beams, and the priority of the group of beams can be determined by the SS with highest priority.

Proposal 2-1: For QCL-TypeD collision handling, support to determine the QCL-TypeD based on the priority for each group of beams, where the beams in a group can be received by UE simultaneously

- For each group of beams, the priority is determined by the corresponding SS with highest priority

## Non-SFN scheme and multi-DCI operation

The multi-DCI operation could be enabled by configuring more than 1 CORESETPoolIndex. However, the non-SFN scheme should reply one some tight coordination between the two TRPs. There is no good use case to transmit PDCCH repetitions from CORESETs with different CORESETPoolIndex. To simplify the design, it is better to preclude such a case.

Proposal 2-2: UE shall not expect the linked SS sets associated with CORESETs with different CORESETPoolIndex.

## PCell/SCell Beam failure recovery related aspects

In Rel-15/Rel-16, PCell/SCell BFR was introduced, where UE can report one beam and after receiving the beam failure recovery response UE can automatically apply the reported beam for PDCCH reception. With regard to Rel-17 PDCCH enhancement, it should be more reasonable for UE to report 2 beams in BFRQ, and UE can update the beams for PDCCH reception after it receives BFR response.

Proposal 2-3: For PCell/SCell BFR, support UE to report 2 new beams in BFRQ for PCell/SCell BFR, and after UE receives BFR response it can apply the 2 beams for reception of PDCCH repetition with 2 beams.

## PDCCH repetitions in PCell

In PCell, gNB may configure CORESET-BFR for CFRA based BFR and CORESET0 for initial access, broadcast signal transmission and so on.

For SS-BFR/CORESET-BFR, the first issue is whether the SS-BFR can be linked to other SS. Since the QCL assumption for SS-BFR is based on the newly identified beam, and UE does not need to monitor SS-BFR if UE did not send CF-PRACH for BFR, the SS-BFR should not be linked to another SS. In addition, to reserve more CORESETs for PDCCH repetitions, the SS-BFR and CORESET-BFR do not need to be 1-to-1 linked. Some normal SSs can be associated with the CORESET-BFR, so that PDCCH repetitions can be supported based on CORESET-BFR.

In addition, for CORESET0, as there are multiple instances of CORESET0 with different beams, PDCCH repetitions with different instances of SS0/CORESET0 can be supported without linking to additional SS/CORESET. One possible way is to provide 2 TCI states for CORESET0, and different instances of the SS0/CORESET0 can be used for PDCCH repetitions.

Proposal 2-4: SS-BFR should not be linked with other SS for PDCCH repetitions.

Proposal 2-5: Support CORESET-BFR to be associated with SS other than SS-BFR.

Proposal 2-6: Support to configure 2 TCI states for CORESET0 for PDCCH repetition with non-SFN scheme.

## PDSCH mapping Type B

In RAN1 #104b, the following agreements on PDSCH mapping Type B were reached.

Agreement If a PDSCH with mapping Type B is scheduled by a DCI in PDCCH candidates that are linked for repetition - For the purpose of the earliest time that the PDSCH can be scheduled as well as for the purpose of the reference symbol for SLIV (when UE is configured with ReferenceofSLIV-ForDCIFormat1_2, and when receiving the PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0), a reference candidate is used. Select one among the following: - Alt1: The candidate that starts later in time - Alt3: The candidate that starts earlier in time - FFS: How to define d1,1 for PDSCH processing time in this case

Currently the single-TRP mode, one scheduling restriction on PDSCH mapping Type B is defined, where the UE is not expected to receive a PDSCH with mapping type B in a slot, if the first symbol of the PDCCH scheduling the PDSCH was received in a later symbol than the first symbol indicated in the PDSCH time domain resource allocation. The general principle is that the PDSCH should not be mapped to symbols before scheduling PDCCH so as not to tight the processing requirement. Thus Alt1 (The candidate that starts later in time) should be preferred.

In addition, another issue is the d1,1 definition. Currently d1,1 depends on the number of PDSCH symbols, number of PDCCH/PDSCH overlapping symbols and number of symbols for a CORESET, which is defined as follows.

- For UE processing capability 1: If the PDSCH is mapping type B as given in clause 7.4.1.1 of [4, TS 38.211], and - if the number of PDSCH symbols allocated is L ≥ 7, then d1,1 = 0, - if the number of PDSCH symbols allocated is L ≥ 4 and L ≤ 6, then d1,1 = 7- L. - if the number of PDSCH symbols allocated is L = 3 then d1,1 = 3 + min (d,1), where d is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH. - if the number of PDSCH symbols allocated is 2, then d1,1 = 3+d, where d is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH. - For UE processing capability 2: If the PDSCH is mapping type B as given in clause 7.4.1.1 of [4, TS 38.211], - if the number of PDSCH symbols allocated is L ≥ 7, then d1,1 = 0, - if the number of PDSCH symbols allocated is L ≥ 3 and L ≤ 6, then d1,1 is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH, - if the number of PDSCH symbols allocated is 2, - if the scheduling PDCCH was in a 3-symbol CORESET and the CORESET and the PDSCH had the same starting symbol, then d1,1 = 3, - otherwise d1,1 is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

For non-SFN based PDCCH, as UE complexity to decode PDCCH increases, more decoding latency could be required. When counting the number of overlapping symbols of PDCCH and scheduled PDSCH, a symbol should be double counted if the symbol contains 2 PDCCH repetitions and PDSCH. If the number of PDSCH symbols allocated is 2 for UE capability 2, if the total number of symbols for the two linked CORESETs is above 2, d1,1 should be equal to the total number of symbols for the two linked CORESETs.

Proposal 2-7: For PDSCH mapping TypeB, the scheduling restriction should be counted based on Alt1 (The candidate that starts later in time).

Proposal 2-8: For PDSCH mapping TypeB, d1,1 should be calculated based on the following scheme

- The number of overlapping symbols of scheduling PDCCH and scheduled PDSCH is counted based on the 2 PDCCH repetitions and a symbol should be double counted when it contains 2 scheduling PDCCH and scheduled PDSCH

- For UE processing capability 2, if the number of PDSCH symbols allocated is 2, d1,1 is equal to the max{number of symbols for both CORESETs starting from the PDSCH symbol, number of overlapping symbols of PDCCH and PDSCH}

## BD counting and interpretation of DCI

In RAN 1 #104b, the following is agreed on BD counting and interpretation of DCI.

Agreement When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET, for the purpose of BD counting and interpretation of a detected DCI, select one option among the following in RAN1#105-e: - Option 1: The individual candidate is not counted for monitoring - Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate). - Option 2: The candidate in a higher SS set ID is not counted for monitoring - Interpretation of the detected DCI depends on which candidate is not counted (either based on Rel. 15/16 rules or based on Rel. 17 PDCCH repetition rules). - FFS: Impact to the other linked PDCCH candidate - Option 3: The candidate associated with SS set(s) with lower priority is not counted for monitoring, where for two linked SS sets, the priority is according to one of the two SS sets with a lower SS set ID - Interpretation of the detected DCI depends on which candidate is not counted (either based on Rel. 15/16 rules or based on Rel. 17 PDCCH repetition rules). - FFS: Impact to the other linked PDCCH candidate - FFS: Whether a max limit on number of such overlaps is needed. Additional specification support may be introduced for the purpose of resolving ambiguity (if any) for interpretation of the detected DCI. For example, - Distinguished by different RNTIs defined for the linked candidate versus the individual candidate - Distinguished by aggregation level restrictions that can be expected by the UE in the case of overlap

If one of the linked PDCCH candidates share the same configuration as an individual candidate, the first issue is whether the BD is counted based on linked PDCCH candidates or individual candidate, and the second issue is how to interpret the decoded DCI. Among the 3 options, option 1 is the simplest one. Option 2 and 3 would have additional impact on the other linked PDCCH candidate. Therefore option 1 is slightly preferred.

Proposal 2-9: When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, Option 1 (The individual candidate is not counted for monitoring) is slightly preferred.

## Monitoring on linked PDCCH candidate

In RAN1 #104b meeting, the following on the monitoring behavior for the linked candidate when one of the PDCCH candidates is dropped is agreed.

Agreement For PDCCH repetition with two linked candidates, if due to Rel. 15/16 procedures, one of the linked candidates is not monitored (is dropped), select one option from Options 1 and 2 in RAN1#105-e: - Option 1: UE still monitors the linked candidate that is not dropped and interprets the DCI based on Rel. 17 PDCCH rules (wrt reference PDCCH candidate) - Option 2: Even the candidate that is not dropped is not monitored (Both linked candidates are dropped if at least one of them is dropped) - FFS: Which of the following Rel. 15/16 rules are applicable for this purpose: - Case 1: Overlap with SSB - Case 2: Overlap with rate matching resources: RateMatchPattern, lte-CRS-ToMatchAround, or LTE-CRS-PatternList-r16, availableRB-SetPerCell-r16 - Case 3: Due to TDD DL/UL related conflicts: Overlap with semi-static / dynamic UL symbols or overlap with PRACH - Case 4: QCL-TypeD prioritization rule among CORESETs result in one of the linked candidates not being monitored - Case 5: Overbooking results in one of the linked candidates not being monitored - Case 6: Overlap with reserved PRB(s) and OFDM symbol(s) indicated by DCI format 2_1 where UE may assume no transmission intended for the UE - Other cases are not precluded - FFS: Whether there is an impact to BD count

If the first PDCCH candidate is dropped, possible way is either to drop the second candidate or not. If the second candidate is not dropped, BD counting should be modified, and the second candidate should be counted without any additional scaling. But sometimes the dropping could happen dynamically, and it would be difficult to change the BD counting dynamically. Therefore, one simple way is to drop the second PDCCH candidate as well.

Proposal 2-10:  For PDCCH repetition with two linked candidates, if one of the linked candidates is dropped, the other linked candidate should be dropped.

# PUCCH reliability enhancement

## PUSCH beam when scheduled by DCI format 0_0

In Rel-15, it is defined that for PUSCH scheduled by DCI format 0_0, the beam should be based on the spatial relation info configured for the PUCCH resource with lowest ID in the same BWP. However currently two PUCCH-spatialRelationInfo are allowed to be configured for a PUCCH resource. Then if this PUCCH resource is selected to determine PUSCH beam, only one PUCCH-spatialRelationInfo should be selected. Therefore, it should be supported that the first PUCCH-spatialRelationInfo in PUCCH resource with lowest ID should be selected to determine the beam of PUSCH when it is scheduled by DCI format 0_0.

Proposal 3-1: It should be supported that the first PUCCH-spatialRelationInfo activated for PUCCH resource with lowest ID should be selected to determine the beam of PUSCH when it is scheduled by DCI format 0_0.

## Spatial relation and power control parameter configuration

It has been agreed to support two configure 2 pucch-spatialRelationInfo for a PUCCH resource in FR2 and 2 power control parameter sets for a PUCCH resource in FR1 by MAC CE. However, there can be up to 128 PUCCH resources in a CC, and to change the beams for the PUCCH resources, gNB needs to send 128 MAC CEs. As many PUCCH resources are configured just for different PUCCH format or time and frequency resources, some PUCCH resources can be grouped, similar to PUCCH resource group based beam indication. Therefore, it is necessary to support to configure 2 pucch-spatialRelationInfo for a group of PUCCH resources in FR2 and 2 power control parameter sets for a group of PUCCH resources by a single MAC CE in FR1.

Proposal 3-2: Support to configure 2 pucch-spatialRelationInfo for a group of PUCCH resources in FR2 by a single MAC CE, where the grouping is based on Rel-16 PUCCH resources grouping scheme.

Proposal 3-3: Support to configure 2 power control parameter sets for a group of PUCCH resources in FR1 by a single MAC CE, where the grouping is based on Rel-16 PUCCH resources grouping scheme.

# PUSCH reliability enhancement

## PT-RS to DMRS association

In RAN1 #104, it was agreed that for rank<2 to indicate the PT-RS to DMRS mapping for repetitions with two beams jointly by a single field. The remaining issue is how to support the indication for rank>2 case.

In RAN1 #104b, the following agreement on PT-RS to DMRS association was reached.

Agreement For single DCI based M-TRP PUSCH Type B repetition, the indication of PTRS-DMRS association for maxRank > 2 is supported, down select one of the following options in RAN1 #105-e meeting, - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - Option 1 (4 bits): with a second PTRS-DMRS association field (similar to the existing field), and each field separately indicating the association between PTRS port and DMRS port for two TRPs. - Option 2 (2 bits): using the existing PTRS-DMRS association field in DCI for the first TRP, and using reserved entries/bits in DM-RS port indication field for the second TRP. - Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports. - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port.

Option 2 is a kind of joint indication for PT-RS and DMRS, which cannot always save overhead since the additional entry may not always be enough and joint coding is not good for forward compatibility. Option 3 cannot cover all the possible cases for PTRS-DMRS port association. Thus option 1 should be simpler and cleaner solution, compared to option 2/3, which can cover all the cases.

In addition, for non-codebook based operation, since the precoder is selected by UE, UE has the information on which layer is the best. Thus, a fixed PT-RS to DMRS port association is sufficient, and UE can allocate the best precoder at the associated DMRS ports.

Proposal 4-1: For PTRS-DMRS port association indication for DG-PUSCH,

- For codebook based transmission scheme, support Option-1 where 4-bit field is used for PTRS-DMRS port association indication

- For non-codebook based transmission scheme, support a fixed association where the PT-RS is always associated with the DMRS port with lowest port index among the DMRS ports that share the same PT-RS port

## PHR report

When actual PHR is transmitted based on PUSCH with multi-beam repetition, how to calculate this actual PHR would be one open issue. The transmission power for different beams could be different for the PUSCH repetitions. Then the two PHR calculated based on each beam could be different. To get more information on the UE transmitting power status, it is better that UE can report two PHRs corresponding to the two beams for the PUSCH repetitions, where each PHR is calculated based on each beam.

In RAN1 #104b, the following agreement was reached.

Agreement For PHR reporting related to M-TRP PUSCH repetition, select one from the following options in RAN1 #105-e meeting. - Option 1: Calculate one PHR associated with the first PUSCH occasion (earliest repetition that overlaps with the first slot in which the PUSCH that carries the PHR MAC-CE is transmitted) - Option 2: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, but report one of them - FFS: How to select the PHR for reporting. - Option 4: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, and report two PHRs - Option 5: No changes to legacy PHR reporting

In RAN1 #105, there was some discussion on option 4, where UE may not be able to predict the scheduling status for the second repetition for PUSCH repetition Type B. One possible way is to report a virtual PHR for the second beam.

Proposal 4-2: For PHR enhancement, support option 4, where two PHRs are reported

- If both beams are transmitted in the same slot, two actual PHRs are reported

- If both beams are transmitted in different slots, an actual PHR and a virtual PHR are reported, where the actual PHR is calculated based on the first beam in the first PUSCH occasion and the virtual PHR is calculated based on the power control parameters for the second beam

## CG-PUSCH

For CG-PUSCH, one open issue is PTRS-DMRS association. To achieve diversity gain, the PT-RS cycling can be supported, where for each repetition, different PT-RS port association can be applied. As shown in Figure 1, cyclic PT-RS to DMRS association is applied for PUSCH repetitions with the same beam.

Figure 1: PT-RS to DMRS port cycling

Figure 2 illustrates the link level simulation results for a rank 2 transmission, where the detailed simulation assumption is shown in Table A-1. It can be observed that PT-RS port cycling can provide better performance than fixed association.

Figure 2: link level simulation results for different PT-RS to DMRS association scheme

Proposal 4-3: For CG-PUSCH with multi-beam repetitions, support PT-RS to DMRS port association cycling.

- The associated DMRS port index for a PT-RS port should be selected based on the repetition index

## Frequency hopping

In RAN1 #104b, the following options on frequency hopping were agreed for further down-selection.

Agreement When inter-slot frequency hopping is configured with Scheme 1, decide one from the below options in RAN1#105-e meeting, - Option 1 - If sequential mapping pattern is configured, frequency hopping is performed on slot level (as in Rel-15). - If cyclical mapping pattern is configured, frequency hopping is performed among the repetitions with the same beam. - Option 2: - gNB always configures sequential mapping pattern and frequency hopping is performed on slot level. (no spec impact) - Option 3: - Frequency hopping is performed on slot level as in Rel-15 (no spec impact).

As network may choose independent decoding for repetitions with the same beam, to achieve diversity gain, it should be supported to perform frequency hopping among the repetitions with the same beam. Option 2 and 3 cannot support frequency hopping for cyclic mapping for scheme 1. If sequency mapping and cyclic mapping is dynamically changed, option 2 or option 3 may be fine, but if it is configured by RRC, most likely cyclic mapping cannot be configured if gNB wants to dynamically enable frequency hopping.

Proposal 4-4: To achieve diversity gain, for multi-TRP PUSCH, support to perform frequency hopping among the repetitions with the same beam.

- For scheme 1, Option 1 should be supported

## PUCCH and PUSCH collision handling

In Rel-16, when PUCCH and TypeB PUSCH collide, the UCI is transmitted on the first actual repetition for TypeB PUSCH. With regard to the reliability for UCI transmission, similar to CSI report, it should be supported that the UCI can be transmitted in the first actual PUSCH repetition that meets Z and Z’ requirement (if applicable) corresponding to each beam.

Proposal 4-5: When mTRP PUSCH collides with PUCCH, support that UCI can be transmitted in the first actual PUSCH repetition corresponding to each beam based on the A-CSI report operation in mTRP PUSCH.

## Dynamic switching between single-TRP and multi-TRP

In RAN1 #105, the following on dynamic switching between sTRP and mTRP was agreed.

Agreement For the new field in the DCI for dynamic switching, support Alt.1 (modified). Alt.1 - Support 2 bits with the following combinations. Codepoint; SRS resource set(s); SRI (for both CB and NCB)/TPMI (CB only) field(s). 00; s-TRP mode with 1st SRS resource set (TRP1); 1st SRI/TPMI field (2nd field is unused). 01; s-TRP mode with 2nd SRS resource set (TRP2); 1st SRI/TPMI field (2nd field is unused). 10; m-TRP mode with (TRP1,TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set; Both 1st and 2nd SRI/TPMI fields. 11; m-TRP mode with (TRP2,TRP1 order) 1st SRI/TPMI field: FFS 2nd SRI/TPMI field: FFS; Both 1st and 2nd SRI/TPMI fields. - The SRS resource set with lower ID is the first SRS resource set, and the other SRS resource set is the second SRS resource set. - For codebook and non-codebook usage, respectively - The same number of SRS resource shall be configured in the two SRS resource sets.

The first and second SRI field should be associated with the first and second SRS resource sets. Thus, the following Table should be endorsed:

Table 4-1: Indication of PUSCH transmission scheme

| Codepoint | SRS resource set(s) | SRI (for both CB and NCB)/TPMI (CB only) field(s) |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set (TRP1) | 1st SRI/TPMI field |
| 01 | s-TRP mode with 2nd SRS resource set (TRP2) | 2nd SRI field and 1st TPMI field |
| 10 | m-TRP mode with (TRP1,TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 11 | m-TRP mode with (TRP2,TRP1 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |

Proposal 4-7: Adopt Table 4-1 for PUSCH transmission scheme indication.

# Conclusion

In this contribution, we discussed multi-TRP reliability enhancement. Based on the discussion, the following proposals have been achieved.

PDCCH

Proposal 2-1: For QCL-TypeD collision handling, support to determine the QCL-TypeD based on the priority for each group of beams, where the beams in a group can be received by UE simultaneously

- For each group of beams, the priority is determined by the corresponding SS with highest priority

Proposal 2-2: UE shall not expect the linked SS sets associated with CORESETs with different CORESETPoolIndex.

Proposal 2-3: For PCell/SCell BFR, support UE to report 2 new beams in BFRQ for PCell/SCell BFR, and after UE receives BFR response it can apply the 2 beams for reception of PDCCH repetition with 2 beams.

Proposal 2-4: SS-BFR should not be linked with other SS for PDCCH repetitions.

Proposal 2-5: Support CORESET-BFR to be associated with SS other than SS-BFR.

Proposal 2-6: Support to configure 2 TCI states for CORESET0 for PDCCH repetition with non-SFN scheme.

Proposal 2-7: For PDSCH mapping TypeB, the scheduling restriction should be counted based on Alt1 (The candidate that starts later in time).

Proposal 2-8: For PDSCH mapping TypeB, d1,1 should be calculated based on the following scheme

- The number of overlapping symbols of scheduling PDCCH and scheduled PDSCH is counted based on the 2 PDCCH repetitions and a symbol should be double counted when it contains 2 scheduling PDCCH and scheduled PDSCH

- For UE processing capability 2, if the number of PDSCH symbols allocated is 2, d1,1 is equal to the max{number of symbols for both CORESETs starting from the PDSCH symbol, number of overlapping symbols of PDCCH and PDSCH}

Proposal 2-9: When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, Option 1 (The individual candidate is not counted for monitoring) is slightly preferred.

Proposal 2-10:  For PDCCH repetition with two linked candidates, if one of the linked candidates is dropped, the other linked candidate should be dropped.

PUCCH

Proposal 3-1: It should be supported that the first PUCCH-spatialRelationInfo activated for PUCCH resource with lowest ID should be selected to determine the beam of PUSCH when it is scheduled by DCI format 0_0.

Proposal 3-2: Support to configure 2 pucch-spatialRelationInfo for a group of PUCCH resources in FR2 by a single MAC CE, where the grouping is based on Rel-16 PUCCH resources grouping scheme.

Proposal 3-3: Support to configure 2 power control parameter sets for a group of PUCCH resources in FR1 by a single MAC CE, where the grouping is based on Rel-16 PUCCH resources grouping scheme.

PUSCH

Proposal 4-1: For PTRS-DMRS port association indication for DG-PUSCH,

- For codebook based transmission scheme, support Option-1 where 4-bit field is used for PTRS-DMRS port association indication

- For non-codebook based transmission scheme, support a fixed association where the PT-RS is always associated with the DMRS port with lowest port index among the DMRS ports that share the same PT-RS port

Proposal 4-2: For PHR enhancement, support option 4, where two PHRs are reported

- If both beams are transmitted in the same slot, two actual PHRs are reported

- If both beams are transmitted in different slots, an actual PHR and a virtual PHR are reported, where the actual PHR is calculated based on the first beam in the first PUSCH occasion and the virtual PHR is calculated based on the power control parameters for the second beam

Proposal 4-3: For CG-PUSCH with multi-beam repetitions, support PT-RS to DMRS port association cycling.

- The associated DMRS port index for a PT-RS port should be selected based on the repetition index

Proposal 4-4: To achieve diversity gain, for multi-TRP PUSCH, support to perform frequency hopping among the repetitions with the same beam.

- For scheme 1, Option 1 should be supported

Proposal 4-5: When mTRP PUSCH collides with PUCCH, support that UCI can be transmitted in the first actual PUSCH repetition corresponding to each beam based on the A-CSI report operation in mTRP PUSCH.

Proposal 4-6: Adopt Table 4-1 for PUSCH transmission scheme indication.

Table 4-1: Indication of PUSCH transmission scheme

| Codepoint | SRS resource set(s) | SRI (for both CB and NCB)/TPMI (CB only) field(s) |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set (TRP1) | 1st SRI/TPMI field |
| 01 | s-TRP mode with 2nd SRS resource set (TRP2) | 2nd SRI field and 1st TPMI field |
| 10 | m-TRP mode with (TRP1,TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 11 | m-TRP mode with (TRP2,TRP1 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |

# Appendix – Simulation Assumption

Table A-1: Link level simulation assumption for PUSCH

| Parameters | Values |
| --- | --- |
| Channel model | CDL-B 30ns |
| gNB antenna configuration | (M, N, P) = (1, 1, 2) |
| UE antenna configuration | (M, N, P) = (1, 1, 2) |
| UE moving speed | 3 km/h |
| # of RBs/symbols | 50 RBs, 12 symbols |
| DMRS pattern | DM-RS configuration type 1 with 1 front-loaded DMRS and 1 additional DMRS |
| # of layers | 2 |
| Code rates | MCS 26 based on MCS Table 1 |
| Frequency hopping | Without hopping |
| Precoder | Fixed precoder |
| PT-RS density | Every 2 RBs and every symbol |
| Number of repetitions | 4 |
| Receiver assumption | MMSE |
| Pathloss different between 2 TRPs | 0 dB |
| blockage | None |