3GPP TSG RAN WG1 #106-e	                                         	      R1-2107724

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.4

Source:	Apple Inc.

Title:	Views on Rel-17 CSI enhancement

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Regarding CSI measurement and reporting enhancement, the following objectives are approved as part of Rel-17 FeMIMO [1]

- Enhancement on CSI measurement and reporting: - Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2 - Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

In the last RAN1 meeting RAN1#105 e-meeting, the following agreement is reached [2]

Agreement For Rel-17 port selection codebook, the maximal value of CSI-RS port number P as Pmax is 32. Conclusion At least for rank 1, no further restriction or condition is applied for polarization-common based free-selection and combinatorial coefficient based port selection for W1. Working Assumption At least for rank 1, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1 - FFS: Further dependence/restriction, e.g. conditioned on N3 or the number of CSI-RS ports, can be applied to above design. If does, how to support a non-consecutive FD bases used for Wf quantization - FFS: Whether to introduce thresholds for N3 and/or P Agreement A polarization-specific bitmap for indication non-zero coefficients should be supported for W2. Agreement For the quantization of W2 coefficient, reusing following Rel-16 quantization mechanism for Rank1 at least: - Two polarization-specific reference amplitudes: - for the polarization associated with the strongest coefficient, the reference amplitude is not reported - for the other polarization, reference amplitude is quantized to 4 bits - The alphabet is{1, 1/2)^(1/4), (1/4)^(1/4), (1/8)^(1/4), …, (1/2^14)^(1/4), [Reserved]} (-1.5dB step size) - For coefficients other than the strongest coefficient - differential amplitude is calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits - The alphabet is {1, 1/sqrt(2), 1/2, 1/(2*sqrt(2)), 1/4, 1/(4*sqrt(2)), 1/8, 1/(8*sqrt(2))} (-3dB step size) - phase is quantized to 16PSK - For the reserved state for reference amplitude, down-select one Alt - Alt 1: it is kept to be reserved - Alt 2: it is replaced as (1/2)^(15/4) - Alt 3: it is replaced as (1/2)^(3/8) Note: whether/how SCI is supported for R17 codebook will be discussed separately Agreement Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis: - It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot. Conclusion Whether to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis, in addition to CSI-IM Alt 2: No, it is not supported Agreement At least for rank 1, candidate values of K1 for port selection matrix W1 in NP*K1 are {2, 4, 8, 12, 16, 24, 32}. - Note: for polarization-common based free-selection, it means to select the same L=K1/2 ports out of P/2 ports for both polarizations Agreement Further reduction for possible parameter combinations among codebook parameters of Rel-17 port selection codebook, e.g. {K1, Mv, Beta}, will be discussed jointly once candidate values are determined - based on trade-off among UPT performance, feedback overhead, and complexity - based on all supported ranks - Limit total number of parameter combinations comparable to Rel-16 eType II - Exact parameters (e.g. with 2 or 3 parameters) within each combination are FFS - Other parameterizations of codebook parameter (e.g. alpha with K1= Alpha*# of CSI-RS ports and Alpha <=1) are not excluded Agreement At least for rank 1 and 2, for the compression coefficient Beta for non-zero coefficients of W2, values of Beta are {[1/4], 1/2, 3/4, 1} - Note: [1/4] means that 1/4 is also a candidate value for the discussion on reduction of parameter combinations, but has a lower priority compared to other beta values Agreement For Wf in CN3*Mv, Mv=2 is supported for R17 PS codebook - FFS: whether further dependence/restriction, i.e. conditioned on the number of CSI-RS ports, can be applied to Mv=2 - FFS: Whether Mv=4 can be supported for # of CSI-RS ports, e.g. 4 or 8 Agreement Whether a NZP CSI-RS resource m can be referred by two CMR pairs (m, a) and (m, b) configured for NCJT measurement hypotheses - Alt 1: It is feasible for FR1 but not for FR2. Agreement A CSI-IM resource is configured to be associated with either a CMR for Single-TRP measurement hypothesis or a CMR pair for NCJT measurement hypothesis: - One-to-one mapping between M+N CSI-IM resources versus M NZP CSI-RS resources for single-TRP measurement hypothesis and N NZP CSI-RS resource pairs for NCJT measurement hypothesis configured in a CSI-RS resource set. - FFS the value/definition of M Note: it is possible to configure the same value of CSI-IM resource ID for both NCJT and Single-TRP measurement hypotheses in FR1 and FR2, subject to QCL-Type D consistency between measurement hypotheses of the shared CMR in FR2 Agreement For a CSI-RS resource set with Ks NZP CSI-RS resources configured for CMR and N NZP CSI-RS resource pairs configured for NCJT measurement hypotheses, study following default value of Ks,max, - Alt 1: Ks,max = 4 - Alt 2: Ks,max = 2 - Alt 3: Ks,max = 4 for FR2, and Ks,max = 2 for FR1 - Note that default value means the minimal supported value for Ks,max in UE capability reporting, if UE support this feature. Agreement For CSI measurement associated with a CSI-ReportConfig for NC-JT, study whether/how to support following dynamic updating on, e.g. by MAC-CE - Alt 1: CMR pairs for NCJT measurement hypotheses - Alt 2: CMRs for Single-TRP measurement hypotheses - Alt 3: TCI states in CMRs - Alt 4: the number of single-TRP CSIs (i.e. X=0/1/2) in a NCJT CSI report Conclusion: There is no consensus to go with either of the following options in RAN1 #105e: - Option 1: Confirm the Working Assumption from RAN1#103e - Option 2: The UE can be expected to report one RI, one PMI, one LI and one CQI per TRP, up to 2 TRPs, for Multi-DCI based NCJT Agreement For Rel-17 Multi-TRP CSI enhancement, companies are encouraged to study following potential specification impact: - CRI codepoint mapping order with CMRs and CMR pairs - Whether/how to configure RI restriction/CBSR configuration for NCJT CSI measurement - Whether/how to enhance the CSI updating rule to address CPU overbooking - Whether/how to introduce new CSI computation delay requirement for NCJT CSI calculation - Whether/how to support wideband CSI report Agreement At least for rank 1 and for Mv>1, Minit for the single window with size N is fixed to be 0 Conclusion For PS codebook enhancements utilizing DL/UL reciprocity of angle and/or delay, there is no consensus of further enhancement for CSI-RS configurations associated with Rel-17 PS codebook. Agreement For CSI measurement associated with a CSI-ReportConfig for NC-JT, down-select one or more Alts in RAN1#106-e: - Alt 2: additional RRC signalling is needed to configure M (M≤ Ks) CMRs from the CSI-RS resource set for CMR for Single-TRP measurement hypotheses - Example: For a given set of {{#0, #1}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypothesis. Additional RRC signaling may select {#0,#3} (if sharing is allowed), or {#1, #3} (if not allowed), or select any from the set for single-TRP measurement hypotheses. - Alt 3: For CMRs configured in the CSI-RS resource set, support RRC signalling to enable/disable single-TRP measurement hypothesis using CMR configured within CMR pairs for NCJT measurement hypothesis - Example: For a given set of {{#0, #1}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypothesis. If gNB enables the sharing, {#0, #1, #2, #3} are for single-TRP measurement. If gNB disable the sharing, {#1, #3} are for single-TRP measurement hypotheses. - Alt 4: CMR sharing between single-TRP measurement hypothesis and NCJT measurement hypothesis is realized by configuring the same value of CMR ID for single-TRP CMR and NCJT CMR pair. - Example: When the UE supports sharing, for a given set of {{#0, #0}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypotheses, the rest {#0, #3} are for single-TRP measurement hypotheses. The CMRs for STRP can be updated by re-configuring the CSI resource set. Note that above examples are only for the purpose of illustrating/discussing Alternatives. Agreement For Option 1 CSI reporting associated with NCJT and X single-TRP measurement hypotheses, study whether to support following PMI/RI sharing mechanisms between NCJT CSI and single-TRP CSI(s): - Enabling/Disabling PMI, RI sharing via higher-layer configuration - Dynamic indication of PMI, RI sharing in the CSI report - FFS: other details - FFS: applicable conditions/restrictions of CMR sharing among Single-TRP and NCJT hypotheses, if above PMI/RI sharing mechanism can be applied For future RAN1 meeting: For a CSI report setting with Option 1 and X=1 or 2, study prioritizing CSI associated with reported CSI hypotheses within a CSI Reporting Setting - FFS potential impact for UCI payload generation - FFS whether/how to update CSI priority formula, and additional specification impact due to updated formula - FFS whether/how to update CSI omission rules for Part 2 CSI based on prioritized CSI - FFS: whether the X+1 CSI hypotheses per CSI Reporting Setting are mapped to a single CSI report or X+1 CSI reports - Companies are encouraged to discuss and justify purposes of prioritizing CSI associated with reported CSI hypotheses. Agreement At least for rank 1 and 2 and Mv > 1, for relationship between N and Mv, study and down-select one alternative from following in RAN1#106-e - Alt 1: N= Mv always, no UE reporting of Wf - Alt 2-1: N >= Mv, Wf is layer-common and reported by UE for N>Mv. - Alt 2-2: N >= Mv, Wf is layer-specific and reported by UE for N>Mv. Note: Wf is layer-common for N=Mv Note: For all alternatives, a layer-common window/set of size N is configured. Agreement Support rank 2 for Rel-17 codebook Agreement For Rel-17 port selection codebook, study following Alternatives and down-select in RAN1 106e: - Alt 1: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length N3. Wf as an all-one vector of length 1 is not needed - Alt 2: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length 1, i.e., a scalar. Wf as an all-one vector of length N3 is not needed. - Alt 3: Keep both Wf OFF and Wf ON with Mv=1. - If PMI format is SB, Wf is an all-one vector of length N3 - Informative note: this case is considered as “Wf ON with Mv=1” in the agreement in RAN1 104e - If PMI format is WB, Wf is an all-one vector of length 1, i.e., a scalar - Informative note: this case is considered as “Wf OFF” in the agreement in RAN1 104e - Note: N3 = NCQISubband*R. - FFS: the case when no SB size is configured. For future RAN1 meeting: Study whether/how the bitmap for indicating non-zero coefficients for W2 can be absent for CSI reporting - FFS: applicable conditions of being absent, .e.g. Mv=1 and Beta =1 for rank 1 or higher ranks - FFS: additional impact for reporting mechanism when/how the bitmap is absent - Note: The principle of UE determining the real number of NZC (same as Rel-15 and Rel-16) is unchanged in Rel-17 - based on trade-off among UPT performance, feedback overhead and complexity

In this contribution, we provide our views on the Rel-17 FeMIMO CSI enhancement in the following areas

- CSI measurement and reporting enhancement for MTRP

- Type II port selection codebook enhancement utilizing partial DL/UL channel reciprocity

# CSI enhancement for MTRP

Rel-15/16 CSI measurement and reporting adopts the design framework of CMR and IMR, in which CMR is configured for channel measurement, and IMR is configurated for interference measurement. In terms of IMR, there are two types of IMR, i.e., NZP-IMR (a.k.a. NZP-CSI-RS) and ZP-IMR (a.k.a. CSI-IM).

For CSI enhancement for MTRP, the main focus is on the NJCT (Non-Coherent Joint Transmission) scheme, in which, the PDSCH transmitted from two TRP can overlap in frequency. For example, Multi-DCI based Multi-TRP operation and Single-DCI based Multi-TRP operation with SDM mode can be used to schedule explicit NCJT operation to the UE.

Rel-16 solution focused on the PDSCH scheduling enhancement, by introducing 6 different modes, 1 for Multi-DCI based Multi-TRP and 5 for Single-DCI based Multi-TRP. However, CSI enhancement is not allowed in Rel-16. Figure 1 illustrates a simple issue for NCJT. Assume TRP1 transmits one CSI-RS resource set and TRP 2 transmits the other CSI-RS resource set, we have the following observation

- There is no clear definition of CMR and IMR for NCJT

  - When TRP1 transmits its PDSCH, the CSI-RS resource set from TRP 1 is CMR, while CSI-RS resource set from TRP 2 is IMR

  - When TRP2 transmits its PDSCH, the CSI-RS resource set from TRP 2 is CMR, while CSI-RS resource set from TRP 1 is IMR

In other words, for UE to derive the optimal CSI reporting, UE needs to jointly consider the reciprocal CMR/IMR relationship between TRP1 and TRP2. Using PMI as an example, UE needs to jointly solve the following optimization problem, note that the problem formulation is loose and for illustration purpose only

MAXw1,w2CH1*W1H2*W2+CH2*W2H1*W1

Whereas

- H1 and H2 are the channel from TRP1 and TRP2 respectively

- W1 and W2 are the precoding matrix (PMI) of TRP1 and TRP2 respectively

- C∙ is the capacity function

Figure 1 Reciprocal relationship between CMR and IMR for NCJT

From the above analysis, we can see that, for UE to provide the optimal CSI reporting. The following two things are required

- The flexible CSI measurement RS configuration to reflect the reciprocal relationship between CMR and IMR

- The flexible CSI report quantity configuration to allow UE to report joint optimization results in terms of PMI/RI/CRI/LI/CQI.

For interference measurement under NCJT, since the transmission from both TRP interfere with each other, it is reasonable that UE should consider the CMR, including RI/PMI decision, from the other TRP as the interference for the intended TRP. In other words, the RI and PMI decision from both TRP should be considered jointly and signals from one TRP should be considered as the interference to the other TRP.

Proposal 1 For interference measurement under NCJT, including RI/PMI/CQI/LI decision, CMR from one TRP should be considered as the interference, i.e. IMR, to the other TRP.

For Rel-17 multi-TRP CSI enhancement, we allow gNB to configure UE to report, in the same CSI-ReportConfig,  both the single TRP measurement results and the multi-TRP measurement results. In the previous RAN1 meetings, there was discussion on whether we need to introduce different priority for single TRP measurement and the multi-TRP measurement in the same CSI-ReportConfig. Currently, the CSI priority is used for the CPU overbooking and the CSI multiplexing and omission. It is important to note that current specification allows gNB to configure more CPU than UE reported capability, in which case, UE is not required to update the CSI with lower priority that exceeds the UE reported capability. Within the same CSI report, it is preferable that UE should have implementation flexibility on whether to process single TRP measurement first, or to process multi-TRP measurement first. Therefore, at least for CPU overbooking, we should not introduce different priority for single TRP measurement and multi-TRP measurement in the same CSI-ReportConfig.

Proposal 2 In the same CSI-ReportConfig, when gNB configures UE to report both the single TRP measurement results and the multi-TRP measurement result, do not introduce different priority for single TRP measurement and multi-TRP measurement in the same CSI-ReportConfig, at least for CPU overbooking scenario

# CSI enhancement with partial DL/UL channel reciprocity

Even for FDD spectrum, it is believed that DL and UL channel still have certain reciprocity, i.e., share similar channel statistic, even though the actual channel can be highly uncorrelated. For DL/UL channel partial reciprocity, it is generally the observation that the following two channel properties can be very similar between DL and UL

- Channel delay and power profile

- Channel angle in terms of AoA/AoD

In general, this is assuming that the DL and UL go through the similar reflection and scattering, and the wavelength of DL and UL signals are also similar even with frequency duplexing separation.

For W1 port selection matrix, majority of the design has been finalized, one issue is whether W1 port selection should be polarization/layer/frequency basis independent or common. We have the following proposal

Proposal 3 For W1 port selection reporting for port selection codebook enhancement, regardless of RI, the reporting should be

- Frequency basis common

- Polarization common

- Layer common

For W2 coefficients matrix, one major remaining issue is how to report SCI (Strongest Coefficient Indicator) . In Rel-16, due to the fact that the frequency basis window location can be reported by the UE, the design is that UE only reports the port indication for SCI, and, it is assumed that SCI is always located at the first frequency basis (DC frequency basis). In Rel-17, for the frequency basis window configuration, we adopted a different design which fixes the starting location of the frequency basis window at the first frequency basis, as results, we need to be more flexible on the SCI reporting, therefore we have the following proposal

Proposal 4 For W2 coefficients reporting for port selection codebook enhancement, UE reports SCI freely in the frequency basis window configured

Proposal 5 For W2 coefficients reporting for port selection codebook enhancement, for the coefficient location reporting (both SCI and bitmap), it is

- Frequency basis independent

- Polarization independent

- Layer independent

For Wf frequency basis, we have the following proposal

Proposal 6 For Wf frequency basis reporting for port selection codebook enhancement, regardless of RI, the Wf is reported

- Polarization common

- Layer independent

Proposal 7 For Wf frequency basis reporting for port selection codebook enhancement, UE reports WB (wideband) PMI under either of the following two conditions

- Mv=1

- Wf not configured

Proposal 8 For Wf frequency basis reporting for port selection codebook enhancement,

- UE always selects the DC frequency basis, i.e., frequency basis with all entries equal to 1.

- Do not support R>1

# Conclusion

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Regarding CSI measurement and reporting enhancement, in this contribution, we provide our views on the following areas

- CSI measurement and reporting enhancement for MTRP

- Type II port selection codebook enhancement utilizing partial DL/UL channel reciprocity

We have the following proposals

For CSI measurement and reporting enhancement for MTRP

Proposal 1 For interference measurement under NCJT, including RI/PMI/CQI/LI decision, CMR from one TRP should be considered as the interference, i.e. IMR, to the other TRP.

Proposal 2 In the same CSI-ReportConfig, when gNB configures UE to report both the single TRP measurement results and the multi-TRP measurement result, do not introduce different priority for single TRP measurement and multi-TRP measurement in the same CSI-ReportConfig, at least for CPU overbooking scenario

For Type II port selection codebook enhancement utilizing partial DL/UL channel reciprocity

Proposal 3 For W1 port selection reporting for port selection codebook enhancement, regardless of RI, the reporting should be

- Frequency basis common

- Polarization common

- Layer common

Proposal 4 For W2 coefficients reporting for port selection codebook enhancement, UE reports SCI freely in the frequency basis window configured

Proposal 5 For W2 coefficients reporting for port selection codebook enhancement, for the coefficient location reporting (both SCI and bitmap), it is

- Frequency basis independent

- Polarization independent

- Layer independent

Proposal 6 For Wf frequency basis reporting for port selection codebook enhancement, regardless of RI, the Wf is reported

- Polarization common

- Layer independent

Proposal 7 For Wf frequency basis reporting for port selection codebook enhancement, UE reports WB (wideband) PMI under either of the following two conditions

- Mv=1

- Wf not configured

Proposal 8 For Wf frequency basis reporting for port selection codebook enhancement,

- UE always selects the DC frequency basis, i.e., frequency basis with all entries equal to 1.

- Do not support R>1

# Reference

- RP-193133, "New WID: Further enhancements on MIMO for NR", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman Notes, 3GPP TSG RAN WG1 #105-e e-Meeting, May 10th – 27th, 2021