3GPP TSG RAN WG1 #106-e                                     R1-2107775

e-Meeting, August 16th – 27th , 2021

Source:	ZTE

Title:	Discussion on timing relationship for NR-NTN

Agenda Item:	8.4.1

Document for:   Discussion

- Introduction

In RAN1#105-e, the following agreements were achieved as potential enhancement for NR-NTN [1]:

Agreement:

If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot , where µ is the SCS configuration for the PUCCH.

Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

Agreement:

The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT.

- The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac.

Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by  TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×TcTTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of  and K_mac.  How to treat  and  can be further discussed.

Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed.

Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

Agreement:

The K_offset value signaled in system information is always used for

- The transmission timing of RAR / fallbackRAR grant scheduled PUSCH

- The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI

  - FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

FFS: how to treat additional transmission timings related to fallback DCI formats

FFS: how to update this formulation with beam-specific K_offset if beam-specific K_offset is agreed to be supported

In this contribution, remaining issues on timing relationship are discussed including initial signaling and updates of K_offset. Some leftovers regarding to Kmac signaling are also discussed and clarification of additional timing relationship cases including PDCCH ordered PRACH, Preamble retransmission is conducted.

- Indication of K_offset

## 2.1 Signaling initial K_offset value

For initial K_offset signaling via system information, following two options [3] were discussed in last meeting:

- Option 1: Signal one offset value for K_offset

  - Note: For example, the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point

- Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values

  - Note: For example, the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link

Basically in case that the reference point is located at serving satellite, there’s no difference between option 1 and option 2. However, if the reference point is the gNB, as shown in Table 1, although, there is potential possibility to reduce the signalling overhead via implicit signaling of partial offset value in Option-2, the benefit is rather trivial, especially with coarse unit for K_offset indication. At the same time, it will result in the high complexity at UE side with more specification impacts, e.g., derivation from common TA. Then, the option 1 with one offset value for K_offset is preferred.

Table 1 Comparison between option 1 and option 2

|  | Complicity | Signaling overhead | Spec impact |
| --- | --- | --- | --- |
| Option 1 | low | low | low |
| Option 2 | high | low | high |

Proposal-1: Option-1 is preferred with single value to cover the RTT of service link plus the RTT between serving satellite and reference point.

## 2.2 Updates of K_offset and TA reporting

In last meeting, the update of K_offset after initial access has been discussed without consensus. In this contribution, continuous analysis on the frequency for K_offset updates with proposed mechanism are elaborated.

- Signaling of updating K_offset

To reducing the scheduling latency, after initial access, at least updating from cell-specific value to UE-specific K_offset is needed if a UL transmission is to be scheduled. During the connected state, it can be expected that updates of this value may not be needed for GEO scenario. However, for LEO with earth moving beam or with earth fixed beam, the frequency of updating K_offset in RRC connected sate depends on the elevation angle of UE and TA drift rate.

With the knowledge of value of UE specific K_offset, the network can indicate the updated value via e.g., either MAC CE signaling or RRC reconfiguration. Based on the discussion of last meeting, following two options were recommended for further discussion [4].

- Option 1: support MAC CE option

- Option 2: support both MAC CE option and RRC reconfiguration option

According to the above proposal, it’s clear that at least MAC CE signaling should be adopted for updating K_offset with higher flexibility and efficiency. If legacy RRC reconfiguration procedure is used for updating K_offset, the problem is that the processing delay for RRC reconfiguration procedure, e.g., 10ms[5] is too much at least for K_offset updating requirement of UE in RRC connected state. Moreover, considering the TA drift along time in LEO satellite case, the updates may not be achieved timely due to interaction delay over RRC procedure (AM mode).

At the same time, significant reduction of signalling overhead can be foreseen if the updates of K_offset is based on adjustment comparing to the previous value as listed in Table 2. On the other side, it may need 1 bit to denote positive or negative value for the second or subsequent update of K_offset.

Table 2 An exemplary comparison between the signaling overhead

|  | Signal a new K_offset | Signal an adjustment K_offset |
| --- | --- | --- |
| Initial K_offset = 542 ms. Maximum differential RTD = 21 ms Minimum UE specific K_offset = 521 ms | 10 bits | Up to 5 bits |

Proposal-2: Updating K_offset via MAC CE only should be supported.

Proposal-3: Signalling of the adjustment value of K_offset is preferred via MAC CE.

- Details on UE reporting (e.g., TA reporting)

In general, regardless of the UE status, to enable the accurate updates of K_offset, the new K_offset (or an adjustment to maintenance K_offset) should be determined based on the information from UE, e.g., TA reporting with the periodicity is equal or smaller than the frequency of updating K_offset as shown in Figure 1.

It can be found that within a period, the K_offset can keep unchanged if the minor changes of RTT can be handled via dynamic scheduling offset K1/K2. However, according to the reported information from UE, if the variant of UE’s TA or RTT exceeds a threshold, updated value will be indicated to UE.

Figure 1 Update UE specific K_offset based on TA reporting

According to the discussion in [4], the content of UE reporting of information and frequency of reporting need to be determined.

- Content of UE reporting of information.

In last meeting, following 5 options were proposed with different content relevant to TA, UE location and K_offset value:

•	Option 1: UE specific TA

•	Option 2: Full TA

•	Option 3: UE location

•	Option 4: Difference between UE-specific K_offset and cell-specific K_offset

•	Option 5: Difference between the latest K_offset and the last applied K_offset

W.r.t these options, in our views, the Option-4 and Option-5 should be precluded firstly since it will lead to the potential error for following up scheduling. More specifically, since the granularity of this K_offset indication is much coarser than the TA variation, after the consideration on the latency for feedback, the scheduling will be failed once the changes of TA is cross the boundary of slot after the latest report.

For the Option-3, the reporting of UE location is mainly to core network for positioning related operation, in NTN case, whether enabling the usage of this information for scheduling is still pending. Moreover, it will lead to additional complexity at gNB side for TA calculation.

W.r.t the Option-1 and 2, in these two solution, the finer granularity of reporting can be ensured to enable the gNB directly track and optimize the scheduling. However, in Option-1, since only the UE specific TA due to the service link is reported, it will lead to the additional complexity on the TA calculation with consideration on the indicated common and corresponding variation. Moreover, errors will occurs once the misalignment time instant for calculation is assumed between UE and gNB. Then, the full TA reported from UE side is preferred.

Proposal-4: Full TA reporting is preferred to enable the updates of K_offset.

- Mechanism of triggering

In last meeting, following 3 options were discussed [4]:

•	Option 1: Event triggered

•	Option 2: Network request

•	Option 3: Periodic

Since the intention of UE reporting is for updating of K_offset, which is configured in SIB, the mechanism of triggering on the TA report should be defined with consideration on the UE’s state. More specifically, in order to optimize the scheduling in initial access, the one-shot TA reporting via PRACH procedure can be expected. For the UE in the RRC connected mode, since the variation of TA is the joint results of UE’s and satellite’s movement, periodic reporting with proper setting is more straightforward with less spec impact and signaling overhead comparing to other Options (e.g., Option-1 and Option-2). For example, w.r.t the event triggered behavior, the adaptive criteria for event triggering should be defined with possibility for updates. And it also lead to additional procedure for resource UL resource acquisition for reporting along with other DL indication, which is similar as Option-2. Moreover, by reception of periodic results of TA reporting from all UEs, the gNB is able to conduct the grouping scheduling with well robust over the time variation.

Proposal-5: For the reporting mechanism, Option 3 is preferred with simple periodic reporting.

In addition, for 2-step RACH, TA report can be executed during the initial access, e.g., via Msg-A PUSCH. In this regard, signalling of K_offset can be conveyed in MsgB or other MAC CE, then finer UE-specific K_offset can be used early for afterward scheduling.

Proposal-6: For 2-step RACH, a refined value of K_offset can be directly configured for a UE if TA is reported via Msg-A PUSCH.

## 2.3 Application of the K_offset

According to the agreement in last meeting [1], following timing relationships is left to identify whether initial K_offset is used or not.

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

After initial access, as to above two cases, it is preferred that initial K_offset can be applied, the reason is that assume updated K_offset is transmitted in Msg4/MsgB, the value can’t be applied for NACK reporting since the value can’t be got when transport block in MsgB/Msg4 isn’t detected.

But for the HARQ-ACK on PUCCH to Msg4, if signaling of K_offset (e.g. Beam specific K_offset) is conveyed in Msg2, in this case, the updated K_offset can be applied.

Proposal-7: It is preferred that initial K_offset can be applied for following timing relationships:

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

Proposal-8: If there is signaling of K_offset conveyed in Msg2 (e.g., beam-specific value), the K_offset value can be applied in the transmission timing of HARQ-ACK on PUCCH to Msg4.

For other timing relationships after initial access, in order to reduce scheduling delay, updated K_offset with finer value may be applied.

Then regarding the transmission timings related to fallback DCI formats, basically there’s no necessary to define a limitation says only cell-specific K_offset is used for all timing relationships. Because it have been agreed that when UE is not provided updated information, the initial K_offset value can be used for all timing relationships that require K_offset enhancement [3]. So basically the initial K_offset can be used, only if the updated value of K_offset is not signaled to UE, or if the UE fails to detect the signaling of updating.

Proposal-9: No need to define the limitation that only cell-specific K_offset is used for transmission scheduled by fallback DCI formats.

- Indication of Kmac

Based on the discussion in [4], when the reference point located at serving satellite, K_mac (satellite-gNB RTT) is not equal to 0, in fact it was agreed to signaling ‘K_mac’ for following two timing relationships:

- MAC CE activation/deactivation

- Start of Msg2/MsgB RAR window

In case of LEO scenario, the feeder link varies, and updating K_mac should be considered, e.g. via MAC CE signaling. Moreover, for the second timing issue, the leftover issue is about the accuracy of UE’s TA, in order to avoid to miss the RAR detection, the minimum RTT within one service cell can also be preferred to postpone the RAR reception. Furthermore, in this way, if the maximal differential delay within one cell is larger than RAR window length, or if retain effective length of RAR monitoring window, further extending of this window can be considered.

Proposal 10: Update K_mac via MAC CE can be supported.

Proposal 11: Determining start of Msg2/MsgB RAR window considering following options:

- Option 1: Introducing an offset of UE specific RTT

  - Note: UE may need to start monitoring earlier

- Option 2: Introducing an offset of Minimum RTT

- PDCCH ordered PRACH

In this case, according to the existing specification of NR in section 8.1 of TS 38.213 [6], selection of available PRACH occasion within first available mapping cycle does not take the impact of TA into account. However, in NTN case, TA is large and should be considered. If the “first mapping cycle” is still interpreted as after PDCCH transmission timing (BS side) or closed to DL reception timing (same mapping cycle at UE side), it may not be suitable for the transmission of PRACH with pre-compensation on the TA.

Then, in NTN case, following options are discussed in previous meetings:

- K_offset from the indication RO (first mapping cycle, PRACH occasion index #y) is used to fix the position of the selected RO.

- Interpreting the determination of the “first available cycle” with consideration on the impact of TA.

From specification impact perspective, assume the impact of large TA should be considered for the determination of RO in implementation of NTN system. Using K_offset for determination of RO can be considered.

Proposal 12: For PDCCH ordered PRACH, using K_offset for determination of RO can be considered.

- Preamble retransmission

According to the existing specification in section 8.2 of TS 38.213 [6], the preamble retransmission is conducted after the last symbol of the DL reception. From UE perspective, UE can follow above rule and proceed the preamble retransmission in absolute time.

Proposal 13: No timing relationship enhancement is needed for preamble retransmission.

- Flexibility scheduling enhancement

In this section, enhancement on enabling the flexibility scheduling are proposed, especially for the potential commercial uses case of ATG/TDD. Specially, supports on the dynamic indication of more K1 value is preferred, because the more K1 value, the more concentrated resource can be allocated for HARQ-ACK on PUCCH, and corresponding UL performance can also be guaranteed [2].

In additional, for keeping the size of DCI bit field, re-interpreting the bit field can also be considered to achieve flexible scheduling.

Proposal 14: Indication of the extended K1 value via enhanced DCI should be supported.

- Conclusions

In this contribution, further discussion on initial signaling K_offset, update of K_offset, and specific additional timing cases is conducted with following observations and proposals:

Proposal-1: Option-1 is preferred with single value to cover the RTT of service link plus the RTT between serving satellite and reference point.

Proposal-2: Updating K_offset via MAC CE only should be supported.

Proposal-3: Signalling of the adjustment value of K_offset is preferred via MAC CE.

Proposal-4: Full TA reporting is preferred to enable the updates of K_offset.

Proposal-5: For the reporting mechanism, Option 3 is preferred with simple periodic reporting.

Proposal-6: For 2-step RACH, a refined value of K_offset can be directly configured for a UE if TA is reported via Msg-A PUSCH.

Proposal-7: It is preferred that initial K_offset can be applied for following timing relationships:

- The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

- The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI

Proposal-8: If there is signaling of K_offset conveyed in Msg2 (e.g, beam-specific value), the K_offset value can be applied in the transmission timing of HARQ-ACK on PUCCH to Msg4.

Proposal-9: No need to define the limitation that only cell-specific K_offset is used for transmission scheduled by fallback DCI formats.

Proposal 10: Update K_mac via MAC CE can be supported.

Proposal 11: Determining start of Msg2/MsgB RAR window considering following options:

- Option 1: Introducing an offset of UE specific RTT

  - Note: UE may need to start monitoring earlier

- Option 2: Introducing an offset of Minimum RTT

Proposal 12: For PDCCH ordered PRACH, using K_offset for determination of RO can be considered.

Proposal 13: No timing relationship enhancement is needed for preamble retransmission.

Proposal 14: Indication of the extended K1 value via enhanced DCI should be supported.

# References

- Chair’s Notes RAN1#105-e final.

- R1-2105189, Discussion on timing relationship for NR-NTN, ZTE, RAN1#105-e.

- Chair’s Notes RAN1#104-e final.

- R1-2106325 FL summary#5 on NTN timing relationships, Ericssion, RAN1#105-e.

- 3GPP TS 38.331: “NR; Radio Resource Control (RRC) protocol specification”.

- 3GPP TS 38.213: “NR; Physical layer procedures for control”.