3GPP TSG RAN WG1 #106-e		                                    R1-2107811

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.6.1.3

Source:	InterDigital, Inc.

Title:	Duplex operation for RedCap UEs

Document for:	Discussion

# Introduction

In RAN1#105-e, the following agreements were taken regarding duplex operation for RedCap UEs [1]:

Agreement:

- For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), a HD-FDD RedCap UE is not required to monitor ULCI

  - No special handling on the priority rule for PDCCH carrying ULCI

Conclusion:

- No consensus of specification support of semi-static UL/DL pattern to HD-FDD RedCap UEs in Rel-17.

Agreement:

- For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, down-select from the following options

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured PDCCH

  - Option 2: Leave to UE implementation whether to receive the configured PDCCH or transmit the PRACH on the valid RO

  - Option 3: If configured PDCCH is in a Type-2 CSS set, then PDCCH is prioritized; otherwise the valid RO is prioritized

  - Option 4: Configured PDCCH is prioritized over valid RO

  - Option 5: Configured by network, e.g. via a priority indicator

  - FFS: whether or not the set of symbols overlapping with PDCCH in CSS set includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD

  - FFS whether a valid RO follows TDD’s or FDD’s definition, and if so, the corresponding impact

- FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

Agreement:

- For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), down-select from the following options

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured DL

  - Option 2: Leave to UE implementation whether to receive the configured DL or transmit the PRACH on the valid RO

  - Option 5: Configured by network, e.g. via a priority indicator

  - Other options are not precluded.

  - FFS: whether or not the set of symbols overlapping with configured DL includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD

- FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

Agreement:

- For Case 8 of valid RO overlapping with dynamically scheduled DL reception, down-select from the following options

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD for operation on a single carrier /single cell in unpaired spectrum

  - Option 2: Leave to UE implementation whether to receive the DL or transmit the PRACH on a valid RO

  - Option 3: Follow the handling of Case 1 to cancel PRACH based on a timeline that when the cancellation timeline is satisfied, the UE cancels the PRACH transmission and receives the DL signal/channels on the symbols overlapping with PRACH occasion (Interpretation 2 in R1-2103809)

  - Option 4: Valid RO is prioritized over dynamic DL that UE performs PRACH transmission and does not perform the DL receptions (Interpretation 3 in R1-2103809)

  - Option 5: When the cancellation timeline is satisfied, the UE neither performs transmission nor receives any DL signal/channels on the symbols overlapping with PRACH occasion (Interpretation 1 in R1-2103809)

  - FFS: whether or not the set of symbols overlapping with dynamic DL reception includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD

- FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

In this contribution, we discuss several remaining details of duplex operation for RedCap UEs.

# Remaining collision cases

- Case 8:

The collision handling for this case has been specified in Rel-16 NR as follows [2]:

“For a set of symbols of a slot corresponding to a valid PRACH occasion and  symbols before the valid PRACH occasion, as described in Clause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols.”

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	If the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2  relative to a last symbol of a CORESET where the UE detects the DCI format; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6, TS38.214], or the PRACH transmission in the set of symbols.

-	If the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6, TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols.

-	The UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the SRS transmission in remaining symbols from the subset of symbols.

Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0.

According to the above specification, for dynamically scheduled DL transmission: The UE does not receive PDCCH, PDSCH, or CSI-RS in a valid RO. The UE may also completely cancel the PRACH (depending on the UE capability) if the UE has sufficient time to decode the DCI before the valid RO; otherwise, the PRACH is transmitted. If partial cancellation is supported, the UE cancels a portion of the PRACH after the DCI is decoded. So, according to the spec, the UE does not receive the DL transmission and also cancels the UL transmission as timeline allows. For UE-dedicated configured DL reception, PRACH is prioritized.

We do not see a strong reason to change the UE behavior in the current spec, so we propose the following:

Proposal 1:

- For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured PDCCH

- For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured DL.

- For Case 8 of valid RO overlapping with dynamically scheduled DL reception, reuse the existing collision handling principles of Rel-15/16 for NR TDD for operation on a single carrier /single cell in unpaired spectrum

For collision between SSB and UL transmission, the UE can decide if SSB is needed (e.g., for RRM, beam management, etc.) in which case UL transmission can be dropped; or if SSB is not necessary in which case UL transmission can be performed.

Proposal 2: Handling collision between UL transmission and SSB is left to UE implementation.

# Conclusion

In this contribution, several remaining collision cases for half duplex RedCap UEs have been discussed and the following are proposed:

Proposal 1:

- For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured PDCCH

- For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured DL.

- For Case 8 of valid RO overlapping with dynamically scheduled DL reception, reuse the existing collision handling principles of Rel-15/16 for NR TDD for operation on a single carrier /single cell in unpaired spectrum

Proposal 2: Handling collision between UL transmission and SSB is left to UE implementation.

# References

- Chairman’s Notes, 3GPP WG1 #105-e

- 3GPP TS 38.213, v16.6.0