3GPP TSG RAN WG1#106-e	 	 	R1-2107863

e-Meeting, August 16th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on positioning for UEs in RRC_INACTIVE state

Agenda Item:	8.5.6

Document for: 	Discussion and Decision

- Introduction

At the RAN#91-e meeting, updated WID on NR positioning was agreed [1]. The work item includes other WG-led objectives including RAN1 as follows:

The objective of this work item is to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning enhancements for improving positioning accuracy, latency, network and/or device efficiency. The objective of this work item also includes the support of GNSS enhancements.

The specific objectives of this work are:

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]:

  - DL NR positioning methods and RAT-independent positioning methods

    - Support of UE positioning measurements for UEs in RRC_INACTIVE state

    - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state

Note: this work will be coordinated with the SDT WI.

  - As 2nd priority:

    - UL and DL+UL NR positioning methods

    - Support of gNB positioning measurements for UEs in RRC_INACTIVE state

- Specify on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, including: [RAN2, RAN1, RAN3]

  - UE-initiated request of on-demand DL PRS transmission;

  - LMF (network)-initiated request of on-demand DL PRS transmission;

Notes:

Solutions for RAT-dependent positioning enhancements are designed to operate in both frequency ranges (i.e. FR1 & FR2)

RAN working groups will take into account the work from other working group SA/CT which involves same objectives.

- The WID is subject to further update in RAN #91 for RAN1/2/3/4 scoping.

In this contribution, we present our view on positioning for UEs in RRC_INACTIVE state for Rel-17 NR.

- UL positioning in RRC_INACTIVE state

Regarding UL positioning in RRC_INACTIVE state, the following agreement was made in the SI phase [2].

Agreement:

- NR positioning for UEs in RRC_IDLE state and UEs in RRC_INACTIVE state will be investigated in Rel-17, including the benefits on latency, network/UE efficiency and UE power consumption

- FFS: which positioning methods to be supported, e.g., DL positioning, UL positioning, DL+UL positioning and/or Multi-RTT

- FFS: the details of how to enable the UE positioning in RRC_IDLE state and RRC_INACTIVE state

  - Reference signals (e.g., based on DL PRS signals, UL SRS signals, both of them, etc.)

  - Signaling and procedures (e.g., based on PRACH procedure, paging triggered UL SRS transmission, etc.)

In Rel-16, when a UE is configured with RRC_INACTIVE state, the UE cannot transmit SRS for positioning. If transmission of SRS for positioning in RRC_INACTIVE state is not supported in also Rel-17, the UE needs to request RRC connection to NW in order to obtain positioning metric from SRS for positioning. It means that there will be a certain delay and an increase of signaling for positioning for UE in RRC_INACTIVE. On the other hand, a UE can transmit RACH preamble regardless of the UE’s RRC state. If NW can obtain UEs’ positioning metric from RACH preamble similar to LTE positioning, UEs in RRC_INACTIVE state can support NR positioning. We think it’s reasonable scheme as it may have minimum impact on RRC_INACTIVE state UE behaviour. Hence, we provide the following proposal.

Proposal 1:

- RACH preamble (i.e. TA based positioning) can be considered for NR positioning of UEs in RRC_INACTIVE state

- Conclusion

In this contribution, we discussed on positioning for UEs in RRC_INACTIVE state for Rel-17 NR. Based on the discussion, we made following proposal.

Proposal 1:

- RACH preamble (i.e. TA based positioning) can be considered for NR positioning of UEs in RRC_INACTIVE state

References

- RP-210903, 3GPP RAN#91-e, “Revised WID on NR Positioning Enhancements”, Electronic Meeting, Mar. 2021.

- 3GPP RAN1#102-e, Chairman’s Notes, Aug. 2020.