3GPP TSG RAN WG1 #106-e			R1-2107871

e-Meeting, August 16th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on extension to DCI-based power saving adaptation

Agenda Item:	8.7.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#105-e meeting, the techniques for extension to DCI-based power saving adaptation were discussed and RAN1 made following agreements [1].

Agreement: - PDCCH schedules data and also indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping for a duration is supported. - At least DCI format(s) 1-1, 0-1, 1-2 and 0-2 can be used for the indication(s) Agreement: - At least one of Alt 1 and Alt 2 is supported, to be decided in RAN1#106, - Alt 1: Supporting SSSG switching to emulate PDCCH skipping functionality, - Alt 1-1: by an ‘empty’ SSSG which no SS set(s) is configured for the ‘empty’ SSSG, UE does not monitoring PDCCH on the ‘empty’ SSSG, - Alt1-2: by a ‘dormant SSSG’ which may have associated SS sets, and monitored conditionally (e.g., depending on HARQ NACK or RTT/ReTx timers) - Alt 2: PDCCH schedules data and also indicates PDCCH monitoring adaptation by PDCCH skipping for a duration is supported. - FFS details, including - e.g., joint / separate indication of SSSG switching and PDCCH skipping - Determination of the duration(s) for PDCCH skipping, e.g., - by RRC signaling, - by DCI indication - Implicitly, to the end of C-DRX active time Agreement: At least SSSG#0 and SSSG#1 switching is supported for Rel-17 SSSG switching indicated by PDCCH scheduling data and/or timer. - FFS: support of more than 2 SSSGs

In this contribution, we discuss on the extension to Rel-16 DCI-based power saving adaptation.

- Discussion

  - PDCCH skipping functionality

It was agreed that for PDCCH skipping functionality, one of SSSG switching (Alt 1) and PDCCH skipping (Alt 2) is supported to be decided in RAN1#106. Although SSSG switching and PDCCH skipping for a duration have the similar functionality in terms of PDCCH monitoring reduction, in our understanding, we prefer SSSG switching (Alt 1). For SSSG switching, one benefit of SSSG switching is that UE can perform PDCCH skipping (or sparse monitoring) multiple times without additional DCI indication, and can stop PDCCH skipping based on the timer or data arrival. It can provide power saving gain for a long term with small DCI overhead.

As some companies mentioned, processing delay of Rel-16 SSSG switching would not be shorter compared to PDCCH skipping. However, we think there are several ways to solve this problem. For example, if ‘empty’ SSSG which no SS set(s) is configured, only when switching to ‘empty’ SSSG, the application delay can be as short as possible to cover PDCCH processing time, i.e., similar as Alt 2. Taking SSSG #0(intensive PDCCH monitoring) and SSSG #1(sparse PDCCH monitoring) as an example, the application of SSSG switching should be based on the HARQ-ACK feedback to avoid the miss-match between the behavior of gNB and the UE due to miss detection of DCI.

On the other hands, only when switching to ‘empty’ SSSG, the miss-match between the behavior of gNB and the UE due to miss detection of DCI is not a critical issue because the UE continue to monitor the PDCCH in the monitoring cycle of previous Search space sets if DCI isn’t detected.

Observation 1: If ‘empty’ SSSG which no SS set(s) is configured, only when switching to ‘empty’ SSSG, the application delay can be as short as possible to cover PDCCH processing time.

Proposal 1:  For PDCCH skipping functionality, SSSG switching should be used.

From the above, if ‘empty’ SSSG which no SS set(s) can be configured, the number of SSSG should be larger than 2. Otherwise, if PDCCH skipping by ‘empty’ SSSG is configured, the flexibility of monitoring periodicity is sacrificed.

Proposal 2: More than 2 SSSGs should be considered for flexible monitoring periodicity and PDCCH skipping functionality.

  - PDCCH skipping along with cross-slot scheduling

In Rel-16, a minimum scheduling offset of K0/K2 was introduced for enhancement on cross-slot scheduling, and the minimum scheduling offset can be indicated by DCI from RRC configured candidate values. When the minimum scheduling offset is indicated by DCI, the actual scheduling offset K0/K2 equal to or larger than the corresponding minimum scheduling offset can be indicated by DCI from RRC configured candidate values. Cross-slot scheduling provides power saving gain by ensuring the sufficient time for PDCCH processing before starting the subsequent PDSCH reception. Because of this, after PDCCH reception UE can sleep without starting PDSCH reception/buffering. Also, in case of cross-slot scheduling, the UE power consumption can be reduced by relaxing the PDCCH processing based on the value of the minimum scheduling offset. However, the typical PDCCH monitoring periodicity seems every slot, and then the benefit of cross-slot scheduling would diminish since the UE needs to wake up to monitor next PDCCH occasion within the duration between PDCCH and PDSCH, and the parallel PDCCH processing may occur if the time for the relaxed PDCCH processing time is more than 1 slot. Basically, cross-slot scheduling would be applied on the condition that power saving is prioritized, e.g., low traffic amount for the UE. Also, even when there is burst traffic for the UE, the data scheduling for the UE might occur once in several slots depending on the number of UEs within the cell. In such cases, PDCCH monitoring in every slot might not be reasonable. Thus, PDCCH skipping along with cross-slot scheduling should be considered so that the benefit of cross-slot scheduling can be maximized.

Observation 2: PDCCH skipping along with cross-slot scheduling can maximize the benefit of cross-slot scheduling.

As mentioned above, assuming the UE monitors PDCCH in slot n, when the UE tries to monitor PDCCH between slot n and slot (n + the value of the applicable minimum scheduling offset), the UE sleep will be disturbed and the UE cannot relax PDCCH processing sufficiently. Thus, it should be considered that the duration of PDCCH skipping is equal to or longer than the applicable minimum scheduling offset.

Proposal 3: It should be considered that the duration of PDCCH skipping is equal to or longer than the applicable minimum scheduling offset.

Figure 1: PDCCH skipping along with cross-slot scheduling

- Conclusion

In this contribution, the extension to Rel-16 DCI-based power saving adaptation was discussed. Based on the discussion, the following observations and proposals were made:

Observation 1: If ‘empty’ SSSG which no SS set(s) is configured, only when switching to ‘empty’ SSSG, the application delay can be as short as possible to cover PDCCH processing time.

Proposal 1:  For PDCCH skipping functionality, SSSG switching should be used.

Proposal 2: More than 2 SSSGs should be considered for flexible monitoring periodicity and PDCCH skipping functionality.

Observation 2: PDCCH skipping along with cross-slot scheduling can maximize the benefit of cross-slot scheduling.

Proposal 3: It should be considered that the duration of PDCCH skipping is equal to or longer than the applicable minimum scheduling offset.

References

- 3GPP RAN1#105-e, Chairman’s note, May 2021.