3GPP TSG RAN WG1 #106-e	R1-2107892

e-Meeting, August 16th – 27th, 2021

Title:	[Draft] Reply LS on synchronous operation between Uu and SL in TDD band

Response to:	LS R1-2106430/R4-2108114 on synchronous operation between Uu and SL in TDD band

Release:	Release 17

Work Item:	NR_SL_enh-Core

Source:	Xiaomi [RAN1]

To:	RAN4

Cc:

Contact person:	Qun Zhao

Email:		         zhaoqun1@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN1 would like to thank RAN4 for the LS (R1-2106430/R4-2108114) on synchronous operation between Uu and SL in TDD band.

RAN1 has discussed questions listed in the LS and would like to provide the following feedback.

Question: is it feasible that RAN4 consider option 2 from RAN1 perspective to define SL transmission timing to align with UL timing when SL is synchronized to a network?

[RAN1 reply] No, it is not feasible.

TA values are only available for RRC_ connected UEs. However, sidelink operations of RRC_idle and RRC_inactive UEs are also supported.

1) It is not feasible for RRC_idle and RRC_inactive UEs to use UL timing as SL transmission timing; and,

2) If RRC_connected UEs use UL timing while RRC_idle/ RRC_inactive UEs use DL timing as SL transmission timing, sidelink communication between RRC_connected UE(s) and RRC_idle/RRC_inactive UE(s) may be broken due to timing misalignment between transmitter and receiver. In addition, if so, severe interference may rise between SL transmissions from RRC_connected UEs and those from RRC_idle/ RRC_inactive UEs.

2	Actions

To RAN4 group

ACTION: 	RAN1 respectfully asks RAN4 to take the above information into account.

3	Dates of next TSG RAN WG 1 meetings

TSG-WG1 Meeting #106bis-e	11th – 19th October 2021			Online Meeting

TSG-WG1 Meeting #107-e	11th – 19th November 2021		Online Meeting