3GPP TSG-RAN WG1 Meeting #106-e	 R1-2107894

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.2.1

Source:	Xiaomi

Title:	Enhancements on Multi-TRP for PDCCH, PUSCH and PUCCH

Document for:	Discussion/Decision

Introduction

The Rel-17 WID for further enhancements on MIMO (FeMIMO) is approved [1], which includes the following objective:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline

In this contribution, we discuss further on the PDCCH, PUCCH and PUSCH transmission using multi-TRP/multi-panel framework in this meeting respectively.

PDCCH

TCI state configuration for Multi-TRP PDCCH

In order to support Option 2+ Case 1, Alt 3 was supported as a working assumption. For Alt 3, if the two SS sets associated with a same CORESET, it needs to configure two TCI states for the CORESET. It is straight forward to reuse the agreement in last meeting to active two TCI states for the CORESET in Alt 3. In addition, the TCI state of each SS set also need to be indicated to UE. And it can also be indicated by a predefined rule, such as a mapping rule between the SS set and the TCI state ID. For example, the TCI state with lower TCI state ID will be applied to the SS set with lower SS set ID.

Proposal 1: To decide the TCI state for each of two SS sets associated with a same CORESET by predefined rule for Alt 3.

If two different CORESETs are necessary for Alt 3, two separately MAC CEs are needed to activate the beam for these two CORESETs if reusing beam indication mechanism for CORESET in Rel-15/16. In order to save the signaling overhead and to make the synchronization between beams updating of these two CORESETs, it is better to indicate these two beams by one signaling. As for the signaling with enhancement, there are two ways to achieve.

- 1st way: by MAC CE

  - One way is to activate at most two beams and each beam for one of two CORESETs respectively. The MAC CE can indicate two CORESET IDs and two TCI states. In addition, for each TCI state, there is 1 bit to indicate the present or not. Thus with this MAC CE signaling, gNB can indicate the TCI state of one CORESET or two CORESETs dynamically.

- 2nd way: by DCI

  - The other way is to reuse the beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework. It means to support Multi-TRP common beam indication by DCI. With this enhancement, one DCI codepoint can indicate one or two TCI states. And the mapping between DCI codepoint and TCI state(s) can be configured by MAC CE. With this DCI signaling, gNB can indicate the TCI state of one CORESET or two CORESETs more dynamically.

Proposal 2: To design one signaling for TCI state indication of two CORESETs for Multi-TRP PDCCH with Alt 3.

Decoding assumption

Agreement

For number of BDs corresponding to two PDCCH candidates that are linked for PDCCH repetition, support

- UE reports one [or more] number(s) as required number of BDs for the two PDCCH candidates

  - Candidate values: 2, 3.

- FFS: Default behavior

- FFS: Whether one of the candidate values imply that UE supports soft combining

- FFS: Whether additional candidate values are supported (e.g. non-integer numbers)

- FFS: RRC configuration based on reported UE capability

As for the number of blind decoding corresponding to two PDCCH candidates that are linked for PDCCH repetition, agreement was achieved in RAN1-104b e-meeting [2]. From our point of view, it is better to support one of the candidate values implying that UE supports soft combining in order for PDCCH resource allocation at gNB side. In addition, we also think it is beneficial to support an additional value between 1 and 2 since only soft combining may be assumed at UE side.

Proposal 3: Support some candidate values implying that UE supports soft combining.

Agreement

When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET, for the purpose of BD counting and interpretation of a detected DCI, select one option among the following in RAN1#105-e:

- Option 1: The individual candidate is not counted for monitoring

  - Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate).

- Option 2: The candidate in a higher SS set ID is not counted for monitoring

  - Interpretation of the detected DCI depends on which candidate is not counted (either based on Rel. 15/16 rules or based on Rel. 17 PDCCH repetition rules).

  - FFS: Impact to the other linked PDCCH candidate

- Option 3: The candidate associated with SS set(s) with lower priority is not counted for monitoring, where for two linked SS sets, the priority is according to one of the two SS sets with a lower SS set ID

  - Interpretation of the detected DCI depends on which candidate is not counted (either based on Rel. 15/16 rules or based on Rel. 17 PDCCH repetition rules).

  - FFS: Impact to the other linked PDCCH candidate

- FFS: Whether a max limit on number of such overlaps is needed.

Additional specification support may be introduced for the purpose of resolving ambiguity (if any) for interpretation of the detected DCI. For example,

- Distinguished by different RNTIs defined for the linked candidate versus the individual candidate

- Distinguished by aggregation level restrictions that can be expected by the UE in the case of overlap

Overlapping a linked candidate with an individual candidate

Three Options were agreed in RAN1-104b e-meeting. As for these three Options, we slightly prefer Option 3. Since with Option 1, there will be a long latency for the services with individual PDCCH candidate. While with Option 2, one of linked PDCCH candidates have similar priority with Rel-15/16. But we think one of linked PDCCH candidates should be configured with different priority from individual PDCCH candidate. Thus Option 3 is a better tradeoff. Only one both two linked PDCCH candidates are configured with higher SS set ID than that of individual PDCCH candidate, one of two linked PDCCH candidate will be not counted. Thus we prefer Option 3.

Proposal 4: Slightly prefer Option 3 When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate.

When one of the two linked candidates is dropped

Two Options were agreed in RAN1-104b e-meeting. As for these two Options, we slightly prefer Option 1. Since with Option 3, there is large waste of PDCCH candidate and long latency for the services with PDCCH repetition. It is also possible that the PDCCH can be decoded successfully even with only one PDCCH candidate.

Proposal 5: Slightly prefer Option 1 if one of the linked candidate is not monitored due to Rel. 15/16 procedures.

Agreement

For PDCCH repetition with two linked candidates, if due to Rel. 15/16 procedures, one of the linked candidates is not monitored (is dropped), select one option from Options 1 and 2 in RAN1#105-e:

- Option 1: UE still monitors the linked candidate that is not dropped and interprets the DCI based on Rel. 17 PDCCH rules (wrt reference PDCCH candidate)

- Option 2: Even the candidate that is not dropped is not monitored (Both linked candidates are dropped if at least one of them is dropped)

- FFS: Which of the following Rel. 15/16 rules are applicable for this purpose:

  - Case 1: Overlap with SSB

  - Case 2: Overlap with rate matching resources: RateMatchPattern, lte-CRS-ToMatchAround, or LTE-CRS-PatternList-r16, availableRB-SetPerCell-r16

  - Case 3: Due to TDD DL/UL related conflicts: Overlap with semi-static / dynamic UL symbols or overlap with PRACH

  - Case 4: QCL-TypeD prioritization rule among CORESETs result in one of the linked candidates not being monitored

  - Case 5: Overbooking results in one of the linked candidates not being monitored

  - Case 6: Overlap with reserved PRB(s) and OFDM symbol(s) indicated by DCI format 2_1 where UE may assume no transmission intended for the UE

  - Other cases are not precluded

- FFS: Whether there is an impact to BD countFFS: Whether additional candidate values are supported (e.g. non-integer numbers)

- FFS: RRC configuration based on reported UE capability

Default spatial setting for PUCCH resource

While for the spatial setting of the PUCCH resource, if PUCCH-SpatialRelationInfo is not provided, the default spatial setting will be applied for PUCCH transmission. And the default spatial setting is same as a spatial setting for PDCCH receptions by the UE in the CORESET with the lowest ID on the active DL BWP of the PCell. If the CORESET with the lowest ID is configured with two TCI states, how to define the default spatial setting for PUCCH transmission?

There are some alternatives to solve these problem.

- Alterative 1 is to restrict the number of the TCI state of the CORESET with the lowest ID is up to 1. But there will introduce some restriction and reduce flexibility on CORESET configuration.

- Alternative 2 is to provide a mechanism to decide the default spatial setting for PUCCH transmission when the CORESET with the lowest ID is configured with two TCI states. For PUCCH transmission with single occasion, only one TCI state with some predefined rule can be applied, such as the TCI state with lower TCI state ID, or the TCI state with the lower SS set ID among the SS sets associated with the CORESET. For PUCCH transmission over multiple occasions, it is possible to apply two TCI states with sequenticalmapping or cyclicmapping.

- Alternative 3 is to define the default spatial setting according to the CORESET for transmitting the PDCCH scheduling the PDSCH or SPS PDSCH release. And the default spatial setting can be same as the CORESET.

- Alternative 4 is to define the spatial setting according to the TCI state(s) in the DCI scheduling the PDSCH or SPS PDSCH release.

- Alternative 5 is to define the spatial setting according to the TCI state(s) in the DCI for joint TCI state indication or separate UL TCI state indication.

Proposal 6: Suggest to study the spatial setting for the PUCCH resource without PUCCH-SpatialRelationInfo.

Intra-slot and inter-slot

Intra-slot Multi-TRP based PDCCH transmission should be supported at least. While for inter-slot, there are many problems listed during the discussion in last meeting. First, it is the value of slot offset for PDSCH/PUSCH transmission. With Option 2, the value of slot offset in each PDCCH transmission should be same since “each repetition has the same number of CCEs and coded bits, and corresponds to the same DCI payload”. In fact, this problem can be solved by deciding it according to the first PDCCH transmission or the last PDCCH transmission. The second problem is the buffer size at UE side. If combined decoding should be applied, UE need to buffer PDCCH candidates of all slots. Which will introduce the complexity at UE side. The third problem is the time domain resource for PDSCH/PUSCH, take PDSCH as an example, whether the PDSCH is transmitted after one PDCCH transmission or after all PDCCH transmissions. If after one PDCCH transmission, UE’s buffer size need to be increased. If after all PDCCH transmission, long latency will be resulted in. Indeed, the latency can be solved by larger SCS.

Since it was agreed that “Maximum number of linked PDCCH candidates is two”, it means that the maximum repetition number of inter-slot Multi-TRP PDCCH transmission is two. Thus the listed problem above will be negligible.

Proposal 7: Support both intra-slot and inter-slot Multi-TRP PDCCH transmission with two as the maximum repetition number.

Linking between SS sets

It was agreed in RAN1-104e meeting that support linking two SS sets by RRC configuration for PDCCH repetition. From our point of view, MAC CE should be supported additionally to reduce the latency. For example, multiple pairs of SS sets can be configured by RRC signaling, and MAC CE will be used to activate/deactivate each pair. In this case, dynamical switching between Multi-TRP PDCCH transmission and single TRP PDCCH transmission can be achieved according to the PDCCH signaling overhead and the channel condition.

Proposal 8: Support MAC CE to activate/deactivate each linked SS set pair to achieve dynamical switching between Multi-TRP PDCCH transmission and single TRP PDCCH transmission.

Agreement

For PDCCH repetition, support linking two SS sets by RRC configuration:

- FFS: Whether MAC-CE can be used additionally

- When PDCCH repetition is monitored in two linked SS sets, the UE does not expect a third monitored SS set to be linked with any of the two linked SS sets.

- The two linked SS sets have the same SS set type (USS/CSS)

  - The two linked SS sets have the same DCI formats to monitor

- For intra-slot PDCCH repetition,

  - The two SS sets should have the same periodicity and offset (monitoringSlotPeriodicityAndOffset), and the same duration

  - For linking monitoring occasions across the two SS sets that exist in the same slot:

    - The two SS sets have the same number of monitoring occasions within a slot and n-th monitoring occasion of one SS set is linked to n-th monitoring occasion of the other SS set

Identification of two QCL Type D for monitoring overlapped CORESETs

Agreement

For a UE supporting reception with two different beams, support identifying two QCL-TypeD properties for multiple overlapping CORESETs

- FFS: How to enhance existing QCL-TypeD priority rules for overlapping CORESETs

- Note: The primary goal of this enhancement for the purpose of this sub-AI is to support time-overlapping PDCCH repetitions in FR2.

It was agreed in RAN1-104b e-meeting that two QCL-Type D properties for multiple overlapping CORESETs. Thus the priority of CORESETs for two QCL-Type D properties determination should be specified. From our point of view, there are some Options:

- Option 1: Reuse Rel-15/16 rule, CSS set has higher priority than USS set, lower SS set ID has higher priority

- Option 2: Configure same priority to two linked SS sets

  - Option 2-1: linked SS sets have higher priority than individual SS set regardless of the SS set ID;

  - Option 2-2: the priority of the linked SS sets can be defined by the lower SS set ID among two linked SS sets.

With Option 1, Rel-15/16 rule will be used regardless of linked SS set or individual SS set. In this case, it is possible to select the QCL Type D of SS set#1 and SS set#2, while SS set#1 and SS set#3 are linked SS sets, SS set#2 and SS set#4 are linked SS sets. With these two selected QCL Type D, only one PDCCH candidate for each of these two linked PDCCH candidate sets can be monitored. The success probability of PDCCH decoding will be decreased.

While with Option 2, two linked SS sets will be configured with same priority, thus QCL Type D of these two linked SS sets will be selected together. With Option 2-1, linked SS sets have higher priority than individual SS set regardless of the SS set ID, in this case, long latency for service scheduled by individual SS set will be introduced. Thus we prefer Option 2-2, the priority of the linked SS sets is defined by the lower SS set ID among two linked SS sets. In this case, when SS set ID of two linked SS sets are higher than individual SS set ID, QCL Type D of individual SS set ID will be selected. Else, the QCL Type D of two linked SS sets will be selected.

Proposal 9: Consider to configure same priority to the two linked SS sets based on the SS set with lower SS set ID for identification of two QCL Type D for monitoring overlapped CORESETs.

PUSCH

In this section, we discuss the following aspects for multi-TRP based PUSCH transmission:

- Dynamic switching enhancements

- UL power control enhancements

- Beam mapping enhancements

- PTRS-DMRS association

- CG PUSCH enhancements

Dynamic switching between sTRP and mTRP

In RAN1#105-e, the following has been agreed on SRI/TPMI indication and the scheme for dynamic switching between single TRP and multi-TRP based PUSCH transmissions [3]:

Agreement Confirm the Working Assumption (with supporting two bits for the new field). - For indicating STRP/MTRP dynamic switching for non-CB/CB based MTRP PUSCH repetition, - Introduce a new field in DCI to indicate at least the S-TRP or M-TRP operation. - The new field is 2 bits Agreement For the new field in the DCI for dynamic switching, support Alt.1 (modified). Alt.1 - Support 2 bits with the following combinations. Codepoint; SRS resource set(s); SRI (for both CB and NCB)/TPMI (CB only) field(s). 00; s-TRP mode with 1st SRS resource set (TRP1); 1st SRI/TPMI field (2nd field is unused). 01; s-TRP mode with 2nd SRS resource set (TRP2); 1st SRI/TPMI field (2nd field is unused). 10; m-TRP mode with (TRP1,TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set; Both 1st and 2nd SRI/TPMI fields. 11; m-TRP mode with (TRP2,TRP1 order) 1st SRI/TPMI field: FFS 2nd SRI/TPMI field: FFS; Both 1st and 2nd SRI/TPMI fields. - The SRS resource set with lower ID is the first SRS resource set, and the other SRS resource set is the second SRS resource set. - For codebook and non-codebook usage, respectively - The same number of SRS resource shall be configured in the two SRS resource sets.

It has been agreed that the dynamic switching of the order of the two TRPs can be supported to reduce the latency for the scheduling, but how to indicate the association between the SRI/TPMI fields and the SRS resource sets via these new DCI codepoints need to be further clarified.

Two options for the FFS part of the fourth codepoint:

- Option 1:   1st SRI/TPMI field: 2nd SRS resource set

2nd SRI/TPMI field: 1st SRS resource set

- Option 2:   1st SRI/TPMI field: 1st  SRS resource set

2nd SRI/TPMI field: 2nd SRS resource set

Since the first SRI/ TPMI field indicates the number of layers for the second SRI/TPMI field, the second SRI /TPMI field alone cannot be used for the indication of single TRP transmission. And it was agreed that the first SRI/TPMI field is used for the single TRP mode with the second SRS resource set (2nd TRP) for both the CB and NCB cases.  For SRI indication of CB case, this has already implies that the number of SRS resources in the first SRS resource set is no less than that in the second SRS resource set. And this could be the limitation for Option.2.

For Option.1, since the two SRI/TPMI fields are pre-determined with fixed bit lengths via RRC configuration, the second SRS resource set should have equal number of resources with the first SRS resource set as not to increase the DCI overhead.

Firstly we don’t see a great need to configure different number of SRS resources in the two SRS resource sets. Secondly, the association between the SRI/TPMI fields and the SRS resource sets may also be used for other indications, like power control parameters, etc., so the mapping should avoid any ambiguity and be as simple and clear as possible, Option 2 is obviously a simpler interpretation.

To make things simple, we prefer option 2 with adding the restriction of configuring the same number of SRS resources in the two SRS resource sets.

Proposal 10: Prefer Option 2 with adding the restriction that same number of SRS resources configured in the two SRS resource sets, as below:

- Support 2 bits with the following combinations.

| Codepoint | SRS resource set(s) | SRI (for both CB and NCB)/TPMI (CB only) field(s) |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set (TRP1) | 1st SRI/TPMI field (2nd field is unused) |
| 01 | s-TRP mode with 2nd SRS resource set (TRP2) | 1st SRI/TPMI field (2nd field is unused) |
| 10 | m-TRP mode with (TRP1,TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 11 | m-TRP mode with (TRP2,TRP1 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |

- The SRS resource set with lower ID is the first SRS resource set, and the other SRS resource set is the second SRS resource set.

  - For codebook and non-codebook usage, respectively

- The same number of SRS resource shall be configured in the two SRS resource sets.

Close-loop power control enhancements

In RAN1#105-e, the following has been agreed on CLPC [3]:

Agreement - To support per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2, a second TPC field can be configured via RRC. - When the second field is configured by RRC, a second TPC field (similar to the existing TPC field) is added in DCI formats 1_1 / 1_2 (option 3). - Each TPC field is for each closed-loop index value respectively - FFS: Whether or not the mapping between the TPC field and the PUCCH transmissions is needed - When the second field is not configured by RRC, a single TPC field (the existing TPC field) is used in DCI formats 1_1 / 1_2, and the TPC value applied for the closed loop index(es) for the scheduled PUCCH - To support per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2, adopt the same solution as with M-TRP PUCCH schemes. - FFS: any additional considerations - Support UE to report the capability on whether it supports the second TPC field - Note1: Per TRP closed-loop power control is only applicable when the “closedLoopIndex” values are not the same for TRPs.

For the closed-loop power control on multi-TRP based PUSCH transmission, it was agreed that the second TPC field is configurable. When the second TPC field exists, the association between the TPC fields and the SRS resource sets should also be specified.

- First, for multi-TRP based transmission, two rules can be considered.

  - Alt.1: the first TPC field is always associated with the first SRS resource set (first TRP), the second TPC field is associated with the second SRS resource set (second TRP) regardless of dynamic switching of the TRP order. And the SRS resource set with lower ID is the first SRS resource set, and the other SRS resource set is the second SRS resource set as agreed last meeting.

  - Alt.2: the same rule as defined for the association of the SRI/TPMI fields and the SRS resource sets of the new DCI field of dynamic switching between sTRP and mTRP.

- Second, for single TRP based transmission, two rules can be considered,

  - Alt.1: the first TPC field is always used, the second field is left unused.

  - Alt.2: the same rule as defined for the case of the new DCI field of dynamic switching between sTRP and mTRP.

To make rules simple, we support alt.2 for both sTRP and mTRP of TPC fields which is aligned with the association rules of other DCI fields.

Proposal 11: When the second TPC field exists, the association rule between the TPC fields and the SRS resource sets should be specified and it is suggested to follow the same rule defined for the association of the SRI/TPMI fields and the SRS resource sets of the new DCI field of dynamic switching between sTRP and mTRP.

Default power control parameters

In RAN1#105-e, the following has been agreed on default power control parameters for down-selection [3]:

Agreement （105-e） For single-DCI based M-TRP PUSCH repetition schemes, when one SRS resource per SRS resource set is configured (i.e., when two SRI fields are absent in DCI formats 0_1 / 0_2), default P0, alpha, PL-RS, and closed loop index is defined per TRP. Select one from the following in RAN1 #106-e meeting, - Alt.1 - The first P0/alpha, PL-RS, and closed loop index are determined by sri-PUSCH-PathlossReferenceRS-Id, sri-P0-PUSCH-AlphaSetId, and sri-PUSCH-ClosedLoopIndex mapped to the first sri-PUSCH-PowerControl associated with the first SRS resource set. - The second P0/alpha, PL-RS, and closed loop index are determined by sri-PUSCH-PathlossReferenceRS-Id, sri-P0-PUSCH-AlphaSetId, and sri-PUSCH-ClosedLoopIndex mapped to the first sri-PUSCH-PowerControl associated with the second SRS resource set. - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2. - Alt.2 - The first set of values {the first value in P0-AlphaSet, the PL-RS corresponded to PUSCH-PathlossReferenceRS-Id = 0 and closed-loop index l = 0} can be used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS corresponded to PUSCH-PathlossReferenceRS-Id = 1 and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise } can be used for TRP2. - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2. - Alt.3 - If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, the first set of values {the first value in P0-AlphaSet, the PL-RS corresponding to the first sri-PUSCH-PowerControl associated with the first SRS resource set and closed-loop index l = 0} is used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS corresponding to the first sri-PUSCH-PowerControlassociated with the second SRS resource set and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise} is used for TRP2. - Otherwise, the first set of values {the first value in P0-AlphaSet, the PL-RS with PUSCH-PathlossReferenceRS-Id=0 and closed-loop index l = 0} can be used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS with PUSCH-PathlossReferenceRS-Id = 1 and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise } can be used for TRP2. - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2.

We prefer Alt.1 as the method to extend the default power control parameters for multi-TRP case.

Alt.1 has less overhead than RRC configuration, less spec impact and provides more flexibility for the configuration of different TRPs among a cluster of TRPs, different coordinated TRPs can be linked together to support the transmissions flexibly with the linking method of Alt.1.

Proposal 12: Prefer Alt.1 which to introduce SRS resource set ID for the linking of SRI fields to two or more power control parameter set(s).

Open-loop power control enhancements

In RAN1#105-e, the following has been agreed on OLPC [3]:

Agreement For indicating per-TRP OLPC set in DCI format 0_1/0_2, if two SRI fields present in the DCI, - Use the existing field (1 bit) for OLPC set indication and a second p0-PUSCH-SetList-r16. - if value of the field equals to ‘0’, the UE determine value of P0 from SRI-PUSCH-PowerControl with a sri-PUSCH-PowerControlId value mapped to the SRI field value corresponding to each TRP. - if value of the field equals to ‘1’, the UE determine value of P0 from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value corresponding to each TRP.

Currently it was agreed that the Open-loop power control (OLPC) parameter should be indicated in a per TRP manner. And in last meeting it was agreed to reuse the existing field for OLPC indication for both TRPs when the SRI fields exist in the DCI and a second p0-PUSCH-SetList-r16 set was introduced. This assumes the same kind of interference scenario for both TRPs and each TRP can choose the power boosting values according to its own configurations. We still think the scenario that different interference to each TRP is valid and useful and should be supported too, so at least a configurable second OLPC indication field should be considered.

For the cases when the SRI fields do not exist in the DCI, the same principle can be applied for the indication of the OLPC parameters. And the detailed interpretation for the indication should be based on Alt.1 on default power control parameters in Section 2.3 as given below.

If the SRI fields do not present in DCI, “Open-loop power control parameter set indication” field in DCI format 0-1 or 0-2 can be used for both TRPs.

  - 0 bit   when P0-PUSCH-Set-r16 is not configured

  - 1bit

- “0”     a first P0-PUSCH-AlphaSet in p0-AlphaSets

- “1”     the  first  and second value in P0-PUSCH-Set-r16 with p0-PUSCH-SetID =0 for each TRP

  - 2bit

- “00”    a first P0-PUSCH-AlphaSet in p0-AlphaSets

- “01”    a first value in first P0-PUSCH-Set-r16 with p0-PUSCH-SetID =0; a first value in second  P0-PUSCH-Set-r16 with p0-PUSCH-SetID=0

- “10”    a second value in first  P0-PUSCH-Set-r16 with p0-PUSCH-SetID=0;  a second value in second P0-PUSCH-Set-r16 with p0-PUSCH-SetID=0

Proposal 13: Currently, the same principle is suggested. If the SRI fields do not present in DCI, “Open-loop power control parameter set indication” field in DCI format 0-1 or 0-2 can be used for both TRPs.

  - 0 bit   when P0-PUSCH-Set-r16 is not configured

  - 1bit

- “0”     a first P0-PUSCH-AlphaSet in p0-AlphaSets

- “1”     the  first  and second value in P0-PUSCH-Set-r16 with p0-PUSCH-SetID =0 for each TRP

  - 2bit

- “00”    a first P0-PUSCH-AlphaSet in p0-AlphaSets

- “01”    a first value in first P0-PUSCH-Set-r16 with p0-PUSCH-SetID =0; a first value in second  P0-PUSCH-Set-r16 with p0-PUSCH-SetID=0

- “10”    a second value in first  P0-PUSCH-Set-r16 with p0-PUSCH-SetID=0;  a second value in second P0-PUSCH-Set-r16 with p0-PUSCH-SetID=0

Proposal 14: FFS: A second OLPC indication field should be considered when considering to support different interference scenarios according to different TRPs.

PHR reporting

In RAN1#105-e, the following has been agreed on PHR reporting [3]:

Agreement For PHR reporting related to M-TRP PUSCH repetition, select one from the following options in RAN1 #105-e meeting. - Option 1: Calculate one PHR associated with the first PUSCH occasion (earliest repetition that overlaps with the first slot in which the PUSCH that carries the PHR MAC-CE is transmitted) - Option 2: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, but report one of them - FFS: How to select the PHR for reporting. - Option 4: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, and report two PHRs - Option 5: No changes to legacy PHR reporting For further study in future meetings: For PHR reporting related to M-TRP PUSCH repetition, study following aspects related to option 4, - Option 4: Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs. - FFS1: How the PHRs are calculated for reporting (actual PHR or virtual PHR) - FFS2: How the PHRs are calculated for reporting for other CCs if the multi-cell PHR MAC CE is applied. - FFS3: Required changes to triggering conditions including the required higher layer parameters (e.g.,’phr-PeriodicTimer’, ‘phr-ProhibitTimer’, ‘phr-Tx-PowerFactorChange’ as TRP specific). - FFS4: Report P-MPR and MPE per TRP within the same MAC-CE extension. Note: Down-selection between Options 1-5 will be based on this study as well as the trade-off between benefit versus UE complexity.

For the multi-TRP based PHR reporting, we suggest that the PHR actual values of both TRPs should be measured and reported via MAC-CE in order to manage the UE power more efficiently. Option 1 does not calculate the PHR in a TRP-specific manner which is not desirable. Option 2 and Option 4 are similar in calculation, but differ in reporting. We prefer two actual PHR values reported which would be beneficial for the scheduling of multi-TRP based operation. Two actual PHRs or two virtual PHRs should be supported, but the third kind of PHRs (actural PHR+virtual PHR) should be avoided because it may provide an arbitrary meaning compared to single PH reporting.

Proposal 15: Prefer option 4 that two PHR measurement values for both TRPs reported to the gNB for the multi-TRP based operation.

Proposal 16: Two actual PHRs or two virtual PHRs should be supported, but the third kind of PHRs (actual PHR + virtual PHR) should be avoided

Additionally, we think the triggering conditions for PHR MAC-CE should be specified in a TRP-specific manner. And the P-MPR and MPE per TRP is also needed.

Proposal 17: The triggering conditions for PHR MAC-CE should be specified in a TRP-specific manner.

Proposal 18: The P-MPR and MPE per TRP should also be supported.

Beam mapping to PUSCH repetitions

In RAN1#104bis-e meeting, the following has been made on beam mapping to PUSCH repetitions [2]:

Agreement Confirm the following working assumption (with removing the last bullet): For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - FFS: Support of half-half mapping. - FFS: Additional considerations on mapping patterns (including required beam switching gaps)

Configurable beam mapping pattern

For PUSCH repetitions, the maximum repetition number is 16 and this may be extended in the future. With the increase of repetition number, the beam switching number increases too. For UE, power consumption at least increases. In some scenarios, frequent beam changing is not desired especially when beam switching gap is involved. So we prefer to also take the half-half mapping into account.

Proposal 19: Support half-half mapping and/or slot-level mapping to avoid the frequent beam changes in some scenarios.

Proposal 20: Support configurable beam mapping pattern by the gNB scheduler for multi-TRP based PUSCH.

Frequency hopping

For PUSCH repetition Type A and Type B, inter-repetition frequency hopping is performed among the repetitions according to the following alternatives,

Alt.1:

- If sequential mapping pattern is configured, frequency hopping is performed on slot/repetition level (as in Rel-15).

- If cyclical mapping pattern is configured, frequency hopping is performed among the repetitions with the same beam.

Alt.2:

- Frequency hopping is performed on repetition level as in Rel-15 (no spec impact).

In our view, sequential beam mapping can always get the beam diversity gain as well as the frequency diversity gain, but when cyclic beam mapping is configured, only Alt.1 can achieve both the beam diversity and frequency divert at the same time which would provide benefits for the performance. Some spec impact on the frequency hopping with cyclical mapping pattern configured is needed to ensure the inter-slot frequency hopping is performed among the repetitions within the same beam. If Alt.1 is not supported for cyclic mapping, it may lead to the first prioritization of sequential beam mapping configuration from the performance perspective, which is not the original intension.

Proposal 21: Support inter-slot frequency hopping performed among the repetitions associated with the same TRP for both cyclic mapping and sequential mapping for PUSCH repetition Type A.

Proposal 22: Support inter-repetition frequency hopping performed among the repetitions associated with the same TRP for both cyclic mapping and sequential mapping for PUSCH repetition Type B.

Enhancement on Orphan symbols

In Rel-16 URLLC WI, the issue of orphan symbol has been discussed. It has been specified that when the time span of an actual repetition is only one symbol, the orphan symbol would be dropped for transmission. Another case that may also lead to orphan symbols is that when the actual coding rate is too high for a successful decoding, e.g. coding rate R>0.94, this is because the TBS determination is based on the indicated symbols of the nominal repetition before segmentation, while channel coding process is based on the actual repetitions. When beam switching gap is considered and the mechanism of dropping (omitting) symbol(s) of the repetition is applied, more orphan symbols would be generated.

With the dropping of the orphan symbols, the transmission gap between the adjacent DMRS symbols targeting the same TRP may become too large in multi-TRP scenarios which would degrade the channel estimation performance. Also it is a kind of waste to only drop the orphan symbols while the resources for uplink transmission is quite limited. So we suggest to still transmit the DMRS of the orphan symbols with the beam allocated for the nominal repetition to enhance the channel estimation performance functioned as additional DMRS for a certain beam.

Proposal 23: Transmitting the DMRS symbol instead of dropping (omitting) the orphan symbol(s) for multi-TRP based PUSCH, applying with the same beam mapped onto that nominal repetition.

PTRS-DMRS association enhancement

In RAN1#104-e meeting, the following has been agreed regarding the PTRS-DMRS association for PUSCH [2]:

Agreement For single DCI based M-TRP PUSCH Type B repetition, the indication of PTRS-DMRS association for maxRank > 2 is supported, down select one of the following options in RAN1 #105-e meeting, - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - Option 1 (4 bits): with a second PTRS-DMRS association field (similar to the existing field), and each field separately indicating the association between PTRS port and DMRS port for two TRPs. - Option 2 (2 bits): using the existing PTRS-DMRS association field in DCI for the first TRP, and using reserved entries/bits in DM-RS port indication field for the second TRP. - Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports. - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port.

Currently, each PTRS port is associated with one DMRS port for cases of one PTRS port and two PTRS ports. For PUSCH transmission of maxRank>2, the PTRS-DMRS association should be enhanced to indicate separately for transmission towards each TRP.

To keep it simple and unified with other DCI design, a second PTRS field can be added for both one PTRS port and two PTRS port which is based on the R15/16 interpretation.

Option 1 provides the full flexibility of the DMRS port allocations for both TRPs while needs more DCI overhead. Option 2 is indicated both explicitly and implicitly, also the entries for DMRS port indication is limited too. The spec impact is larger to save more DCI overhead. Option 3 has limitation in

Proposal 24:  For multi-TRP operation of maxRank>2, option 1 is preferred to indicate the PTRS-DMRS association per TRP.

Support of CG PUSCH

In RAN1#104b-e meeting, the following has been agreed regarding the CG PUSCH [2] :

Agreement For type 1 or type 2 CG based multi-TRP PUSCH repetition, - Introduce the second fields of 'p0-PUSCH-Alpha' and 'powerControlLoopToUse' in 'ConfiguredGrantConfig’ - For type 1 CG based m-TRP PUSCH repetition, introduce the second fields of ‘pathlossReferenceIndex’, 'srs-ResourceIndicator' and 'precodingAndNumberOfLayers' in 'rrc-ConfiguredUplinkGrant'. - For type 2 CG based M-TRP PUSCH, two SRIs/TPMIs are indicated via the activating DCI. - FFS1: UL PT-RS port(s) and DM-RS port(s) for CG type 1 - FFS3: Details on RV mapping. - FFS4: Possible transmission occasion for initial transmission - FFS5: Other TRP specific parameters in 'rrc-ConfiguredUplinkGrant', e.g., 'dmrs-SeqInitialization'.

Single CG configuration is more aligned with single-DCI based PUSCH transmission, most of the enhancements can be reused.

- For Type 1 CG PUSCH, two sets of transmission parameters are needed to be configured by RRC.

- For Type 2 CG PUSCH, two sets of transmission parameters are configured by DCI for the activation of the CG configuration.

Here we discuss the RV mapping for CG PUSCH.

For single CG configuration based multi-TRP operation, RV sequence (e.g.{0,0,0,0}, {0,3,0,3}, {0,2,3,1}) should be applied to each of the transmission occasions targeting each TRP separately.

The RV sequence applied to each TRP can be configured by RRC with the following options:

- Alt.1: 2 RV sequences can be configured to one CG configuration, each applied to a set of transmissions targeting one TRP.

- Alt.2: 1 RV sequence and a RV offset value can be applied to different sets of transmission occasions associated with one TRP.

Alt.2 is traditionally used, but here we can see the benefits of configuring more than one RV sequences to different TRPs.

Firstly, different RV strategies for different TRPs can be selected as a combination to enhance the whole reliability for URLLC service mainly considering the decoding performance.

Secondly, UE starts a CG repetition transmission only at a PUSCH occasion with RV=0 when data arrives. So the RV sequence is always expected to be mapped starting with 0. Different RV sequence can provide different chances to start a transmission. With different RV sequences for each TRP, different strategies to enable faster data transmission and more repetitions within the configured period would be possible and further enhance the reliability of the transmission.

So alt.1 can provide more benefits than alt.2.

Proposal 25: Support 2 RV sequences can be configured to single CG configuration for the multi-TRP based PUSCH transmission.

Proposal 26: RV sequences should be mapped onto each set of the transmission occasions targeting each TRP respectively.

Proposal 27: UE can start a CG repetition transmission at a PUSCH transmission occasion with RV=0 of either TRP.

PUCCH

In this section, we provide our views on several issues of PUCCH transmissions using multi-TRP/multi-panel framework.

Transmission schemes

In RAN1#104-e meeting, the following WA has been made on the Scheme 3 of PUCCH [2]:

Agreement Confirm the working assumption with removing brackets on [consecutive] and adding UE capability. - For PUCCH reliability enhancement, support multi-TRP intra-slot repetition (Scheme 3) for all PUCCH formats. - The same PUCCH resource carrying UCI is repeated for X = 2 [consecutive] sub-slots within a slot. - Refer the design details related to sub-slot configurations (e.g. other values of X) to Rel-17 eIIoT - Note1: The decision of supporting scheme 3 is only applicable for multi-TRP operation. - This feature is optional.

PUCCH Scheme 2 has been discussed among companies, and we can’t see a strong reason to against it technically.

To enhance the multi-TRP operation within a slot, either Scheme 2 or Scheme 3 alone cannot support multi-TRP based intra-slot repetitions for all cases.  Since Scheme 3 support only sub-slot PUCCH, and Scheme 2 can be specified as another appealing approach for UEs not implementing sub-slot operations.

Also Scheme 2 is an important approach especially for cases when twice of the PUCCH duration cannot be satisfied to repeat within a slot and very low latency is highly required.

Some companies have concerns about the self-decodable capability for each beam hop in the case of blockage, we think Scheme 2 is quite similar to FDM Scheme 2a (not self-decodable for each TRP) for multi-TRP based PDSCH,  and Scheme 3 is somewhat similar to FDM Scheme 2b(self-decodable for each TRP)，both schemes can be applied to different scenarios, so this would not be a problem. How to apply the appropriate scheme (scheme 2 or scheme 3 as candidate schemes for intra-slot approaches) is up to the scheduler of the network.

Based on the above, Scheme 2 is as important as Scheme 3 to achieve the diversity gain within a slot for different scenarios from Scheme 3. So we suggest to also adopt Scheme 2 for the intra-slot PUSCH repetition for multi-TRP based operation.

Proposal 28: Support scheme 2 to be adopted for the intra-slot PUSCH repetition for multi-TRP based operation, and more than 2 hops are not expected.

Proposal 29: For scheme 3, only 2 repetitions within a slot are considered.

Dynamic indication of PUCCH repetition number

In RAN1#103-e meeting, the following agreement has been made on the repetition number of PUCCH indication:

Agreement For configuration/indication of the number of PUCCH repetitions for Scheme 1, there is no restriction on using Rel-15 framework on configuring the number of repetitions. - Rel-17 feMIMO may additionally consider supporting the dynamic indication of the number of repetitions in RAN1 #104 meeting.

It is beneficial to support more dynamic indication method of the number of repetitions. Currently, the repetition number is configured on PUCCH format level and fixed to all PUCCH resources with the same PUCCH format. If both the repetition number and the repetition scheme can be configured per PUCCH resource, dynamic indication on a resource level can be achieved. Also this can be achieved by activating the suitable repetition number for the certain PUCCH resource set with the MAC-CE signaling from a set of RRC-configured candidate values.

Proposal 30: Dynamic indication can be achieved on a resource level configuration, or by activating the suitable repetition number for the certain PUCCH resource with the MAC-CE signaling from a set of RRC-configured candidate values.

Beam mapping to PUCCH repetitions

In RAN1#104b-e meeting, the following has been made on beam mapping to PUCCH repetitions [2]:

Agreement Confirm the following Working Assumption: For PUCCH multi-TRP enhancements in Scheme 1, it is possible to configure either cyclic mapping or sequential mapping of spatial relation info’s over PUCCH repetitions. - FFS: Applicability of mapping patterns for different beam switching gaps - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - Note: For Scheme 1, cyclical mapping pattern and sequential mapping pattern are as follows, - Cyclical mapping pattern: the first and second beam are applied to the first and second PUCCH repetition, respectively, and the same beam mapping pattern continues to the remaining PUCCH repetitions. - Sequential mapping pattern: the first beam is applied to the first and second PUCCH repetitions, and the second beam is applied to the third and fourth PUCCH repetitions, and the same beam mapping pattern continues to the remaining PUCCH repetitions.

In RAN1#104b-e meeting, the following has been made on frequency hopping for PUCCH scheme 1 [2]:

Agreement When inter-slot frequency hopping is configured with Scheme 1, decide one from the below options in RAN1#105-e meeting, - Option 1 - If sequential mapping pattern is configured, frequency hopping is performed on slot level (as in Rel-15). - If cyclical mapping pattern is configured, frequency hopping is performed among the repetitions with the same beam. - Option 2: - gNB always configures sequential mapping pattern and frequency hopping is performed on slot level. (no spec impact) - Option 3: - Frequency hopping is performed on slot level as in Rel-15 (no spec impact).

Only Option 1 can achieve both the beam diversity and frequency divert at the same time. Some spec impact on the frequency hopping with cyclical mapping pattern configured is needed to ensure the inter-slot frequency hopping is performed among the repetitions within the same beam.

Proposal 31: Support Option 1 for the inter-slot frequency hopping with PUCCH repetition.

Conclusion

In this contribution, we discuss about the PDCCH, PUCCH and PUSCH reception by Multi-TRP. Based on above discussions, we provide the following proposals.

TRP. Based on above discusses, we provide the following proposals.

PDCCH：

Proposal 1: To decide the TCI state for each of two SS sets associated with a same CORESET by predefined rule for Alt 3.

Proposal 2: To design one signaling for TCI state indication of two CORESETs for Multi-TRP PDCCH with Alt 3.

Proposal 3: Support some candidate values implying that UE supports soft combining.

Proposal 4: Slightly prefer Option 3 When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate.

Proposal 5: Slightly prefer Option 1 if one of the linked candidate is not monitored due to Rel. 15/16 procedures.

Proposal 6: Suggest to study the spatial setting for the PUCCH resource without PUCCH-SpatialRelationInfo.

Proposal 7: Support both intra-slot and inter-slot Multi-TRP PDCCH transmission with two as the maximum repetition number.

Proposal 8: Support MAC CE to activate/deactivate each linked SS set pair to achieve dynamical switching between Multi-TRP PDCCH transmission and single TRP PDCCH transmission.

Proposal 9: Consider to configure same priority to the two linked SS sets based on the SS set with lower SS set ID for identification of two QCL Type D for monitoring overlapped CORESETs.

PUSCH:

Dynamic switching between sTRP and mTRP:

Proposal 10: Prefer Option 2 with adding the restriction that same number of SRS resources configured in the two SRS resource sets, as below:

- Support 2 bits with the following combinations.

| Codepoint | SRS resource set(s) | SRI (for both CB and NCB)/TPMI (CB only) field(s) |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set (TRP1) | 1st SRI/TPMI field (2nd field is unused) |
| 01 | s-TRP mode with 2nd SRS resource set (TRP2) | 1st SRI/TPMI field (2nd field is unused) |
| 10 | m-TRP mode with (TRP1,TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 11 | m-TRP mode with (TRP2,TRP1 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |

- The SRS resource set with lower ID is the first SRS resource set, and the other SRS resource set is the second SRS resource set.

  - For codebook and non-codebook usage, respectively

- The same number of SRS resource shall be configured in the two SRS resource sets.

Power control issues:

Proposal 11: When the second TPC field exists, the association rule between the TPC fields and the SRS resource sets should be specified and it is suggested to follow the same rule defined for the association of the SRI/TPMI fields and the SRS resource sets of the new DCI field of dynamic switching between sTRP and mTRP.

Proposal 12: Prefer Alt.1 which to introduce SRS resource set ID for the linking of SRI fields to two or more power control parameter set(s).

Proposal 13: Currently, the same principle is suggested. If the SRI fields do not present in DCI, “Open-loop power control parameter set indication” field in DCI format 0-1 or 0-2 can be used for both TRPs.

  - 0 bit   when P0-PUSCH-Set-r16 is not configured

  - 1bit

- “0”     a first P0-PUSCH-AlphaSet in p0-AlphaSets

- “1”     the  first  and second value in P0-PUSCH-Set-r16 with p0-PUSCH-SetID =0 for each TRP

  - 2bit

- “00”    a first P0-PUSCH-AlphaSet in p0-AlphaSets

- “01”    a first value in first P0-PUSCH-Set-r16 with p0-PUSCH-SetID =0; a first value in second  P0-PUSCH-Set-r16 with p0-PUSCH-SetID=0

- “10”    a second value in first  P0-PUSCH-Set-r16 with p0-PUSCH-SetID=0;  a second value in second P0-PUSCH-Set-r16 with p0-PUSCH-SetID=0

Proposal 14: FFS: A second OLPC indication field should be considered when considering to support different interference scenarios according to different TRPs.

Proposal 15: Prefer option 4 that two PHR measurement values for both TRPs reported to the gNB for the multi-TRP based operation.

Proposal 16: Two actual PHRs or two virtual PHRs should be supported, but the third kind of PHRs (actual PHR + virtual PHR) should be avoided

Proposal 17: The triggering conditions for PHR MAC-CE should be specified in a TRP-specific manner.

Proposal 18: The P-MPR and MPE per TRP should also be supported.

Beam mapping issues:

Proposal 19: Support half-half mapping and/or slot-level mapping to avoid the frequent beam changes in some scenarios.

Proposal 20: Support configurable beam mapping pattern by the gNB scheduler for multi-TRP based PUSCH.

Proposal 21: Support inter-slot frequency hopping performed among the repetitions associated with the same TRP for both cyclic mapping and sequential mapping for PUSCH repetition Type A.

Proposal 22: Support inter-repetition frequency hopping performed among the repetitions associated with the same TRP for both cyclic mapping and sequential mapping for PUSCH repetition Type B.

Proposal 23: Transmitting the DMRS symbol instead of dropping (omitting) the orphan symbol(s) for multi-TRP based PUSCH, applying with the same beam mapped onto that nominal repetition.

PTRS related issue:

Proposal 24:  For multi-TRP operation of maxRank>2, option 1 is preferred to indicate the PTRS-DMRS association per TRP.

CG PUSCH related issue:

Proposal 25: Support 2 RV sequences can be configured to single CG configuration for the multi-TRP based PUSCH transmission.

Proposal 26: RV sequences should be mapped onto each set of the transmission occasions targeting each TRP respectively.

Proposal 27: UE can start a CG repetition transmission at a PUSCH transmission occasion with RV=0 of either TRP.

PUCCH:

Proposal 28: Support scheme 2 to be adopted for the intra-slot PUSCH repetition for multi-TRP based operation, and more than 2 hops are not expected.

Proposal 29: For scheme 3, only 2 repetitions within a slot are considered.

Proposal 30: Dynamic indication can be achieved on a resource level configuration, or by activating the suitable repetition number for the certain PUCCH resource with the MAC-CE signaling from a set of RRC-configured candidate values.

Proposal 31: Support Option 1 for the inter-slot frequency hopping with PUCCH repetition

References

- RP-193133, “New WID: Further enhancements on MIMO for NR”

- RAN 1#104bis-e Meeting chairman notes

- RAN 1#105-e Meeting chairman notes