3GPP TSG RAN WG1 Meeting#106e			R1-2107952

e-Meeting, August 16th – August 27th , 2021

Agenda item:	8.12.3

Source:	Chengdu TD Tech, TD Tech

Title:	NR MBS related discussion for RRC_IDLE/RRC_INACTIVE UEs

Document for:	Discussion and Decision

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e to provide MBS in a cell. Based on the agreement on NR MBS, the open questions for RRC_IDLE/RRC_INACTIVE UEs are discussed and the related proposals are suggested in the contribution.

2 Open questions for RRC_IDLE/RRC_INACTIVE UEs

During the past RAN1 meetings, many open issues on group scheduling for RRC_CONNECTED UEs are proposed for the discussion. In the section, these open issues will be discussed one by one.

Figure 1: CFRs and sub-BWPs

Issue 1: The CFR for RRC_IDLE/RRC_INACTIVE UEs denoted by CFR-II can be initial BWP for DL. Whether or not CFR-II can be configured greater than the initial BWP for DL?

The bandwidth of CFR-II is determined by the MBS bandwidth requirement for RRC_IDLE/RRC_INACTIVE UEs. We can make the initial BWP for DL is big enough to meet both the bandwidth requirement for the SI/paging transmission and the MBS bandwidth requirement. But such processing will lead to unnecessary power consumption in the UEs which don’t support MBS or are not interest in the current MBS sessions in the cell. Therefore, there exists the scenario that the bandwidth of CFR-II is greater than the initial BWP for DL. When CFR-II is greater than initial BWP for DL, CFR-II shall contain the initial BWP for DL and has the same numerology as the initial BWP for DL to avoid the BWP switch by UE between CFR-II and the initial BWP for DL.

From gNB side, only one CFR-II can be configured. But if CFR-II is far greater than the initial BWP for DL, it can be divided into several sub-BWP as shown in Figure 1. Each sub-BWP has the following features:

- Each sub-BWP contains the initial BWP for DL, the CORESETs for the MCCH and MTCH monitoring, and MCCH

- Each sub-BWP has the same numerology as the initial BWP for DL.

- An MBS session can be provided in a sub-BWP instead of CFR-II.

The introduction of sub-BWPs for CFR-II is used to reduce the power consumption for a UE just receiving an MBS session scheduled in a sub-BWP. For example, the MBS sessions of voice type are scheduled in a sub-BWP instead of the entire CFR-II. The sub-BWP is indicated to the UEs receiving the MBS sessions of voice type. Such processing will reduce the power consumption in the UEs.

Based on the above discussion, the following proposals are suggested.

Proposal 1: CFR-II can be configured to contain the initial BWP for DL.

Proposal 2: If CFR-II is far greater than the initial BWP for DL, it can be divided into several sub-BWPs. Each sub-BWP contains the initial BWP for DL, the CORESETs for the MCCH and MBS session monitoring, and MCCH.

Proposal 3: If an MBS session is scheduled in a sub-BWP, the sub-BWP instead of CFR-II is indicated in the configuration information of the MBS session on MCCH.

Issue 2: How to define the CORESETs/search spaces for MCCH and the CORESETs/search spaces for MTCH?

In order to simplifying the MCCH and MBS session monitoring, MCCH and MTCH can share the same CORESETs/search spaces. Based on the agreement that MCCH is transmitted in the initial BWP for DL, the CORESETs/search spaces for MCCH and MTCH are configured in the initial BWP for DL. Therefore, the following proposal is suggested.

Proposal 4: MCCH and MTCH share the same CORESETs/search spaces which are configured in the initial BWP for DL.

Issue 3: How to send the MCCH change notification? How to deal with the missing MCCH change notification by UE?

The MCCH change notification can be sent with the following two options:

Option 1: MCCH change notification is sent with a new RNTI other than the MCCH specific RNTI

Option 2: MCCH change notification is sent in the DCI format on the MCCH specific PDCCH

In order to avoid the missing MCCH change notification, it’s better to send the MCCH change notification several times per the MCCH repetition period.

Another question needs to be considered with issue 3: whether or not the MCCH change notification can provide other new information in addition to the new MBS session indicator and the MBS configuration update indicator? The question needs to wait for the conclusion from RAN2.

Based on the above discussion, it’ better to provide the MCCH change notification with a new RNTI, which means a specific DCI format is used for the MCCH change notification. The specific DCI format has the enough bit to carry any new information needed from RAN2. Furthermore, the repetitions of the MCCH change notification per the MCCH repetition period is independent from the MCCH transmission. Therefore, the following proposal is suggested.

Proposal 5: A new RNTI is used for the MCCH change notification. The MCCH change notification can be transmitted several times per the MCCH repetition period. The MCCH change notification is sent in the same CORESET/search space as MCCH.

Proposal 6: The MCCH specific RNTI is configured with a fixed value. The MCCH change notification specific RNTI is configured with fixed values.

Proposal 7: Alternatively, the MCCH change notification can be sent in the DCI format on the MCCH specific PDCCH.

If the MCCH change notification is sent in the DCI format on the MCCH specific PDCCH, how to receive the MCCH change notification is left for the UE implementation. For example, if the content of MCCH is big, the content of MCCH can be divided into K segments. In order to send the scheduling information of the K segments, the MCCH specific PDCCH will be transmitted in the B*K slots, where B is the number of the beams for the PBCH/SS block. The DCI format on each PDCCH carries the MCCH change notification. If K=1, the MCCH change notification is transmitted only once in each beam direction. Under such scenario, there’s no repetition for the MCCH change notification per the MCCH repetition period.

- Conclusion

Based on the discussion in the above sections, the following proposals are suggested.

Proposal 1: CFR-II can be configured to contain the initial BWP for DL.

Proposal 2: If CFR-II is far greater than the initial BWP for DL, it can be divided into several sub-BWPs. Each sub-BWP contains the initial BWP for DL, the CORESETs for the MCCH and MBS session monitoring, and MCCH.

Proposal 3: If an MBS session is scheduled in a sub-BWP, the sub-BWP instead of CFR-II is indicated in the configuration information of the MBS session on MCCH.

Proposal 4: MCCH and MTCH share the same CORESETs/search spaces which are configured in the initial BWP for DL.

Proposal 5: A new RNTI is used for the MCCH change notification. The MCCH change notification can be transmitted several times per the MCCH repetition period. The MCCH change notification is sent in the same CORESET/search space as MCCH.

Proposal 6: The MCCH specific RNTI is configured with a fixed value. The MCCH change notification specific RNTI is configured with fixed values.

Proposal 7: Alternatively, the MCCH change notification can be sent in the DCI format on the MCCH specific PDCCH.

- References

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e.