3GPP TSG RAN WG1 Meeting#106-e			R1-2107953

e-Meeting, August 16th – August 27th , 2021

Agenda item:	8.12.4

Source:	Chengdu TD Tech, TD Tech

Title:	Other aspects for NR MBS

Document for:	Discussion and Decision

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e to provide MBS in a cell. Based on the agreements on NR MBS so far, the two delivery modes were agreed to provide an MBS session in a cell. Delivery mode 1 is used to provide an MBS session with both the PTM bearer and the PTP bearer per UE. Delivery mode 2 is used to provide an MBS session with the PTM bearer. Before the MBS sessions with delivery mode 1 can be provided in the predefined area covered by the intra-frequency network, the MBS related network planning will be done to make the frequency resource budget for delivery mode 1. The discussion on the MBS related network planning for delivery mode 1 is made with the related proposals suggested in the contribution.

- MBS related network planning for delivery mode 1

In the predefined area covered by the intra-frequency network, the same carrier is used in each cell in the predefined area. In order to provide MBS with delivery mode 1 in the predefined area, the MBS related network planning will be done with the frequency resource budget for delivery mode 1 as output.

Usually several unicast BWPs for DL will be planned firstly with each unicast BWP applied in each cell within the predefined area. Within each unicast BWP for DL, at least one common frequency resource for delivery mode 1 will be planned with each common frequency resource applied in the same unicast BWP in each cell within the predefined area.

Based on the above network planning, the further network planning can be done.

- For each unicast BWP, the service types to be supported can be defined with each service type applied in the unicast BWP in each cell within the area.

- For each unicast BWP, the service types to be supported in each common frequency resource for delivery mode 1 can be defined.

Based on the above MBS related network planning, the following configuration can be supported.

- The unicast BWP can be area specific in the predefined area.

- Each common frequency resource for delivery mode 1 within a unicast BWP can be area specific.

- The service type configuration information in each unicast BWP can be area specific.

- The service type configuration information in each common frequency resource for delivery mode 1 can be area specific.

Based on the above MBS related network planning, an MBS session with delivery mode 1 may be configured in the same common frequency resource within the same unicast BWP in each cell transmit the MBS session, which means the configuration information of the MBS session with delivery mode 1 may be area specific.

If an MBS session has the same configuration information in each cell transmitting it, the UE receiving the MBS session can use the same configuration information at the edge of the source cell to receive the MBS session in both the source cell and the target cell, which can improve the reception performance of the MBS session.

Based on the above discussion on the MBS related network planning, the following proposals are suggested.

Proposal 1: Discuss the MBS related network planning to derive the typical configuration for MBS.

Proposal 2: The typical configuration for MBS as output of the MBS related network planning should be supported to simplify the MBS reception during the UE mobility between the different cells within the predefined area.

- Conclusion

Based on the discussion in the above section, the following proposals are suggested.

Proposal 1: Discuss the MBS related network planning to derive the typical configuration for MBS.

Proposal 2: The typical configuration for MBS as output of the MBS related network planning should be supported to simplify the MBS reception during the UE mobility between the different cells within the predefined area.

- Reference

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e.